3GPP TSG-RAN5 Meeting #102	R5-240805

Athens, February 26 - March 1 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2974 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR_U test case 11.5.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR_U test case 11.5.1.3 is missing in 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 11.5.1.3 will remain missingin TS 38.533 and NR_U WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.1.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

#### 11.5.1.3	SA FR1 Event-triggered reporting tests on PCC with per-UE gaps under non-DRX

Editor's Note:

- MU and TT analysis is incomplete

- Message contents is missing

- Test procedure is missing

- Applicability needs to be updated

- Statistical analysis to determine test case verdict is FFS

- Test requirements and parameters are missing in RAN4

11.5.1.3.1	Test purpose

The purpose of this test is to verify that the UE makes correct reporting of an event with per-UE gaps in non-DRX and with CCA. This test will partly verify the intra-frequency cell search requirements in clause 11.5.1.0.

11.5.1.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR-U and intra-frequency measurements on shared channel access.

11.5.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clauses 11.5.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.11.5.1.3.

11.5.1.3.4	Test description

Supported test configurations are shown in Table 11.5.1.3.4.1-1. Intra-frequency measurements are tested by using the parameters in Table 11.5.1.3.5-1. In all test cases, Cell 1 is the PCell, and Cell 2 is the target cell.

11.5.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 11.5.1.3.4.1-1.

Table 11.5.1.3.4.1-1: Supported test cofigurations

TBD

Configure the test equipment and the DUT according to the parameters in Table 11.5.1.3.4.1-2.

Table 11.5.1.3.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.8-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 10.4.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2a | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | Without LTE part. For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part. |  |  |

1.	The general test parameter settings are set up according to Table 11.5.1.3.4.1-3.

2.	Message contents are defined in clause 11.5.1.3.4.3.

3.	Cell 1 is the NR PCell with the power level set according to clauses C.1.2 and C.1.3 for this test. Cell 2 is the NR neighbour cell also with the power level set according to clauses C.1.2 and C.1.3 for this test.

Table 11.5.1.3.4.1-3 General test parameters

TBD

11.5.1.3.4.2	Test procedure

TBD

11.5.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 11.5.1.3.4.3-1: Common Exception messages for NR SA FR1 intra-frequency SS-RSRP measurements

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | TBD |

11.5.1.3.5	Test requirements

Table 11.5.1.3.5-1 defines the primary level settings including test tolerances for NR SA FR1 intra-frequency SS-RSRP measurements.

Table 11.5.1.3.5-1: Cell specific test parameters for NR SA FR1 intra-frequency SS-RSRP measurements

TBD

The UE shall send one Event A3 triggered measurement report (SS-RSRP in Test 1, SS-RSRQ in Test 2, SS-SINR in Test 3), with a measurement reporting delay less than D1 ms from the beginning of time period T2.

Editor’s note: D1=TBD.

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

FFS: NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

///End of changes