3GPP TSG RAN WG5#102	R5-240817

# Athens, Greece, 26 February - 1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2663 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test case 6.3F.4.3 relative power tolerance for shared spectrum channel access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 6.3F.4.3 is missing in 38.521-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test case 6.3F.4.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR-U work item cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.43F.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP analysis in R5-240858 MU and TT in R5-240819 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes >>

Table 6.3F.4.2.5-3: Test Tolerance

|  | 4.2GHz < f ≤ 7.125GHz |
| --- | --- |
| BW ≤ 100MHz | 1.4 dB |

#### 6.3F.4.3	Relative power tolerance for shared spectrum channel access

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

- Test coverage for UL-MIMO

- Message exceptions

- Test state and generic procedure are TBD in 38.508-1

6.3F.4.3.1	Test purpose

To verify the ability of the UE transmitter to set its output power in a target sub-frame(1ms) relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is less than or equal to 20ms.

6.3F.4.3.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR standalone shared spectrum channel access.

6.3F.4.3.3	Minimum conformance requirement

The relative power tolerance requirements of clause 6.3.4.3.3 apply if the transmission gap between the target sub-frame and the reference sub-frame is less than or equal to 40 ms.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3F.4.3.

6.3F.4.3.4	Test description

6.3F.4.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3F.4.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3F.4.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Mid, Highest |  |
| Test SCS as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Ch BW | Downlink Configuration |  | Uplink Configuration |  |
|  | Modulation | RB Allocation | Modulation | RB allocation |
| 20, 40, 60, 80MHz | N/A |  | CP-OFDM QPSK | See Table 6.3F.4.3.5-1 See Table 6.3F.4.3.5-2 See Table 6.3F.4.3.5-3 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.3F.4.3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5 Message contents are defined in clause 6.3F.4.3.4.3.

6.3F.4.3.4.2	Test procedure

The procedure is separated in various subtests to verify different aspects of relative power control. The power patterns of the subtests are described in figure 6.3F.4.3.4.2-1 thru figure 6.3F.4.3.4.2-5.

Figure 6.3F.4.3.4.2-1: FDD ramping up test power patterns

Figure 6.3F.4.3.4.2-2: FDD ramping down test power patterns

Figure 6.3F.4.3.4.2-3: TDD ramping up test power patterns

Figure 6.3F.4.3.4.2-4: TDD ramping down test power patterns

Figure 6.3F.4.3.4.2-5: Alternating Test Power patterns

1.	Sub test: ramping up pattern

1.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3F.4.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level -33 dBm, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3F.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

1.2.	Schedule the UE's PUSCH data transmission as described in Figure 6.3F.4.3.4.2-1 (FDD pattern A: sub-test is divided in 4 arbitrary radio frames with 10 active uplink sub-frames per radio frame) and Figure 6.3F.4.3.4.2-3 (TDD pattern A: sub-test is divided in 20 arbitrary radio frames with 2 active uplink sub-frames per radio frame). Uplink RB allocation as defined in table 6.3F.4.3.5-1/6.3F.4.3.5-3/ 6.3F.4.3.5-5 depending on channel bandwidth. On the PDCCH format 0_1 for the scheduling of the PUSCH the SS will transmit a +1dB TPC command for every first slot in a sub-frame. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

1.3.	Measure the power of PUSCH transmissions to verify the UE relative power control meet test requirements 6.3F.4.3.5. For power transients between sub-frames, transient periods of 20us between sub-frames are excluded.

1.4.	Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3F.4.3.5-1/6.3F.4.3.5-3/ 6.3F.4.3.5-5 to force bigger UE power steps at various points in the power range.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.2.

2.	Sub test: ramping down pattern

2.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3F.4.3.4.1-1.  Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -(MU + Uplink power control window size) to -MU dB of the target power level 20.7 dBm, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size is same as defined in step 1.1.

2.2.	Schedule the UE's PUSCH data transmission as described in Figure 6.3F.4.3.4.2-2 (FDD pattern A: sub-test is divided in 4 arbitrary radio frames with 10 active uplink slots per radio frame) and Figure 6.3F.4.3.4.2-4 (TDD pattern A: sub-test is divided in 20 arbitrary radio frames with 2 active uplink sub-frames per radio frame). Uplink RB allocation as defined in table 6.3F.4.3.5-2/6.3F.4.3.5-4/ 6.3F.4.3.5-6 depending on channel bandwidth. On the PDCCH format 0_1 for the scheduling of the PUSCH the SS will transmit a -1dB TPC command for every first slot in a sub-frame. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

2.3.	Measure the power of PUSCH transmissions to verify the UE relative power control meet test requirements 6.3.4.4.5. For power transients between sub-frames, transient periods of 20us between sub-frames are excluded.

2.4.	Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3F.4.3.5-2/6.3F.4.3.5-4/ 6.3F.4.3.5-6 to force bigger UE power steps at various points in the power range.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F, clause F.4.3.

3.	Sub test: alternating pattern

3.1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3F.4.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.  Send uplink power control commands for PUSCH to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as – (Uplink power control window size / 2) dB to + (Uplink power control window size / 2) dB of the target power level -10 dBm, where:

-	Uplink power control window size is same as defined in step 1.1.

3.2.	Schedule the UE's PUSCH data transmission as described in Figure 6.3.5.2.4.2-5 for 10 sub-frames an uplink RB allocation alternating pattern as defined in table 6.3F.4.3.5-7 while transmitting 0dB TPC command for PUSCH via the PDCCH.

3.3.	Measure the power of PUSCH transmissions to verify the UE relative power control meet test requirements specified in clause 6.3F.4.3.5. For power transients between sub-frames, transient periods of 20us between sub-frames are excluded.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.4.

6.3F.4.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.3F.4.3.5	Test requirement

Each UE power step measured in the test procedure 6.3F.4.3.4.2 should satisfy the test requirements specified in Table 6.3F.4.3.5-1 thru 6.3F.4.3.5-7.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each of ramping up and ramping down test patterns. For these exceptions the power tolerance limit is a maximum of ± (6.0 + TT) dB. If there is an exception in the power step caused by the RB change for all test patterns (A, B, C) then fail the UE.

Table 6.3F.4.3.5-1: Test Requirements Relative Power Tolerance for Transmission, ramp up sub-test

| BW [MHz] | Test SCS [kHz] | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 11RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 15 | RB change | 11RBs to 106 RBs | TPC=+1dB | 10.84 | 10dB ≤ ΔP < 15dB | 10.84 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 106 RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 20 |  | Subframes before RB change | Fixed = 11RB | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 30 | RB change | 11RB to 51 RBs | TPC=+1dB | 7.66 | 4dB ≤ ΔP < 10dB | 7.66 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 51RBs | TPC=+1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  |  |  | Subframes before RB change |  | Fixed = 22RBs |  | TPC=+1dB |  | 1 |  | ΔP ≤ 1 dB |  | 1 +/- (0.7 + TT) |  |
|  |  | 15 |  | RB change |  | 22RBs to 216RBs |  | TPC=+1dB |  | 10.92 |  | 10dB ≤ ΔP < 15dB |  | 10.92 +/- (4 + TT) |  |
|  |  |  |  | Subframes after RB change |  | Fixed = 216RBs |  | TPC=+1dB |  | 1 |  | ΔP ≤ 1 dB |  | 1 +/- (0.7 + TT) |  |
| 40 |  |  |  | Subframes before RB change |  | Fixed = 22RB |  | TPC=+1dB |  | 1 |  | ΔP ≤ 1 dB |  | 1 +/- (0.7 + TT) |  |
|  |  | 30 |  | RB change |  | 22RB to 106 RBs |  | TPC=+1dB |  | 7.83 |  | 4dB ≤ ΔP < 10dB |  | 7.83 +/- (3.5 + TT) |  |
|  |  |  |  | Subframes after RB change |  | Fixed = 106RBs |  | TPC=+1dB |  | 1 |  | ΔP ≤ 1 dB |  | 1 +/- (0.7 + TT) |  |
|  |  |  |  | Subframes before RB change |  | Fixed = 33RBs |  | TPC=+1dB |  | 1 |  | ΔP ≤ 1 dB |  | 1 +/- (0.7 + TT) |  |
| 60 |  | 30 |  | RB change |  | 33RBs to 162RBs |  | TPC=+1dB |  | 7.91 |  | 4dB ≤ ΔP < 10dB |  | 7.91 +/- (3.5 + TT) |  |
|  |  |  |  | Subframes after RB change |  | Fixed = 162RBs |  | TPC=+1dB |  | 1 |  | ΔP ≤ 1 dB |  | 1 +/- (0.7 + TT) |  |
|  |  |  |  | Subframes before RB change |  | Fixed = 44RBs |  | TPC=+1dB |  | 1 |  | ΔP ≤ 1 dB |  | 1 +/- (0.7 + TT) |  |
| 80 |  | 30 |  | RB change |  | 44PBs to 217RBs |  | TPC=+1dB |  | 7.93 |  | 4dB ≤ ΔP < 10dB |  | 7.93 +/- (3.5 + TT) |  |
|  |  |  |  | Subframes after RB change |  | Fixed = 217RBs |  | TPC=+1dB |  | 1 |  | ΔP ≤ 1 dB |  | 1 +/- (0.7 + TT) |  |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: Allocation as per Table 6.1F-1. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3F.1. |  |  |  |  |  |  |  |

Table 6.3F.4.3.5-2: Test Requirements Relative Power Tolerance for Transmission, ramp down sub-test

| BW [MHz] | Test SCS [kHz] | Applicable sub-frames | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  |  | Subframes before RB change | Fixed = 106RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 15 | RB change | 106RBs to 11 RBs | TPC=-1dB | 10.84 | 10dB ≤ ΔP < 15dB | 10.84 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 11 RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 20 |  | Subframes before RB change | Fixed = 51RB | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 30 | RB change | 51RB to 11RBs | TPC=-1dB | 7.66 | 4dB ≤ ΔP < 10dB | 7.66 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 11RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 216RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 15 | RB change | 216RBs to 22RBs | TPC=-1dB | 10.92 | 10dB ≤ ΔP < 15dB | 10.92 +/- (4 + TT) |
|  |  | Subframes after RB change | Fixed = 22RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 40 |  | Subframes before RB change | Fixed = 106RB | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  | 30 | RB change | 106RB to 22 RBs | TPC=-1dB | 7.83 | 4dB ≤ ΔP < 10dB | 7.83 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 22RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 162RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 60 | 30 | RB change | 162RBs to 33RBs | TPC=-1dB | 7.91 | 4dB ≤ ΔP < 10dB | 7.91 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 33RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
|  |  | Subframes before RB change | Fixed = 217RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| 80 | 30 | RB change | 217PBs to 44RBs | TPC=-1dB | 7.93 | 4dB ≤ ΔP < 10dB | 7.93 +/- (3.5 + TT) |
|  |  | Subframes after RB change | Fixed = 44RBs | TPC=-1dB | 1 | ΔP ≤ 1 dB | 1 +/- (0.7 + TT) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink Subframes. Pattern B the position of RB uplink allocation change is after 20 active uplink Subframes Pattern C the position of RB uplink allocation change is after 30 active uplink Subframes. Note 2: Allocation as per Table 6.1F-1. Note 3: TT=0.7dB Note 4: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3F.1. |  |  |  |  |  |  |  |

Table 6.3F.4.3.5-3: Test Requirements Relative Power Tolerance for Transmission, alternating sub-test

| BW | Test SCS [kHz] | Uplink RB allocation | TPC command | Expected power step size (Up or Down) | Power step size range (Up or Down) | PUSCH |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
|  | 15 | Alternating 11 and 106 | TPC=0dB | 9.84 | 4dB ≤ ΔP < 10dB | 9.84 +/- (3.5 + TT) |
| 20 | 30 | Alternating 11 and 51 | TPC=0dB | 6.66 | 4dB ≤ ΔP < 10dB | 6.66 +/- (3.5 + TT) |
|  | 15 | Alternating 22 and 216 | TPC=0dB | 9.92 | 4dB ≤ ΔP < 10dB | 9.92 +/- (3.5 + TT) |
| 40 | 30 | Alternating 22 and 106 | TPC=0dB | 6.83 | 4dB ≤ ΔP < 10dB | 6.83 +/- (3.5 + TT) |
| 60 | 30 | Alternating 33 and 162 | TPC=0dB | 6.91 | 4dB ≤ ΔP < 10dB | 6.91 +/- (3.5 + TT) |
| 80 | 30 | Alternating 44 and 217 | TPC=0dB | 6.93 | 4dB ≤ ΔP < 10dB | 6.93 +/- (3.5 + TT) |
| Note 1: Allocation as per Table 6.1F-1. Note 2: TT=0.7dB Note 3: Applicable if PUMAX ≥ P ≥ Pmin. Pmin as defined in sub-clause 6.3F.1. |  |  |  |  |  |  |

#### << End of changes >>