3GPP TSG-RAN WG5 Meeting # 102	R5-240846

# Athens, Greece, February 26 – 01 March, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0009 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on 6.5.2 for out of band emission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some test cases for spectrum emission mask are missing and the message contents should be updated. The test requirements for ACLR should also be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add test cases for spectrum emission mask in Table 6.5.2.2.4.1-1. - Update the message contents in clause 6.5.2.2.4.3 for SEM test. - Update the message contents and test requirements for ALCLR test. - Other editorial corrections on the tables and references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for NTN UE out of band eimission test will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2.3, 6.5.2.2.4.1, 6.5.2.2.4.2, 6.5.2.2.4.3, 6.5.2.4.1.3, 6.5.2.4.1.4.1, 6.5.2.4.1.4.3, 6.5.2.4.1.4.5, 6.5.2.4.1.5, 6.5.2.4.2.3, 6.5.2.4.2.4.1, 6.5.2.4.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

### 6.5.2	Out of band emission

#### 6.5.2.1	[to be updated]

#### 6.5.2.2	Spectrum emission mask

Editor's Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support NR satellite access is to be updated

- Message exceptions specific to satellite access is to be updated

- Annex F MU/TT is to be updated

##### 6.5.2.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth

##### 6.5.2.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

##### 6.5.2.2.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2.3-1 for the specified channel bandwidth.

Table 6.5.2.2.3-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10, 15, 20 |  |
| ± 0-1 | -13 | -13 | 1 % of channel BW |
| ± 1-5 | -10 | -10 | 1 MHz |
| ± 5-6 | -13 |  |  |
| ± 6-10 | -25 |  |  |
| ± 5-BWChannel |  | -13 |  |
| ± BWChannel-(BWChannel+5) |  | -25 |  |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.5.2.2.

##### 6.5.2.2.4	Test description

6.5.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.5.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 38.521-1 [2] Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [2] Annex C.2.

Table 6.5.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [12] subclause 4.1 |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [12] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Spectrum Emission Mask test case | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 14 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 24 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 34 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Edger_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 256 QAM | Outer_Full |
| 16 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 17 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 18 | Default |  |  |  | CP-OFDM QPSK | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 22 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 23 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 24 | Default |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| 25 | Low |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 26 | High |  |  |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 27 | Default |  |  |  | CP-OFDM 256 QAM | Outer_Full |
| 285,6 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 295,6 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 305,6 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports Pi/2 BPSK in FR1. NOTE 3: VOID NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: For Power Class 3 testing, UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n77, n78 and n79. NOTE 6: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] Annex G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions in service link and Common TA delay according to SIB19 configuration in TS 38.508-1 [12].

8.	Ensure the UE is in State [to be updated] with generic procedure parameters [to be updated], Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.5.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5.2.2.4.1-1, Table 6.5.2.2.4.1-2, Table 6.5.2.2.4.1-2a and Table 6.5.2.2.4.1-3. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control "up" commands to the UE until the UE transmits at PUMAX level. Allow at least 200 ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.2.5-1 to 6.2.2.5-9. The period of the measurement shall be at least the continuous duration of 1 ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.5.2.2.5-1. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with the following exceptions:

Table 6.5.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [12] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.5.2.2.4.3-2: DMRS-UplinkConfig (Test ID 28 – 30 in Table 6.5.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

##### 6.5.2.2.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.2.5-1, and the power of any UE emission shall fulfil requirements in Table 6.5.2.2.5-1.

Table 6.5.2.2.5-1: General NR spectrum emission mask

| ΔfOOB (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  | Measurement bandwidth |
| --- | --- | --- | --- |
|  | 5 | 10, 15, 20 |  |
| ± 0-1 | -13+TT | -13+TT | 1 % of channel BW |
| ± 1-5 | -10+TT | -10+TT | 1 MHz |
| ± 5-6 | -13+TT |  |  |
| ± 6-10 | -25+TT |  |  |
| ± 5-BWChannel |  | -13+TT |  |
| ± BWChannel-(BWChannel+5) |  | -25+TT |  |

Table 6.5.2.2.5-2: Test Tolerance (Spectrum Emission Mask)

[to be updated]

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5.2.3	[to be updated]

#### 6.5.2.4	Adjacent channel leakage ratio

##### 6.5.2.4.0	General

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.5.2.4.1	NR Adjacent channel leakage ratio

Editor's Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support NR satellite access is to be updated

- Message exceptions specific to satellite access is to be updated

- Annex F MU/TT is to be updated

6.5.2.4.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5.2.4.1.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

6.5.2.4.1.3	Minimum conformance requirements

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency at nominal channel spacing.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1.3-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1.3-2.

Table 6.5.2.4.1.3-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20 |
| --- | --- | --- |
| REF_SCS | (kHz) | 15 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |

Table 6.5.2.4.1.3-2: NR ACLR requirement

|  | Power class 3 |
| --- | --- |
| NR ACLR | 30 dB |

The normative reference for this requirement is TS 38.101-5 [11] clause 6.5.2.4.1.

6.5.2.4.1.4	Test description

6.5.2.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.4.1.4.1-1: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.2.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to clauses C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] clauses G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.5.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through AT commands or any other preconfigured means.

7.	Test equipment shall emulate Zero Doppler conditions in service link and Common TA delay according to SIB19 configuration in TS 38.508-1 [12].

8.	Ensure the UE is in State [to be updated] with generic procedure parameters according to TS 38.508-1 [12] clause [to be updated], Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.5.2.4.1.4.3.

6.5.2.4.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the test configuration tables in clause 6.2.2.4.1T. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously power control "up" commands to the UE until the UE transmits at PUMAX level. Allow at least 200 ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, as measured in step 3 of 6.2.2.4.2, which shall meet the requirements described in clause 6.2.2.5 as appropriate.

4.	Measure the rectangular filtered mean power for the assigned NR channel.

5.	Measure the rectangular filtered mean power of the first NR adjacent channel on both lower and upper side of the assigned NR channel, respectively.

6.	Calculate the ratios of the power between the values measured in step 4 over step 5 for lower and upper NR ACLR, respectively.

NOTE:	When switching to DFT-s-OFDM waveform, as specified in the test configuration tables in clause 6.2.2.4.1, send an NR RRCReconfiguration message according to TS 38.508-1 [12] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.5.2.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with the following exceptions:

Table 6.5.2.4.1.4.3-1: P-Max (Step 7)

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1 UE |

Table 6.5.2.4.1.4.3-1a: Void

Table 6.5.2.4.1.4.3-2: PUSCH-Config

| Derivation Path: TS 38.508-1 [12] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.5.2.4.1.4.3-3: DMRS-UplinkConfig (Test ID 28-30 in Table 6.5.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [12], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 { |  |  |  |
| Setup SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.5.2.4.1.4.3-4: ServingCellConfig

| Derivation Path: TS 38.508-1 [12] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.2.4.1.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in clause 6.2.2.5 as appropriate, and if the measured adjacent channel power is greater than –50 dBm then the measured NR ACLR, derived in step 6, shall be higher than the limits in Table 6.5.2.4.1.5-2.

Table 6.5.2.4.1.5-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 5,10,15,20 |
| --- | --- | --- |
| REF_SCS | (kHz) | 15 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |

Table 6.5.2.4.1.5-2: NR ACLR requirement

|  | Power class 3 |
| --- | --- |
| NR ACLR | 30 - TT dB |

Table 6.5.2.4.1.5-3: Test Tolerance (NR ACLR)

[to be updated]

##### 6.5.2.4.2	UTRA Adjacent channel leakage ratio

Editor's Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support NR satellite access is to be updated

- Message exceptions specific to satellite access is to be updated

- Test Points analysis is to be updated

- Test configuration is to be updated

- Annex F MU/TT is to be updated

6.5.2.4.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

6.5.2.4.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 3 UE release 17 and forward that support satellite access operation.

6.5.2.4.2.3	Minimum conformance requirements

UTRA adjacent channel leakage power ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRAACLR is specified for the first adjacent UTRA channel (UTRAACLR1) which center frequency is ± 2.5 MHz from NR channel edge and for the 2nd adjacent UTRA channel (UTRAACLR2) which center frequency is ± 7.5 MHz from NR channel edge.

The UTRA channel power is measured with a RRC filter with roll-off factor = 0.22 and bandwidth of 3.84 MHz. The assigned NR channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.5.2.4.1.3-1.

If the measured adjacent channel power is greater than – 50 dBm then the UTRAACLR1 and UTRAACLR2 shall be higher than the value specified in Table 6.5.2.4.2.3-1.

Table 6.5.2.4.2.3-1: UTRA ACLR requirement

|  | Power class 3 |
| --- | --- |
| UTRAACLR1 | 33 dB |
| UTRAACLR2 | 36 dB |

UTRA ACLR requirement is applicable when signalled by the network with network signalling value indicated by the field additionalSpectrumEmission.

The normative reference for this requirement is TS 38.101-5 [11] clause 6.5.2.4.2.

6.5.2.4.2.4	Test description

6.5.2.4.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2.4.2.4.1-1: Test Configuration Table for power class 3

[to be updated]

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [12] Annex A, Figure A.3.1.2.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to TS 38.521-1 [2] clauses G.0, G.1, G.2, and G.3.1.

4.	The UL Reference Measurement Channel is set according to Table 6.5.2.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [12] clause 4.5. Message contents are defined in clause 6.5.2.4.2.4.3.

6.5.2.4.2.4.2	Test procedure

[to be updated]

6.5.2.4.2.4.3	Message contents

Message contents are according to TS 38.508-1 [12] subclause 4.6 and 5.4 with the following exceptions:

[to be updated]

6.5.2.4.2.5	Test requirement

[to be updated]

==============================================================

<< End of changes >>