Title: 	Discussion on test point selection of Configured Transmitted Power for shared spectrum channel access in FR1

Source: 	Ericsson

# Introduction

In this contribution, we discuss some points in deciding the test points of Configured transmitted power for shared spectrum channel access in FR1.

Firstly the single transmission case is analysed, and then exceptions for UL-MIMO case based on the discrepancy of the waveform is provided.

# Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

  - Test Environment

General rule for shared spectrum channel access applies. Use the same test environment as in test case 6.2.4, Configured Transmitted Power.

Proposal 1: Propose Test Environment as Normal, TL/VL, TL/VH, TH/VL, TH/VH, for Configured transmitted power for shared spectrum channel access.

  - Test Frequencies

General rule for shared spectrum channel access applies. Use the same test frequencies as in test case case 6.2.4, Configured Transmitted Power.

Proposal 2: Only Mid frequency range is tested for Configured transmitted power for shared spectrum channel access.

  - Test Channel Bandwidth

General rule for shared spectrum channel access applies. Use the same test channel bandwidts as in test case 6.2.4, Configured Transmitted Power.

Proposal 3: The Lowest, Mid and Highest Channel Bandwidth shall be tested for Configured transmitted power for shared spectrum channel access.

  - Test Subcarrier Spacing (SCS)

General rule for shared spectrum channel access applies. Use the same test subcarrier spacing as in test case 6.2.4, Configured Transmitted Power.

Proposal 4: The Lowest subcarrier spacing shall be tested for Configured transmitted power.

  - Modulations and RB allocations

General rule for shared spectrum channel access could apply. However, there is no allocation that satisfies MPR = 0 for NR-U as per [3], table 6.2F.2. Selecting the configuration with lowest MPR value becomes the second best choice. Select DFT-s-OFDM Pi/2 BPSK + full RB allocation and DFT-s-OFDM QPSK + full RB allocation.

Proposal 5: The Configured transmitted power for shared spectrum channel access shall be tested with DFT-s-OFDM Pi/2 BPSK + full RB allocation and DFT-s-OFDM QPSK + full RB allocation.

  - Number of Test Points

Table 2.6-2 Maximum Number of Test Steps for Configured Transmitted Power for shared spectrum channel access

| Environment conditions | Number SCS | Maximum Number of ChBW | Maximum Number of Frequencies | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 5 | 1 | 3 | 1 | 2 | 30 |

# Conclusion

In this document, we discussed several points in deciding the test points of Configured transmitted power for shared spectrum access in FR1. It is proposed that RAN5 endorse following proposals.

Proposal 1: Propose Test Environment as Normal, TL/VL, TL/VH, TH/VL, TH/VH, for Configured transmitted power for shared spectrum channel access.

Proposal 2: Only Mid frequency range is tested for Configured transmitted power for shared spectrum channel access.

Proposal 3: The Lowest, Mid and Highest Channel Bandwidth shall be tested for Configured transmitted power for shared spectrum channel access.

Proposal 4: The Lowest subcarrier spacing shall be tested for Configured transmitted power power for shared spectrum channel access.

Proposal 5: The Configured transmitted power for shared spectrum channel access shall be tested with DFT-s-OFDM Pi/2 BPSK + full RB allocation and DFT-s-OFDM QPSK + full RB allocation.

# Reference

[1]		R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] TS 38.521-1 v18.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

[3] TS 38.101-1 v16.18.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone