3GPP TSG-RAN5 Meeting #102	R5-240903

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-7 | CR | 0055 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC 6.4.1 Emergency Alert CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There was no test case emergency alerts CO |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added TC 6.4.1 and new clause 6.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCData Clients could fail in the field without conformance verification of emergency alerts |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4, 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 6.4	Emergency Alert

### 6.4.1	On-network / Emergency Alert / Cancel Emergency Alert / Client Originated (CO)

6.4.1.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorized for MCDATA Service, including authorized to initiate an emergency alert }

ensure that {
  when { the MCDATA User requests to send an emergency alert with the location of emergency }

then { UE (MCDATA Client) sends a SIP MESSAGE initiating an emergency alert and reporting location information and gives the MCDATA User an indication of success upon the receipt of a SIP 200 (OK) message }

}

(2)

with { UE (MCDATA Client) in the “MDEA3: emergency-alert-initiated” state}

ensure that {

when { the MCDATA User requests to cancel the emergency alert}

then { UE (MCDATA Client) sends a SIP MESSAGE requesting the cancelation of the emergency alert}

}

6.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clauses 16.2.1.2, 16.2.1.2. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-16 requirements.

[TS 24.282 clause 16.2.1.1]

Upon receiving a request from the MCData user to send an MCData emergency alert, the MCData client shall determine whether or not it is authorised to originate an emergency alert, as follows:

1)	if the <allow-activate-emergency-alert> element of the <actions> element of a <rule> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index associated with MCData user (see 3GPP TS 24.484 [12]) is present and is set to a value of "true", then the MCData emergency alert request shall be considered to be an authorised request for an MCData emergency alert. In all other cases, the MCData client shall indicate to the MCData user that the request for sending an MCData emergency alert is unauthorised and shall terminate this procedure.

If the request was authorised, but the MCData user has not indicated the identity of the MCData group to receive the emergency alert, the MCData client shall use, in descending order of preference, one of the following: the value of the <entry> element of the <GroupEmergencyAlert> element of the <Common> element in the MCData user profile, if present; if not, the identity of the MCData group to which the most recent communication or affiliation request was made by the MCData client since last acquiring the MCData service. If an MCData group identity cannot be determined, the MCData client shall indicate the fact to the MCData user and shall terminate this procedure.

The MCData client shall generate a SIP MESSAGE as an out-of-dialog request, in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6], and:

1)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7] in the SIP MESSAGE request;

2)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [5];

4)	shall include an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element (see clause D.1) with:

a)	the <mcdata-request-uri> element set to the MCData group identity;

b)	the <alert-ind> element set to a value of "true";

c)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client; and

d)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias;

5)	shall include an application/vnd.3gpp.mcdata-location-info+xml MIME body with a <Report> element included in the <location-info> root element (see clause D.x);

6)	shall include in the <Report> element the specific location information configured for the MCData emergency alert location trigger;

7)	shall set the MCData emergency state if not already set;

8)	shall set the MCData emergency alert state to "MDEA 2: emergency-alert-confirm-pending";

9)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the group identity; and

10)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP 24.229 [5];

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCData client shall set the MCData emergency alert state to "MDEA 3: emergency-alert-initiated" and shall give the MCData user an indication of success.

On receiving a SIP 4xx response a SIP 5xx response or a SIP 6xx response to the SIP MESSAGE request, the MCData client shall set the MCData emergency alert state to "MDEA 1: no-alert" and shall indicate the failure to the MCData user.

NOTE:	If no response is received after an implementation dependent amount of time or if there is an indication of communication failure, the MCData client can inform the user, and can clear the MCData emergency alert state or can retry sending the emergency alert to the MCData participating server. The MCData emergency state is left unchanged, as the MCData user presumably is in the best position to determine whether or not there still is an emergency situation and can use manual clearing, as necessary.

[TS 24.282 clause 16.2.1.2]

Upon receiving a request from the MCData user to send an MCData emergency alert cancellation, the MCData client shall determine whether or not it is authorised to cancel an emergency alert, as follows:

1)	if the MCData emergency cancellation request is for an MCData emergency alert originated by this MCData user, then the request shall be considered authorised if <allow-cancel-emergency-alert> element of the <actions> element of a <rule> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index associated with MCData user (see 3GPP TS 24.484 [12]) is present and is set to a value of "true"; and

2)	if the MCData emergency cancellation request is for an MCData emergency alert originated by a different MCData user, then the request shall be considered authorised if <allow-cancel-emergency-alert-any-user> element of the <actions> element of a <rule> element of the <ruleset> element of the MCData user profile document identified by the MCData ID and profile index associated with MCData user (see 3GPP TS 24.484 [12]) is present and is set to a value of "true".

If the MCData emergency cancellation request is not considered authorised, the MCData client shall indicate this fact to the requesting MCData user and shall terminate this procedure.

If the authorised MCData emergency cancellation request is for an MCData emergency alert originated by this MCData user and if there are more than one outstanding emergency alerts from this MCData user and the MCData user has not indicated which one to cancel, the MCData client shall terminate this procedure after giving an indication of the condition to the MCData user.

The MCData client shall generate a SIP MESSAGE out-of dialog request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] and:

1)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata" (coded as specified in 3GPP TS 24.229 [5]), in a P-Preferred-Service header field according to IETF RFC 6050 [7];

2)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [8];

3)	may include a P-Preferred-Identity header field containing a public user identity as specified in 3GPP TS 24.229 [5];

4)	if the MCData emergency alert was originated by this MCData user, shall include an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element (see clause D.1) with:

a)	the <mcdata-request-uri> element set to the MCData group identity;

b)	the <alert-ind> element set to a value of "false";

c)	the <mcdata-client-id> element set to the MCData client ID of this MCData client; amd

d)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias;

5)	if the MCData emergency alert was originated by a different MCData user, shall include an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element (see clause D.1) with:

a)	the <mcdata-request-uri> element set to the MCData group identity;

b)	the <alert-ind> element set to a value of "false";

c)	the <originated-by> element set to the MCData ID of the MCData user who originated the MCData emergency alert; and

d)	if the MCData client is aware of active functional aliases, and an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> set to the URI of the used functional alias;

6)	shall set the Request-URI to the public service identity identifying the participating MCData function serving the group identity;

7)	if the generated SIP MESSAGE request does not contain an <originated-by> element in the application/vnd.3gpp.MCData-info+xml MIME body, shall set the MCData emergency alert state to "MDEA 4: emergency-alert-cancel-pending"; and

8)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

On receipt of a SIP MESSAGE request containing an application/vnd.3gpp.mcdata-info+xml MIME body with an <alert-ind-rcvd> element set to "true" and an <mcdata-client-id> matching the MCData client ID included in the sent SIP MESSAGE request and if the sent SIP MESSAGE request did not contain an <originated-by> element in its application/vnd.3gpp.mcdata-info+xml MIME body, the MCData client shall:

1)	if the <alert-ind> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the received SIP MESSAGE request is set to a value of "false":

a)	set the MCData emergency alert state to "MDEA 1: no-alert"; and

b)	clear the MCData emergency state if not already cleared; and

2)	if the <alert-ind> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the received SIP MESSAGE request is set to a value of "true" and if the MCData emergency alert state is set to "MDEA 4: emergency-alert-cancel-pending":

a)	set the MCData emergency alert state to "MDEA 1: no-alert".

NOTE:	It would appear to be an unusual situation for the initiator of an MCData emergency alert to not be able to clear their own alert. Nevertheless, an MCData user can be configured to be authorised to initiate MCData emergency alerts but not have the authority to clear them. Hence, the case is covered here.

On receiving a SIP 4xx response, SIP 5xx response or SIP 6xx response to the sent SIP MESSAGE emergency alert cancellation request, if the sent SIP MESSAGE request did not contain an <originated-by> element in the application/vnd.3gpp.mcdata-info+xml MIME body and the MCData emergency alert state is set to "MDEA 4: emergency-alert-cancel-pending":

1)	if the received SIP 4xx response, SIP 5xx response or SIP 6xx response does not contain an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element containing the  <alert-ind> element OR if it contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with the <alert-ind> element set to a value of "true" (see clause D.1), the MCData client shall set the MCData emergency alert state to "MDEA 3: emergency-alert-initiated".

6.4.1.3	Test description

6.4.1.3.1	Pre-test conditions

System Simulator:

-	SS (MCData server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCDATA operation in the MCDATA configuration document).

IUT:

-	UE (MCData client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

Preamble:

-	The UE has performed procedure 'MCData UE registration' as specified in TS 36.579-1 [2] clause 5.4.2B.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCData Client Application has been activated and User has registered-in as the MCDATA User with the Server as active user at the Client.

6.4.1.3.2	Test procedure sequence

Table 6.4.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCDATA Client) send an emergency alert. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCDATA Client) correctly perform procedure 'MCX SIP MESSAGE Request - Accept CO' as described in TS 36.579-1 [2] Table 5.3.30.3-1 to request an emergency alert providing location information? | - | - | 1 | P |
| 3 | Check: Does the UE (MCDATA Client) give the MCDATA User an indication of success? | - | - | 1 | P |
| 4 | Make the UE (MCDATA Client) cancel the emergency alert. (NOTE 1) | - | - | - | - |
| 5 | Check: Does the UE (MCDATA Client) correctly perform procedure 'MCX SIP MESSAGE Request - Accept CO' as described in TS 36.579-1 [2] Table 5.3.30.3-1 to cancel the emergency alert? | - | - | 2 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.4.1.3.3	Specific message contents

Table 6.4.1.3.3-1: SIP MESSAGE from the UE (step 2, Table 6.4.1.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.1.3.3-2 |  |  |  |
| MIME body part |  | Location-info |  |  |
| MIME-part-body | Location-info as described in Table 6.4.1.3.3-3 |  |  |  |

Table 6.4.1.3.3-2: MCData-Info in SIP MESSAGE (Table 6.4.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | not present |  |  |  |
| alert-ind | Encrypted <alert-ind> with mcdataBoolean set to true | Encryption according to NOTE 2 in TS 36.579-1 [2] Table 5.5.3.2.1-3 |  |  |

Table 6.4.1.3.3-3: Location-Info in SIP MESSAGE (Table 6.4.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.4.1-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| location-info |  |  |  |  |
| Report |  |  |  |  |
| CurrentLocation |  |  |  |  |
| CurrentCoordinate |  |  |  |  |
| longitude | Encrypted <longitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |
| latitude | Encrypted <latitude> with any content | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.4.1-3 |  |  |

Table 6.4.1.3.3-4: SIP MESSAGE from the SS (step 2, Table 6.4.1.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.1.3.3-5 |  |  |  |

Table 6.4.1.3.3-5: MCData-Info in SIP MESSAGE (Table 6.4.1.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| alert-ind | Encrypted <alert-ind> with mcdataBoolean set to true | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.2.2-3 |  |  |
| mcdata-client-id | Encrypted <mcdata-client-id> with mcdataString set to the client-id as received from the UE | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.2.2-3 |  |  |
| anyEXT |  |  |  |  |
| alert-ind-rcvd | mcdataBoolean set to true |  |  |  |

Table 6.4.1.3.3-6: SIP MESSAGE from the UE (step 5, Table 6.4.1.3.2-1; 
step 2, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.1.3.3-7 |  |  |  |

Table 6.4.1.3.3-7: MCData-Info in SIP MESSAGE (Table 6.4.1.3.3-6)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-3, condition MCD_grp |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| request-type | not present |  |  |  |
| alert-ind | Encrypted <alert-ind> with mcdataBoolean set to false | Encryption according to NOTE 2 in TS 36.579-1 [2] Table 5.5.3.2.1-3 |  |  |

Table 6.4.1.3.3-8: SIP MESSAGE from the SS (step 5, Table 6.4.1.3.2-1;
step 4, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCData-Info |  |  |
| MIME-part-body | MCData-Info as described in Table 6.4.1.3.3-9 |  |  |  |

Table 6.4.1.3.3-9: MCData-Info in SIP MESSAGE (Table 6.4.1.3.3-8)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-info |  |  |  |  |
| mcdata-Params |  |  |  |  |
| mcdata-calling-user-id | not present |  |  |  |
| alert-ind | Encrypted <alert-ind> with mcdataBoolean set to false | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.2.2-3 |  |  |
| mcdata-client-id | Encrypted <mcdata-client-id> with mcdataString set to the client-id as received from the UE | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.2.2-3 |  |  |
| anyExt |  |  |  |  |
| alert-ind-rcvd | mcdataBoolean set to true |  |  |  |

<END OF CHANGES>