3GPP TSG-RAN5 Meeting #102	R5-240939

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1461 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional CA_30A-48A for CA signalling test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Bureau Veritas ADT, Sporton International, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CA_30A-48A is missing test frequency for CA signalling test. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Default test frequencies CA_30A-48A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA_30A-48A will miss test frequency for CA signalling test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## {Start of changes}

### 6.2.3	Default test frequencies

<Unchanged Sections Skipped>

#### 6.2.3.2	Test frequencies for CA signalling test

Test frequencies for CA signalling testing are specified in Table 6.2.3.2-1 for CA Intra-Band contiguous case; in Tables 6.2.3.2-2 for CA Inter-band case and in Table 6.2.3.2-4 for Intra-Band Non-Contiguous case.

NOTE 1:	Alternative test frequencies for additional channel bandwidth combinations may need to be specified when new CA configurations or CA Bandwidth Combination Sets are introduced in TS 36.101 subclause 5.6A.

For CA Intra-Band contiguous scenarios then f1, f2 and f3 are used.

NOTE 2:	f2 and f3 are not adjacent frequencies; hence those cannot be used simultaneously as Pcell and Scell for CA Intra-Band contiguous scenarios.

For CA Inter-Band scenarios then f1, f2, f5 and f6 are used.

NOTE 3:	Table 6.2.3.2-2 specifies the test frequencies for operation with PCell in the frequency band mentioned first (= carrier 1) and SCell in the frequency band mentioned second (= carrier 2) in the E-UTRA CA Configuration name acc. to TS 36.101 subclause 5.6A (e.g. PCell in band 1 and SCell in band 5 for CA_1A-5A configuration), for operation with switched allocation of PCell and SCell. SCell in the frequency band mentioned first (= carrier 1) and PCell in the frequency band mentioned second (= carrier 2) in the E-UTRA CA Configuration name (given that UL is supported in both frequency bands in question).

NOTE 4:	For operation with switched allocation f1 is mapped to f5, f2 is mapped to f6, f5 is mapped to f1, f6 is mapped to f2

For CA Intra-Band non-contiguous scenarios then f1, f2 and f3 are used.

<Unchanged Sections Skipped>

Table 6.2.3.2-2: Test frequencies for E-UTRA PCell and SCell for CA Inter-band operation and E-UTRA PCell and PSCell for DC Inter-band operation (two bands)

| E-UTRA CA Configuration | Width of Operating bands [MHz+MHz] | CC Combination / NRB_agg | Test Frequency | CC NRB | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A, DC_1A-3A | 60+75 | 50 + 50 | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |
|  |  |  | f2 | CC 50 | 18550 | 1975 | 550 | 2165 |
|  |  |  | f5 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |
|  |  |  | f6 | CC 50 | 19900 | 1780 | 1900 | 1875 |
| CA_1A-5A, DC_1A-5A | 60+25 | 50 + 50 | f1 | CC 50 | 18300 | 1950 | 300 | 2140 |
|  |  |  | f2 | CC 50 | [18550] | [1975] | [550] | [2165] |
|  |  |  | f5 | CC 50 | 20450 | 829 | 2450 | 874 |
|  |  |  | f6 | CC 50 | 20600 | 844 | 2600 | 889 |
| <Unchanged Sections Skipped> |  |  |  |  |  |  |  |  |
| CA_29A-70A1 | 11+(15+25)2 | 50+75 | f1 | CC 50 | N/A | N/A | 9720 | 723 |
|  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |
|  |  |  | f5 | CC 75 | 133047 | 1702.5 | 68411 | 2002.5 |
|  |  |  | f6 | CC 75 | N/A | N/A | N/A | N/A |
| CA_29A-70C1 | 11+(15+25)2 | 50+75+50 | f1 | CC 50 | N/A | N/A | 9720 | 723 |
|  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |
|  |  |  | f5 | CC 75 | 133047 | 1702.5 | 68411 | 2002.5 |
|  |  |  | f6 | CC 50 | N/A | N/A | 68531 | 2014.5 |
| CA_30A-48A | 10 + 150 | 50 + 100 | f1 | CC 50 | 27710 | 2310 | 9820 | 2355 |
|  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |
|  |  |  | f5 | CC 100 | 55990 | 3625 | 55990 | 3625 |
|  |  |  | f6 | CC 100 | 56640 | 3690 | 56640 | 3690 |
| CA_30A-66A | 10+(70+90)2 | 50+100 | f1 | CC 50 | 27710 | 2310 | 9820 | 2355 |
|  |  |  | f2 | N/A | N/A | N/A | N/A | N/A |
|  |  |  | f5 | CC 100 | 132422 | 1755 | 66886 | 2155 |
|  |  |  | f6 | CC 100 | 132572 | 1770 | 67036 | 2170 |
| CA_39A-41A, DC_39A-41A | 40+194 | 100+100 | f1 | CC 100 | 38350 | 1890 | 38350 | 1890 |
|  |  |  | f2 | CC 100 | 38550 | 1910 | 38550 | 1910 |
|  |  |  | f5 | CC 100 | 40620 | 2593 | 40620 | 2593 |
|  |  |  | f6 | CC 100 | 41490 | 2680 | 41490 | 2680 |

## {End of changes}