3GPP TSG-RAN5 Meeting #102	R5-240944

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

Agenda item:	5.3.19.9

Source: 	Apple

Title: 	Time synchronization required for UE beamlock function during initial access

Document for:	Discussion and Endorsement

# Introduction

To support beam correspondence testing during initial access in RAN5, UE beam lock function is required. This paper discusses the significance of time alignment needed between TE and UE before sending and acknowledging the activation of beam lock command during the initial access RACH process.

As per the LS [1] received from RAN4#108bis meeting, to develop the beam lock function, to lock the UE after requested by test equipment (TE) when UE is in RRC_Idle State.

The goal is to align UE/TE before sending/acknowledging the beam lock command and to identify the problems.

# Discussion

The intention of this paper is to discuss and address the issues that may arise with the development of UE beam lock test procedure in initial access.

The considerations listed below should be considered to define the Beam Lock Function:

- Beam lock has to occur after UE optimal Tx Beam selection, as per LS[1].

- Beam lock has to occur before the expiry of preambleTransMax, the maximum number of Random-Access Preamble transmission counter, IE defined in TS 38.331.

- TE needs at least 1ms duration to perform PRACH measurements, as per LS[1]

- UE also needs to send acknowledgement to TE for the beam lock command, as specified in TS 38.509, Section 5.4.

- TE needs to send MSG2 before the expiry of preambleTransMax counter, for successful RACH[2].

RACH is a time sensitive process, it is important for TE that it should be time aligned and synchronized with UE.As specified in [2] clause 5.1.3 and clause 5.1.4, during RACH MSG1, UE can send RA-preamble and TE can responds with random access response (RAR) in MSG2. PreambleTransMax counter is initiated on UE side with the transmission of first preamble transmission and UE declares RACH failure upon its expiry.

TE is supposed to send the UE beam lock command, complete PRACH measurements and send MSG2 before this counter expiry.

Observation 1: TE and UE shall be time aligned before sending and acknowledging beam lock command during initial access.

Proposal 1: Further discuss the following aspects

- Time synchronization aspects between UE and TE

- Additional details to be finalized to ensure PRACH measurements are completed before PreambleTransMax counter reaches the limit.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1: TE and UE shall be time aligned before sending and acknowledging beam lock command during initial access.

Proposal 1: Further discuss the following aspects

- Time synchronization aspects between UE and TE

- Additional details to be finalized to ensure PRACH measurements are completed before PreambleTransMax counter reaches the limit.

# References

- R4-2317764, “LS on testability issues of beam correspondence requirements”, RAN4#108-bis, October 9 – October 13, 2023

- TS 38.321, Medium Access Control (MAC) protocol specification.