3GPP TSG WG RAN5 Meeting #102	R5-240965

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5496 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 of CA Configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SGS Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 doesn’t include correct CA Configurations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct Table 7.5A.8.4.1-1 & Table 7.5A.9.4.1-1 & Table 7.6.1A.8.4.1-1 of CA Configurations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA Configurations will still incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 7.5A.8.4.1-1 Table 7.5A.9.4.1-1 Table 7.6.1A.8.4.1-1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.5A.8	Adjacent Channel Selectivity (ACS) for 5DL CA

< Start of Change-1 >

Table 7.5A.8.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  |  | SCC1 |  |  |  |  |  |  |  | SCC2 |  |  |  |  | SCC3 |  |  |  |  |  | SCC4 |  |  |  |  |  |  |  |  |  |  |
|  | Band | Range |  | NRB |  | Band |  | Range | NRB |  |  |  |  | Band | Range | NRB |  |  | Band | Range |  | NRB |  |  | Band |  | Range |  |  |  | NRB |  |  |  |  |
|  | UL MOD | DL MOD |  | ULalloc (Note 2,3,4) | DL alloc | UL MOD |  | DL MOD | UL alloc |  |  | DL alloc |  | UL MOD | UL MOD | DL MOD | UL alloc |  | UL MOD | DL MOD |  | UL alloc | DL alloc |  |  | UL MOD |  |  | DL MOD |  |  |  | UL alloc |  | DL alloc |
| Default Test Settings for a CA_XF Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/ CC1 |  | REF SENS | ALL RBs | X |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | X | Mid/ CC3 | N/A | ALL RBs |  | X | Mid/ CC4 |  | N/A | ALL RBs |  |  | X |  |  | Mid/ CC5 |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Lowest NRB_agg | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A | QPSK |  | Lowest NRB_agg |  | N/A | QPSK |  |  | Lowest NRB_agg |  |  | N/A |  |  | QPSK |  |  |  |  |  | Lowest NRB_agg |
| 2 | X | Mid/ CC1 |  | REF SENS | ALL RBs | X |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | X | Mid/ CC3 | N/A | ALL RBs |  | X | Mid/ CC4 |  | N/A | ALL RBs |  |  | X |  |  | Mid/ CC5 |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB_agg | N/A |  | QPSK |  |  |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  |  | Highest NRB_agg |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB_agg |
| Default Test Settings for a CA_XA-YA-ZA-RA-SA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid |  | REF SENS | ALL RBs | Y |  | Mid | N/A |  |  | ALL RBs |  | Z | Mid | N/A | ALL RBs |  | R | Mid |  | N/A | ALL RBs |  |  | S |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 2 | Y | Mid |  | REF SENS | ALL RBs | X |  | Mid | N/A |  |  | ALL RBs |  | Z | Mid | N/A | ALL RBs |  | R | Mid |  | N/A | ALL RBs |  |  | S |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 3 | Z | Mid |  | REF SENS | ALL RBs | X |  | Mid | N/A |  |  | ALL RBs |  | Y | Mid | N/A | ALL RBs |  | R | Mid |  | N/A | ALL RBs |  |  | S |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 4 | R | Mid |  | REF SENS | ALL RBs | X |  | Mid | N/A |  |  | ALL RBs |  | Y | Mid | N/A | ALL RBs |  | Z | Mid |  | N/A | ALL RBs |  |  | S |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 5 | S | Mid |  | REF SENS | ALL RBs | X |  | Mid | N/A |  |  | ALL RBs |  | Y | Mid | N/A | ALL RBs |  | Z | Mid |  | N/A | ALL RBs |  |  | R |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| Default Test Settings for a CA_XE-YA, YA-XE Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/ CC1 |  | REF SENSE | ALL RBs | X |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | X | Mid/ CC3 | N/A | ALL RBs |  | X | Mid/ CC4 |  | N/A | ALL RBs |  |  | Y |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 2 | X | Mid/ CC1 |  | REF SENSE | ALL RBs | X |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | X | Mid/ CC3 | N/A | ALL RBs |  | X | Mid/ CC4 |  | N/A | ALL RBs |  |  | Y |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Lowest NRB | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A | QPSK |  | Lowest NRB |  | N/A | QPSK |  |  | Lowest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 3 | Y | Mid |  | REF SENSE | ALL RBs | X |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | X | Mid/ CC2 | N/A | ALL RBs |  | X | Mid/ CC3 |  | N/A | ALL RBs |  |  | X |  |  | Mid/ CC4 |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 4 | Y | Mid |  | REF SENSE | ALL RBs | X |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | X | Mid/ CC2 | N/A | ALL RBs |  | X | Mid/ CC3 |  | N/A | ALL RBs |  |  | X |  |  | Mid/ CC4 |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A | QPSK |  | Lowest NRB |  | N/A | QPSK |  |  | Lowest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Lowest NRB |
| Default Test Settings for CA_YA-ZA-XB-RA and CA_YA-ZA-XC-RA Configuration (Intra-band contiguous + Inter-band) Default Test Settings for CA_YA-ZA-RA-XB and CA_YA-ZA-RA-XC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/ CC1 |  | REF SENS | ALL RBs | X |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | Y | Mid | N/A | ALL RBs |  | Z | Mid |  | N/A | ALL RBs |  |  | R |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB_agg | N/A |  | QPSK |  |  |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 2 | X | Mid/ CC1 |  | REF SENS | ALL RBs | X |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | Y | Mid | N/A | ALL RBs |  | Z | Mid |  | N/A | ALL RBs |  |  | R |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Lowest NRB_agg | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 3 | Y | Mid |  | REF SENS | ALL RBs | X |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | X | Mid/ CC2 | N/A | ALL RBs |  | Z | Mid |  | N/A | ALL RBs |  |  | R |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB_agg |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 4 | Y | Mid |  | REF SENS | ALL RBs | X |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | X | Mid/ CC2 | N/A | ALL RBs |  | Z | Mid |  | N/A | ALL RBs |  |  | R |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A | QPSK |  | Lowest NRB_agg |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 5 | Z | Mid |  | REF SENS | ALL RBs | Y |  | Mid | N/A |  |  | ALL RBs |  | X | Mid/ CC1 | N/A | ALL RBs |  | X | Mid/ CC2 |  | N/A | ALL RBs |  |  | R |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  |  | Highest NRB_agg |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 6 | Z | Mid |  | REF SENS | ALL RBs | Y |  | Mid | N/A |  |  | ALL RBs |  | X | Mid/ CC1 | N/A | ALL RBs |  | X | Mid/ CC2 |  | N/A | ALL RBs |  |  | R |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Lowest NRB_agg |  | N/A | QPSK |  |  | Lowest NRB_agg |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 7 | R | Mid |  | REF SENS | ALL RBs | Y |  | Mid | N/A |  |  | ALL RBs |  | X | Mid/ CC1 | N/A | ALL RBs |  | X | Mid/ CC2 |  | N/A | ALL RBs |  |  | Z |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | N/A | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB_agg |  | N/A | QPSK |  |  | Highest NRB_agg |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 8 | R | Mid |  | REF SENS | ALL RBs | Y |  | Mid | N/A |  |  | ALL RBs |  | X | Mid/ CC1 | N/A | ALL RBs |  | X | Mid/ CC2 |  | N/A | ALL RBs |  |  | Z |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | N/A | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Lowest NRB_agg |  | N/A | QPSK |  |  | Lowest NRB_agg |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| Default Test Settings CA_XB-YB-ZA, CA_XB-YC-ZA, CA_XC-YB-ZA, and CA_XC-YC-ZA Configuration (Intra-band contiguous + Intra-band contiguous + Inter-band) Default Test Settings CA_ZA-XB-YB, CA_ZA-XB-YC, CA_ZA-XC-YB, and CA_ZA-XC-YC Configuration (Intra-band contiguous + Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/ CC1 |  | REF SENS | ALL RBs | X |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | Y | Mid/ CC1 | N/A | ALL RBs |  | Y | Mid/ CC2 |  | N/A | ALL RBs |  |  | Z |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 2 | X | Mid/ CC1 |  | REF SENS | ALL RBs | X |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | Y | Mid/ CC1 | N/A | ALL RBs |  | Y | Mid/ CC2 |  | N/A | ALL RBs |  |  | Z |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Lowest NRB | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A | QPSK |  | Lowest NRB |  | N/A | QPSK |  |  | Lowest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 3 | Y | Mid/ CC1 |  | REF SENS | ALL RBs | Y |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | X | Mid/ CC1 | N/A | ALL RBs |  | X | Mid/ CC2 |  | N/A | ALL RBs |  |  | Z |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 4 | Y | Mid/ CC1 |  | REF SENS | ALL RBs | Y |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | X | Mid/ CC1 | N/A | ALL RBs |  | X | Mid/ CC2 |  | N/A | ALL RBs |  |  | Z |  |  | Mid |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Lowest NRB | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A | QPSK |  | Lowest NRB |  | N/A | QPSK |  |  | Lowest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Highest NRB |
| 5 | Z | Mid |  | REF SENS | ALL RBs | X |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | X | Mid/ CC2 | N/A | ALL RBs |  | Y | Mid/ CC1 |  | N/A | ALL RBs |  |  | Y |  |  | Mid/ CC2 |  |  |  | N/A |  | ALL RBs |
|  | QPSK |  | QPSK |  | Highest NRB | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  | Highest NRB |  | N/A | QPSK |  |  | Highest NRB |  |  | N/A |  |  |  | QPSK |  |  |  |  | Highest NRB |
| Default Test Settings for CA_XC-YD, YD-XC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 |  | REFSENS | ALL RBs | X | Mid/CC2 |  |  | N/A | ALL RBs |  | Y |  | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 |  | N/A | ALL RBs |  | Y |  |  |  | Mid/C C3 |  |  |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  | Highest NRB | N/A | QPSK |  |  | Highest NRB |  | N/A |  |  |  | QPSK |  |  |  |  |  | Highest NRB |  |
| 2 | X | Mid/CC1 |  | REFSENS | ALL RBs | X | Mid/CC2 |  |  | N/A | ALL RBs |  | Y |  | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 |  | N/A | ALL RBs |  | Y |  |  |  | Mid/C C3 |  |  |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  |  | Lowest NRB | N/A | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  | Highest NRB | N/A | QPSK |  |  | Highest NRB |  | N/A |  |  |  | QPSK |  |  |  |  |  | Highest NRB |  |
| 3 | Y | Mid/CC1 |  | REFSENS | ALL RBs | Y | Mid/CC2 |  |  | N/A | ALL RBs |  | Y |  | Mid/CC3 | N/A | ALL RBs | X | Mid/CC1 |  | N/A | ALL RBs |  | X |  |  |  | Mid/CC2 |  |  |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  | Highest NRB | N/A | QPSK |  |  | Highest NRB |  | N/A |  |  |  | QPSK |  |  |  |  |  | Highest NRB |  |
| 4 | Y | Mid/CC1 |  | REFSENS | ALL RBs | Y | Mid/CC2 |  |  | N/A | ALL RBs |  | Y |  | Mid/CC3 | N/A | ALL RBs | X | Mid/CC1 |  | N/A | ALL RBs |  | X |  |  |  | Mid/CC2 |  |  |  | N/A |  | ALL RBs |  |
|  | QPSK | QPSK |  |  | Highest NRB | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  | Highest NRB | N/A | QPSK |  |  | Lowest NRB |  | N/A |  |  |  | QPSK |  |  |  |  |  | Lowest NRB |  |
| 5 | Y | Mid |  | REF SENS | ALL RBs | X |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | X | Mid/ CC2 | N/A | ALL RBs |  | Y | Mid/ CC1 |  | N/A | ALL RBs |  |  | Y |  |  | Mid/ CC2 |  |  |  | N/A |  | ALL RBs |
|  | QPSK | QPSK |  |  | Highest NRB | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A | QPSK |  | Lowest NRB |  | N/A | QPSK |  |  | Lowest NRB |  |  | N/A |  |  | QPSK |  |  |  |  |  | Lowest NRB |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band contiguous & Intra-band contiguous + Inter-band: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band: Use CA Configuration – specific test points if present in the table, Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41F, X=41. Note 6: Inter-band: X,Y,Z,R,S correspond to the different bands in the CA Configuration. E.g. for CA_1A-3A-7A-20A-42A,X=1,Y=3,Z=7,R=20,S=42. Note 7: Intra-band contiguous + Inter-band: X,Y,Z,R correspond to the different bands in the CA Configuration. E.g. for CA_3C-7A-28A, X=3,Y=7 and Z=28. Note 8: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 10: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< End of Change-1 >

7.5A.9	Adjacent Channel Selectivity (ACS) for 6DL CA

< Start of Change-2 >

Table 7.5A.9.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC |  |  |  | SCC1 |  |  |  | SCC2 |  |  |  | SCC3 |  |  |  | SCC4 |  |  |  | SCC5 |  |  |  |
|  | Band | Range | NRB |  | Range | NRB | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | ULalloc | UL MOD | DL MOD | DLalloc | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc | UL MOD | DL MOD | ULalloc | DLalloc |
| Default Test Settings for a CA_XA-YA-ZA-RA-SA-UA (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | REFSENS | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs | S | Mid | N/A | ALL RBs | U | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 2 | Y | Mid | REFSENS | ALL RBs | X | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs | S | Mid | N/A | ALL RBs | U | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 3 | Z | Mid | REFSENS | ALL RBs | X | Mid | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs | S | Mid | N/A | ALL RBs | U | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 4 | R | Mid | REFSENS | ALL RBs | X | Mid | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | S | Mid | N/A | ALL RBs | U | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 5 | S | Mid | REFSENS | ALL RBs | X | Mid | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs | U | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 6 | U | Mid | REFSENS | ALL RBs | X | Mid | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs | S | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XE-YC, XE-YB, YC-XE, YB-XE (Intra-band contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 3 | Y | Mid/CC1 | REFSENS | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | X | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 4 | Y | Mid/CC1 | REFSENS | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | X | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XE-YA-ZA, YA-XE-ZA, YA-ZA-XE Configuration (Intra-band contiguous + Inter-band) Default Test Settings for a CA_XE-YA-ZA, YA-XE-ZA,YA-ZA-XE Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 3 | Y | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 4 | Y | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 5 | Z | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 6 | Z | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | X | Mid/CC4 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XD-YA-ZA-RA, YA-XD-ZA-RA, YA-ZA-XD-RA, YA-ZA-RA-XD Configuration (Intra-band contiguous + Inter-band(2)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 3 | Y | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | N/A | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 4 | Y | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | N/A | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 5 | Z | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | N/A | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 6 | Z | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | N/A | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 7 | R | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | N/A | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 8 | R | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | N/A | QPSK |  | Highest NRB_agg | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XD-YC-ZA, XD-ZA-YC, YC-XD-ZA, YC-ZA-XD, ZA-XD-YC, ZA-YC-XD Configuration (Intra-band contiguous + Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 3 | Y | Mid/CC1 | REFSENS | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 4 | Y | Mid/CC1 | REFSENS | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 5 | Z | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 6 | Z | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | X | Mid/CC3 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XC-YC-ZA-RA, ZA-XC-YC-RA, ZA-RA-XC-YC Configuration (Intra-band contiguous + Intra-band contiguous + Inter-band(2)) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 3 | Y | Mid/CC1 | REFSENS | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 4 | Y | Mid/CC1 | REFSENS | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 5 | Z | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | R | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 6 | R | Mid | REFSENS | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | Z | Mid | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Default Test Settings for a CA_XC-YC-ZC Configuration (Intra-band contiguous + Intra-band contiguous + Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC1 | N/A | ALL RBs | Z | Mid/CC1 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 2 | X | Mid/CC1 | REFSENS | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC1 | N/A | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 3 | Y | Mid/CC1 | REFSENS | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC1 | N/A | ALL RBs | Z | Mid.CC1 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 4 | Y | Mid/CC1 | REFSENS | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs | Z | Mid/CC1 | N/A | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| 5 | Z | Mid/CC1 | REFSENS | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC1 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg | N/A | QPSK |  | Lowest NRB_agg |
| 6 | Z | Mid/CC1 | REFSENS | ALL RBs | Z | Mid/CC2 | N/A | ALL RBs | Y | Mid/CC1 | N/A | ALL RBs | Y | Mid/CC2 | N/A | ALL RBs | X | Mid/CC1 | N/A | ALL RBs | X | Mid/CC2 | N/A | ALL RBs |
|  | QPSK | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg | N/A | QPSK |  | Highest NRB_agg |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band contiguous & Intra-band contiguous + Inter-band: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band: Use CA Configuration – specific test points if present in the table, Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 4: Inter-band & Intra-band contiguous + Inter-band: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_42F, X=42. Note 6: Inter-band: X,Y,Z,R,S,U correspond to the different bands in the CA Configuration. E.g. for CA_1A-3A-7A-20A-28A-42A,X=1,Y=3,Z=7,R=20,S=28.U=42 Note 7: Intra-band contiguous + Inter-band: X,Y,Z,R correspond to the different bands in the CA Configuration. E.g. for CA_1A-5A-7A-46C, X=1,Y=5,Z=7 and R=46. Note 8: REFSENS refers to the PCC band’s and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 10: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg , choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< End of Change-2 >

7.6.1A.8	In-band blocking for 5DL CA

< Start of Change-3 >

Table 7.6.1A.8.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to “PCC NRB” and “SCCs NRB” columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID |  | PCC |  |  |  |  |  |  |  | SCC1 |  |  |  |  |  |  |  | SCC2 |  |  |  |  |  |  |  |  | SCC3 |  |  |  |  |  |  |  |  | SCC4 |  |  |  |  |
|  |  | Band |  | Range |  | NRB |  |  |  | Band |  | Range |  | NRB |  |  |  | Band |  |  | Range | NRB |  |  |  |  | Band |  | Range |  | NRB |  |  |  |  | Band | Range | NRB |  |  |  |
|  |  | UL MOD |  | DL MOD |  | ULalloc (Note 2,3,4) |  | DL alloc |  | UL MOD |  | DL MOD |  | UL alloc |  | DL alloc |  | UL MOD |  |  | UL MOD | DL MOD |  |  | UL alloc |  | UL MOD |  | DL MOD |  | UL alloc |  | DL alloc |  |  | UL MOD | DL MOD | UL alloc |  | DL alloc |  |
| Default Test Settings for a CA_XF Configuration (Intra-band contiguous) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC3 | N/A |  |  | ALL RBs |  | X |  | Mid/ CC4 |  | N/A |  | ALL RBs |  |  | X | Mid/ CC5 | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  |  | N/A | QPSK |  |  | Lowest NRB_agg |  |
| 2 |  | X |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC3 | N/A |  |  | ALL RBs |  | X |  | Mid/ CC4 |  | N/A |  | ALL RBs |  |  | X | Mid/ CC5 | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  | N/A |  |  | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |  | N/A | QPSK |  |  | Highest NRB_agg |  |
| Default Test Settings for a CA_XA-YA-ZA-RA-SA Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  | Mid |  | REF SENS |  | ALL RBs |  | Y |  | Mid |  | N/A |  | ALL RBs |  | Z |  |  | Mid | N/A |  |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |  | S | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 2 |  | Y |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  | Z |  |  | Mid | N/A |  |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |  | S | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 3 |  | Z |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  | Y |  |  | Mid | N/A |  |  | ALL RBs |  | R |  | Mid |  | N/A |  | ALL RBs |  |  | S | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 4 |  | R |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  | Y |  |  | Mid | N/A |  |  | ALL RBs |  | Z |  | Mid |  | N/A |  | ALL RBs |  |  | S | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 5 |  | S |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  | Y |  |  | Mid | N/A |  |  | ALL RBs |  | Z |  | Mid |  | N/A |  | ALL RBs |  |  | R | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| Default Test Settings for a CA_XE-YA, YA-XE Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC3 | N/A |  |  | ALL RBs |  | X |  | Mid/ CC4 |  | N/A |  | ALL RBs |  |  | Y | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 2 |  | X |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC3 | N/A |  |  | ALL RBs |  | X |  | Mid/ CC4 |  | N/A |  | ALL RBs |  |  | Y | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A |  |  | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 3 |  | Y |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC1 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | X |  | Mid/ CC3 |  | N/A |  | ALL RBs |  |  | X | Mid/ CC4 | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 4 |  | Y |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC1 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | X |  | Mid/ CC3 |  | N/A |  | ALL RBs |  |  | X | Mid/ CC4 | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A |  |  | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  |  | N/A | QPSK |  |  | Lowest NRB |  |
| Default Test Settings for CA_XA-ZA-RA-YB and CA_XA-ZA-RA-YC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Y |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | X |  |  | Mid | N/A |  |  | ALL RBs |  | Z |  | Mid |  | N/A |  | ALL RBs |  |  | R | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 2 |  | Y |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | X |  |  | Mid | N/A |  |  | ALL RBs |  | Z |  | Mid |  | N/A |  | ALL RBs |  |  | R | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 3 |  | X |  | Mid |  | REF SENS |  | ALL RBs |  | Y |  | Mid/ CC1 |  | N/A |  | ALL RBs |  | Y |  |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | Z |  | Mid |  | N/A |  | ALL RBs |  |  | R | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  | N/A |  |  | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 4 |  | X |  | Mid |  | REF SENS |  | ALL RBs |  | Y |  | Mid/ CC1 |  | N/A |  | ALL RBs |  | Y |  |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | Z |  | Mid |  | N/A |  | ALL RBs |  |  | R | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 5 |  | Z |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  | Y |  |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  |  | R | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 6 |  | Z |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  | Y |  |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  |  | R | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 7 |  | R |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  | Y |  |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  |  | Z | Mid | N/A |  | ALL RBs |  |
|  |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB_agg |  | N/A |  | QPSK |  |  |  | Highest NRB_agg |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 8 |  | R |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid |  | N/A |  | ALL RBs |  | Y |  |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  |  | Z | Mid | N/A |  | ALL RBs |  |
|  |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Lowest NRB_agg |  | N/A |  | QPSK |  |  |  | Lowest NRB_agg |  |  | N/A | QPSK |  |  | Highest NRB |  |
| Default Test Settings CA_XB-YB-ZA, CA_XB-YC-ZA, CA_XC-YB-ZA, and CA_XC-YC-ZA Configuration (Intra-band contiguous + Intra-band contiguous + Inter-band) Default Test Settings CA_ZA-XB-YB, CA_ZA-XB-YC, CA_ZA-XC-YB, and CA_ZA-XC-YC Configuration (Intra-band contiguous + Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | Y |  |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  |  | Z | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 2 |  | X |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | Y |  |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  |  | Z | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A |  |  | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 3 |  | Y |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | X |  | Mid/ CC2 |  | N/A |  | ALL RBs |  |  | Z | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 4 |  | Y |  | Mid/ CC1 |  | REF SENS |  | ALL RBs |  | Y |  | Mid/ CC2 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC1 | N/A |  |  | ALL RBs |  | X |  | Mid/ CC2 |  | N/A |  | ALL RBs |  |  | Z | Mid | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A |  |  | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| 5 |  | Z |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC1 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | Y |  | Mid/ CC1 |  | N/A |  | ALL RBs |  |  | Y | Mid/ CC2 | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A |  |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  |  | N/A | QPSK |  |  | Highest NRB |  |
| Default Test Settings for CA_XC-YD, YD-XC Configuration (Intra-band contiguous + Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | X |  | Mid/CC1 |  | REFSENS |  | ALL RBs |  | X |  | Mid/CC2 |  | N/A |  | ALL RBs |  | Y | Mid/CC1 |  |  | N/A | ALL RBs |  | Y |  | Mid/CC2 |  | N/A |  | ALL RBs |  | Y | Mid/C C3 |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  |  |  | Highest NRB |  |
| 2 |  | X |  | Mid/CC1 |  | REFSENS |  | ALL RBs |  | X |  | Mid/CC2 |  | N/A |  | ALL RBs |  | Y | Mid/CC1 |  |  | N/A | ALL RBs |  | Y |  | Mid/CC2 |  | N/A |  | ALL RBs |  | Y | Mid/C C3 |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  |  |  | Highest NRB |  |
| 3 |  | Y |  | Mid/CC1 |  | REFSENS |  | ALL RBs |  | Y |  | Mid/CC2 |  | N/A |  | ALL RBs |  | Y | Mid/CC3 |  |  | N/A | ALL RBs |  | X |  | Mid/CC1 |  | N/A |  | ALL RBs |  | X | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  |  |  | Highest NRB |  |
| 4 |  | Y |  | Mid/CC1 |  | REFSENS |  | ALL RBs |  | Y |  | Mid/CC2 |  | N/A |  | ALL RBs |  | Y | Mid/CC3 |  |  | N/A | ALL RBs |  | X |  | Mid/CC1 |  | N/A |  | ALL RBs |  | X | Mid/CC2 |  | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Highest NRB |  | N/A | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A | QPSK |  |  |  | Lowest NRB |  |
| 5 |  | Y |  | Mid |  | REF SENS |  | ALL RBs |  | X |  | Mid/ CC1 |  | N/A |  | ALL RBs |  | X |  |  | Mid/ CC2 | N/A |  |  | ALL RBs |  | Y |  | Mid/ CC1 |  | N/A |  | ALL RBs |  |  | Y | Mid/ CC2 | N/A |  | ALL RBs |  |
|  |  | QPSK |  | QPSK |  |  |  | Highest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  | N/A |  |  | QPSK |  |  |  | Lowest NRB |  | N/A |  | QPSK |  |  |  | Lowest NRB |  |  | N/A | QPSK |  |  | Lowest NRB |  |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: Intra-band contiguous & Intra-band contiguous + Inter-band: Use CA Configuration – specific test points if present in the table, otherwise use Default Test Settings test points. Note 3: Inter-band: Use CA Configuration – specific test points if present in the table, Otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 4: If, according to the UE declared capability, UE does not support UL in an individual band within the CA Configuration, test points with that individual band as PCC are not applicable. Note 5: Intra-band contiguous: X corresponds to the band of the CA Configuration. E.g. for CA_41F, X=41. Note 6: Inter-band: X,Y,Z,R,S correspond to the different bands in the CA Configuration. E.g. for CA_1A-3A-7A-20A-42A,X=1,Y=3,Z=7,R=20,S=42. Note 7: Intra-band contiguous + Inter-band: X,Y,Z,R correspond to the different bands in the CA Configuration. E.g. for CA_3C-7A-28A, X=3,Y=7 and Z=28. Note 8: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2. Note 9: Intra-band contiguous: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose for testing the Combination with maximum NRB_PCC and then select maximum NRB_SCC1 for the chosen NRB_PCC. Note 10: Intra-band contiguous + Inter-band: If in the CA Configuration UE supports multiple CC Combinations with the same NRB_agg, choose the Combination with NRB_PCC = NRB_SCC1 for testing. If no such combination is supported, choose Combination with maximum NRB_PCC for testing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< End of Change-3 >