Title: 	Discussion on test point selection for AMPR, ASEM and ASE for NR V2X in FR1

Source: 	Huawei

Introduction

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NR V2X Additional Maximum Power Reduction (AMPR), Additional Spectrum Emission mask (ASEM) and Additional Spurious Emission (ASE) tests for NS_33 in FR1.

Discussion

Minimum requirement

The A-MPR requirements are shown in Table 2.1-1 and Table 2.1-2 for PSSCH/PSCCH, Table 2.1-3 for PSFCH and Table 2.1-4 for S-SSB.

Table 2.1-1: A-MPR for PSSCH/PSCCH by NS_33 (at Fc =5860MHz)

| Carrier frequency [MHz] | Resources Blocks (LCRB) | Start Resource Block | A-MPRBase (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | QPSK/16QAM | 64QAM | 256QAM |
| 5860 | ≥ 10 and ≤ 15 | 0 | ≤ 24 |  |  |
|  |  | ≥ 1 and ≤ 3 | ≤19 |  |  |
|  | ≥ 10 and ≤ 15 | ≥ 26 and ≤ 38 | ≤6 |  |  |
|  | ≥ 10 and ≤ 15 | ≥38 | ≤ 6 |  |  |
|  | ≥ 10 and ≤ 20 | ≥ 12 and ≤ 14 | ≤11 |  |  |
|  |  | ≥ 15 and ≤ 19 | ≤9.5 |  |  |
|  |  | ≥ 20 and ≤ 25 | ≤8.0 |  |  |
|  | > 15 and < 25 | ≥ 25 | ≤ 8 |  |  |
|  | ≥ 10 and < 40 | ≥ 4 and ≤7 | ≤ 16 |  |  |
|  |  | ≥ 8 and ≤ 11 | ≤ 13.5 |  |  |
|  | ≥ 20 and < 40 | ≥ 0 and ≤ 3 | ≤ 22 |  |  |
|  | ≥ 25 and < 40 | ≥ 16 and ≤ 21 | ≤ 9.5 |  |  |
|  |  | ≥ 22 and ≤ 27 | ≤ 8.0 |  |  |
|  | ≥ 24 and ≤ 40 | ≥ 12 and ≤ 15 | ≤ 12 |  |  |
|  | 40 and 45 | 0 and 1 | ≤ 19 |  |  |
|  |  | ≥ 2 and ≤ 5 | ≤ 16 |  |  |
|  |  | ≥ 6 and ≤ 11 | ≤ 13.5 |  |  |
|  | >45 | ≥ 0 | ≤ 16 |  |  |
| NOTE 1: A-MPRstep =1.2 dB is applied for RBstart 0 and 1 and A-MPRstep =0.7 dB is applied for all other RBstart NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |

Table 2.1-2: A-MPR for PSSCH/PSCCH by NS_33 (at other carrier frequency)

| Carrier frequency [MHz] | RB allocations | A-MPRBase (dB) |  |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | QPSK | 16QAM | 64QAM | 256QAM |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | Inner | ≤ 3.0 |  | ≤ 5.0 | ≤ 6.0 | 0.5 |
|  | Outer | ≤ 4.5 |  |  |  |  |
| NOTE 1: Inner and Outer RB allocations are defined in clause 6.2E.2.2 NOTE 2: Applicable for Channel Bandwidth = 10 MHz |  |  |  |  |  |  |

Table 2.1-3: A-MPR for simultaneous PSFCH by NS_33

| Channel Bandwidth [MHz] | Center Frequency [MHz] | RB allocation | A-MPRBase (dB) |  |  | A-MPRstep (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 ≤ NGap / NRB < 0.15 | 0.15≤ NGap / NRB < 0.3 | 0.3≤ NGap / NRB ≤ 1 |  |
| 10 | 5860 | NRB =1 | 19.0 |  |  | 1.0 |
|  |  | NRB > 1 | 22.0 |  |  |  |
|  | 5870, 5880, 5890, 5900, 5910, 5920 | NRB =1 | 5 |  |  | 0.8 |
|  |  | NRB > 1 | 14 | 7 | 18.5 |  |
| Note 1: NGap is the gap RB amount between RBstart and RBend for contiguous and non-contiguous allocation simultaneous PSFCH transmission. (NGap = RBend - RBstart) |  |  |  |  |  |  |

Table 2.1-4: A-MPR for S-SSB transmission by NS_33

| Carrier Frequency (MHz) | RBStart * 12*SCS [MHz] | A-MPRBase (dB) | AMPRStep (dB) |
| --- | --- | --- | --- |
| 5860 | ≤1.0 | ≤ 25 | 0.6 |
|  | >1.0 and ≤2.0 | ≤ 19 |  |
|  | >2.0 and ≤3.24 | ≤ 12 |  |
|  | >3.24 and ≤3.6 | ≤ 10 |  |
|  | >3.6 | ≤ 9 |  |
| 5870, 5880, 5890, 5900, 5910, 5920 | ≤1.0 | ≤ 7.0 | 0.85 |
|  | >1.0 and ≤1.6 | ≤ 6.5 |  |
|  | >1.6 and ≤2.6 | ≤ 5.8 |  |
|  | >2.6 and ≤3.24 | ≤ 4.5 |  |
|  | >3.24 and ≤4.32 | ≤ 5.5 |  |
|  | >4.32 | ≤ 6.5 |  |

The additional spectrum mask requirement in Table 2.1-5 for NS_33 applies for NR V2X UE within 5 855 MHz to 5 950 MHz.

Table 2.1-5: Additional spectrum mask requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

The additional spurious emission requirements for NS_33 are in Table 2.1-6.

Table 2.1-6: Additional requirements for "NS_33"

| Protected band |  | Frequency range (MHz) |  |  | Maximum Level (EIRP2) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 5925 |  | - | 5950 | -30 | 1 | 1 |
| Frequency range | 5815 |  | - | 5855 | -30 | 1 | 3 |
| NOTE 1: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 2: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I in [11]. NOTE 3: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set larger than (symbol length)*(number of points in sweep) to improve the measurement accuracy. |  |  |  |  |  |  |  |

Test Environment

The test environment for basic NR FR1 test cases is selected as Normal for AMPR, A-SEM and A-SE. The same environment selection can be adopted for V2X test cases.

Proposal 1: Define test environment for V2X AMPR, A-SEM and A-SE as Normal.

Test Subcarrier Spacing

Test subcarrier spacing is selected as Lowest and Highest for AMPR, A-SEM and A-SE for basic test cases in FR1. The same approach can be followed for NR V2X test cases.

Proposal 2: Select Lowest and Highest SCS supported for UE for AMPR, A-SEM and A-SE for NR V2X in FR1.

  - Test Frequencies

The AMPR requirements are different between frequency sets {5860MHz} and {5870, 5880, 5890, 5900, 5910, 5920} MHz. Test frequency needs to cover both frequency sets. Considering the ASEM and ASE requirements apply below and above the frequency range of band n47, both Low and High range need to be tested, i.e. 5860MHz and 5920MHz.

Proposal 3: Test frequencies are selected as fc=5860MHz and fc=5920MHz for AMPR, A-SEM and A-SE for NR V2X in FR1.

Test Channel Bandwidth

The AMPR requirements are specified only for 10MHz. This needs to be selected as test channel bandwidth.

Proposal 4: Test 10MHz channel bandwidth for AMPR, A-SEM and A-SE for NR V2X in FR1.

Modulations and RB allocation

AMPR requirements are specified for PSSCH/PSCCH, PSFCH and S-SSB respectively. The modulation and RB allocation shall also be considered for each physical channel.

For PSSCH/PSCCH with fc=5860MHz, the AMPR requirements are defined for different combinations of RB number and RB start. The test points need to cover all the possible AMPR values. To balance the test coverage and test load, the RB allocation could be selected as the maximum RB number with lowest RB start position as long as proper sub-channel could be configured. The AMPR values are the same for each applicable modulation type, 64QAM could be selected as 256QAM is optional for V2X.

For PSSCH/PSCCH with fc other than 5860MHz, the AMPR requirements are defined for Inner and Outer RB allocations, and applicable modulation types. For each AMPR value, the maximum RB allocation and highest modulation type could be selected.

Table 2.6-1 RB allocation for PSSCH/PSCCH

| Test frequency (MHz) | RB allocation | Modulation type |
| --- | --- | --- |
| 5860 | 15@0 | 64QAM |
|  | 15@1 | 64QAM |
|  | 15@30 | 64QAM |
|  | 15@45 | 64QAM |
|  | 20@12 | 64QAM |
|  | 15@15 | 64QAM |
|  | 20@20 | 64QAM |
|  | 24@25 | 64QAM |
|  | 36@4 | 64QAM |
|  | 36@8 | 64QAM |
|  | 36@0 | 64QAM |
|  | 36@16 | 64QAM |
|  | 36@22 | 64QAM |
|  | 36@12 | 64QAM |
|  | 45@0 | 64QAM |
|  | 45@2 | 64QAM |
|  | 45@6 | 64QAM |
|  | 50@0 | 64QAM |
| 5920 | Inner | 16QAM |
|  | Inner | 64QAM |
|  | Inner | 256QAM |
|  | Outer | 16QAM |
|  | Outer | 64QAM |
|  | Outer | 256QAM |

For PSFCH, UE transmits pseudo-random sequence as specified in 8.3.4.2 in TS 38.211. AMPR requirements are defined for single RB, and simultaneous PSFCH transmission with different Ngap values.

For single RB transmission, it’s proposed to allocate the RB at the edge of channel bandwidth. In order to reduce the testing workload, Edge_1RB_left is tested only on Low range frequency and Edge_1RB_Right is tested only on High range frequency.

For simultaneous PSFCH transmission, it’s proposed to test 2RBs and the maximum Ngap for each AMPR value

Table 2.6-2 RB allocation for PSFCH

| Test frequency (MHz) | RB allocation |  |
| --- | --- | --- |
| 5860 | Edge_1RB_left |  |
|  | 1@0 | 1@RBmax |
| 5920 | Edge_1RB_Right |  |
|  | 1@0 | 1@7 |
|  | 1@0 | 1@15 |
|  | 1@0 | 1@RBmax |

For S-SSB the frequency range is fixed as 11 RBs. The AMPR requirements are defined at different frequency locations. For each AMPR value, the tested RB allocation could be selected as lowest possible for Low frequency range and highest possible for High frequency range.

Table 2.6-3 RB allocation for S-SSB

| Test frequency (MHz) | RB allocation |
| --- | --- |
| 5860 | 11@0 |
|  | 11@6 |
|  | 11@12 |
|  | 11@19 |
|  | 11@21 |
| 5920 | 11@5 |
|  | 11@8 |
|  | 11@14 |
|  | 11@18 |
|  | 11@24 |
|  | 11@42 |

Proposal 5: The combination of modulation and RB allocation is selected as Table 2.6-1, 2.6-2 and 2.6-3 for PSSCH/PSCCH, PSFCH and S-SSB.

Number of Test Points

Based on above proposal, the total test points are tabulated below:

Table 2.7-1 Total test points of MPR, SEM and ACLR

| Test case | Environment conditions | Test SCS | Test Frequencies | Test ChBW | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- | --- |
| AMPR ASEM ASE | 1 | 2 | 2 | 1 | PSSCH/PSCCH: 24 for 2 test frequencies PSFCH: 6 for 2 test frequencies S-SSB: 11 for 2 test frequencies | 82 |

Exceptions for SL MIMO test

Extracted from Clause 6.2E.3.1 [2]

For UE with two transmit antenna connectors, the A-MPR values specified in clause 6.2E.3.2 and 6.2E.3.3 shall apply to the maximum output power specified in Table 6.2E.1.1-1. The requirements shall be met with the SL MIMO configurations specified in Table 6.2D.1-2. For UE supporting SL MIMO, the maximum output power is defined as the sum of the maximum output power from each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Extracted from Clause 6.5E.2.1 [2]

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier shall apply to each transmit antenna connector. The requirements shall be met with SL MIMO configurations described in clause 6.2D.1.

Observation 1: Same AMPR and out of band emission requirements as specified for single transmission apply for UEs supporting SL-MIMO.

SL-MIMO only applies to PSSCH, therefore PSBCH and PSFCH could be excluded.

Proposal 6: Adopt the same PSSCH test configuration of single carrier test case for SL-MIMO measurement in FR1.

Exceptions for con-current test.

Requirements of AMPR for con-current operation are extracted from Clause 6.2E.3.4 [2]

For the inter-band con-current NR V2X operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in clause 6.2.3 apply for NR Uu operation in licensed band, and the A-MPR requirements in clause 6.2E.3.2 and 6.2E.3.3 apply for NR sidelink operation in Band n47.

Observation 1: No exceptional requirements are specified for inter-band concurrent NR V2X operation. The NR Uu requirements and V2X requirements apply respectively.

Proposal 7: The A-MPR requirement for con-current operation is covered by NR test and V2X non-concurrent test respectively. No need for double testing.

Conclusion

It is proposed that RAN5 agrees below proposals on test points of MPR, SEM and ACLR for NR V2X:

Proposal 1: Define test environment for V2X AMPR, A-SEM and A-SE as Normal.

Proposal 2: Select Lowest and Highest SCS supported for UE for AMPR, A-SEM and A-SE for NR V2X in FR1.

Proposal 3: Test frequencies are selected as fc=5860MHz and fc=5920MHz for AMPR, A-SEM and A-SE for NR V2X in FR1.

Proposal 4: Test 10MHz channel bandwidth for AMPR, A-SEM and A-SE for NR V2X in FR1.

Proposal 5: The combination of modulation and RB allocation is selected as Table 3-1, 3-2 and 3-3 for PSSCH/PSCCH, PSFCH and S-SSB

Table 3-1 Combinations of modulation and RB allocation for PSSCH/PSCCH

| Test frequency (MHz) | RB allocation | Modulation type |
| --- | --- | --- |
| 5860 | 15@0 | 64QAM |
|  | 15@1 | 64QAM |
|  | 15@30 | 64QAM |
|  | 15@45 | 64QAM |
|  | 20@12 | 64QAM |
|  | 15@15 | 64QAM |
|  | 20@20 | 64QAM |
|  | 24@25 | 64QAM |
|  | 36@4 | 64QAM |
|  | 36@8 | 64QAM |
|  | 36@0 | 64QAM |
|  | 36@16 | 64QAM |
|  | 36@22 | 64QAM |
|  | 36@12 | 64QAM |
|  | 45@0 | 64QAM |
|  | 45@2 | 64QAM |
|  | 45@6 | 64QAM |
|  | 50@0 | 64QAM |
| 5920 | Inner | 16QAM |
|  | Inner | 64QAM |
|  | Inner | 256QAM |
|  | Outer | 16QAM |
|  | Outer | 64QAM |
|  | Outer | 256QAM |

Table 3-2 RB allocation for PSFCH

| Test frequency (MHz) | RB allocation |  |
| --- | --- | --- |
| 5860 | Edge_1RB_left |  |
|  | 1@0 | 1@RBmax |
| 5920 | Edge_1RB_Right |  |
|  | 1@0 | 1@7 |
|  | 1@0 | 1@15 |
|  | 1@0 | 1@RBmax |

Table 3-3 RB allocation for S-SSB

| Test frequency (MHz) | RB allocation |
| --- | --- |
| 5860 | 11@0 |
|  | 11@6 |
|  | 11@12 |
|  | 11@19 |
|  | 11@21 |
| 5920 | 11@5 |
|  | 11@8 |
|  | 11@14 |
|  | 11@18 |
|  | 11@24 |
|  | 11@42 |

Proposal 6: Adopt the same PSSCH test configuration of single carrier test case for SL-MIMO measurement in FR1.

Proposal 7: The A-MPR requirement for con-current operation is covered by NR test and V2X non-concurrent test respectively. No need for double testing.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v16.6.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[3] 3GPP TS 38.521-1 v16.7.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

[4] Discussion on test point selection for MPR, NR ACLR and SEM in FR1 in TR 38.905