Title:	Discussion on test point selection for NS_47 A-MPR up to PC 1.5 in FR1

Document for:	Discussion and approval

Background

The purpose of this paper is to provide an analysis of the parameters for the A-MPR test configuration table with a selection of test points that minimize testing and still achieve good test coverage for NS_47. The intention with NS_47 is to protect services in adjacent services below 2545 MHz in band n41 (2496-2690 MHz). See Table 1.

Table 1: Additional requirements for NR channels assigned within 2545 - 2575 MHz for "NS_47"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 30 MHz |  |
| 2530 ≤ f ≤ 2535 | -25 | 1 MHz |
| 2505 ≤ f ≤ 2530 | -30 | 1 MHz |

Discussion

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for A-MPR FR1 test point selection.

The selection of test points below is based on approved test point analyzes for SEM [2, 3] for FR1. The A-MPR requirements are shown in Table 2 and Table 3.

Table 2: A-MPR regions and types for NS_47 (from [4])

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |

Table 3: A-MPR for modulation and waveform type (from [4])

| Modulation/Waveform | A1(dB) |  | A2(dB) |  | A3(dB) |  | A4(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 |  | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 |  | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  |  |  | ≤ 7 |

Note that regions A3 and A4 do not have A-MPR for all modulations.

For PC 1.5 operation, the following tables are defined in [6]:

Table 3A: A-MPR regions and types for NS_47 (Power Class 1.5)

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | >6.12, ≤7.92 | >10, ≤14.4 | A5 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.18-4. |  |  |  |  |

Table 3B: A-MPR for NS_47 (Power Class 1.5)

| Modulation/Waveform |  | A1(dB) | A2(dB) | A3(dB) | A4(dB) | A5(dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | QPSK | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | 16 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | 64 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 |  |
|  | 256 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 |  |
| CP-OFDM | QPSK | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 | ≤ 4 |
|  | 16 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 | ≤ 4 |
|  | 64 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 |  |
|  | 256 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 |  |
| NOTE 1: PC1.5 assumes dual Tx. |  |  |  |  |  |  |

Note that a new region A5 is defined while A1-A4 remain as per PC3 and PC2 cases.

Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR NS_47.

Test Frequencies

UL test carrier centre frequencies (Fc) are defied in Table 2 for NS_47 A-MPR.

PROPOSAL 2: Select 2560 MHz as test frequency for NS_47.

Test Channel Bandwidth

Only 30 MHz channel bandwidth is specified for NS_47.

PROPOSAL 3: Select 30 MHz channel bandwidth.

  - Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. Lowest and Highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_47.

  - UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms).

PROPOSAL 5: Test all UL modulation waveforms that have A-MPR requirements and are supported by the UE.

  - Uplink RB allocation

The proposed RB allocation and test frequency for NS_47 is shown in Table 4.

Table 4: UL RB allocations for NS_47

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart | LCRB | Proposed RB allocation |  | A-MPR |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DFT-s (SCS 15/30/60 kHz) | CP (SCS 15/30/60 kHz) |  |
| 30 MHz | Fc=2560 | 15 kHz: RBstart ≤ 28 30 kHz: RBstart ≤ 14 60 kHz: RBstart ≤ 7 | LCRB ≤ 8 LCRB ≤ 4 LCRB ≤ 2 | Edge_1RB_Left | Edge_1RB_Left | A1 |
|  |  | 15 kHz: 28 < RBstart ≤ 53.3 30 kHz: 14 < RBstart ≤ 26.6 60 kHz: 7 < RBstart ≤ 13.3 | LCRB ≤ 8 LCRB ≤ 4 LCRB ≤ 2 | 1@29 1@15 1@8 | 1@29 1@15 1@8 | A2 |
|  |  | 15 kHz: RBstart > 136 30 kHz: RBstart > 68 60 kHz: RBstart > 34 | LCRB ≤ 8 LCRB ≤ 4 LCRB ≤ 2 | Edge_1RB_Right | Edge_1RB_Right | A3 |
|  |  | 15 kHz: RBstart ≤ 53.3 30 kHz: RBstart ≤ 26.6 60 kHz: RBstart ≤ 13.3 | LCRB > 116.6 LCRB > 58.3 LCRB > 29.2 | Outer_Full | Outer_Full | A2 |
|  |  |  | 80 < LCRB < 116.7 40 < LCRB < 58.3 20 < LCRB < 29.1 | 108@0 54@0 27@0 | 117@0 58@0 29@0 | A4 |
|  |  | 15 kHz: RBstart ≤ 34 30 kHz: RBstart ≤ 17 60 kHz: RBstart ≤ 8.5 | 55.5 < LCRB ≤ 80 27.7 < LCRB ≤ 40 13.8 < LCRB ≤ 20 | 80@0 40@0 20@0 | 80@0 40@0 20@0 | A4 |
|  |  |  | 8 < LCRB < 55.5 4 < LCRB < 27.2 2 < LCRB < 13.8 | 54@0 27@0 12@0 | 55@0 27@0 13@0 | A2 |
|  |  | 15 kHz: 34 < RBstart ≤ 44 30 kHz: 17 < RBstart ≤ 22 60 kHz: 8.5 < RBstart ≤ 11 | 55.5 < LCRB ≤ 80 27.7 < LCRB ≤ 40 13.8 < LCRB ≤ 20 | 80@35 40@18 20@9 | 80@35 40@18 20@9 | A5 |

The UL RB allocation can be based on the agreed proposals for SEM [2], i.e. Edge_1RB_left, Edge_1RB_Right and Outer_Full defined in [5] except A5 dedicated for PC 1.5 case

The proposals for LCRB@RBstart in Table 4 when LCRB belong to an interval are based on allocating as many RBs as possible close to the lower frequency edge. For the case where RBstart belongs to an interval the proposal above is based on one RB located as close as possible to the lower edge of the band as it can be considered a worst-case scenario for spurious emissions towards adjacent services.

PROPOSAL 6: Select RB allocation according to table 4.

2.7.	Number of Test Points

The maximum number of test points for A-MPR tests for NS_47:

| Power Class | Environment conditions | Maximum Number of Steps (Freq. X Ch BW X RB alloc. X SCS) | Maximum Number of Modulations | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- |
| PC2, PC3 | 1 (NC) | 7 | 9 | 70 |
| PC1.5 | 1 (NC) | 8 | 9 | 72 |

Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_47 A-MPR test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

PROPOSAL 2: Select 2560 MHz as test frequency for NS_47.

PROPOSAL 3: Select 30 MHz channel bandwidth.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_47.

PROPOSAL 5: Test all UL modulation waveforms that have A-MPR requirements and are supported by the UE.

PROPOSAL 6: Select RB allocation according to table 4.

References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- 3GPP TS 38.101-1, V16.3.0 (2020-03), Table 6.2.3.6-1

- 3GPP TS 38.521-1, V16.3.1 (2020-03), Table 6.1-1

- 3GPP TS 38.101-1, V16.18.0 (2024-01)