3GPP TSG-RAN5 Meeting #82	R5-192025

# Athens, Greece, 25th Feb 2019 - 1st Mar 2019

Agenda item:	5.3.8.16

Source: 	China Unicom

Title: 	Discussion on test point selection for Time alignment error for UL-MIMO in FR1

Document for:	Discussion and endorsement

Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Time alignment error for UL-MIMO test case in FR1.

Discussion

Time alignment error for UL-MIMO in FR1 is defined in Transmitter power section in [3]. We use the test point choice from the relevant LTE time alignment error for UL-MIMO test case from TS 36.521-1 clause 6.8B.4.1 and NR single antenna test case from TS 38.521-1 clause 6.3.1 can be used as inspiration: as inspiration:

Table 6.8B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] clause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS36.508 [7] clause 4.3.1 |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] clause 4.3.1 |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD | TDD |
| 1.4MHz |  |  | QPSK | 6 | 6 |
| 3MHz |  |  | QPSK | 15 | 15 |
| 5MHz |  |  | QPSK | 25 | 25 |
| 10MHz |  |  | QPSK | 50 | 50 |
| 15MHz |  |  | QPSK | 75 | 75 |
| 20MHz |  |  | QPSK | 100 | 100 |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. |  |  |  |  |  |

Table 6.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for minimum output power | Modulation | RB allocation (NOTE 1) |
| 1 | test case | CP-OFDM QPSK | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

Following sub clauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

Test Environment

Based on the baseline agreed in [1], test environment for time alignment error for UL-MIMO for FR1 can be defined as in LTE.

Proposal 1: Define Test Environment as ‘Normal’ for time alignment error for UL-MIMO measurement in FR1.

Test Subcarrier Spacing

It is proposed to test both the Lowest and Highest SCS.

Proposal 2: Select Lowest and Highest SCS supported for UE for Time alignment error for UL-MIMO in FR1.

  - Test Frequencies

In LTE, Time alignment error for UL-MIMO is tested for Mid frequency. For NR it is proposed to adopt the same selection of test frequencies.

Proposal 3: Test Mid Frequency for Time alignment error for UL-MIMO in FR1.

Test Channel Bandwidth

In LTE, this test case is tested for Lowest, 5 MHz, and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE transmitter related to different frequency selectivities with different channel bandwidths. For NR it is proposed to adopt the same selection of test channel bandwidth.

Proposal 4: Test the Lowest, Mid and Highest Channel Bandwidth for Time alignment error for UL-MIMO in FR1.

Modulations

In LTE, only QPSK is tested, which is generating a stable power for testing. For NR it is proposed to also test QPSK for Time alignment error for UL-MIMO measurement.

Since DFT-s-OFDM is disable for UL-MIMO case. It is proposed to test CP-OFDM for Time alignment error for UL-MIMO measurement in FR1

Proposal 5: Test CP-OFDM QPSK in Time alignment error for UL-MIMO.

Uplink Configuration

In LTE, Time alignment error for UL-MIMO is tested with maximum uplink allocation, which will generate the lowest power spectrum density. For NR the same configuration can be adopted.

Proposal 6: Use Full RB allocation in the Time alignment error for UL-MIMO in FR1.

Handling of TAE for intra-band CA with UL MIMO

RAN4 specified the requirements of time alignment error for intra-band contiguous CA with UL MIMO in section 6.4H.1.3 of TS 38.101-1. The requirement is same as the TAE requirement on single carrier for each component carrier. The test case could be executed with RB allocation on one carrier, then the test points of 6.4D.3 could be reused.

Proposal 7: For 6.4H.1.3 TAE for intra-band contiguous CA, reuse the test points of 6.4D.3 for each component carrier.

Number of Test Points

Based on above proposal, the total test points are tabulated below:

Table 2.6-1 Total test points of Maximum output power

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | 2 | 1 | 6 |

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Time alignment error for UL-MIMO test case definition:

Proposal 1: Define Test Environment as ‘Normal’ for time alignment error for UL-MIMO measurement in FR1.

Proposal 2: Select Lowest and Highest SCS supported for UE for Time alignment error for UL-MIMO in FR1.

Proposal 3: Test Mid Frequency for Time alignment error for UL-MIMO in FR1.

Proposal 4: Test the Lowest, Mid and Highest Channel Bandwidth for Time alignment error for UL-MIMO in FR1.

Proposal 5: Test CP-OFDM QPSK in Time alignment error for UL-MIMO.

Proposal 6: Use Full RB allocation in the Time alignment error for UL-MIMO in FR1.

Proposal 7: For 6.4H.1.3 TAE for intra-band contiguous CA, reuse the test points of 6.4D.3 for each component carrier.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] 3GPP TS 38.521-1 v15.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)