Title: 	Test point analysis for Time mask for switching between two uplink carriers or two bands

Source: 	Huawei

- Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Time mask for switching between two uplink carriers test case in FR1.

- Discussion

The minimum requirement of Time mask for switching between two uplink carriers is extracted as below:

In addition to the requirements in 6.3A.3.3.1 and the maximum output power requirement specified in Table 6.2A.1.3-1 with uplink assigned to two NR bands, the switching time mask specified in this sub-clause is applicable for an uplink band pair of a inter-band UL CA configuration when the capability uplinkTxSwitchingPeriod is present, and is only applicable for uplink switching mechanisms specified in sub-clause 6.1.0 of TS 38.214 [10], where NR UL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors with 3dB boosting on the maximum output power when the capability uplinkTxSwitchingPowerBoosting is present and the IE powerboostingTxSwitching is set to 1, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3A.3.3.2-1a and Figure 6.3A.3.3.2-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3A.3.3.2-1a: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3A.3.3.2-1b: Time mask for switching between UL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in sub-clause 4.2 of TS 38.213 [8].

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

## 2.1	Test Environment

In NR test case General ON/OFF Time Mask measurement for CA (2UL CA) in FR1, test environment are defined as Normal, TL/VL, TL/VH, TH/VL, TH/VH. However this test case is only applicable to UEs which support uplink carrier switching but not to all UEs. So only testing under normal condition is enough to verifying the UE capability and meanwhile avoid costing too much time. And absolute power control is used in test procedure. The requirements on power control accuracy apply under normal conditions. So in Time Mask for inter-band CA uplink carrier switching testing, only Normal condition is tested.

Proposal 1: Define Test Environment as Normal for Time mask for switching between two uplink carriers testing

## 2.2	Test Frequencies

In NR, General ON/OFF Time Mask for 2UL inter-band CA is tested for Low and High range frequencies based on the same principle in LTE. Considering the minimum requirement of Time mask for switching between two uplink carriers is independent with test frequency range, to reduce the cost of testing time, selecting Mid range will be enough.

Proposal 2: Selecting Mid range for Time mask for switching between two uplink carriers testing

## 2.3	Test Channel Bandwidths

In NR, General ON/OFF Time Mask for 2UL inter-band CA is tested for Lowest and Highest channel bandwidth based on the same selection in LTE. Since generally we consider highest channel bandwidth to be the worst case because of possible frequency selectivity and BW limitation issues, testing Highest channel bandwidth will be enough to reduce testing time without losing test coverage.

Proposal 3: Test the highest NRB_agg for Time mask for switching between two uplink carriers testing

## 2.4	Test Subcarrier Spacing

The output power is measured in a duration of at least one slot. The slot length varies with different SCS. The highest SCS comes with the shortest slot length Testing highest SCS makes ON power measurement smaller and potential transients more visible. So highest SCS should be selected.

Proposal 4: Select Highest SCS supported for UE for Time mask for switching between two uplink carriers testing

## 2.5	Uplink Modulation and RB allocation

In General ON/OFF Time Mask for 2UL inter-band CA testing, since 2-layer UL-MIMO would be scheduled on the carrier supporting 2Tx, CP-OFDM is chosen, with  QPSK + Inner Full RB allocation to provide minimum MPR. In Time mask for switching between two uplink carriers testing, the same approach should be selected.

Proposal 5: Use CP-OFDM QPSK + Inner Full RB allocation for Time mask for switching between two uplink carriers testing

  - Void

## 2.7	Number of Test Points

Based on above proposals, the total test points of Time mask for switching between two uplink carriers testing are tabulated below.

Table 2.6-1 Total test points of Time mask for switching between two uplink carriers testing

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

## 2.8	Handling of test point of switching between two uplink carriers with two transmit antenna connectors

The minimum requirement of 2Tx-2Tx UL switching is specified in 6.3A.3.3.3 of TS 38.101, where the main difference from 1Tx-2Tx switching is both carriers are capable of two transmit antenna connectors. The ON/OFF time mask period and location is the same as 1Tx-2Tx UL switching. The TP analysis of 1Tx-2Tx UL switching could be reused.

Proposal 7: The test points of 2Tx-2Tx UL switching is the same as that of 1Tx-2Tx UL switching.

## 2.9	Handling of test point of 1Tx-2Tx and 2Tx-2Tx switching between two bands

The minimum requirements of 1Tx-2Tx and 2Tx-2Tx UL switching between two bands are specified in 6.3A.3.3.4 and 6.3A.3.3.5 of TS 38.101, where the main difference from switching between two uplink carriers is one of the bands is configured with intra-band contiguous CA. The ON/OFF time mask period and location is the same as 1Tx-2Tx UL switching. The TP analysis of UL switching between two uplink carriers could be reused except the RB allocation should be replaced by inner_full allocation for intra-band contiguous CA.

Proposal 8: The test points of UL switching between two bands are the same as that of UL switching between two uplink carriers, except the RB allocation is replaced by inner_full allocation for intra-band contiguous CA.

- Conclusion

Proposal 1: Define Test Environment as Normal for Time mask for switching between two uplink carriers testing

Proposal 2: Selecting Mid range for Time mask for switching between two uplink carriers testing

Proposal 3: Test the highest NRB_agg for Time mask for switching between two uplink carriers testing

Proposal 4: Select Lowest and Highest SCS supported for UE for Time mask for switching between two uplink carriers testing

Proposal 5: Use CP-OFDM QPSK + Inner Full RB allocation for Time mask for switching between two uplink carriers testing

Proposal 6: Void.

Proposal 7: The test points of 2Tx-2Tx UL switching is the same as that of 1Tx-2Tx UL switching.

Proposal 8: The test points of UL switching between two bands are the same as that of UL switching between two uplink carriers, except the RB allocation is replaced by inner_full allocation for intra-band contiguous CA.

- References

[1] 3GPP TS 38.521-1 V16.6.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 16)

[2] 	3GPP TS 38.101-1 v16.6.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[3]	R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[4] 	R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019