Title: 	Test point analysis for General Time mask for switching between two uplink carriers for SUL

Source: 	Huawei

- Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for General Time mask for switching between two uplink carriers for SUL.

- Discussion

The minimum requirement of Time mask for switching between two uplink carriers is extracted as below:

The switching time mask specified in this clause is applicable for an uplink band pair of a SUL configuration when the capability uplinkTxSwitchingPeriod is present, is only applicable for uplink switching mechanisms specified in clause 6.16 of TS 38.214 [10], where NR SUL carrier 1 is capable of one transmit antenna connector and NR UL carrier 2 is capable of two transmit antenna connectors, and the two uplink carriers are in different bands with different carrier frequencies. The UE shall support the switch between single layer transmission with one antenna port and two-layer transmission with two antenna ports on the two uplink carriers following the scheduling commands and rank adaptation, i.e., both single layer and two-layer transmission with 2 antenna ports, and single layer transmission with 1 antenna port shall be supported on NR UL carrier 2.

The switching periods described in Figure 6.3C.3.1-1a and Figure 6.3C.3.1-1b are located in either NR carrier 1 or carrier 2 as indicated in RRC signalling uplinkTxSwitchingPeriodLocation [7], and the length of uplink switching period X is less than the value indicated by UE capability uplinkTxSwitchingPeriod.

Figure 6.3C.3.1-1a: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 1

Figure 6.3C.3.1-1b: Time mask for switching between SUL carrier 1 and UL Carrier 2, where the switching period is located in carrier 2

The requirements apply for the case of co-located and synchronized network deployment for the two uplink carriers.

The requirements apply for the case of single TAG for the two uplink carriers, i.e., the same uplink timing for the two carriers as described in clause 4.2 of TS 38.213 [8].

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

## 2.1	Test Environment

In NR test case General ON/OFF Time Mask measurement for SUL in FR1, test environment are defined as Normal, TL/VL, TL/VH, TH/VL, TH/VH. However this test case is only applicable to UEs which support uplink carrier switching but not to all UEs. So only testing under normal condition is enough to verifying the UE capability and meanwhile avoid costing too much time. And relative power control is used in test procedure. The requirements on power control accuracy apply under normal conditions. So in Time Mask for SUL two uplink carrier switching testing, only Normal condition is tested.

Proposal 1: Define Test Environment as Normal

## 2.2	Test Frequencies

In NR, General ON/OFF Time Mask for SUL is tested for Low, Mid and High range frequencies for SUL carrier. Considering the minimum requirement of Time mask for switching between two uplink carriers is independent with test frequency range, to reduce the cost of testing time, selecting Mid range will be enough.

Proposal 2: Selecting Mid range for SUL General time mask for switching between two uplink carriers testing

## 2.3	Test Channel Bandwidths

In NR, General ON/OFF Time Mask for SUL is tested for Lowest, Mid and Highest channel bandwidth for SUL carrier based on the same selection in single-carrier test case. And Mid channel bandwidth is selected for NUL carrier.

In this test case, uplink is scheduled on NUL carrier and SUL carrier in different slot. And generally we consider highest channel bandwidth to be the worst case because of possible frequency selectivity and BW limitation issues. So testing Highest channel bandwidth for both NUL and SUL carrier will be enough to reduce testing time without losing test coverage.

Proposal 3: Select highest channel bandwidth for NUL carrier and SUL carrier for SUL General time mask for switching between two uplink carriers testing

## 2.4	Test Subcarrier Spacing

For SUL test case, 15kHz for SUL carrier and lowest supported SCS for NUL carrier should be selected.

Proposal 4: Select 15kHz SCS for SUL carrier and Lowest supported SCS for Non-SUL carrier.

## 2.5	Uplink Modulation and RB allocation

In General ON/OFF Time Mask for SUL testing, DFT-s-OFDM QPSK is chosen. Inner Full RB allocation is adopted for it has the lowest MPR. In SUL General Time mask for switching between two uplink carriers testing, the same approach should be selected.

Proposal 5: Use DFT-s-OFDM QPSK + Inner Full RB allocation for Time mask for switching between two uplink carriers testing

## 2.6	Number of Test Points

Based on above proposals, the total test points of Time mask for switching between two uplink carriers testing are tabulated below.

Table 2.6-1 Total test points of Time mask for switching between two uplink carriers testing

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 |

## 2.7	Handling of test point of switching between two uplink carriers with two transmit antenna connectors

The minimum requirement of 2Tx-2Tx UL switching for SUL configuration is specified in 6.3C.3.2 of TS 38.101, where the main difference from 1Tx-2Tx switching is both carriers are capable of two transmit antenna connectors. The ON/OFF time mask period and location is the same as 1Tx-2Tx UL switching. The TP analysis of 1Tx-2Tx UL switching could be reused.

Proposal 6: The test points of 2Tx-2Tx UL switching is the same as that of 1Tx-2Tx UL switching.

## 2.8	Handling of test point of 1Tx-2Tx and 2Tx-2Tx switching between two bands

The minimum requirements of 1Tx-2Tx and 2Tx-2Tx UL switching for SUL configuration between two bands are specified in 6.3C.3.3 and 6.3C.3.4 of TS 38.101, where the main difference from switching between two uplink carriers is the NUL band is configured with intra-band contiguous CA. The ON/OFF time mask period and location is the same as 1Tx-2Tx UL switching. The TP analysis of UL switching between two uplink carriers could be reused except the RB allocation should be replaced by inner_full allocation for intra-band contiguous CA.

Proposal 7: The test points of UL switching between two bands are the same as that of UL switching between two uplink carriers, except the RB allocation is replaced by inner_full allocation for intra-band contiguous CA.

- Conclusion

Proposal 1: Define Test Environment as Normal

Proposal 2: Selecting Mid range for General time mask for switching between two uplink carriers testing for SUL

Proposal 3: Select highest channel bandwidth for NUL carrier and SUL carrier for General time mask for switching between two uplink carriers testing for SUL

Proposal 4: Select 15kHz SCS for SUL carrier and Lowest supported SCS for Non-SUL carrier

Proposal 5: Use DFT-s-OFDM QPSK + Inner Full RB allocation for Time mask for switching between two uplink carriers testing

Proposal 6: The test points of 2Tx-2Tx UL switching is the same as that of 1Tx-2Tx UL switching.

Proposal 7: The test points of UL switching between two bands are the same as that of UL switching between two uplink carriers, except the RB allocation is replaced by inner_full allocation for intra-band contiguous CA.

- References

[1] 3GPP TS 38.521-1 V17.5.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 16)

[2] 	3GPP TS 38.101-1 v16.12.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[3]	R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.