3GPP TSG-RAN5 Meeting #102	R5-241063

# Athens, Greece, 26th Feb – 1rd Mar 2024

Source:	NTTDOCOMO, INC

Title:	Proposal for new test case related to ATTACH NEEDED STATE

Agenda item:	6.4.7

Document for:	Endorsement

Background

According to current TS24.301[1], there are 8 states of EMM DEREGISTERED. Please refer as follows.

EMM-DEREGISTERED.NORMAL-SERVICE

EMM-DEREGISTERED.LIMITED-SERVICE

EMM-DEREGISTERED.ATTEMPTING-TO-ATTACH

EMM-DEREGISTERED.PLMN-SEARCH

EMM-DEREGISTERED.NO-IMSI

EMM-DEREGISTERED.ATTACH-NEEDED

EMM-DEREGISTERED.NO-CELL-AVAILABLE

EMM-DEREGISTERED.eCALL-INACTIVE

As to EMM-DEREGISTERED.ATTACH-NEEDED, the UE is allowed to make a emergency call.

TS24.301 Rel-18.5

==

          - 5.1.3.2.3.7	EMM-DEREGISTERED.ATTACH-NEEDED

Valid subscriber data are available for the UE and for some reason an attach must be performed as soon as possible. This substate can be entered if the access class is blocked due to access class control, or if the network rejects the NAS signalling connection establishment.

==

##### 5.2.2.3.6	ATTACH-NEEDED

The UE shall initiate the attach or combined attach procedure, if still needed, as soon as the access is allowed in the selected cell for one of the access classes of the UE.

The UE may initiate attach for emergency bearer services.

The UE may initiate attach for access to RLOS.

==

So, the UE can establish the RRC connection with the RRC establishmentCause set to ‘emergency’ and transmits an ATTACH REQUEST message to attach for emergency bearer services as well as “EMM-DEREGISTERED. LIMITED-SERVICE”.

TS24.301 Rel-18.5

==

          - 5.1.3.2.3.3	EMM-DEREGISTERED.LIMITED-SERVICE

The substate EMM-DEREGISTERED.LIMITED-SERVICE is chosen in the UE, when it is known that a selected cell is unable to provide normal service (e.g. the selected cell is in a forbidden PLMN, is in a forbidden tracking area or the selected cell is a CSG cell whose CSG ID and associated PLMN identity are not included in the UE's Allowed CSG list or in the UE's Operator CSG List).

##### 5.2.2.3.2	LIMITED-SERVICE

The UE shall initiate an attach or combined attach procedure when entering a cell which provides normal service.

The UE may initiate attach for emergency bearer services.

The UE may initiate attach for access to RLOS.

==

According to current TS36.523-1[2],  there is no test case which checks whether the UE performs an emargencycall from the substate “EMM-DEREGISTERED.ATTACH-NEEDED” although there is a test case which checks whether the UE performs an emargencycall from the substate “EMM-DEREGISTERED. LIMITED-SERVICE”

TS36.523-1 Rel-18.3

==

### 11.2.2		Emergency bearer services / Normal cell / LIMITED-SERVICE / Attach / PDN connect

==

Proposal

I have a proposal to check emergency call from substate of EMM-DEREGISTERED.ATTACH-NEEDED.

Proposal: RAN5 make a new test case to check above.

This test case is applied to UEs supporting E-UTRA and IMS emergency call.

If this proposal is agreed, following new test case has already been submitted.

11.2.13 Emergency bearer services / Normal cell / ATTACH-SERVICE / ATTACH-NEEDED STATE / PDN connect

# test case:

R5-241065

# Test applicability:

R5-241067

Reference

[1]	3GPP TS 24.301: "3rd Generation Partnership Project;Technical Specification Group Core Network and Terminals;

Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS);Stage 3(Release 18)".

[2]	3GPP TS 36.523-1: "3rd Generation Partnership Project;Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC);User Equipment (UE) conformance specification;Part 1: Protocol conformance specification (Release 18)".

[3]	3GPP TS 36.523-2: "3rd Generation Partnership Project;Technical Specification Group Radio Access Network;Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification; Part 2: Implementation Conformance Statement (ICS) proforma specification (Release 18)".