3GPP TSG-RAN5 Meeting #102	R5-241087

# Athens, Greece, 26th Feb – 1st Mar 2024

Title:							QoQZ up to FR2d

Source:			Anritsu

Agenda Item:			5.4.20

Document for:			Endorsement

# 1.	Introduction

In the past meeting [1], the action point AP#100.21 was created to progress the work on FR2 bands beyond FR2b. This paper provides QoQZ measurement results up to FR2d.

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#100.21 | RF | System vendors to provide QoQZ MU measurements for EIRP/EIS and TRP metric for NTC and ETC (including FR2d) | KEYS, R&S, Anritsu | R5-235754 | RAN5#102 | Open |

# 2.	Discussion

As shown in Table B.2.2.3-1 of TR 38.903 [2], QoQZ is still TBD for  some conditions, e.g., FR2d, QZ = 40 cm and ETC. To define the value of QoQZ for these conditions, we provide the following measurement results of QoQZ.

Table 1: Standard Deviations for EIRP/EIS metric (QZ size = 40 cm, ETC)

| Positions | QoQZ MU [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 23.450 GHz | 32.125 GHz | 40.800 GHz | 44.300 GHz | 49.000 GHz |
| P1-P7 (Stage 2) | 0.87 | 0.85 | 0.53 | 0.73 | 0.68 |
| P1 only (Stage 1) | 0.58 | 0.38 | 0.15 | 0.16 | 0.41 |

Table 2: Standard Deviations for TRP metric (QZ size = 40 cm, ETC)

| Positions | QoQZ MU [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 23.450 GHz | 32.125 GHz | 40.800 GHz | 44.300 GHz | 49.000 GHz |
| P1-P7 (Stage 2) | 0.33 | 0.31 | 0.35 | 0.50 | 0.56 |
| P1 only (Stage 1) | 0.25 | 0.13 | 0.20 | 0.49 | 0.22 |

Considering the above measurement results and commonality with the already endorsed QoQZ values, we propose the follwing QoQZ values for QZ size = 40 cm and ETC.

Table 3: Endorsed/Proposed QoQZ MU for EIRP/EIS and TRP metric for P1-P7 (Stage 2)

| Condition | QoQZ MU [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | FR2a | FR2b | FR2c | FR2d |
| QZ size ≤ 30 cm and NTC | 0.6 | 0.6 | 0.7 | 0.7 |
| QZ size ≤ 30 cm and ETC | 0.9 | 0.9 | 0.9 (EIRP/EIS) 0.9 (TRP) | 0.9 |
| QZ size = 40 cm and NTC | 0.7 | 0.7 | 0.8 | 0.8 |
| QZ size = 40 cm and ETC | 0.9 | 0.9 | 0.9 | 0.9 |
| Note: Black text indicates endorsed values, and green text indicates proposed values in this paper. |  |  |  |  |

Table 4: Endorsed/Proposed QoQZ MU for EIRP/EIS and TRP metric for P1 only (Stage 1)

| Condition | QoQZ MU [dB] |  |  |  |
| --- | --- | --- | --- | --- |
|  | FR2a | FR2b | FR2c | FR2d |
| QZ size ≤ 40 cm and NTC | 0.4 | 0.4 | 0.5 | 0.5 |
| QZ size ≤ 40 cm and ETC | 0.6 | 0.6 | 0.6 (EIRP/EIS) 0.6 (TRP) | 0.6 |
| Note: Black text indicates endorsed values, and green text indicates proposed values in this paper. |  |  |  |  |

Proposal 1: Define QoQZ MU values as shown in Table 3 and Table 4.

# 3.	Conclusion

RAN5 is asked to endorse following proposals.

Proposal 1: Define QoQZ MU values as shown in Table 3 and Table 4.

# 4.	References

[1] R5-235754, “On QoQZ ETC MU beyond FR2b”, Keysight Technologies, RAN5#100

[2] TR 38.903, “Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases”, V18.1.0 (2023-12)