3GPP TSG-RAN5 Meeting #102 	R5-241092

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3084 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CBW selection to avoid unintentional asymmetric CBW |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | High CBWs of n5, n8, n50, n70, n91, n93 are defined as UL and DL separately in Table 4.3.1.0C-1. Such a CBW definition causes asymmetric CBW in Tx test cases which allocates both UL and DL. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added note to avoid asymmetric CBW in test cases where asymmetric is not intended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Asymmetric CBW will be used unintentionally. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.0C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

4.3.1.0C	High test channel bandwidth

The high test channel bandwidth definition for RF is given in Table 4.3.1.0C-1 and Table 4.3.1.0C-2 for FR1 and FR2 respectively.

Table 4.3.1.0C-1: High Test Channel bandwidths for each NR band, FR1

| NR Band | UE High Test Channel bandwidth [MHz]10, 11, 12 | RedCap UE High Test Channel bandwidth [MHz] 10, 11, 12 |
| --- | --- | --- |
| n1 | 50 | 20 |
| n2 | 40 | 20 |
| n3 | 50 | 20 |
| n5 | 201,14, 252 | 20 |
| n7 | 50 | 20 |
| n8 | 201,14, 352 | 20 |
| n12 | 15 | 15 |
| n13 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 20 | 20 |
| n24 | 10 | 10 |
| n25 | 40 | 20 |
| n26 | 20 | 20 |
| n28 | 30 | 20 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 15 | 15 |
| n38 | 25, 4013 | 15 |
| n39 | 40 | 20 |
| n40 | 80 | 20 |
| n41 | 100 | 20 |
| n46 | 80 | N/A |
| n48 | 403, 1004 | 20 |
| n50 | 601,14, 802 | 20 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 20 | 20 |
| n66 | 40 | 20 |
| n70 | 151,14, 252 | 151, 202 |
| n71 | 20 | 20 |
| n74 | 20 | 20 |
| n75 | 202 | N/A |
| n76 | 52 | N/A |
| n77 | 100 | 20 |
| n78 | 100 | 20 |
| n79 | 100 | 20 |
| n80 | 301 | [20] |
| n81 | 201 | [20] |
| n82 | 201 | [20] |
| n83 | 301 | [20] |
| n84 | 501 | [20] |
| n86 | 401 | [20] |
| n91 | 101, 52,14 | [10] |
| n92 | 20 | [20] |
| n93 | 101, 52,14 | [10] |
| n94 | 20 | [20] |
| n95 | 151 | [15] |
| n96 | 80 | N/A |
| n97 | 1001 | [20] |
| n99 | 101 | [10] |
| n100 | 5 | 5 |
| n101 | 10 | 10 |
| Note 1: This UE channel bandwidth is applicable only to uplink. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: Applicable for use as single carrier, PCell in CA or PCell in DC configuration. Note 4: Applicable for use as DL SCell in CA or DL SCell in DC configuration. Note 5: Void Note 6: Void Note 7: Void Note 8: Void Note 9: Void Note 10: Maximum values among all the possible channel BW combinations per band in Table 5.3.5-1 of TS 38.521-1 [14] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-1 [7] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 11: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-1 [7]is mandatory without capability parameter (i.e., purely mandatory) as defined in [55] TS 38.306 clause 4,2,1 for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). Note 12: For CA, DC, SDL and SUL, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA, DC, SDL or SUL, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth is tested instead values described in Note 12 in Table 4.3.1.0C-1. Note 13: These UE channel bandwidths are applicable to sidelink operation. Note 14: This UE channel bandwidth is applicable to both uplink and downlink when both uplink and downlink are allocated and testing asymmetric BW is not explicitly intended. |  |  |

NOTE 1 (Informative): In case values listed in table above are higher than those signalled by the UE in supportedBandwidthDL/supportedBandwidthUL, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so the value signalled by the UE in supportedBandwidthDL/supportedBandwidthUL is used in single carrier operation instead values described in Table 4.3.1.0C-1.

Table 4.3.1.0C-2: High Test Channel bandwidths for each NR band, FR2

| NR Band | UE High Test Channel bandwidth [MHz]1, 2, 3 |
| --- | --- |
| n257 | 400 |
| n258 | 400 |
| n259 | 400 |
| n260 | 400 |
| n261 | 400 |
| Note 1: Maximum values among all the possible channel BW combinations per band in Table 5.3.5-1 of TS 38.521-2 [15] are listed. In case such bandwidth is not applicable for a given subcarrier spacing, the maximum bandwidth applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-2 [8] Table 5.3.5-1, the maximum bandwidth defined for that band in the UE release specification shall be tested. In case such bandwidth is optional in the UE release specification according to 38.101-2 [8] Table 5.3.5-1 and not supported by the UE, the maximum non-optional bandwidth for the UE release specification shall be tested. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-2 [8] is mandatory without capability parameter (i.e., purely mandatory) as defined in [55] TS 38.306 clause 4.2.1 for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). Note 3: For CA and DC, the High-test channel bandwidth per component carrier is chosen to allow maximum aggregated bandwidth defined for a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve maximum aggregated bandwidths in CA and DC, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so one combination of bandwidth per component carrier within the bandwidth combination set that maximizes the aggregated bandwidth is tested instead values described in Note 3 in Table 4.3.1.0C-2. |  |

NOTE 1 (Informative): In case values listed in table above are higher than those signalled by the UE in supportedBandwidthDL/supportedBandwidthUL, some flexibility could be provided to the ecosystem for Rel-15 and Rel-16 so the value signalled by the UE is used in single carrier operation instead values described in Table 4.3.1.0C-2.

Table 4.3.1.0C-3: High Test Channel bandwidths for each NR band, NTN

| NR Band | UE High Test Channel bandwidth [MHz]1, 2 |
| --- | --- |
| n255 | 20 |
| n256 | 20 |
| Note 1: Maximum values among all the possible channel BW combinations per band in Table 5.3.5-1 of TS 38.521-5 [57] are listed. Note 2: Values listed in this table assume that the maximum (non-optional) channel bandwidth specified in Table 5.3.5-1 of TS 38.101-5 [56] is mandatory without capability parameter (i.e., purely mandatory) as defined in TS 38.306 clause 4,2,1 [55] for supportedBandwidthDL/ supportedBandwidthUL parameters in a band combination with a single band entry and a single CC entry (i.e., non-CA band combination). |  |

<<End of change>>