3GPP TSG-RAN5 Meeting #102 	R5-241101

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2708 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Rel-16 A-MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_bands_BW_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - LCRB of some test points in Table 6.2.3.4.1-12 does not fulfill the following definition for DFT-s OFDM in TS 38.211 6.3.1.4. - Required A-MPR of Test ID 3, 17, 18 in Table 6.2.3.4.1-25 is not applied due to its incorrect RB allocation. - Required A-MPR of Test ID 7, 8 in Table 6.2D.3.4.1-8 is not applied due to its incorrect RB allocation. - Invalid modulation is used in Table 6.2D.3.4.1-9 - Incorrect A-MPR regions in Table 6.2D.3.4.1-10 - Incorrect PCMAX_L values in Table 6.2D.3.5-13 - In Table 6.2D.3.5-13, the row for Test ID 18 ChBW 10 MHz is loceted for Test ID 19, 20. - Incorrect A-MPR values in Table 6.2D.3.5-14 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected RB allocation of Test ID 64-72, 76-84, 110-112, 114-116, 140-142, 144-146 in Table 6.2.3.4.1-12 - Corrected RB allocation of Test ID 3, 17, 18 in Table 6.2.3.4.1-25 - Corrected RB allocation of Test ID 7, 8 in Table 6.2D.3.4.1-8 - Corrected modulation from “CP-s OFDM” to “CP-OFDM” in Table 6.2D.3.4.1-9 - Corrected A-MPR regions in Table 6.2D.3.4.1-10 - Corrected test rquirements in Table 6.2D.3.5-13 - Location of the row for Test ID 18 ChBW 10 MHz was corrected in Table 6.2D.3.5-13. - Corrected test requirements in Table 6.2D.3.5-14 Editorial correction: - Merged cells of modulation in Table 6.2D.3.4.1-8 - Merged cells of modulation in Table 6.2D.3.4.1-9 - Corrected text style in Table 6.2D.3.4.1-10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Tests cannot be performed correctly because the correct A-MPR is not applied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.2D.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

<<Unchanged sections skipped>>

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

<<Unchanged tables skipped>>

Table 6.2.3.4.1-12: Test Configuration table for NS_24

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | Refer to test parameters (5, 10, 15, 20 MHz) |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest |  |  |  |  |
| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 1-3 | 1992.5 | 5 | Default | N/A for A-MPR testing. | DFT-s-OFDM | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 4-6 | 1997.5 | 5 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 7-15 | 2002.5 | 5 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 16-18 | 1985 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | N/A | N/A |
| 19-21 | 1985 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 22-30 | 1995 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 31-39 | 2000 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 40-42 | 2000 | 10 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 43-51 | 1987.5 | 15 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 52-60 | 1997.5 | 15 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 61-63 | 1997.5 | 15 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 64-72 | 1990 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 75@0 |
| 73-75 | 1990 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 76-84 | 1995 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Edge_1RB_Right | Edge_1RB_Left | 64@0 |
| 85-87 | 1995 | 20 | Default |  |  | PI/2 BPSK QPSK 16 QAM | Outer_Full | N/A | N/A |
| 88 | 1992.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 89 | 1997.5 | 5 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 90-92 | 2002.5 | 5 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 93 | 1985 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | N/A | N/A |
| 94 | 1985 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |

| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 95-97 | 1995 | 10 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 98-100 | 2000 | 10 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 101 | 2000 | 10 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 103-105 | 1987.5 | 15 | Default |  |  | 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 106-108 | 1997.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 109 | 1997.5 | 15 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 110-112 | 1990 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 75@0 |
| 113 | 1990 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 114-116 | 1995 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 64@0 |
| 117 | 1995 | 20 | Default |  |  | 64 QAM | Outer_Full | N/A | N/A |
| 118 | 1992.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 119 | 1997.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 120-122 | 2002.5 | 5 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 123 | 1985 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | N/A | N/A |
| 124 | 1985 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 125-127 | 1995 | 10 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 128-130 | 2000 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 36@0 |
| 131 | 2000 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 133-135 | 1987.5 | 15 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 136-138 | 1997.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 50@0 |
| 139 | 1997.5 | 15 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 140-142 | 1990 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 75@0 |
| 143 | 1990 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 144-146 | 1995 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 64@0 |
| 147 | 1995 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 148-150 | 1992.5 | 5 | Default |  | CP-OFDM | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 151-153 | 1997.5 | 5 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 154-162 | 2002.5 | 5 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 163-165 | 1985 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | N/A | N/A |
| 166-168 | 1985 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 169-177 | 1995 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |

| A-MPR test parameters for NS_24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (NOTE 2, 3) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | Region A | Region B | Region C |
| 178-186 | 2000 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 187-189 | 2000 | 10 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 190-198 | 1987.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 199-207 | 1997.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 208-210 | 1997.5 | 15 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 211-219 | 1990 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 220-222 | 1990 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 223-231 | 1995 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 232-234 | 1995 | 20 | Default |  |  | QPSK 16 QAM 64 QAM | Outer_Full | N/A | N/A |
| 235 | 1992.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 236 | 1997.5 | 5 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 237-239 | 2002.5 | 5 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 240 | 1985 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | N/A | N/A |
| 241 | 1985 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 242-244 | 1995 | 10 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 245-247 | 2000 | 10 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 37@0 |
| 248 | 2000 | 10 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 250-252 | 1987.5 | 15 | Default |  |  | 256 QAM | Outer_Full | Edge_1RB_Right | Edge_1RB_Left |
| 253-255 | 1997.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 51@0 |
| 256 | 1997.5 | 15 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 257-259 | 1990 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 74@0 |
| 260 | 1990 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 261-263 | 1995 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Left | 69@0 |
| 264 | 1995 | 20 | Default |  |  | 256 QAM | Outer_Full | N/A | N/A |
| 265 | 1992.5 | 5 | Default |  |  | QPSK | Edge_1RB_Right | N/A | N/A |
| 266 | 1997.5 | 5 | Default |  |  | 16 QAM | Edge_1RB_Right | N/A | N/A |
| 267 | 1985 | 10 | Default |  |  | QPSK | Edge_1RB_Left | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: In test IDs with multiple modulations, each UL Modulation shall be tested separately against Range A, B, and C. |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

Table 6.2.3.4.1-25: Test Configuration table for NS_46

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 |  |  |  | N/A for A-MPR testing. | DFT-s-OFDM |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |
| 3 | Default | 25 | Default |  |  | 64QAM | 60@73 (A3) |  | 30@35 (A3) | 15@16 (A3) |
| 4 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 5 | Default | 50 | Default |  |  | 16QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 6 | Default | 50 | Default |  |  | 64QAM | 162@0 (A6) |  | 81@0 (A6) | 40@0 (A6) |
| 7 | Default | 50 | Default |  |  | 256QAM | 216@0 (A7) |  | 108@0 (A7) | 54@0 (A7) |
| 8 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| 9 |  |  |  |  | CP-OFDM |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |
| 11 | Default | 25 | Default |  |  | QPSK | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 12 | Default | 25 | Default |  |  | 256QAM | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 13 | Default | 50 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |  |
| 14 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 15 | Default | 50 | Default |  |  | QPSK | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 16 | Default | 50 | Default |  |  | 64QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 17 | Default | 50 | Default |  |  | QPSK | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 18 | Default | 50 | Default |  |  | 64QAM | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 19 | Default | 50 | Default |  |  | QPSK | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 20 | Default | 50 | Default |  |  | 256QAM | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 21 | Default | 50 | Default |  |  | QPSK | Outer_Full (A8) |  |  |  |
| 22 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.2D.3	UE additional maximum output power reduction for UL MIMO

Editor’s Note:

- Supporting of ULFPTx is only completed for NS_04, NS_03, NS_03U.

6.2D.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated a unique with network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2D.1.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2D.3.2	Test applicability

The requirements of this test apply in test case 6.5D.2.3 Additional Spectrum Emission mask for UL MIMO for network signalling value NS_03, NS_03U, NS_04 to all types of NR UE release 15 and forward that support UL MIMO.

The requirements of this test apply in test case 6.5D.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, and NS_100 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5D.3.3 Additional Spurious Emissions for network signalling value NS_04, NS_47, NS_21, NS_56, NS_50 to all types of NR UE release 15 and forward that support UL-MIMO.

NOTE:	Test execution is not necessary if 6.5D.2.3, 6.5D.2.4.2 and 6.5D.3.3 are executed.

6.2D.3.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the A-MPR values specified in clause 6.2.3.3 shall apply to the maximum output power specified in Table 6.2D.1.3-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1.3-3, based upon UE’s support of uplink full power transmission mode.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2D.4.3 apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3.3 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2D.3.

6.2D.3.4	Test description

6.2D.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2D.3.4.1-1 to 6.2D.3.4.1-11. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

<<Unchanged tables skipped>>

Table 6.2D.3.4.1-8: Test Configuration table for NS_46

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 15MHz, 20MHz, 25 MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_46 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS |  | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  | (Note 2) |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 25 | Default |  | CP-OFDM | QPSK | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 2 | Default | 25 | Default |  |  | 256QAM | 90@43 (A3) |  | 45@20 (A3) | 23@8 (A3) |
| 3 | Default | 50 | Default |  |  | QPSK | Edge_1RB_Left (A4) |  |  |  |
| 4 | Default | 50 | Default |  |  | 64QAM | Edge_1RB_Left (A4) |  |  |  |
| 5 | Default | 50 | Default | Downlink Configuration |  | QPSK | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 6 | Default | 50 | Default |  |  | 64QAM | 120@0 (A5) |  | 60@0 (A5) | 30@0 (A5) |
| 7 | Default | 50 | Default |  |  | QPSK | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 8 | Default | 50 | Default |  |  | 64QAM | 175@0 (A6) |  | 88@0 (A6) | 44@0 (A6) |
| 9 | Default | 50 | Default |  |  | QPSK | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 10 | Default | 50 | Default |  |  | 256QAM | 220@0 (A7) |  | 110@0 (A7) | 55@0 (A7) |
| 11 | Default | 50 | Default |  |  | QPSK | Outer_Full (A8) |  |  |  |
| 12 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A8) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-9: Test Configuration table for NS_21

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| A-MPR test parameters for NS_21 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink | Uplink Configuration |  |  |  |  |  |
|  |  |  |  | Configuration | Modulation |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  | (Note 2) |  |  | SCS 15 kHz | SCS 30 kHz |  |
| 1 | Low | Default | Default |  | CP-OFDM | QPSK |  | Edge_1RB_Left |  |  |
| 2 | High | Default | Default |  |  | QPSK |  | Edge_1RB_Right |  |  |
| 3 | Default | Default | Default |  |  | QPSK |  | Outer_Full |  |  |
| 4 | Default | 10 MHz | Default |  |  | QPSK |  | 4@0 |  | 2@0 |
| 5 | Default | 10 MHz | Default |  |  | QPSK |  | 4@48 |  | 2@22 |
| 6 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |  |
| 7 | High | Default | Default |  |  | 16 QAM |  | Edge_1RB_Right |  |  |
| 8 | Default | Default | Default | N/A |  | 16 QAM |  | Outer_Full |  |  |
| 9 | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@0 |  | 2@0 |
| 10 | Default | 10 MHz | Default |  |  | 16 QAM |  | 4@48 |  | 2@22 |
| 11 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |  |
| 12 | High | Default | Default |  |  | 64 QAM |  | Edge_1RB_Right |  |  |
| 13 | Default | Default | Default |  |  | 64 QAM |  | Outer_Full |  |  |
| 14 | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@0 |  | 2@0 |
| 15 | Default | 10 MHz | Default |  |  | 64 QAM |  | 4@48 |  | 2@22 |
| 16 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |  |
| 17 | High | Default | Default |  |  | 256 QAM |  | Edge_1RB_Right |  |  |
| 18 | Default | Default | Default |  |  | 256 QAM |  | Outer_Full |  |  |
| 19 | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@0 |  | 2@0 |
| 20 | Default | 10 MHz | Default |  |  | 256 QAM |  | 4@48 |  | 2@22 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.4.1-10: Test Configuration table for NS_44

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Defined for each test ID |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_44 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc | Ch BW | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  | (MHz) | (MHz) | (kHz) |  | Modulation |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  | (Note 2) |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | 2602.5 | 25 | Default |  | CP-OFDM | 64 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 2 | 2602.5 | 25 | Default |  |  | 64 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 3 | 2602.5 | 25 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 4 | 2602.5 | 25 | Default |  |  | 256 QAM | 100@0 (A3) |  | 50@0 (A3) | 25@0 (A3) |
| 5 | 2602.5 | 25 | Default | N/A |  | 256 QAM | 39@94 (A3) |  | 19@46 (A3) | 9@22 (A3) |
| 6 | 2602.5 | 25 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 7 | 2600 | 30 | Default |  |  | 64 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 8 | 2600 | 30 | Default |  |  | 64 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 9 | 2600 | 30 | Default |  |  | 64 QAM | Outer_Full (A6) |  |  |  |
| 10 | 2600 | 30 | Default |  |  | 256 QAM | 120@0 (A3) |  | 60@0 (A3) | 30@0 (A3) |
| 11 | 2600 | 30 | Default |  |  | 256 QAM | 69@91 (A3) |  | 34@44 (A3) | 17@21 (A3) |
| 12 | 2600 | 30 | Default |  |  | 256 QAM | Outer_Full (A6) |  |  |  |
| 13 | 2595 | 40 | Default |  |  | 64 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 14 | 2595 | 40 | Default |  |  | 64 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 15 | 2595 | 40 | Default |  |  | 64 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 16 | 2595 | 40 | Default |  |  | 64 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 17 | 2595 | 40 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |  |
| 18 | 2595 | 40 | Default |  |  | 256 QAM | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 19 | 2595 | 40 | Default |  |  | 256 QAM | 80@0 (A2) |  | 40@0 (A2) | 20@0 (A2) |
| 20 | 2595 | 40 | Default |  |  | 256 QAM | 130@0 (A3) |  | 65@0 (A3) | 33@0 (A3) |
| 21 | 2595 | 40 | Default |  |  | 256 QAM | 180@0 (A4) |  | 90@0 (A4) | 45@0 (A4) |
| 22 | 2595 | 40 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

<<Unchanged tables skipped>>

<<Unchanged sections skipped>>

6.2D.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2D.3.5-1 to table 6.2D.3.5-14. The allowed A-MPR values specified in table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2D.1.3-1 apply.

<<Unchanged tables skipped>>

Table 6.2D.3.5-13: UE Power Class 3 test requirements (NS_21) for band n30

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) |  | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1,2 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 3 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 3 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 3 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 4,5 | 10 | 23 | 3 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 6,7 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 3 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 8 | 5 | 23 | 3 | 0 | 0 |  | 20 | 6 | 3 | 25+TT | 14-TT |
|  | 10 | 23 | 3 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 9,10 | 10 | 23 | 3 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 11,12 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 6 | 0 |  | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 5 | 23 | 3.5 | 0 | 0 |  | 19.5 | 5 | 3 | 25+TT | 14.5-TT |
|  | 10 | 23 | 3.5 | 5.5 | 0 |  | 17.5 | 5 | 3 | 25+TT | 12.5-TT |
| 14,15 | 10 | 23 | 3.5 | 4 | 0 |  | 19 | 5 | 3 | 25+TT | 14-TT |
| 16,17 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 6 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 18 | 5 | 23 | 6.5 | 0 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
|  | 10 | 23 | 6.5 | 5.5 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19,20 | 10 | 23 | 6.5 | 4 | 0 |  | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.3.5-14: UE Power Class 3 test requirements (NS_44) for band n38

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 2 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 3 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 5 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 6 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 7 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 8 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 9 | 23 | 0 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 10 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 11 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 12 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 13 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 14 | 23 | 0 | 3.5 | 4 | 0 | 19 | 5 | 3 | 25+TT | 14-TT |
| 15 | 23 | 0 | 3.5 | 5 | 0 | 18 | 5 | 3 | 25+TT | 13-TT |
| 16 | 23 | 0 | 3.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 17 | 23 | 0 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 18 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 19 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 20 | 23 | 0 | 6.5 | 0 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 21 | 23 | 0 | 6.5 | 8 | 0 | 15 | 6 | 3 | 25+TT | 9-TT |
| 22 | 23 | 0 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.3.5-0. |  |  |  |  |  |  |  |  |  |  |

<<End of change>>

<< Informal note for the implementer: Please merge the cells for modulation “CP-OFDM” in Table 6.2D.3.4.1-8 and Table 6.2D.3.4.1-9 during implementation. Merging cells was not recorded in MS Word.>>