3GPP TSG-RAN5 Meeting #102 	R5-241120

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2996 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to QuantityConfigu-DEFAULT in H.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Table H.3.1-5: QuantityConfig-DEFAULT does not specify setting for quantityConfigRS-Index. <TS 38.331> |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added setting for quantityConfigRS-Index in Table H.3.1-5: QuantityConfig-DEFAULT: - quantityConfigRS-Index is configured when SSB Index (RS Index) is to be reported, otherwise, it is Not present. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration

<<Unchanged sections skipped>>

QuantityConfig-DEFAULT

NR quantity configuration when L3 filtering is not used.

Table H.3.1-5: QuantityConfig-DEFAULT: NR quantity configuration when L3 filtering is not used

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-127 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig::= SEQUENCE { |  |  |  |
| quantityConfigNR-List SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF SEQUENCE { | 1 entry |  |  |
| quantityConfigCell[1] SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigRS-Index | Not present |  |  |
| quantityConfigRS-Index SEQUENCE { |  |  | SSB Index |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigEUTRA | Not present |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  | INTER-RAT |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| filterCoefficientRS-SINR | fc0 |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |
| SSB Index | SSB Index (RS Index) is reported |

<<End of change>>