3GPP TSG-RAN5 Meeting #102	R5-241179

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5295 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 8.1.3.8a Redirection to GERAN-Redir-policy bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Fake eNB can with an unprotected rrcConnectionRelease message redirect UE to fake GERAN BS. GERAN provides less security protection than LTE, and from the Fake GERAN BS, e.g. fraud SMS can be sent to UE. Network support for redir-policy bit was introduced in Rel-15 which allows network operators to indicate to the UE whether insecured redirection to a GERAN cell is not allowed or allowed in the current PLMN due to operator policies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case 8.1.3.8a RRC connection release / Redirection from E-UTRAN to GERAN / redir-policy bit has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage for redir-policy bit remains inclomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.8a |  |  |  |  |  |  |  |  |
| sss |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.523-2 CR 1445 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.1.3.8a	RRC connection release / Redirection from E-UTRAN to GERAN / Redir-policy bit

8.1.3.8a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state }

ensure that {

when { UE receives an RRCConnectionRelease message including an IE redirectedCarrierInfo set to geran and upper layers indicate that redirect to GERAN without AS security is not allowed}

then { UE discards redirection to GERAN}

}

8.1.3.8a.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clause 5.5.1.2.4 and 5.5.3.2.4, TS 36.331, clause 5.3.8.3

[TS 24.301, clause 5.5.1.2.4]

…

If the redir-policy bit is set to "Unsecured redirection to GERAN not allowed" in the Network policy IE of the ATTACH ACCEPT message, the UE should set the network policy on unsecured redirection to GERAN for the current PLMN to "Unsecured redirection to GERAN not allowed" and indicate to the lower layers that unsecured redirection to a GERAN cell is not allowed. If the redir-policy bit is set to "Unsecured redirection to GERAN allowed" or if the Network policy IE is not included in the ATTACH ACCEPT message, the UE should set the network policy for the current PLMN to "Unsecured redirection to GERAN allowed" and indicate to the lower layers that unsecured redirection to a GERAN cell is allowed. If the Network policy IE is not provided by the network in the ATTACH ACCEPT message, the UE should set the network policy on unsecured redirection to GERAN to "Unsecured redirection to GERAN not allowed" and indicate this to the lower layers when any of the following events occurs:

-	the UE initiates an EPS attach or tracking area updating procedure in a PLMN different from the PLMN where the UE performed the last successful EPS attach or tracking area updating procedure;

-	the UE is switched on; or

-	the UICC containing the USIM is removed.

…

[TS 24.301, clause 5.5.1.2.4]

…

If the redir-policy bit is set to "Unsecured redirection to GERAN not allowed" in the Network policy IE of the TRACKING AREA UPDATE ACCEPT message, the UE should set the network policy on unsecured redirection to GERAN for the current PLMN to "Unsecured redirection to GERAN not allowed" and indicate to the lower layers that unsecured redirection to a GERAN cell is not allowed. If the redir-policy bit is set to "Unsecured redirection to GERAN allowed" or if the Network policy IE is not included in the TRACKING AREA UPDATE ACCEPT message, the UE should set the network policy on unsecured redirection to GERAN for the current PLMN to "Unsecured redirection to GERAN allowed" and indicate to the lower layers that unsecured redirection to a GERAN cell is allowed. If the Network policy IE is not provided by the network in the TRACKING AREA UPDATE ACCEPT message, the UE should set the network policy on unsecured redirection to GERAN to "Unsecured redirection to GERAN not allowed" and indicate this to the lower layers when any of the following events occurs:

-	the UE initiates an EPS attach or tracking area updating procedure in a PLMN different from the PLMN where the UE performed the last successful EPS attach or tracking area updating procedure;

-	the UE is switched on; or

-	the UICC containing the USIM is removed.

…

[TS 36.331, clause 5.3.8.3]

The UE shall:

1>	if the RRCConnectionRelease message includes redirectedCarrierInfo indicating redirection to geran:

…

2>	if AS security has not been activated, and

2>	if upper layers indicate that redirect to GERAN without AS security is not allowed:

3>	ignore the content of the RRCConnectionRelease;

3>	perform the actions upon leaving RRC_CONNECTED or RRC_INACTIVE as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

…

8.1.3.8a.3	Test description

8.1.3.8a.3.1	Pre-test conditions

System Simulator:

-	2 cells, one E-UTRA and one GERAN cell and different location areas:

-	Cell 1 E-UTRA serving cell

-	Cell 24 GERAN neighbouring cell

-	Cell power levels are selected according to [18] so that camping on Cell 1 is guaranteed

-	System information combination 5 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

-	Cell 24 with AS security deactivated

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

8.1.3.8a.3.2	Test procedure sequence

Table 8.1.3.8a.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is powered up or switched on. |  |  | - | - |
| 2 | The UE sends an ATTACH REQUEST message on serving Cell 1 with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach" | --> | ATTACH REQUEST | - | - |
| 3-9 | Steps 5-11 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 10 | The SS sends ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message and with the network policy information element redir-policy set to “unsecured redirection to GERAN not allowed” | <-- | ATTACH ACCEPT | - | - |
| 11 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 12 | The UE send ATTACH COMPLETE message with the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message | --> | ATTACH COMPLETE | - | - |
| 13 | SS transmits an RRCConnectionRelease message (IE redirectedCarrierInfo including startingARFCN of Cell 24). | <-- | RRCConnectionRelease | - | - |
| 14 | Check: Does the UE send a CHANNEL REQUEST message on the cell 24 to transmit a ROUTING AREA UPDATE REQUEST message? | --> | CHANNEL REQUEST | 1 | F |
| 15 | Check: Does the UE leaving RRC CONNECTED with release cause 'other' in Cell 1 NOTE: The UE discards the RAU procedure. | --> | - | 1 | P |

8.1.3.8.3.3	Specific message or IE contents

Table 8.1.3.8a.3.3-1: SystemInformationBlockType7 for cell 1 (preamble and all steps, Table 8.1.3.8a.3.2-1)

| Derivation Path: 36.508 table 4.4.3.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType7 ::= SEQUENCE { |  |  |  |
| carrierFreqsInfoList SEQUENCE (SIZE (1..maxGNFG)) OF SEQUENCE { |  |  |  |
| carrierFreqs SEQUENCE [n] { |  |  |  |
| startingARFCN | Same as cell 24 |  |  |
| bandIndicator | Same as cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| equallySpacedARFCNs SEQUENCE { |  |  |  |
| arfcn-Spacing | Same as cell 24 |  |  |
| numberOfFollowingARFCNs | Same as cell 24 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| commonInfo SEQUENCE { |  |  |  |
| cellReselectionPriority | 3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.3.8a.3.3-2: Message ATTACH REQUEST (step 2, Table 8.1.3.8a.3.2-1)

| Derivation path: 36.508 table 4.7.2-4 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| EPS attach type | '0010'B |  | ''combined EPS/IMSI attach'' |  |  |
| Old GUTI or IMSI | GUTI-1 |  |  |  |  |
| Last visited registered TAI | TAI-1 |  |  |  |  |
| Old location area identification | LAI-1 |  |  |  |  |
| TMSI status | Not Present |  |  |  |  |

Table 8.1.3.8a.3.3-3: Message ATTACH ACCEPT (step 10, Table 8.1.3.8a.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment | Condition |
| EPS attach result | '010'B |  | ''Combined EPS/IMSI attach'' |  |  |
| GUTI | GUTI-2 |  |  |  |  |
| LAI | LAI-1 |  |  |  |  |
| MS identity | TMSI-1 |  |  |  |  |
| Network policy |  |  |  |  |  |
| Redir-policy | 0001’B |  | Unsecured redirection to GERAN not allowed |  |  |

Table 8.1.3.8a.3.3-2: RRCConnectionRelease message (step 13, Table 8.1.3.8a.3.2-1)

| Derivation Path: 36.508 table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| redirectedCarrierInfo CHOICE { |  |  |  |
| geran SEQUENCE { |  |  |  |
| startingARFCN | Downlink ARFCN of Cell 24 |  |  |
| bandIndicator | The same band indicator of the Cell 24 |  |  |
| followingARFCNs CHOICE { |  |  |  |
| explicitListOfARFCNs | 0 Entries |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>