3GPP TSG-RAN5 Meeting #102	R5-241198

Athens, Greece, Feb 26 – March 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3008 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 4.7.8.2 EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM Test Case 4.7.8.2 EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell is defined in RAN4 [6] core-spec., but not added in RAN5 TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case 4.7.8.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Cases will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.8.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing.

-	TT analysis is missing.

-	Test Procedure is TBD.

-	Test applicability needs to be updated

-	Cell mapping is missing in Annex E

4.7.8.2.1	Test purpose

The purpose of this test is to verify that the CSI-RSRP measurement accuracy is within the specified limits for all bands.

4.7.8.2.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 16 onwards.

4.7.8.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.8.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.4.7.8.2.

4.7.8.2.4	Test description

4.7.8.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.7.8.2.4.1-1.

Table 4.7.8.2.4.1-1: Applicable NR configurations for FR1 inter-frequency CSI-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB and CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB and CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations on each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 4.7.8.2.4.1-2.

Table 4.7.8.2.4.1-2: Test Environment parameters for EN-DC CSI-RSRP Inter-frequency measurement accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.7.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 4.7.8.2.4.3.

2.	In this set of test cases there are three cells in the test, E-UTRAN PCell (Cell 1), FR1 PSCell (Cell 2) and a FR1 neighbour cell (Cell 3) on a different frequency than the PSCell. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 is the target cell for CSI-RSRP measurement. The connection setup is done according to the settings in clause C.1.1.

4.7.8.2.4.2	Test procedure

TBD

4.7.8.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

TBD

4.7.8.2.5	Test requirement

Table 4.7.8.2.5-1 defines the primary level settings including test tolerances for all tests.

Each CSI-RSRP measurement report for each of the tests in Table 4.7.8.2.5-1 shall meet the corresponding absolute accuracy requirements in Table 4.7.8.2.5-2 for test configuration 1, 2, 4 and 5, and the corresponding absolute accuracy requirements in Table 4.7.8.2.5-3 for test configuration 3 and 6.

Table 4.7.8.2.5-1: CSI-RSRP inter-frequency test parameters

| Parameter |  | Config | Unit | Test 1 |  |  | Test 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 3 | Cell 2 |  | Cell 3 |  |
| SSB ARFCN |  | 1~6 |  | freq1 |  | freq2 | freq1 |  | freq2 |  |
| BWchannel |  | 1,4 | MHz | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |  |
|  |  | 2,5 |  | 10: NRB,c = 52 |  |  | 10: NRB,c = 52 |  |  |  |
|  |  | 3,6 |  | 40: NRB,c = 106 |  |  | 40: NRB,c = 106 |  |  |  |
| Gap pattern ID |  |  |  | 0 |  |  | 0 |  |  |  |
| Duplex mode |  | 1,4 |  | FDD |  |  | FDD |  |  |  |
|  |  | 2,5 |  | TDD |  |  | TDD |  |  |  |
|  |  | 3,6 |  | TDD |  |  | TDD |  |  |  |
| TDD configuration |  | 1,4 |  | N/A |  |  | N/A |  |  |  |
|  |  | 2,5 |  | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |  |
|  |  | 3,6 |  | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |  |
| PDSCH Reference measurement channel |  | 1,4 |  | SR.1.1 FDD |  | - | SR.1.1 FDD |  | - |  |
|  |  | 2,5 |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |  |
|  |  | 3,6 |  | SR.2.1 FDD |  |  | SR.2.1 FDD |  |  |  |
| RMSI CORESET Reference Channel |  | 1,4 |  | CR.1.1 FDD |  | - | CR.1.1 FDD |  | - |  |
|  |  | 2,5 |  | CR.1.1 TDD |  | - | CR.1.1 TDD |  | - |  |
|  |  | 3,6 |  | CR.2.1 FDD |  | - | CR.2.1 FDD |  | - |  |
| Dedicated CORESET Reference Channel |  | 1,4 |  | CCR.1.1 FDD |  | - | CCR.1.1 FDD |  | - |  |
|  |  | 2,5 |  | CCR.1.1 TDD |  | - | CCR.1.1 TDD |  | - |  |
|  |  | 3,6 |  | CCR.2.1 TDD |  | - | CCR.2.1 TDD |  | - |  |
| SSB configuration |  | 1,4 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |  |
|  |  | 2,5 |  | SSB.1 FR1 |  |  | SSB.1 FR1 |  |  |  |
|  |  | 3,6 |  | SSB.2 FR1 |  |  | SSB.2 FR1 |  |  |  |
| CSI-RS configuration for RRM |  | 1,4 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |
|  |  | 2,5 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |
|  |  | 3,6 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  | 1~6 |  | OP.1 |  |  | OP.1 |  |  |  |
| TRS configuration |  | 1,4 |  | TRS.1.1 FDD |  | - | TRS.1.1 FDD |  |  | - |
|  |  | 2,5 |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |  |
|  |  | 3,6 |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |  |
| Initial BWP Configuration |  | 1~6 |  | DLBWP.0.1 ULBWP.0.1 |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |  |
| Dedicated BWP configuration |  | 1~6 |  | DLBWP.1.1 ULBWP.1.1 |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |  |
| Time offset with Cell 2 |  | 1,2,4,5 | s | - | 4.7 |  | - | 4.7 |  |  |
|  |  | 3,6 | s | - | 2.35 |  | - | 2.35 |  |  |
| SMTC configuration |  | 1,4 |  | SMTC.2 |  |  | SMTC.2 |  |  |  |
|  |  | 2,3,5,6 |  | SMTC.1 |  |  | SMTC.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | 1~6 | dB | 0 |  | 0 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1~6 | dBm/15kHz | -94.65 |  |  | ( for Cell 3 +8dB) |  | -115 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |  |
|  | NF_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |  |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 1,2,4,5 | dBm/SSB SCS | -94.65 |  |  | ( for Cell 3 +8dB) |  | -115 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.5 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113 |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.5 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.5 |  |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.5 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, NR_SDL_FR1_A | 3,6 |  | -91.65 |  |  | ( for C 3 +8dB) |  | -112.00 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -112.50 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -112.00 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -111.50 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -111.00 |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -110.50 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -110.00 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -110.50 |  |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -107.50 |  |
|  |  | 1~6 | dB | 10 |  | 10 | 13 |  | -3 |  |
| CSI-RSRPNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2,4,5 | dBm/SCS | -84.65 |  |  | (RSRP for Cell 3 +25dB) |  | -118.00 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -117.50 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -117.00 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -116.50 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -116.00 |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -115.50 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -115.00 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -114.50 |  |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -111.50 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5, | 3,6 |  | -81.65 |  |  | (RSRP for Cell 3 +25dB) |  | -115.00 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -114.50 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -114.00 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -113.50 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -113.00 |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -112.50 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -112.00 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -111.50 |  |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -108.50 |  |
| IoNote3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, | 1,2,4,5 | dBm/ 9.36MHz | -56.28 |  |  | (Io for Channel 3 +19.75dB) |  | -85.28 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -84.78 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -84.28 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -83.78 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -83.28 |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -82.78 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -82.28 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -81.78 |  |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -78.78 |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 6, | 3,6 | dBm/ 38.16MHz | -50.19 |  |  | (Io for Channel 3 +19.75dB) |  | -79.19 |  |
|  | NR_FDD_FR1_B |  |  |  |  |  |  |  | -78.69 |  |
|  | NR_TDD_FR1_C |  |  |  |  |  |  |  | -78.19 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  |  |  |  |  | -77.69 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  |  |  |  |  | -77.19 |  |
|  | NR_FDD_FR1_F |  |  |  |  |  |  |  | -76.69 |  |
|  | NR_FDD_FR1_G |  |  |  |  |  |  |  | -76.19 |  |
|  | NR_FDD_FR1_H |  |  |  |  |  |  |  | -75.69 |  |
|  | NR_FDD_FR1_N |  |  |  |  |  |  |  | -72.69 |  |
|  |  | 1~6 | dB | 10 |  | 10 | 13 |  | -3 |  |
| Propagation condition |  | 1~6 | - | AWGN |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 1x2 |  |  | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: CSI-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5 The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |  |  |  |  |

Table 4.7.8.2.5-2: CSI-RSRP Intra-frequency absolute accuracy requirements for
the reported values for test configurations 1, 2, 4 and 5

TBD

Table 4.7.8.2.5-3: CSI-RSRP Intra-frequency absolute accuracy requirements for
the reported values for test configurations 3 and 6

TBD

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

## << End of changes >>