3GPP TSG-RAN5 Meeting #102	R5-241201

Athens, Greece, Feb 26 – March 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | - | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct of Test Tolerance into Annex F for EN-DC FR1 addition and release delay of known PSCell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM TT is incorrect for Test Case 4.5.7.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRM TT has been corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 4.5.7.1 will remain using an incorrect TT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.903 CR 0684 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

F.1.3.2	Measurement of RRM requirements

Many information are skipped

Table F.1.3.2-1: Derivation of test requirements for EN-DC FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 4.3.2.2.1 | Absolute uplink power: Normal conditions ±9dB Relative uplink power step: Normal conditions ±2.5dB Uplink timing: 15kHz SCS Te ±12*64*Tc 30kHz SCS Te ±8*64*Tc | 2.1dB 0.7dB 112Tc 112Tc | Absolute uplink power: Normal conditions ±11.1dB Relative uplink power step: Normal conditions ±3.2dB Uplink timing: 15kHz SCS Te ±880*Tc 30kHz SCS Te ±624*Tc |
| 4.3.2.2.2 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.3 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.3.2.2.4 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 | Same as 4.3.2.2.1 |
| 4.4.1.1 EN-DC FR1 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Max step size Tq: 5.5*64*Tc Min adjust rate Tp: 5.5*64*Tc Max adjust rate: 5.5*64*Tc Ês2 / Noc: +3.00dB Noc = -98 dBm/15 kHz (Config 1,2,3) Test 2 (with DRX): ±12*64 Tc for 15 KHz SSB SCS,15 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,30 kHz UL SCS ±10*64 Tc for 15 KHz SSB SCS,60 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,15 kHz UL SCS ±8*64 Tc for 30 KHz SSB SCS,30 kHz UL SCS ±7*64 Tc for 30 KHz SSB SCS,60 kHz UL SCS Ês2 / Noc: +3.00dB | ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.5*64Tc -3.6*64*Tc +1.1*64*Tc +0.3 dB +1.5 dB ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc ±1.75*64*Tc +0.3dB | Test 1 (10MHz Ch BW): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Max step size Tq: 6.0*64*Tc Min adjust rate: 1.9*64*Tc Max adjust rate: 6.6*64*Tc Ês2 / Noc: +3.30dB Noc = -98 dBm/15 kHz (Config 1,2,3) +1.5 dB Test 2 (with DRX): Uplink timing: ±13.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±11.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±9.75*64*Tc Uplink timing: ±8.75*64*Tc Ês2 / Noc: +3.30dB |
| 4.4.3.1 EN-DC FR1 timing advance adjustment accuracy | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE Timing Advance Adjustment Accuracy for 15kHz SCS = ±256 Tc+ TT UE Timing Advance Adjustment Accuracy for 30kHz SCS = ±256 Tc+ TT | 0 0 0 +/- 88 Tc +/- 88 Tc | Noc = -98 dBm/15 kHz (Config 1, 2, 4, 5) Noc = -95 dBm/15 kHz (Config 3, 6) Êsx / Noc = 3 dB UE TAAA for 15kHz SCS = ±344 Tc UE TAAA for 30kHz SCS = ±344 Tc |
| 4.5.1.1 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB |
| 4.5.1.2 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode | SNR during T1: 1dB T2: -7dB T3: -15dB T4: -4.5dB T5: 1dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: +0.8dB | SNR during T1: 1.8dB T2: -6.2dB T3: -15.8dB T4: -5.3dB T5: 1.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3 and T4 is modified as specified in clause D.4.1.1 |
| 4.5.1.3 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.4 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 4.5.1.5 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.6 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in non-DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 4.5.1.7 EN-DC FR1 radio link monitoring out-of-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.1 | Same as 4.5.1.1 | Same as 4.5.1.1 |
| 4.5.1.8 EN-DC FR1 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 4.5.1.2 | Same as 4.5.1.2 | Same as 4.5.1.2 |
| 4.5.1.9 EN-DC FR1 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | SNR during T1: 1dB T2: 1dB T3: -15dB | Offset during T1: +0.8dB T2: +0.8dB T3: -0.8dB | SNR during T1: 1.8dB T2: 1.8dB T3: -15.8dB |
| 4.5.2.1 EN-DC FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC | SNRs as specified | 0.6dB | Formula: SNR + TT |
| 4.5.2.2 EN-DC FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC | Same as 4.5.2.1 | Same as 4.5.2.1 | Same as 4.5.2.1 |
| 4.5.2.3 EN-DC FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.2.4 EN-DC FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC | Same as 4.5.2.3 | Same as 4.5.2.3 | Same as 4.5.2.3 |
| 4.5.2.5 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês1 / Noc1: +17dB Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.2.6 EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC | Same as 4.5.2.5 | Same as 4.5.2.5 | Same as 4.5.2.5 |
| 4.5.2.10 EN-DC FR1 interruptions due to RRM and RLM/BFD measurements on deactivated NR PSCell | During T1: Noc: -104dBm/15kHz Ês2 / Noc: +17dB | During T1: 0dB 0dB | During T1: Noc: -104dBm/15kHz Ês2 / Noc: +17dB |
| 4.5.3.1 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 160ms SCell measurement cycle | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.3.2 EN-DC FR1 SCell activation and deactivation of known SCell in non-DRX for 640ms SCell measurement cycle | Same as 4.5.3.1 | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.3 EN-DC FR1 SCell activation and deactivation of unknown SCell in non-DRX | Same as 4.5.3.1 | Same as 4.5.3.1 | Same as 4.5.3.1 |
| 4.5.3.5 EN-DC FR1 direct SCell activation at SCell addition of known SCell | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB |
| 4.5.3.6 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 160ms SCell measurement cycle | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB In signaling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signaling: 0 dB | During T1: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Ês2 / Noc2: +17dB Ês3 / Noc3: +17dB In signaling: A3-offset = -15dB |
| 4.5.3.7 EN-DC FR1 fast SCell Activation of known SCell in non-DRX for 640 ms SCell measurement cycle | Same as 4.5.3.6 | Same as 4.5.3.6 | Same as 4.5.3.6 |
| 4.5.4.1 EN-DC FR1 UE UL carrier RRC reconfiguration delay | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T2: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB During T3: Noc2: -102dBm/15kHz Noc3: -102dBm/15kHz Ês2 / Noc2: +16dB Ês3 / Noc3: +16dB |
| 4.5.5.1 EN-DC FR1 SSB-based beam failure detection and link recovery in non-DRX | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.2 EN-DC FR1 SSB-based beam failure detection and link recovery in DRX | Same as 4.5.5.1 | Same as 4.5.5.1 | Same as 4.5.5.1 |
| 4.5.5.3 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in non-DRX | q0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.4 EN-DC FR1 CSI-RS-based beam failure detection and link recovery in DRX | Same as 4.5.5.3 | Same as 4.5.5.3 | Same as 4.5.5.3 |
| 4.5.5.5 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | q0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q1 SSB during T1: 0dB -0.2dB q1 SSB during T2: 0dB -0.2dB q1 SSB during T3, T4 and T5: 0dB +0.2dB q1 CSI-RS during T1: 0dB -0.2dB q1 CSI-RS during T2: 0dB -0.2dB q1 CSI-RS during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB q1 CSI-RS during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1 CSI-RS during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.6 EN-DC FR1 Scell CSI-RS-based beam failure detection and SSB-based link recovery in DRX | Same as 4.5.5.5 | Same as 4.5.5.5 | Same as 4.5.5.5 |
| 4.5.5.7 EN-DC FR1 PSCell TRP specific SSB-based beam failure detection and link recovery in non-DRX | q0,0 SSB SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q0,1 SSB SNR during: T1-T5: 5dB q1,0 and q1,1, SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 and q1,1, SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 and q1,1, SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | q0,0, Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q0,1, Offset during: T1-T5: +0.8dB q1,0 and q1,1, SSB during T1: 0dB -0.2dB q1,0 and q1,1, SSB during T2: 0dB -0.2dB q1,0 and q1,1, SSB during T3, T4 and T5: 0dB +0.2dB | q0,0, SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in clause D.4.1.1 q0,1, SNR during: T1-T5: +5.8dB q1,0 and q1,1, SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 and q1,1, SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 and q1,1, SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.5.8 EN-DC FR1 SCell TRP specific CSI-RS-based beam failure detection and SSB-based link recovery in non-DRX | q0,0 CSI-RS SNR during: T1: 5dB T2: -3dB T3: -12dB T4: -12dB T5: -12dB q0,1 CSI-RS SNR during: T1-T5: 5dB q1,0 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,0 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB q1,1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10dB q1,1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10dB | Offset during: T1: +0.8dB T2: +0.8dB T3: -0.8dB T4: -0.8dB T5: -0.8dB q0,1, Offset during: T1-T5: +0.8dB q1,0 SSB during T1: 0dB -0.2dB q1,0 SSB during T2: 0dB -0.2dB q1,0 SSB during T3, T4 and T5: 0dB +0.2dB q1,1 SSB during T1: 0dB -0.2dB q1,1 SSB during T2: 0dB -0.2dB q1,1 SSB during T3, T4 and T5: 0dB +0.2dB | SNR during: T1: +5.8dB T2: -2.2dB T3: -12.8dB T4: -12.8dB T5: -12.8dB For testing of a UE which supports 4RX on all bands, the SNR during T3, T4 and T5 is modified as specified in section D.4.1.1 q0,1, SNR during: T1-T5: +5.8dB q1,0 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,0 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB q1,1 SSB during T1: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,1 SSB during T2: Noc2: -98dBm/15kHz Ês2 / Noc2: -10.2dB q1,1 SSB during T3, T4 and T5: Noc2: -98dBm/15kHz Ês2 / Noc2: +10.2dB |
| 4.5.6.1.1 EN-DC FR1 DCI-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB | During T1: 0dB 0dB During T2: 0dB 0dB During T3: 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |
| 4.5.6.1.2 EN-DC FR1 DCI-based DL active BWP switch with SCell in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T2: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB During T3: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB Noc3: -104dBm/15kHz Ês3 / Noc3: +17dB |
| 4.5.6.2.1 EN-DC FR1 RRC-based DL active BWP switch in non-DRX in synchronous EN-DC | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB | During T1: 0dB 0dB | During T1: Noc2: -104dBm/15kHz Ês2 / Noc2: +17dB |
| 4.5.7.1 EN-DC FR1 addition and release delay of known PSCell | For Test Configurations 1,2,4,5 Noc: -85dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS For Test Configurations 3,6 Noc: -85dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -93 dBm/SCS | 0dB 0dB -3dB -4dB 0dB -3dB | Noc: -85dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS Noc: -89dBm/15kHz Ês / Noc: 0.0dB B1-Threshold: -96 dBm/SCS |  |
| 4.5.8.1 EN-DC FR1 interruptions at switching between two uplink carriers | Noc: -104dBm/15kHz Ês / Noc: 17dB | 0dB 0dB | Noc: -104dBm/15kHz Ês / Noc: 17dB |  |

## << End of changes >>