3GPP TSG-RAN5 Meeting #102	R5-241203

Athens, Greece, Feb 26 – March 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-2 | CR | 1026 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add information in Power Control test cases specific to Power Class 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some Power Control Test Cases are marked for 'Power Class 3' in the comment column, but lack relevant information in the 'Additional Information' column. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add information in Power Control test cases specific to Power Class 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This could potentially cause confusion for those reviewing the specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

4.1	RF conformance test cases

NOTE:	To determine applicability of a test case, FGI support in combined or fdd-Add-UE-EUTRA-Capabilities or tdd-Add-UE-EUTRA-Capabilities is taken into account.

Table 4.1-1: Applicability of RF conformance test cases, ref. TS 36.521-1 [1]

| Clause | Title | Release | Applicability |  | Tested Bands / CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comments |  |  |  |
| Transmitter Characteristics |  |  |  |  |  |  |  |
| 6.2.2 | UE Maximum Output Power | Rel-8 | C186 | UE supporting E-UTRA Power Class 3 | D01 | FDD, TDD | Not required to be tested in Band 41 for Power Class 2 UE |
| 6.2.2_1 | UE Maximum Output Power for HPUE | Rel-10 | C39a | UE supporting E-UTRA Power Class 1 | D16 |  |  |
|  |  |  | C39b | UE supporting E-UTRA Power Class 2 | D17 |  |  |
| 6.2.2A.1 | UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) | Rel-10 | C19 | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E01 | FDD, TDD | Not required to be tested in Band 41 for Power Class 2 UE |
| 6.2.2A.1_3 | UE Maximum Output Power for CA and HPUE (intra-band contiguous DL CA and UL CA) | Rel-10 | C39b | UE supporting intraband contiguous CA and Power Class 2 | E01 | TDD |  |
| 6.2.2A.2 | UE Maximum Output Power for CA (inter-band DL CA and UL CA) | Rel-11 | C116 | UE supporting E-UTRA and inter-band DL CA and UL CA | E03 | FDD, TDD |  |
|  |  | Rel-14 | C305 | UE supporting E-UTRA and eLAA |  |  |  |
| 6.2.2A.3 | UE Maximum Output Power for CA (intra-band non-contiguous DL CA and UL CA) | Rel-11 | C115 | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA | E02 | FDD, TDD |  |
| 6.2.2A.4 | UE Maximum Output Power for CA (3UL CA) | Rel-13 | C19a | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E18 | TDD |  |
|  |  |  | C116a | UE supporting E-UTRA and inter-band DL CA and UL CA | E18 | FDD, TDD |  |
| 6.2.2B | UE Maximum Output Power for UL-MIMO | Rel-10 | C07 | UE supporting E-UTRA Power Class 3 and UL-MIMO | D05 | FDD, TDD |  |
| 6.2.2B_1 | HPUE Maximum Output Power for UL-MIMO | Rel-10 | C202 | UE supporting E-UTRA Power Class 2 and UL-MIMO | D05 | TDD |  |
| 6.2.2E | UE Maximum Output Power for UE category 0 | Rel-12 | C112 | UE supporting E-UTRA (UE category 0) | D01 | FDD, HD-FDD, TDD |  |
| 6.2.2EA | UE Maximum Output Power for UE category M1 | Rel-13 | C112a | UE supporting E-UTRA and UE category M1 | D01 | FDD, HD-FDD, TDD |  |
| 6.2.2EB | UE Maximum Output Power for UE category 1bis | Rel-13 | C112c | UE supporting E-UTRA and UE category 1bis | D01 |  |  |
| 6.2.2EC | UE Maximum Output Power for UE category M2 | Rel-14 | C112d | UE supporting E-UTRA and UE category M2 | D01 | FDD, HD-FDD, TDD |  |
| 6.2.2F | UE Maximum Output Power for category NB1 and NB2 | Rel-13 | C112b | UE supporting NB-IoT | D11, D18 | HD-FDD, TDD |  |
| 6.2.2FA | UE Maximum Output Power for category NB1 and NB2/Power Class 6 | Rel-14 | C325 | UE supporting NB-IoT and Power Class 6 | D11, D18 | HD-FDD, TDD |  |
| 6.2.2G.2 | UE Maximum Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Rel-14 | C320 | UE supporting E-UTRA and V2X Sidelink communication | E16 | FDD,TDD |  |
| 6.2.2G.4.1 | UE Maximum Output Power for V2X Communication / Power class 2 / Non-concurrent with E-UTRA uplink transmissions | Rel14 | C335 | UE supporting V2X Sidelink communication and Power Class 2 | D14 | TDD |  |
| 6.2.2_s | UE Maximum Output Power for subslot/slot TTI | Rel-15 | C352 | UE supporting E-UTRA FDD and Power Class 3 and subslot/slot TTI | D02 | FDD |  |
|  |  |  | C352a | UE supporting E-UTRA TDD and Power Class 3 and slot TTI | D03 | TDD | Not required to be tested in Band 41 for Power Class 2 UE |
| 6.2.5 | Configured UE transmitted Output Power | Rel-8 | C186 | UE supporting E-UTRA Power Class 3 | D01 | FDD, TDD | Not required to be tested in Band 41 for Power Class 2 UE |
| 6.2.5_s | Configured UE transmitted Output Power for subslot/slot TTI | Rel-15 | C352 | UE supporting E-UTRA FDD and Power Class 3 and subslot/slot TTI | D02 | FDD |  |
|  |  |  | C352a | UE supporting E-UTRA TDD and Power Class 3 and slot TTI | D03 | TDD | Not required to be tested in Band 41 for Power Class 2 UE |
| 6.2.5_1 | Configured UE transmitted Output Power for HPUE | Rel-10 | C39a | UE supporting E-UTRA Power Class 1 | D16 |  |  |
|  |  |  | C39b | UE supporting E-UTRA Power Class 2 | D17 |  |  |
| 6.2.5A.1 | Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) | Rel-10 | C19 | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E01 | FDD, TDD |  |
| 6.2.5A.3 | Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) | Rel-11 | C116 | UE supporting E-UTRA and inter-band DL CA and UL CA | E03 | FDD, TDD |  |
| 6.2.5A.4 | Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) | Rel-11 | C115 | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA | E02 | FDD, TDD |  |
| 6.2.5A.5 | Configured UE transmitted Output Power for CA (3UL CA) | Rel-13 | C19a | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E18 | TDD |  |
|  |  |  | C116a | UE supporting E-UTRA and inter-band DL CA and UL CA | E18 | FDD, TDD |  |
| 6.2.5B | Configured transmitted power for UL-MIMO | Rel-10 | C07 | UE supporting E-UTRA and UL_MIMO | D05 | FDD, TDD |  |
| 6.2.5E | Configured transmitted power for UE category 0 | Rel-12 | C112 | UE supporting E-UTRA (UE category 0) | D01 | FDD, HD-FDD, TDD |  |
| 6.2.5EA | Configured UE transmitted Power for UE category M1 | Rel-13 | C112a | UE supporting E-UTRA and UE category M1 | D01 | FDD, HD-FDD, TDD |  |
| 6.2.5EB | Configured UE transmitted Power for UE category 1bis | Rel-13 | C112c | UE supporting E-UTRA and UE category 1bis | D01 |  |  |
| 6.2.5EC | Configured UE transmitted Power for UE category M2 | Rel-14 | C112d | UE supporting E-UTRA and UE category M2 | D01 | FDD, HD-FDD, TDD |  |
| 6.2.5F | Configured UE transmitted Output Power for category NB1 and NB2 | Rel-13 | C112b | UE supporting NB-IoT | D12, D13, D18 | HD-FDD, TDD |  |
| 6.2.5FA | Configured UE transmitted Output Power for category NB1 and NB2/Power Class 6 | Rel-14 | C325 | UE supporting NB-IoT and Power Class 6 | D11, D18 | HD-FDD, TDD |  |
| 6.2.5G.1 | Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission | Rel-14 | C313 | UE supporting V2X Sidelink communication | D14 | TDD |  |
| 6.2.5G.2 | Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Rel-14 | C320 | UE supporting E-UTRA and V2X Sidelink communication | E16 | FDD,TDD |  |
| 6.2.5G.3 | Configured UE transmitted Output Power for V2X Communication / Intra-band contiguous multi-carrier operation | Rel-14 | C333 | UE supporting V2X Sidelink communication and multi-carrier configurations | E17 | TDD |  |
| 6.3.1 | Void |  |  |  |  |  |  |
| 6.3.2 | Minimum Output Power | Rel-8 | C113 | UE supporting E-UTRA | D01 | FDD, TDD |  |
| 6.3.2A.1 | Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) | Rel-10 | C19 | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E01 | FDD, TDD |  |
| 6.3.2A.2 | Minimum Output Power for CA (inter-band DL CA and UL CA) | Rel-11 | C116 | UE supporting E-UTRA and inter-band DL CA and UL CA | E03 | FDD, TDD |  |
| 6.3.2A.3 | Minimum Output Power for CA (intra-band non-contiguous DL CA and UL CA) | Rel-11 | C115 | UE supporting E-UTRA and inter-band DL CA and UL CA | E02 | FDD |  |
| 6.3.2A.4 | Minimum Output Power for CA (3UL CA) | Rel-13 | C19a | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E18 | TDD |  |
|  |  |  | C116a | UE supporting E-UTRA and inter-band DL CA and UL CA | E18 | FDD, TDD |  |
| 6.3.2B | Minimum Output Power for UL-MIMO | Rel-10 | C07 | UE supporting E-UTRA and UL_MIMO | D05 | FDD, TDD |  |
| 6.3.2E | Minimum Output Power for UE category 0 | Rel-12 | C112 | UE supporting E-UTRA (UE category 0 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.2EA | Minimum Output Power for UE category M1 | Rel-13 | C112a | UE supporting E-UTRA and UE category M1 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.2EB | Minimum Output Power for UE category 1bis | Rel-13 | C112c | UE supporting E-UTRA and UE category 1bis | D01 |  |  |
| 6.3.2EC | Minimum Output Power for UE category M2 | Rel-14 | C112d | UE supporting E-UTRA and UE category M2 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.2F | Minimum Output Power for category NB1 and NB2 | Rel-13 | C112b | UE supporting NB-IoT | D12, D13, D18 | HD-FDD, TDD |  |
| 6.3.2G.1 | Minimum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 | UE supporting V2X Sidelink communication | D14 | TDD |  |
| 6.3.2G.2 | Minimum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Rel-14 | C320 | UE supporting E-UTRA and V2X Sidelink communication | E16 | FDD,TDD |  |
| 6.3.2G.3 | Minimum output power for V2X Communication / Intra-band contiguous multi-carrier operation | Rel-14 | C333 | UE supporting V2X Sidelink communication and multi-carrier configurations | E17 | TDD |  |
| 6.3.3 | Transmit OFF Power | Rel-8 | C113 | UE supporting E-UTRA | D01 | FDD, TDD |  |
| 6.3.3A.1 | Transmit OFF Power for CA (intra-band contiguous DL CA and UL CA) | Rel-10 | C19 | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E01 | FDD, TDD |  |
| 6.3.3A.2 | UE Transmit OFF power for CA (inter-band DL CA and UL CA) | Rel-11 | C116 | UE supporting E-UTRA and inter-band DL CA and UL CA | E03 | FDD, TDD |  |
| 6.3.3A.3 | Transmit OFF Power for CA (intra-band non-contiguous DL CA and UL CA) | Rel-11 | C115 | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA | E02 | FDD, TDD |  |
| 6.3.3B | UE Transmit OFF power for UL-MIMO | Rel-10 | C07 | UE supporting E-UTRA and UL_MIMO | D05 | FDD, TDD |  |
| 6.3.3E | UE Transmit OFF power for UE category 0 | Rel-12 | C112 | UE supporting E-UTRA (UE category 0 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.3EA | UE Transmit OFF power for UE category M1 | Rel-13 | C112a | UE supporting E-UTRA and UE category M1 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.3EB | UE Transmit OFF power for UE category 1bis | Rel-13 | C112c | UE supporting E-UTRA and UE category 1bis | D01 |  |  |
| 6.3.3EC | UE Transmit OFF power for UE category M2 | Rel-14 | C112d | UE supporting E-UTRA and UE category M2 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.3F | Transmit OFF power for category NB1 and NB2 | Rel-13 | C112b | UE supporting NB-IoT | D12, D13, D18 | HD-FDD, TDD |  |
| 6.3.3G.1 | UE Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 | UE supporting V2X Sidelink communication | D14 | TDD |  |
| 6.3.3G.2 | UE Transmit OFF power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Rel-14 | C320 | UE supporting E-UTRA and V2X Sidelink communication | E16 | FDD,TDD |  |
| 6.3.3G.3 | UE Transmit OFF power for V2X Communication / Intra-band contiguous multi-carrier operation | Rel-14 | C333 | UE supporting V2X Sidelink communication and multi-carrier configurations | E17 | TDD |  |
| 6.3.4.1 | General ON/OFF time mask | Rel-8 | C113 | UE supporting E-UTRA | D01 | FDD, TDD |  |
| 6.3.4.1_s | General ON/OFF time mask for subslot/slot TTI | Rel-15 | C353 | UE supporting E-UTRA FDD and subslot/slot TTI | D02 | FDD |  |
|  |  |  | C353a | UE supporting E-UTRA TDD and slot TTI | D03 | TDD |  |
| 6.3.4.2.1 | PRACH time mask | Rel-8 | C113 | UE supporting E-UTRA | D01 | FDD, TDD |  |
| 6.3.4.2.2 | SRS time mask | Rel-8 | C113 | UE supporting E-UTRA | D01 | FDD, TDD |  |
| 6.3.4A.1.1 | General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | Rel-10 | C19 | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E01 | FDD, TDD |  |
| 6.3.4A.1.2 | General ON/OFF time mask for CA (inter-band DL CA and UL CA) | Rel-11 | C116 | UE supporting E-UTRA and inter-band DL CA and UL CA | E03 | FDD, TDD |  |
| 6.3.4A.1.3 | General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) | Rel-11 | C115 | UE supporting E-UTRA and intra-band non-contiguous DL CA and UL CA | E02 | FDD, TDD |  |
| 6.3.4A.1.4 | General ON/OFF time mask for CA (3UL CA) | Rel-13 | C19a | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E18 | TDD |  |
|  |  |  | C116a | UE supporting E-UTRA and inter-band DL CA and UL CA |  | FDD, TDD |  |
| 6.3.4A.1.5 | General ON/OFF time mask for CA (4UL CA) | Rel-11 | C334 | UE supporting E-UTRA and 4DL CA and 4UL CA | E20 | TDD |  |
| 6.3.4B.1 | ON/OFF time mask for UL-MIMO | Rel-10 | C07 | UE supporting E-UTRA and UL_MIMO | D05 | FDD, TDD |  |
| 6.3.4C.1 | General ON/OFF time mask for Dual Connectivity | Rel-12 | C224 | UE supporting Dual Connectivity | E03 | FDD, TDD |  |
| 6.3.4C.1_1 | General ON/OFF time mask for asynchronous Dual Connectivity | Rel-12 | C225 | UE supporting asynchronous Dual Connectivity | E03 | FDD, TDD |  |
| 6.3.4E.1 | General ON/OFF time mask for UE category 0 | Rel-12 | C112 | UE supporting E-UTRA (UE category 0 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.4E.2 | Prach and SRC ON/OFF time mask for UE category 0 | Rel-12 | C112 | UE supporting E-UTRA (UE category 0 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.4EA.1 | General ON/OFF time mask for UE category M1 | Rel-13 | C112a | UE supporting E-UTRA and UE category M1 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.4EA.2.1 | PRACH time mask for UE category M1 | Rel-13 | C112a | UE supporting E-UTRA and UE category M1 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.4EA.2.2 | SRS time mask for UE category M1 | Rel-13 | C112a | UE supporting E-UTRA and UE category M1 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.4EB.1 | General ON/OFF time mask for UE category 1bis | Rel-13 | C112c | UE supporting E-UTRA and UE category 1bis | D01 |  |  |
| 6.3.4EB.2.1 | PRACH time mask for UE category 1bis | Rel13 | C112c | UE supporting E-UTRA and UE category 1bis | D01 |  |  |
| 6.3.4EB.2.2 | SRS time mask for UE category 1bis | Rel13 | C112c | UE supporting E-UTRA and UE category 1bis |  |  |  |
| 6.3.4EC.1 | General ON/OFF time mask for UE category M2 | Rel-14 | C112d | UE supporting E-UTRA and UE category M2 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.4EC.2.1 | PRACH time mask for UE category M2 | Rel-14 | C112d | UE supporting E-UTRA and UE category M2 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.4EC.2.2 | SRS time mask for UE category M2 | Rel-14 | C112d | UE supporting E-UTRA and UE category M2 | D01 | FDD, HD-FDD, TDD |  |
| 6.3.4F.1 | General ON/OFF time mask for category NB1 and NB2 | Rel-13 | C112b | UE supporting NB-IoT | D12, D13, D18 | HD-FDD, TDD |  |
| 6.3.4F.2 | NPRACH time mask for category NB1 and NB2 | Rel-13 | C112b | UE supporting NB-IoT | D12, D13, D18 | HD-FDD, TDD |  |
| 6.3.4G.1 | General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 | UE supporting V2X Sidelink communication | D14 | TDD |  |
| 6.3.4G.2 | General ON/OFF time mask for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Rel-14 | C320 | UE supporting E-UTRA and V2X Sidelink communication | E16 | FDD,TDD |  |
| 6.3.4G.3 | General ON/OFF time mask for V2X Communication / Intra-band contiguous multi-carrier operation | Rel-14 | C333 | UE supporting V2X Sidelink communication and multi-carrier configurations | E17 | TDD |  |
| 6.3.4G.4 | PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 | UE supporting V2X Sidelink communication | D14 | TDD |  |
| 6.3.5.1 | Power Control Absolute Power Tolerance | Rel-8 | C186 | UE supporting E-UTRA Power Class 3 | D01 | FDD, TDD | Not required to be tested in Band 41 for Power Class 2 UE |
| 6.3.5.2 | Power Control Relative Power Tolerance | Rel-8 | C186 | UE supporting E-UTRA Power Class 3 | D01 | FDD, TDD | Not required to be tested in Band 41 for Power Class 2 UE |
| 6.3.5.3 | Aggregate Power Control Tolerance | Rel-8 | C186 | UE supporting E-UTRA Power Class 3 | D01 | FDD, TDD | Not required to be tested in Band 41 for Power Class 2 UE |
| 6.3.5A.1.1 | Power Control Absolute Power Tolerance for CA (intra-band contiguous DL CA and UL CA) | Rel-10 | C19 | UE supporting E-UTRA and intra-band contiguous DL CA and UL CA | E01 | FDD, TDD |  |

## << End of changes >>