3GPP TSG-RAN5 Meeting #102	R5-241269

# Athens, Greece, 26th February 2024 – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3040 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NTN SS-SINR measurement accuracy tests 14.6.3.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NTN SS-SINR measurement accuracy tests are missing in the specification. This CR implements these test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added core section 14.6.3.0 - Added new test cases 14.6.3.1 and 14.6.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would be missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.6.3.0, 14.6.3.1, 14.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 14.6	Measurement Performance Requirements

### 14.6.3	SS-SINR Requirements for SAN

#### 14.6.3.0	Minimum conformance requirements

##### 14.6.3.0.1	Intra-Frequency SS-SINR accuracy requirements in FR1

14.6.3.0.1.1	Absolute SS-SINR accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 14.6.3.0.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.2 for a corresponding Band.

-	Valid information for the SAN serving the target cell has been provided.

Table 14.6.3.0.1.1-1: SS-SINR Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 3 | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [3] | [4] | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| [3.5] | [4] | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB under non-HST scenarios. NOTE 4: NR operating band groups for satellite access in FR1 are as defined in clause 3A.4.1A. NOTE 5: The requirements apply for SSB Ês/Iot ≤5 dB with SCS 15kHz or 30kHz under NR high speed scenarios. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.12C.1.1.

##### 14.6.3.0.2	Inter-Frequency SS-SINR accuracy requirements in FR1

14.6.3.0.2.1	Absolute SS-SINR accuracy

The requirements for absolute accuracy of SS-SINR in this clause apply to a cell on a frequency in FR1 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 14.6.3.0.2.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.3 for a corresponding Band.

-	Valid information for the SAN serving the target cell has been provided.

Table 14.6.3.0.2.1-1: SS-SINR Inter frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 3 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [3] | [4] | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| [3.5] | [4] | -6 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The requirements apply for SSB Ês/Iot ≤ 25 dB. NOTE 4: NR operating band groups in FR1 are as defined in clause 3A.4.1A. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.14C.1.1.

14.6.3.0.2.2	Relative SS-SINR accuracy

The relative accuracy of SS-SINR in inter frequency case is defined as the SS-SINR measured from one cell on a frequency in FR1 compared to the SS-SINR measured from another cell on a different frequency in FR1.

The accuracy requirements in Table 14.6.3.0.2.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to 38.133 [6] Annex B.2.3 for a corresponding Band.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	| Channel 1_Io -Channel 2_Io |  20 dB

-	Valid information for the SAN serving the target cell has been provided.

Table 14.6.3.0.2.2-1: SS-SINR Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  | Note 2,4 | NR operating band groups Note 5 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 120 kHz | SCSSSB = 240 kHz |  |  |
| [3.5] | [4] | -3 | NR_FDD_SAB_FR1_A | -121 | -118 | N/A | -50 |
| [4] | [4] | -6 | Note 3 | Note 3 | Note 3 | Note 3 | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: The requirements apply for SSB Ês/Iot ≤ [25] dB. NOTE 5: NR operating band groups in FR1 are as defined in clause 3A.4.1A. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.14C.1.2.

#### 14.6.3.1	NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure needs to be updated

- Applicability may need to be updated

- Exceptions to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

#### Annex E and F need to be updated

14.6.3.1.1	Test purpose

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 14.6.3.0.1.

14.6.3.1.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access, SS-SINR measurements in FR1 and [inter-satellite measurements].

14.6.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.6.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.14.6.3.1.

14.6.3.1.4	Test description

In this test case all cells are on the same carrier frequency. Supported test configuration are shown in Table 14.6.3.1.4.1-1. The absolute accuracy of SS-SINR intra-frequency measurement is tested by using the parameters in Table 14.6.3.1.5-1. In all test cases, Cell 1 is the PCell and Cell 2 is the target cell.

14.6.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.6.3.1.4.1-1.

Table 14.6.3.1.4.1-1: Supported test configurations for NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.6.3.1-1 | GSO, NR FDD, 15kHz SSB SCS, data SCS 15kHz, 10 MHz BW |
| 14.6.3.1-2 | NGSO, NR FDD, 15kHz SSB SCS, data SCS 15kHz, 10 MHz BW |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.6.3.1.4.1-2

Table 14.6.3.1.4.1-2: Initial conditions for NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.6.3.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.6.3.1.4.1-3.

2.	Message contents are defined in clause 14.6.3.1.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

4.	The initial test environment conditions are setup according to section 14.0.5.

14.6.3.1.4.2	Test procedure

The test scenario comprises of two satellite access NR FDD intra-frequency cells as given in table 14.6.3.1.5-1.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to Table 14.6.3.1.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1 to configure periodic SS-SINR measurements on the UE.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodic MeasurementReport messages.

6.	After 10s wait from Step 4, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS-SINR value of Cell 2 reported by the UE is compared to the expected SS-SINR. If the value is outside the limits in Table 14.6.3.1.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 14.6.3.1.5-1 as appropriate and repeat steps 5-7.

14.6.3.1.4.3	Message contents

Editor’s notes: Message contents need to be updated.

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 14.6.3.1.4.3-1: Common Exception messages for NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-3 with Condition SS-SINR Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ and SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |

Table 14.6.3.1.4.3-2: ReportConfigNR for NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

14.6.3.1.5	Test requirements

Table 14.6.3.1.5-1 defines the primary level settings including test tolerances for NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access. Table 14.6.3.1.5-2 defines the requirement that the SS-SINR values contained in the MeasurementReport shall meet.

Table 14.6.3.1.5-1: Cell specific parameters for NR SA FR1 SS-SINR Measurement Accuracy for Satellite Access

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |
| TRS configuration |  | Config 1, 2 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |
| RMSI CORESET Reference Channel |  | Config 1, 2 |  | CR.1.1 FDD | - | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  | Config 1, 2 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| SMTC configuration |  | Config 1, 2 |  | SMTC.2 |  |  |  |
| Time offset with Cell 1 |  | Config 1, 2 | ms | - | 3 | - | 3 |
| SSB configuration |  | Config 1, 2 |  | SSB.1 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 | kHz | 15 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | NR_FDD_SAB_FR1_A, | dBm/15kHz | -93+TT |  | -116+TT |  |
|  |  | NR_FDD_SAB_FR1_B |  |  |  | -115.5+TT |  |
|  |  | NR_FDD_SAB_FR1_C |  |  |  | -115+TT |  |
|  |  | NR_FDD_SAB_FR1_D |  |  |  | -114.5+TT |  |
|  |  | NR_FDD_SAB_FR1_E |  |  |  | -114+TT |  |
|  |  | NR_FDD_SAB_FR1_F, |  |  |  | -113.5+TT |  |
|  |  | NR_FDD_SAB_FR1_G |  |  |  | -113+TT |  |
|  |  | NR_FDD_SAB_FR1_H |  |  |  | -112.5+TT |  |
|  |  | NR_FDD_SAB_FR1_I |  |  |  | -112+TT |  |
|  |  | NR_FDD_SAB_FR1_J |  |  |  | -111.5+TT |  |
| Note2 | Config 1, 2 |  | dBm/SCS | -93+TT |  | Same as Noc for 15kHz |  |
|  |  |  | dB | 0+TT | -3.19+TT | -5.46+TT | -5.46+TT |
|  |  |  | dB | 4.54+TT | 2.66+TT | -4+TT | -4+TT |
| SS-RSRPNote3 | Config 1, 2 | NR_FDD_SAB_FR1_A, | dBm/SCS | -88.46+TT | -90.34+TT | -120+TT | -120+TT |
|  |  | NR_FDD_SAB_FR1_B |  |  |  | -119.5+TT | -119.5+TT |
|  |  | NR_FDD_SAB_FR1_C |  |  |  | -119+TT | -119+TT |
|  |  | NR_FDD_SAB_FR1_D |  |  |  | -118.5+TT | -118.5+TT |
|  |  | NR_FDD_SAB_FR1_E |  |  |  | -118+TT | -118+TT |
|  |  | NR_FDD_SAB_FR1_F, |  |  |  | -117.5+TT | -117.5+TT |
|  |  | NR_FDD_SAB_FR1_G |  |  |  | -117+TT | -117+TT |
|  |  | NR_FDD_SAB_FR1_H |  |  |  | -116.5+TT | -116.5+TT |
|  |  | NR_FDD_SAB_FR1_I |  |  |  | -116+TT | -116+TT |
|  |  | NR_FDD_SAB_FR1_J |  |  |  | -115.5+TT | -115.5+TT |
| SS-SINR Note3 |  | NR_FDD_SAB_FR1_A, | dB | 0+TT | -3.19+TT | -5.46+TT | -5.46+TT |
|  |  | NR_FDD_SAB_FR1_B |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_C |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_D |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_E |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_F, |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_G |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_H |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_I |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_J |  |  |  |  |  |
| IoNote3 | Config 1, 2 | NR_FDD_SAB_FR1_A, | dBm/ 9.36MHz | -57.5+TT |  | -85.51+TT |  |
|  |  | NR_FDD_SAB_FR1_B |  |  |  | -85.01+TT |  |
|  |  | NR_FDD_SAB_FR1_C |  |  |  | -84.51+TT |  |
|  |  | NR_FDD_SAB_FR1_D |  |  |  | -84.01+TT |  |
|  |  | NR_FDD_SAB_FR1_E |  |  |  | -83.51+TT |  |
|  |  | NR_FDD_SAB_FR1_F, |  |  |  | -83.01+TT |  |
|  |  | NR_FDD_SAB_FR1_G |  |  |  | -82.51+TT |  |
|  |  | NR_FDD_SAB_FR1_H |  |  |  | -82.01+TT |  |
|  |  | NR_FDD_SAB_FR1_I |  |  |  | -81.51+TT |  |
|  |  | NR_FDD_SAB_FR1_J |  |  |  | -81.01+TT |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3A.4.1A. |  |  |  |  |  |  |  |

Table 14.6.3.1.5-2: SS-SINR Intra frequency absolute accuracy requirements for the reported values

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | [All bands] |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | FFS | FFS |
| Highest reported value (Cell 2) | FFS | FFS |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | FFS | FFS |
| Highest reported value (Cell 2) | FFS | FFS |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 14.6.3.2	NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access

Editor's Note:

- MU and TT analysis is incomplete

- Message contents may need to be updated

- Call setup and test procedure needs to be updated

- Applicability may need to be updated

- Exceptions to connection diagram may need to be updated

- Several sections are in brackets

- Several test parameters and configuration are still in brackets

- Annex E and F need to be updated

14.6.3.2.1	Test purpose

The purpose of this test is to verify that the SS-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 14.6.3.0.2.

14.6.3.2.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access, SS-SINR measurements in FR1 and [inter-satellite measurements].

14.6.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.6.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.14.6.3.2.

14.6.3.2.4	Test description

In this test case the two cells (i.e., Cell 1 and Cell 2) are on different carrier frequencies and measurement gaps are provided. Supported test configurations are shown in Table 14.6.3.2.4.1-1. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-frequency measurement are tested by using test parameters in Table 14.6.3.2.5-1. In all test cases, Cell 1 is the PCell and Cell 2 is target cell.

14.6.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.6.3.2.4.1-1.

Table 14.6.3.2.4.1-1: Supported test configurations for NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.6.3.2-1 | GSO, NR FDD, 15kHz SSB SCS, data SCS 15kHz, 10 MHz BW |
| 14.6.3.2-2 | NGSO, NR FDD, 15kHz SSB SCS, data SCS 15kHz, 10 MHz BW |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.6.3.2.4.1-2

Table 14.6.3.2.4.1-2: Initial conditions for NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.6.3.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | [For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part] |  | [4Rx support is FFS, exceptions may need to be removed] |

1.	The general test parameter settings are set up according to Table 14.6.3.2.4.1-3.

2.	Message contents are defined in clause 14.6.3.2.4.3.

3.	Cell 1 and Cell 2 are NR cells (PCell and neighbor cell, respectively) with the power level set according to clauses [C.1.2 and C.1.3] for this test. [Both Cell 1 and Cell 2 are satellite access cells].

4.	The initial test environment conditions are setup according to section 14.0.5.

14.6.3.2.4.2	Test procedure

The test scenario comprises of two satellite access NR FDD inter-frequency cells as given in table 14.6.3.2.5-1.

1.	Ensure the UE is in [state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5]. Cell 1 is the active cell.

2.	Set the parameters according to Table 14.6.3.2.5-1 as appropriate.

3.	The SS shall transmit an RRCReconfiguration message on Cell 1 to configure periodic SS-SINR measurements on the UE.

4.	The UE shall transmit an RRCReconfigurationComplete message.

5.	The UE shall transmit periodic MeasurementReport messages.

6.	After 10s wait from Step 4, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS-SINR value of Cell 2 reported by the UE is compared to the expected SS-SINR. If the value is outside the limits in Table 14.6.3.2.5-2 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 14.6.3.2.5-1 as appropriate and repeat steps 5-7.

14.6.3.2.4.3	Message contents

Editor’s notes: Message contents need to be updated.

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 14.6.3.2.4.3-1: Common Exception messages for NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ and GAP NEEDED Table H.3.1-3 with Condition SS-SINR Table H.3.1-5 Table H.3.1-6 with Condition Pattern #0 Table H.3.1-7 with Condition INTER-FREQ and SINR Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.2 |

Table 14.6.3.2.4.3-2: ReportConfigNR for NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access

| Derivation Path: 38.508-1 [14] Table 4.6.3-142 with condition PERIODICAL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  | PERIODICAL |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | false |  |  |
| rsrq | false |  |  |
| sinr | true |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

14.6.3.2.5	Test requirements

Table 14.6.3.2.5-1 defines the primary level settings including test tolerances for NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access. Table 14.6.3.2.5-2 and 14.6.3.2.5-3 define the absolute and relative requirements, correspondingly, that the SS-SINR values contained in the MeasurementReport shall meet.

Table 14.6.3.2.5-1: Cell specific parameters for NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| SSB ARFCN |  |  |  | freq1 | freq2 | freq1 | freq2 |  | freq1 |  | freq2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  | DLBWP.1.1 |  |  |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  |  |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| Gap pattern ID |  |  |  | 0 | - | 0 | - | 0 |  | - |  |
| TRS configuration |  | Config 1, 2 |  | TRS.1.1 FDD |  | TRS.1.1 FDD |  |  | TRS.1.1 FDD |  |  |
| PDSCH Reference measurement channel |  | Config 1, 2 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
| RMSI CORESET Reference Channel |  | Config 1, 2 |  | CR.1.1 FDD | - | CR.1.1 FDD | - |  | CR.1.1 FDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1, 2 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1, 2 | ms | - | 3 | - | 3 | - |  | 3 |  |
| SMTC configuration |  | Config 1, 2 |  | SMTC pattern 2 |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1, 2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1, 2 | kHz | 15 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1, 2 | NR_FDD_SAB_FR1_A, | dBm/15kHz | -88+TT |  | -108.5+TT |  |  | -119.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_B |  |  |  |  |  |  | -119+TT |  |  |
|  |  | NR_FDD_SAB_FR1_C |  |  |  |  |  |  | -118.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_D |  |  |  |  |  |  | -118+TT |  |  |
|  |  | NR_FDD_SAB_FR1_E |  |  |  |  |  |  | -117.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_F, |  |  |  |  |  |  | -117+TT |  |  |
|  |  | NR_FDD_SAB_FR1_G |  |  |  |  |  |  | -116.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_H |  |  |  |  |  |  | -116+TT |  |  |
|  |  | NR_FDD_SAB_FR1_I |  |  |  |  |  |  | -115.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_J |  |  |  |  |  |  | -115+TT |  |  |
| Note2 | Config 1, 2 |  | dBm/SCS | -88 |  | -108.5+TT |  |  | Same as Noc for 15kHz |  |  |
|  |  |  | dB | -1.75+TT | -1.75+TT | 20+TT | 20+TT |  | -4.0+TT |  | -4.0+TT |
|  |  |  | dB | -1.75+TT |  | 20+TT |  |  | -4.0+TT |  |  |
| SS-RSRP Note3 | Config 1, 2 | NR_FDD_SAB_FR1_A, | dBm/SCS | -89.75+TT |  | -88.5+TT |  |  | -123.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_B |  |  |  |  |  |  | -123+TT |  |  |
|  |  | NR_FDD_SAB_FR1_C |  |  |  |  |  |  | -122.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_D |  |  |  |  |  |  | -122+TT |  |  |
|  |  | NR_FDD_SAB_FR1_E |  |  |  |  |  |  | -121.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_F, |  |  |  |  |  |  | -121+TT |  |  |
|  |  | NR_FDD_SAB_FR1_G |  |  |  |  |  |  | -120.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_H |  |  |  |  |  |  | -120+TT |  |  |
|  |  | NR_FDD_SAB_FR1_I |  |  |  |  |  |  | -119.5+TT |  |  |
|  |  | NR_FDD_SAB_FR1_J |  |  |  |  |  |  | -119+TT |  |  |
| SS-SINRNote3 |  | NR_FDD_SAB_FR1_A, | dB | -1.75+TT |  | 20+TT |  |  | -4.0+TT |  |  |
|  |  | NR_FDD_SAB_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_F, |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_I |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_SAB_FR1_J |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1, 2 | NR_FDD_SAB_FR1_A, | dBm/ 9.36MHz | -57.83+TT |  | -60.5+TT |  |  | -90.09+TT |  |  |
|  |  | NR_FDD_SAB_FR1_B |  |  |  |  |  |  | -89.59+TT |  |  |
|  |  | NR_FDD_SAB_FR1_C |  |  |  |  |  |  | -89.09+TT |  |  |
|  |  | NR_FDD_SAB_FR1_D |  |  |  |  |  |  | -88.59+TT |  |  |
|  |  | NR_FDD_SAB_FR1_E |  |  |  |  |  |  | -88.09+TT |  |  |
|  |  | NR_FDD_SAB_FR1_F, |  |  |  |  |  |  | -87.59+TT |  |  |
|  |  | NR_FDD_SAB_FR1_G |  |  |  |  |  |  | -87.09+TT |  |  |
|  |  | NR_FDD_SAB_FR1_H |  |  |  |  |  |  | -86.59+TT |  |  |
|  |  | NR_FDD_SAB_FR1_I |  |  |  |  |  |  | -86.09+TT |  |  |
|  |  | NR_FDD_SAB_FR1_J |  |  |  |  |  |  | -85.59+TT |  |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3A.4.1A. |  |  |  |  |  |  |  |  |  |  |  |

Table 14.6.3.2.5-2: Reported SS-SINR Intra frequency absolute accuracy requirements for NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | [All bands] |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | FFS | FFS |
| Highest reported value (Cell 2) | FFS | FFS |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | FFS | FFS |
| Highest reported value (Cell 2) | FFS | FFS |

Table 14.6.3.2.5-3: Reported SS-SINR Intra frequency relative accuracy requirements for NR SA FR1-FR1 SS-SINR Measurement Accuracy for Satellite Access

|  | Test 1 | Test 2 |
| --- | --- | --- |
|  | All bands | [All bands] |
| Normal Conditions |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x - FFS | SS-SINR_x - FFS |
| Highest reported value (Cell 2) | SS-SINR_x + FFS | SS-SINR_x + FFS |
| Extreme Conditions |  |  |
| Lowest reported value (Cell 2) | SS-SINR_x - FFS | SS-SINR_x - FFS |
| Highest reported value (Cell 2) | SS-SINR_x + FFS | SS-SINR_x + FFS |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

<< End of Changes >>