3GPP TSG-RAN5 Meeting #102	R5-241275

# Athens, Greece, 26th February 2024 – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3045 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR-U general sections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | General sections for NR-U need some updates. This CR implements these. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the semi-static channel access period and the SSB-MTC configurations for shared channel access in Annex H. - Added new table for RSSI and Channel Occupancy message contents |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## H.3.10	RRC messages and information elements contents exceptions for Shared Spectrum Access

Table H.3.10-1: ServingCellConfigCommon for Shared Spectrum Access

| Derivation Path: TS 38.508-1, Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| mediumBitmap | 10000000 |  | SSB.1 CCA |
|  | 10100000 |  | SSB.2 CCA |
|  | 11000000 |  | SSB.3 CCA |
|  | 11110000 |  | SSB.4 CCA |
| } |  |  |  |
| channelAccessMode-r16 CHOICE { |  |  |  |
| dynamic | NULL |  | Dynamic |
| semiStatic SEQUENCE { |  |  | SemiStatic |
| period | ms2 |  |  |
| } |  |  |  |
| } |  |  |  |
| discoveryBurstWindowLength-r16 | ms1 | DBT.1 |  |
| ssb-PositionQCL-r16 | n1 |  | SSB.1 CCA SSB.2 CCA |
|  | n2 |  | SSB.3 CCA SSB.4 CCA |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB.X CCA | SSB pattern SSB.X CCA configured as per section A.3A (X = {1,2,3,4}) |
| Dynamic | Dynamic channel access |
| SemiStatic | Semi-static channel access |

Table H.3.10-2: SSB-MTC for Shared Spectrum Access

| Derivation Path: Table 38.508-1 Table 7.3.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SSB-MTC ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sf20 | 0 |  | SMTC.1 SMTC.2 |
|  | 10 |  | SMTC.4 SMTC.5 |
| } |  |  |  |
| duration | sf1 |  | SMTC.1 SMTC.4 |
|  | sf5 |  | SMTC.2 SMTC.5 |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SMTC.X | SMTC pattern SMTC.X configured as per section A.4 (X = {1,2,4,5}) |

Table H.3.10-3: MeasResultForRSSI for Shared Spectrum Access

| Derivation Path: TS 38.508-1, Table 4.6.3-78AA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultForRSSI ::= SEQUENCE { |  |  |  |
| rssi-Result-r16 | FFS |  |  |
| channelOccupancy-r16 | FFS |  |  |
| } |  |  |  |

<< End of Changes >>