3GPP TSG-RAN5 Meeting #102	R5-241325

# Athens, Greece, February 26th - March 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3078 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell test case 6.7.11.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM test case 6.7.11.2 is missing in TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRM test case 6.7.11.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test case 6.7.11.2 will remain missing in TS 38.533 and WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****Start of changes

#### 6.7.11.2	SA inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell

Editor's Note: This test case is incomplete in following aspects:

-	Message contents are missing

-	TT analysis is missing

##### 6.7.11.2.1	Test purpose

The purpose of this test is to verify that the CSI-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.9.2.1 and 10.1.9.2.2 in TS 38.133 [6].

##### 6.7.11.2.2			Test applicability

This test applies to all types of NR UE Rel-16 and forward supporting NR SA.

##### 6.7.11.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in Clause 6.7.11.0.2.

The normative reference for this requirement is in TS 38.133 [6] A.6.7.11.2.

##### 6.7.11.2.4	Test description

6.7.11.2.4.1	Initial conditions

Supported test configurations are shown in Table 6.7.11.2.4.1-1.

Table 6.7.11.2.4.1-1: CSI-RSRQ Inter frequency CSI-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.11.2.4.1-2.

Table 6.7.11.2.4.1-2: Initial conditions for CSI-RSRQ inter frequency accuracy in FR1

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] Clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] Clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.11.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without the LTE link |  |  |

1. Message contents are defined in Clause 6.7.11.1.4.3.

2. Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is an NR FR1 cell in the different frequency as Cell 1. Cell 2 is the target cell for CSI-RSRQ measurements. The connection setup is done according to the settings in Annex C.1.1.

6.7.11.2.4.2	Test procedure

1. Ensure the UE is in state RRC_CONNECTED CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] Clause 4.5.

2. Set the parameters according to Table 6.7.11.2.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message on Cell 1.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 3, the SS shall check the CSI-RSRQ reported values in the periodic MeasurementReport. The CSI-RSRQ value of Cell 2 reported by the UE is compared to the CSI-RSRQ value of Cell 1 reported by the UE. If the difference between both values is outside the limits (determined by Table 6.7.11.0.2.1-1 and Table 6.7.11.0.2.1-2) or the UE fails to report the measurement value for Cell 2 or Cell 1, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each sub-test in Table 6.7.11.2.5-1 as appropriate and repeat steps 5-7.

6.7.11.2.4.3	Message contents

[TBD]

##### 6.7.11.2.5	Test Requirements

The CSI-RSRQ measurement accuracy shall fulfil the requirements in Clause 6.7.11.0.2.

Table 6.7.11.2.5-1: CSI-RSRQ Inter frequency test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |  | Test 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| Gap pattern ID |  | Config 1,2,3 |  | 0 |  |  |  |  |  |  |  |
| BWP BW |  | Config 1 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | SR.1.1 FDD | - | SR.1.1 FDD | - |  | SR.1.1 FDD |  | - |
|  |  | Config 2,5 |  | SR.1.1 TDD |  | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | Config 3,6 |  | SR.2.1 TDD |  | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD | - | R.1.1 FDD | - |  | CR.1.1 FDD |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  | CR2.1 TDD |  |  | CR2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  | Config 1 |  | CCR.1.1 FDD | - | CCR.1.1 FDD | - |  | CCR.1.1 FDD |  | - |
|  |  | Config 2 |  | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | Config 3 |  | CCR2.1 TDD |  | CCR2.1 TDD |  |  | CCR2.1 TDD |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD | - | TRS.1.1 FDD | - |  | TRS.1.1 FDD |  | - |
|  |  | Config 2 |  | TRS.1.1 TDD |  | TRS.1.1 TDD |  |  | TRS.1.1 TDD |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  | TRS.1.2 TDD |  |  | TRS.1.2 TDD |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |  |
| Time offset with Cell 1 |  | Config 1,2 | s | - | 4.7 | - | 4.7 | - |  | 4.7 |  |
|  |  | Config 3 | s | - | 2.35 | - | 2.35 | - |  | 2.35 |  |
| CSI-RS configuration for RRM |  | Config 1 |  | CSI-RS.RRM.FR1.1 FDD |  |  |  |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.RRM.FR1.1 TDD |  |  |  |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.RRM.FR1.2 TDD |  |  |  |  |  |  |  |
| SSB configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.2 |  |  |  |  |  |  |  |
|  |  | Config 3 |  | SMTC.1 |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  | -106 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
| Note2 | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -86.27 |  | -113 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
| Note2 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/15kHz | -80.18 |  | -106 |  |  | -116 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -115.5 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -115 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -114.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -114 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -113.5 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -109.5 |  |  |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -83.27 |  | -110 |  |  | -113 |  |  |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -112.5 |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -112 |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -111.5 |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -111 |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.5 |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -110 |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -109.5 |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -106.5 |  |  |
|  |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
|  |  |  | dB | -1.75 |  | -1.75 |  |  | 3 |  | -1.75 |
| SS-RSRP/CSI-RSRPNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -81.93 | -81.93 | -107.75 | -107.75 |  | -113 |  | -117.75 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -112.5 |  | -117.25 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -112 |  | -116.75 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -111.5 |  | -116.25 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -111 |  | -115.75 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -110.5 |  | -115.2 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -110 |  | -114.75 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -109.5 |  | -114.25 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -106.5 |  | -111.25 |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -85.02 | -85.02 | -111.75 | -111.75 |  | -110 |  | -114.75 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -109.5 |  | -114.25 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -109 |  | -113.75 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -108.5 |  | -113.25 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -108 |  | -112.75 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -107.5 |  | -112.2 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -107 |  | -111.75 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -106.5 |  | -111.25 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -103.5 |  | -108.25 |
| SS-RSRQ/CSI-RSRQNote3 |  | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dB | -14.77 | -14.77 | -40.59 | -40.59 |  | 12.56T |  | 14.76T |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |  |  |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |  |  |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 | dBm/SCS | -50 |  | -75.83 |  |  | -83.28 |  | -85.83 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -82.78 |  | -85.33 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -82.28 |  | -84.83 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -81.78 |  | -84.33 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -81.28 |  | -83.83 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -80.78 |  | -83.33 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -80.28 |  | -82.83 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -79.78 |  | -82.33 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -76.78 |  | -79.33 |
|  | Config 3 | NR_FDD_FR1_A NR_TDD_FR1_A NOTE 6 |  | -50 |  | -76.73 |  |  | -77.19 |  | -79.73 |
|  |  | NR_FDD_FR1_B |  |  |  |  |  |  | -76.69 |  | -79.23 |
|  |  | NR_TDD_FR1_C |  |  |  |  |  |  | -76.19 |  | -78.73 |
|  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  | -75.69 |  | -78.23 |
|  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  | -75.19 |  | -77.73 |
|  |  | NR_FDD_FR1_F |  |  |  |  |  |  | -74.69 |  | -77.23 |
|  |  | NR_FDD_FR1_G |  |  |  |  |  |  | -74.19 |  | -76.73 |
|  |  | NR_FDD_FR1_H |  |  |  |  |  |  | -73.69 |  | -76.53 |
|  |  | NR_FDD_FR1_N |  |  |  |  |  |  | -70.69 |  | -73.53 |
| Propagation condition |  |  | - | AWGN | AWGN | AWGN | AWGN |  | AWGN |  | AWGN |
| Antenna configuration |  |  |  | 1x2 | 1x2 | 1x2 | 1x2 |  | 1x2 |  | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ/CSI-RSRQ, SS-RSRP/CSI-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in Clause 3.5.2 in TS 38.133 [6]. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

****End of changes