3GPP TSG-RAN5 Meeting #102	R5-241332

# Athens, Greece, February 26th - March 1st 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3085 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Clause 7.7.9.0 minimum conformance requirements for CSI-SINR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 7.7.9.0 minimum conformance requirements for CSI-SINR is missing in TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Clause 7.7.9.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clause 7.7.9.0 will remain missing in TS 38.533 and WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.9.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****Start of changes

### 7.7.9	CSI-SINR

### 7.7.9.0	Minimum conformance requirements

#### 7.7.9.0.1	Intra-frequency CSI-SINR accuracy requirements

##### [TS 38.133, Clause 10.1.13.2 and Clause 10.1.16]

##### 7.7.9.0.1.1	Absolute CSI-SINR accuracy

Unless otherwise specified, the requirements for absolute accuracy of CSI-SINR in this clause apply to a cell on the same frequency as that of the serving cell in FR2.

The accuracy requirements in Table 7.7.9.0.1.1-1 are valid under the following conditions:

-	Conditions defined in Clause 7.3 of TS 38.101-2 [3] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.12 in TS 38.133 [6] for a corresponding Band.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in Clause 7.3.4 of TS 38.101-2 [3].

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

•	Note: The reference measurement timing for intra-frequency measurement is serving cell timing.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 7.7.9.0.1.1-1.

Table 7.7.9.0.1.1-1: CSI-SINR Intra frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| ±3 | ±4 | ≥-3 | Same value as CSI_RP in Table B.2.8-2 in TS 38.133 [6], according to UE Power class, operating band and angle of arrival |  | -50 |
| ±3.5 | ±4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in Clauses 7.3.2 and 7.3.4 of TS 38.101-2 [3]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the reference point and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |

The reporting range of CSI-SINR for reporting is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 7.7.9.0.1.1-2. The range in the signaling may be larger than the guaranteed accuracy range.

Table 7.7.9.0.1.1-2: CSI-SINR measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CSI-SINR_0 | CSI-SINR <-23 | dB |
| CSI-SINR_1 | -23≤ CSI-SINR <-22.5 | dB |
| CSI-SINR_2 | -22.5≤ CSI-SINR <-22 | dB |
| CSI-SINR_3 | -22≤ CSI-SINR <-21.5 | dB |
| CSI-SINR_4 | -21.5≤ CSI-SINR <-21 | dB |
| .. | .. | … |
| CSI-SINR_123 | 38≤ CSI-SINR <38.5 | dB |
| CSI-SINR_124 | 38.5≤ CSI-SINR <39 | dB |
| CSI-SINR_125 | 39≤ CSI-SINR <39.5 | dB |
| CSI-SINR_126 | 39.5≤ CSI-SINR <40 | dB |
| CSI-SINR_127 | 40≤ CSI-SINR | dB |

#### 7.7.9.0.2	Inter-frequency CSI-SINR accuracy requirements

##### [TS 38.133, Clause 10.1.15.2 and Clause 10.1.16]

##### 7.7.9.0.2.1	Absolute CSI-SINR accuracy

The requirements for absolute accuracy of CSI-SINR in this clause apply to a cell on a frequency in FR2 that has different carrier frequency from the serving cell.

The accuracy requirements in Table 7.7.9.0.2.1-1 are valid under the following conditions:

-	Conditions defined in Clause 7.3 of TS 38.101-2 [3] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.13 in TS 38.133 [6] for a corresponding Band.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in Clause 7.3.4 of TS 38.101-2 [3].

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

•	Note: The reference measurement timing for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 7.7.9.0.2.1-1.

Table 7.7.9.0.2.1-1: CSI-SINR Inter frequency absolute accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| ±3 | ±4 | ≥-3 | Same value as CSI_RP in Table B.2.9-2 in TS 38.133 [6], according to UE Power class, operating band and angle of arrival |  | -50 |
| 3.5 | ±4 | ≥-4 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in Clauses 7.3.2 and 7.3.4 of TS 38.101-2 [3]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the reference point and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |

The reporting range of CSI-SINR for reporting is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 7.7.9.0.2.1-2. The range in the signaling may be larger than the guaranteed accuracy range.

Table 7.7.9.0.2.1-2: CSI-SINR measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CSI-SINR_0 | CSI-SINR <-23 | dB |
| CSI-SINR_1 | -23≤ CSI-SINR <-22.5 | dB |
| CSI-SINR_2 | -22.5≤ CSI-SINR <-22 | dB |
| CSI-SINR_3 | -22≤ CSI-SINR <-21.5 | dB |
| CSI-SINR_4 | -21.5≤ CSI-SINR <-21 | dB |
| .. | .. | … |
| CSI-SINR_123 | 38≤ CSI-SINR <38.5 | dB |
| CSI-SINR_124 | 38.5≤ CSI-SINR <39 | dB |
| CSI-SINR_125 | 39≤ CSI-SINR <39.5 | dB |
| CSI-SINR_126 | 39.5≤ CSI-SINR <40 | dB |
| CSI-SINR_127 | 40≤ CSI-SINR | dB |

##### 7.7.9.0.2.2	Relative CSI-SINR accuracy

The relative accuracy of CSI-SINR in inter frequency case is defined as the CSI-SINR measured from one cell on a frequency in FR2 compared to the CSI-SINR measured from another cell on a different frequency in FR2.

The accuracy requirements in Table 7.7.9.0.2.2-1 are valid under the following conditions:

-	Conditions defined in Clause 7.3 of TS 38.101-2 [3] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.13 in TS 38.133 [6] for a corresponding Band.

-	|CSI_RP1dBm - CSI_RP2dBm| £ 27 dB

-	| Channel 1_Io -Channel 2_Io | £ 20 dB

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in Clause 7.3.4 of TS 38.101-2 [3].

-	The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

•	Note: The reference measurement timing for inter-frequency measurement is up to UE implementation and shall be based on the timing of one of the target cells.

-	The bandwidth of CSI-RS is 48 PRBs and the density is 3.

•	The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements in Table 7.7.9.0.2.2-1.

Table 7.7.9.0.2.2-1: CSI-SINR Inter frequency relative accuracy in FR2

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS Ês/Iot | Io Note 2 range |  |  |
|  |  |  | Minimum Io |  | Maximum Io |
| dB | dB | dB | dBm / SCSCSI-RS Note 1 |  | dBm/BWChannel |
|  |  |  | SCSCSI-RS = 60kHz | SCSCSI-RS = 120kHz |  |
| ±3.5 | ±4 | ≥-3 | Same value as CSI_RP in Table B.2.9-2 in TS 38.133 [6], according to UE Power class, operating band and angle of arrival |  | -50 |
| ±4 | ±4 | ≥-6 |  |  |  |
| Note 1: Values based on Refsens and EIS spherical coverage as defined in Clauses 7.3.2 and 7.3.4 of TS 38.101-2 [3]. Applicable side condition selected depending on angle of arrival. Note 2: Io specified at the reference point and assumed to have constant EPRE across the bandwidth. Note 3: In the test cases, the CSI-RS Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. Note 4: The requirements apply for CSI-RS Ês/Iot ≤XdB. X=15 if timing offset between the reference measurement timing and the target CSI-RS is no larger than 0.5*CP, and X=4 if timing offset between the reference measurement timing and the target CSI-RS is larger than 0.5*CP but no larger than CP. |  |  |  |  |  |

****End of changes