3GPP TSG-RAN5 Meeting #102	R5-241342

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 51.010-1 | CR | 5170 | rev | - | Current version: | 13.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of coding scheme in USF testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TCs 14.16.1 and 14.16.1a testing of both USF/CS-2 and USF/CS-4 is performed. Since for USF/CS-4 testing much lower power levels are applied compared to PDTCH/CS-4 testing, call drops can occur as the UE cannot decode the PDTCH data. However, the decoding of the PDTCH is tested in a different step with appropriate power levels and is not the test purpose of the USF testing step. Moreover, USF/CS-4 and USF/CS-2 apply the same coding so that testing both is redundant. In order to avoid call drops and to avoid redundant testing, the USF/CS-4 test step is to be removed. In TCs 14.18.1, 14.18.1b, and 14.18.5 the usage of USF/MCS-4 and USF/MCS-9 can result in call drops and inconclusive testing results, since due to the low power levels applied for USF testing the UE cannot decode the PDTCH. As a remedy, a MCS with high error protection of the PDTCH has to be applied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TC 14.16.1 and 14.16.1a testing of USF/CS-4 has been skipped. In TCs 14.18.1, 14.18.1b, and 14.18.5, USF/MCS-4 and USF/MCS-9 have been replaced by USF/MCS-1 and USF/MCS-5, respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing of USF/CS-4 will remain instable in TCs 14.16.1, 14.16.1a, 14.18.1, 14.18.1b, and 14.18.5 resulting in inconclusive test case verdicts. Redundant testing will persist for TCs 14.16.1 and 14.16.1a. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.16.1.4.2, 14.16.1a.4.2, 14.18.1.4, 14.18.1b.4, 14.18.5.4, 14.18.5.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

## 14.16	GPRS receiver tests

Statistical testing of receiver BLER performance

Error Definition

Block Error Ratio (BLER):

The Block Error Ratio is the ratio of blocks received in error to the total number of received blocks, where a block is defined as received in error if the error detection functions in the receiver, operating in accordance with 3GPP TS 05.03, indicate an error as a the result of the Block Check Sequence (BCS).

For USF the Block Error Ratio is the ratio of incorrectly interpreted USF to the total number of received USF.

Test criteria

In the receiver tests for circuit switched channels, test error rates have been defined in order not to pass MS with a performance worse than the specification by 1 dB, with tests to be performed at the sensitivity and interference levels defined in 3GPP TS 05.05. For circuit switched channels 3GPP TS 05.05 defines the error rates at a fixed sensitivity or interference level.

For packet switched channels 3GPP TS 05.05 defines the receive or interference level at which a fixed Block Error Ratio is met. Therefore, for GPRS the receiver is tested with a 1 dB offset in the receive level and the interference level.

If the error events can be assumed to be random independent variables, outputs of stationary random processes with identical Gaussian distributions, the previous figures suggest a number of events not lower than 200 in AWGN channel and not lower than 600 in a multipath environment.

For multipath propagation conditions the hypothesis of stationary random processes does not generally hold. In case of non frequency hopping operation mode, the radio channel may be assumed to change 10 times per wavelength of travelled distance and to be short term stationary in between. So, in this case, the required observation time for having good statistical properties should not be lower (with some rounding) than that reported in table 14.16-1.

Table 14.16-1: Minimum test time according to propagation profile

|  | GSM 400, GSM 700, T-GSM 810, GSM 850 and GSM 900 |  |  |  | DCS 1 800 and PCS 1 900 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation Conditions | TUlow | TUhigh | HT | RA | TUlow | TUhigh | HT | RA |
| Min. test time (s) | 500 | 30 | 15 | 6 | 500 | 15 | 7,5 | 6 |

Table 14.16-2 details, for the different test conditions, the minimum number of blocks required in order to meet points 1) to 3): the corresponding test time (point 4) can be consequently computed.

Table 14.16-2: Test conditions

| Type of test | Type of sub-test | Propagation/ frequency conditions | Specified BLER % | Minimum No of blocks |
| --- | --- | --- | --- | --- |
| Sensitivity | PDTCH/CS-1 | static | 10 | 2000 |
| " | PDTCH/CS-1 | TUhigh/no FH | 10 | 6000 |
| " | PDTCH/CS-1 | TUhigh/FH | 10 | 6000 |
| " | PDTCH/CS-1 | RA/no FH | 10 | 6000 |
| " | PDTCH/CS-1 | HT/no FH | 10 | 6000 |
| " | PDTCH/CS-2 | static | 10 | 2000 |
| " | PDTCH/CS-2 | TUhigh/no FH | 10 | 6000 |
| " | PDTCH/CS-2 | TUhigh/FH | 10 | 6000 |
| " | PDTCH/CS-2 | RA/no FH | 10 | 6000 |
| " | PDTCH/CS-2 | HT/no FH | 10 | 6000 |
| " | PDTCH/CS-3 | static | 10 | 2000 |
| " | PDTCH/CS-3 | TUhigh/no FH | 10 | 6000 |
| " | PDTCH/CS-3 | TUhigh/FH | 10 | 6000 |
| " | PDTCH/CS-3 | RA/no FH | 10 | 6000 |
| " | PDTCH/CS-3 | HT/no FH | 10 | 6000 |
| " | PDTCH/CS-4 | static | 10 | 2000 |
| " | PDTCH/CS-4 | TUhigh/no FH | 10 | 6000 |
| " | PDTCH/CS-4 | TUhigh/FH | 10 | 6000 |
| " | USF/CS-1 | static | 1 | 20000 |
| " | USF/CS-1 | TUhigh/no FH | 1 | 60000 |
| " | USF/CS-1 | TUhigh/FH | 1 | 60000 |
| " | USF/CS-1 | RA/no FH | 1 | 60000 |
| " | USF/CS-1 | HT/no FH | 1 | 60000 |
| " | USF/CS-2/CS-3/CS-4 | static | 1 | 20000 |
| " | USF/CS-2/CS-3/CS-4 | TUhigh/noFH | 1 | 60000 |
| " | USF/CS-2/CS-3/CS-4 | TUhigh/FH | 1 | 60000 |
| " | USF/CS-2/CS-3/CS-4 | RA/no FH | 1 | 60000 |
| " | USF/CS-2/CS-3/CS-4 | HT/no FH | 1 | 60000 |
| Co-channel | PDTCH/CS-1 | TUlow/no FH | 10 | 6000, but minimum of 500s |
| ,, | PDTCH/CS-1 | TUhigh/no FH | 10 | 6000 |
| ,, | PDTCH/CS-1 | TUhigh/FH | 10 | 6000 |
| ,, | PDTCH/CS-1 | RA/no FH | 10 | 6000 |
| ,, | PDTCH/CS-2 | TUlow/no FH | 10 | 6000, but minimum of 500s |
| ,, | PDTCH/CS-2 | TUhigh/no FH | 10 | 6000 |
| ,, | PDTCH/CS-2 | TUhigh/FH | 10 | 6000 |
| ,, | PDTCH/CS-2 | RA/no FH | 10 | 6000 |
| ,, | PDTCH/CS-3 | TUlow/no FH | 10 | 6000, but minimum of 500s |
| ,, | PDTCH/CS-3 | TUhigh/no FH | 10 | 6000 |
| ,, | PDTCH/CS-3 | TUhigh/FH | 10 | 6000 |
| ,, | PDTCH/CS-3 | RA/no FH | 10 | 6000 |
| ,, | PDTCH/CS-4 | TUlow/no FH | 10 | 6000, but minimum of 500s |
| ,, | PDTCH/CS-4 | TUhigh/no FH | 10 | 6000 |
| " | PDTCH/CS-4 | TUhigh/FH | 10 | 6000 |
| " | USF/CS-1 | TUlow/no FH | 1 | 60000 |
| " | USF/CS-1 | TUhigh/no FH | 1 | 60000 |
| " | USF/CS-1 | TUhigh/FH | 1 | 60000 |
| " | USF/CS-1 | RA/no FH | 1 | 60000 |
| " | USF/CS-2/CS-3/CS-4 | TUlow/no FH | 1 | 60000 |
| " | USF/CS-2/CS-3/CS-4 | TUhigh/no FH | 1 | 60000 |
| " | USF/CS-2/CS-3/CS-4 | TUhigh/FH | 1 | 60000 |
| " | USF/CS-2/CS-3/CS-4 | RA/no FH | 1 | 60000 |
| NOTE 1: For PDTCH sub-tests under fading conditions, the number of RLC blocks indicated above shall be transmitted on each downlink timeslot of the multislot configuration. NOTE 2: For USF sub-tests under fading conditions, the number of RLC blocks indicated above shall be per uplink timeslot of the multislot configuration. |  |  |  |  |

### 14.16.1	Minimum Input level for Reference Performance

14.16.1.1	Definition

The minimum input level is the signal level at the MS receiver input at which a certain BLER is met.

14.16.1.2	Conformance requirement

1.	The block error rate (BLER) performance shall not exceed 10 % at input levels according to the table below.

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| PDTCH/CS-1 | dBm | -104 | -104 | -104 | -104 | -103 |
| PDTCH/CS-2 | dBm | -104 | -100 | -101 | -101 | -99 |
| PDTCH/CS-3 | dBm | -104 | -98 | -99 | -98 | -96 |
| PDTCH/CS-4 | dBm | -101 | -90 | -90 | * | * |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| PDTCH/CS-1 | dBm | -104 | -104 | -104 | -104 | -103 |
| PDTCH/CS-2 | dBm | -104 | -100 | -100 | -101 | -99 |
| PDTCH/CS-3 | dBm | -104 | -98 | -98 | -98 | -94 |
| PDTCH/CS-4 | dBm | -101 | -88 | -88 | * | * |

The input levels given in the above Table are referenced to normal GSM 900 MS, and have to be corrected by the following values for other MS:

GSM 400, GSM 700, GSM 850 and GSM 900 small MS	+2 dB

DCS 1800 class 1 or 2 MS	+2/+4 dB**

DCS 1800 class 3 and PCS 1 900 class 1 or 2 MS	+2 dB

PCS 1 900 class 3 MS		0 dB

**	For all DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

3GPP TS 05.05, table 1a; 3GPP TS 05.05, subclause 6.2.

2	The block error rate (BLER) performance shall not exceed 1 % at input levels according to the table below.

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| USF/CS-1 | dBm | < -104 | -101 | -103 | -103 | -101 |
| USF/CS-2 to 4 | dBm | < -104 | -103 | -104 | -104 | -104 |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| USF/CS-1 | dBm | < -104 | -103 | -103 | -103 | -101 |
| USF/CS-2 to 4 | dBm | < -104 | -104 | -104 | -104 | -103 |

The input levels given in the above Table are referenced to normal GSM 900 MS, and have to be corrected by the following values for other MS:

GSM 400, GSM 700, GSM 850 and GSM 900 small MS		+2 dB

DCS 1800 class 1 or 2 MS	+2/+4 dB**

DCS 1800 class 3 and PCS 1 900 class 1 or 2 MS	+2 dB

PCS 1 900 class 3 MS	0 dB

**	For all DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

3GPP TS 05.05, table 1a; 3GPP TS 05.05, subclause 6.2.

3.	The BLER shall not exceed the conformance requirements given in 1. - 2. under extreme conditions; 3GPP TS 05.05, subclause 6.2 and annex D subclauses D.2.1 and D.2.2.

4.	The reference sensitivity performance specified above need not be met in the following cases:

for MS at the static channel, if the received level on either of the two adjacent timeslots to the wanted exceed the wanted timeslot by more than 20 dB;

for MS on a multislot configuration, if the received level on any of the timeslots belonging to the same multislot configuration as the wanted time slot, exceed the wanted time slot by more than 6 dB;

The interfering adjacent time slots shall be static with valid GSM signals in all cases;

3GPP TS 05.05, subclause 6.2.

5)	For an MS allocated a USF on a PDCH with a random RF input or a valid PDCH signal with a random USF not equal to the allocated USF, the overall reception shall be such that the MS shall detect the allocated USF in less than 1% of the radio blocks. This requirement shall be met for all input levels up to -40 dBm.

3GPP TS 05.05, subclause 6.4

3GPP TS 45.05 subclause 2:

For T-GSM 810 the requirements for GSM 900 shall apply, apart for those parameters for which a separate requirement exists.

14.16.1.3	Test purpose

NOTE:	This test is performed under STATIC propagation conditions to allow implicit testing of the ability of the MS to hop over the full band. The tests under dynamic propagation conditions are better suited to test the minimum input level for reference BLER performance conformance but cannot test hopping over the full band due to limited bandwidth of available fading simulators.

1.	To verify that that the MS sends a Packet Not Acknowledge in the Packet Downlink Ack/Nack in case of a the Block Check Sequence indicating a Block Error.

2.	To verify that the MS does not exceed conformance requirement 1 for CS-3 and CS-4 under STATIC, TUhigh, HT and RA propagation conditions with an allowance for the statistical significance of the test.

3.	To verify that the MS does not exceed conformance requirement 2 under HT propagation conditions with an allowance for the statistical significance of the test.

4.	To verify that the MS does not exceed conformance requirement 3 under STATIC, TUhigh, HT and RA propagation conditions for the PDTCH, and HT propagation conditions for the USF, with an allowance for the statistical significance of the test.

5.	To verify that the MS meets the conformance requirements also 1 and 2 for the conditions allowed by conformance requirement 4, with an allowance for the statistical significance of the test.

6.	To verify that the MS meets conformance requirement 5, with an allowance for the statistical significance of the test.

14.16.1.4	Method of test

14.16.1.4.1	Initial conditions

NOTE 1:	The BA list sent on the BCCH and SACCH will indicate at least six surrounding cells with at least one near to each band edge. It is not necessary to generate any of these BCCHs but, if provided the signal strengths of BCCHs shall be in the range 15 dBVemf(  ) to 35 dBVemf(  ). Surrounding cell signal levels and cell reselection parameters are set so that the MS will not try a cell reselection.

NOTE 2:	The ARFCN of any BCCH shall not be co-channel or on adjacent channels to the wanted traffic channel.

NOTE 3:	When frequency hopping is used, the traffic channel may fall on any of the ARFCNs defined in clause 6.

NOTE 4:	The PSI1 message is included in the PACCH  when the MS is in packet transfer mode. The PBCCH_CHANGE_MARK value in PSI1 is not changed. This, together with preventing cell reselection as per Note 1, ensures that the MS is highly unlikely to suspend the TBF (3GPP TS 04.60 subclause 5.5.1.4.2 Suspension of operation to receive system operation), and thus making the effect of TBF suspension statistically insignificant for the test result.

A call is set up according to the generic call set up procedure for packet switched on an ARFCN in the Mid range, on the maximum number of receive timeslots, with the MS transmitting at maximum power. The power control parameter ALPHA (α) is set to 0.

For the ACK/NACK BLER and the BCS BLER parts of the test case, a downlink TBF will be established.

For the USF BLER parts of the test case the Test Mode defined in 3GPP TS 04.14 (subclause 5.4) will be used for uplink TBF. If the MS is capable of both:

Mode (a)	transmitting pseudo-random data sequence in RLC data blocks;

Mode (b)	transmitting looped-back RLC data blocks;

then Mode (a) will be used.

If Mode (b) is used then the SS sends the pseudo-random data sequence specified for Mode (a) on the downlink for loopback on the uplink.

14.16.1.4.2	Procedure

a)	The SS transmits packets under Static propagation conditions, using CS-3 coding at a level of 1 dB above the level given in conformance reference 1. Out of the 400 blocks transmitted by the SS, 20 blocks are sent with incorrect BCS, at (pseudo) random positions. The SS checks, for the blocks it transmitted with incorrect BCS, whether or not the MS Packet Downlink Ack/Nack as sent by the MS indicates these blocks as not acknowledged.

b)	The SS transmits packets under static conditions, with the MS commanded to hopping mode using the hopping sequence used in clause 6, and using CS-3 coding to the MS on all allocated timeslots, at a level of 1 dB above the level given in the table in conformance requirement 1. On the time slots not allocated to the MS, the SS transmits at a level of 20 dB above the level given in the table in conformance requirement 1. This implicitly tests adjacent time slot rejection.

c)	The SS counts the number of blocks transmitted with CS-3 and the number of these blocks not acknowledged based on the content of the Ack/Nack Description information element (see 3GPP TS 04.60, subclause 12.3) in the Packet Downlink Ack/Nack as sent from the MS to the SS on the PACCH.

NOTE:	Due to the error rates related to the USF, the MS is likely to occasionally miss its USF for transmitting the Packet Downlink Ack/Nack. As this requirement is not verified in this part of the test, the SS then again assigns uplink resources so the MS can sent this message.

d)	Once the number of blocks transmitted with CS-3 as counted in step c) reaches or exceeds the minimum number of blocks as given in table 14.16-2, the SS calculates the Block error ratio. The SS resets both counters.

e)	The SS repeats step b) to d) with the following four fading conditions and hopping modes: TUhigh/noFH, TUhigh/FH, HT/noFH and RA/noFH. For these tests with fading channels , the SS does not transmit on the timeslots not allocated to the MS.

f)	The SS repeats steps b) to d) using CS-4 coding with the following three fading conditions:  Static/FH, TUhigh/noFH and TUhigh/FH. For these tests with fading channels , the SS does not transmit on the timeslots not allocated to the MS.

g)	The SS repeats steps b) to f) under extreme test conditions.

h)	This step is only performed for a multislot MS. The SS establishes the normal test conditions with the exceptions in the parameter settings of Packet Downlink Assignment message:

- P0 = 14 dBm;

- BTS_PWR_CTRL_MODE = Mode A;

- PR_MODE = B.

Furthermore, the SS has to set the PR fields in the MAC headers of each downlink RLC data block to correspond the applied downlink power level, as defined below. The SS repeats steps b) to d) with only one of the active timeslots at 1 dB above the level at which the reference sensitivity performance shall be met, and all other timeslots belonging to the same multislot configuration at a level of 6 dB above this timeslot.

i)	The SS establishes the normal test conditions, and sets the fading function to HT/noFH.

j)	The SS sets the value of the USF/CS-1 such as to allocate the uplink to the MS, transmitting at a level of 1 dB above the level given in the table in conformance requirement 2.

k)	The SS counts the number of times the USF is allocated to the MS, and the number of times the MS does not transmit while being allocated the uplink.

l)	Once the number of USF/CS-1 allocating the uplink for the MS as counted in step k) reaches or exceeds the minimum number of blocks as given in table 14.16-2, the SS calculates the Block error ratio. The SS resets both counters.

m)	The SS repeats steps j) to l) using USF/CS2 coding.

NOTE:	Since coding for USF-bits is identical for CS2, CS3, and CS4, it’s not required to perform the step for CS3 or CS4.

n)	The SS repeats steps i) to m) under extreme test conditions.

o)	The SS establishes normal test condition and a static channel. The SS sets the value of the USF/CS-1 to all values randomly, with the exception of the one allocated to the MS, transmitting at 3 dB below the level at which reference performance shall be met, and counts the number of times the MS transmits on the uplink. This is done for 2 000 blocks.

14.16.1.5	Test requirements

In step a) the Packet Downlink Ack/Nack as sent by the MS shall indicate every block transmitted by the SS with incorrect BCS as not acknowledged.

The block error ratio, as calculated by the SS for different channels and under the different propagation conditions, under any combination of normal and extreme test voltages and ambient temperatures, shall not exceed the conformance requirement.

In step o) the MS shall transmit no more than 25 times.

In the case when downlink power control is not used and the output power used on the transmitted blocks is not equal to (BCCH level – Pb) then the MS is not required to fulfil 3GPP TS 05.05 requirements for the first 25 blocks addressed to this MS (3GPP TS 05.08, subclause 10.2.2).

NOTE:	This is stated in the Rel 99 version of 3GPP TS 05.08.

### 14.16.1a	Minimum Input level for Reference Performance in TIGHTER configuration

14.16.1a.1	Definition

The minimum input level is the signal level at the MS receiver input at which a certain BLER is met.

14.16.1a.2	Conformance requirement

For a MS indicating support for TIGHTER Capability (see 3GPP TS 24.008), the minimum input signal levels for which the reference performance shall be met are specified in table 1w, according to the propagation condition. The performance requirements for GSM 400 and GSM 700 systems are as for GSM 900 in table 1w, except that the GSM 400 MS speed is doubled from that of GSM 900, e.g. TU50 becomes TU100, and the GSM 700 MS speed is increased by a factor of 1.2, e.g. TU50 becomes TU60.

1.	The block error rate (BLER) performance shall not exceed 10 % at input levels according to the table below.

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| PDTCH/CS-1 | dBm | -105 | -106 | -106 | -105,5 | -104,5 |
| PDTCH/CS-2 | dBm | -105 | -102 | -103 | -102,5 | -100,5 |
| PDTCH/CS-3 | dBm | -105 | -100 | -101 | -99,5 | -97,5 |
| PDTCH/CS-4 | dBm | -102 | -92 | -92 | (note 2) | (note 2) |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| PDTCH/CS-1 | dBm | (note 3) | -106 | -106 | -105,5 | -104,5 |
| PDTCH/CS-2 | dBm | (note 3) | -102 | -102 | -102,5 | -100,5 |
| PDTCH/CS-3 | dBm | (note 3) | -100 | -100 | -99,5 | -95,5 |
| PDTCH/CS-4 | dBm | (note 3) | -90 | -90 | (note 2) | (note 2) |
| NOTE 2: PDTCH for MCS-x cannot meet the reference performance for some propagation conditions. NOTE 3: The requirements for the DCS 1800 & PCS 1900 Static propagation condition are the same as for the GSM 850 & GSM 900 Static propagation condition, the requirements for the GSM 850 & GSM 900 TU50 (ideal FH) and DCS 1800 & PCS 1900 TU50 (ideal FH) propagation conditions are the same as for the DCS 1800 & PCS 1900 TU50 (no FH) propagation condition, and the requirements for the DCS 1800 & PCS 1900 RA130 (no FH) propagation condition are the same as for the GSM 850 & GSM 900 RA250 (no FH) propagation condition. |  |  |  |  |  |  |

The input levels given in the above Table are referenced to normal GSM 900 MS, and have to be corrected by the following values for other MS:

GSM 400, GSM 700, GSM 850 and GSM 900 small MS	+2 dB

DCS 1800 class 1 or 2 MS	+2/+4 dB**

DCS 1800 class 3 and PCS 1 900 class 1 or 2 MS	+2 dB

PCS 1 900 class 3 MS		0 dB

**	For all DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

3GPP TS 05.05, table 1a; 3GPP TS 05.05, subclause 6.2.

2.	The block error rate (BLER) performance shall not exceed 1 % at input levels according to the table below.

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| USF/CS-1 | dBm | < -104 | -101 | -103 | -103 | -101 |
| USF/CS-2 to 4 | dBm | < -104 | -103 | -104 | -104 | -104 |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| USF/CS-1 | dBm | < -104 | -103 | -103 | -103 | -101 |
| USF/CS-2 to 4 | dBm | < -104 | -104 | -104 | -104 | -103 |

The input levels given in the above Table are referenced to normal GSM 900 MS, and have to be corrected by the following values for other MS:

GSM 400, GSM 700, GSM 850 and GSM 900 small MS		+2 dB

DCS 1800 class 1 or 2 MS	+2/+4 dB**

DCS 1800 class 3 and PCS 1 900 class 1 or 2 MS	+2 dB

PCS 1 900 class 3 MS	0 dB

**	For all DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

3GPP TS 05.05, table 1a; 3GPP TS 05.05, subclause 6.2.

3.	The BLER shall not exceed the conformance requirements given in 1. - 2. under extreme conditions; 3GPP TS 05.05, subclause 6.2 and annex D subclauses D.2.1 and D.2.2.

4.	The reference sensitivity performance specified above need not be met in the following cases:

for MS at the static channel, if the received level on either of the two adjacent timeslots to the wanted exceed the wanted timeslot by more than 20 dB;

for MS on a multislot configuration, if the received level on any of the timeslots belonging to the same multislot configuration as the wanted time slot, exceed the wanted time slot by more than 6 dB;

The interfering adjacent time slots shall be static with valid GSM signals in all cases;

3GPP TS 05.05, subclause 6.2.

5.	For an MS allocated a USF on a PDCH with a random RF input or a valid PDCH signal with a random USF not equal to the allocated USF, the overall reception shall be such that the MS shall detect the allocated USF in less than 1% of the radio blocks. This requirement shall be met for all input levels up to -40 dBm.

3GPP TS 05.05, subclause 6.4

3GPP TS 45.05 subclause 2:

For T-GSM 810 the requirements for GSM 900 shall apply, apart for those parameters for which a separate requirement exists.

14.16.1a.3	Test purpose

NOTE:	This test is performed under STATIC propagation conditions to allow implicit testing of the ability of the MS to hop over the full band. The tests under dynamic propagation conditions are better suited to test the minimum input level for reference BLER performance conformance but cannot test hopping over the full band due to limited bandwidth of available fading simulators.

1.	To verify that that the MS sends a Packet Not Acknowledge in the Packet Downlink Ack/Nack in case of a the Block Check Sequence indicating a Block Error.

2.	To verify that the MS does not exceed conformance requirement 1 for CS-3 and CS-4 under STATIC, TUhigh, HT and RA propagation conditions with an allowance for the statistical significance of the test.

3.	To verify that the MS does not exceed conformance requirement 2 under HT propagation conditions with an allowance for the statistical significance of the test.

4.	To verify that the MS does not exceed conformance requirement 3 under STATIC, TUhigh, HT and RA propagation conditions for the PDTCH, and HT propagation conditions for the USF, with an allowance for the statistical significance of the test.

5.	To verify that the MS meets the conformance requirements also 1 and 2 for the conditions allowed by conformance requirement 4, with an allowance for the statistical significance of the test.

6.	To verify that the MS meets conformance requirement 5, with an allowance for the statistical significance of the test.

14.16.1a.4	Method of test

14.16.1a.4.1	Initial conditions

NOTE 1:	The BA list sent on the BCCH and SACCH will indicate at least six surrounding cells with at least one near to each band edge. It is not necessary to generate any of these BCCHs but, if provided the signal strengths of BCCHs shall be in the range 15 dBVemf(  ) to 35 dBVemf(  ). Surrounding cell signal levels and cell reselection parameters are set so that the MS will not try a cell reselection.

NOTE 2:	The ARFCN of any BCCH shall not be co-channel or on adjacent channels to the wanted traffic channel.

NOTE 3:	When frequency hopping is used, the traffic channel may fall on any of the ARFCNs defined in clause 6.

NOTE 4:	The PSI1 message is included in the PACCH  when the MS is in packet transfer mode. The PBCCH_CHANGE_MARK value in PSI1 is not changed. This, together with preventing cell reselection as per Note 1, ensures that the MS is highly unlikely to suspend the TBF (3GPP TS 04.60 subclause 5.5.1.4.2 Suspension of operation to receive system operation), and thus making the effect of TBF suspension statistically insignificant for the test result.

A call is set up according to the generic call set up procedure for packet switched on an ARFCN in the Mid range, on the maximum number of receive timeslots, with the MS transmitting at maximum power. The power control parameter ALPHA (α) is set to 0.

For the ACK/NACK BLER and the BCS BLER parts of the test case, a downlink TBF will be established.

For the USF BLER parts of the test case the Test Mode defined in 3GPP TS 04.14 (subclause 5.4) will be used for uplink TBF. If the MS is capable of both:

Mode (a)	transmitting pseudo-random data sequence in RLC data blocks;

Mode (b)	transmitting looped-back RLC data blocks;

then Mode (a) will be used.

If Mode (b) is used then the SS sends the pseudo-random data sequence specified for Mode (a) on the downlink for loopback on the uplink.

14.16.1a.4.2	Procedure

a)	The SS transmits packets under Static propagation conditions, using CS-3 coding at a level of 1 dB above the level given in conformance reference 1. Out of the 400 blocks transmitted by the SS, 20 blocks are sent with incorrect BCS, at (pseudo) random positions. The SS checks, for the blocks it transmitted with incorrect BCS, whether or not the MS Packet Downlink Ack/Nack as sent by the MS indicates these blocks as not acknowledged.

b)	The SS transmits packets under static conditions, with the MS commanded to hopping mode using the hopping sequence used in clause 6, and using CS-3 coding to the MS on all allocated timeslots, at a level of 1 dB above the level given in the table in conformance requirement 1. On the time slots not allocated to the MS, the SS transmits at a level of 20 dB above the level given in the table in conformance requirement 1. This implicitly tests adjacent time slot rejection.

c)	The SS counts the number of blocks transmitted with CS-3 and the number of these blocks not acknowledged based on the content of the Ack/Nack Description information element (see 3GPP TS 04.60, subclause 12.3) in the Packet Downlink Ack/Nack as sent from the MS to the SS on the PACCH.

NOTE:	Due to the error rates related to the USF, the MS is likely to occasionally miss its USF for transmitting the Packet Downlink Ack/Nack. As this requirement is not verified in this part of the test, the SS then again assigns uplink resources so the MS can sent this message.

d)	Once the number of blocks transmitted with CS-3 as counted in step c) reaches or exceeds the minimum number of blocks as given in table 14.16-2, the SS calculates the Block error ratio. The SS resets both counters.

e)	The SS repeats step b) to d) with the following four fading conditions and hopping modes: TUhigh/noFH, TUhigh/FH, HT/noFH and RA/noFH. For these tests with fading channels , the SS does not transmit on the timeslots not allocated to the MS.

f)	The SS repeats steps b) to d) using CS-4 coding with the following three fading conditions:  Static/FH, TUhigh/noFH and TUhigh/FH. For these tests with fading channels , the SS does not transmit on the timeslots not allocated to the MS.

g)	The SS repeats steps b) to f) under extreme test conditions.

h)	This step is only performed for a multislot MS. The SS establishes the normal test conditions with the exceptions in the parameter settings of Packet Downlink Assignment message:

- P0 = 14 dBm;

- BTS_PWR_CTRL_MODE = Mode A;

- PR_MODE = B.

Furthermore, the SS has to set the PR fields in the MAC headers of each downlink RLC data block to correspond the applied downlink power level, as defined below. The SS repeats steps b) to d) with only one of the active timeslots at 1 dB above the level at which the reference sensitivity performance shall be met, and all other timeslots belonging to the same multislot configuration at a level of 6 dB above this timeslot.

i)	The SS establishes the normal test conditions, and sets the fading function to HT/noFH.

j)	The SS sets the value of the USF/CS-1 such as to allocate the uplink to the MS, transmitting at a level of 1 dB above the level given in the table in conformance requirement 2.

k)	The SS counts the number of times the USF is allocated to the MS, and the number of times the MS does not transmit while being allocated the uplink.

l)	Once the number of USF/CS-1 allocating the uplink for the MS as counted in step k) reaches or exceeds the minimum number of blocks as given in table 14.16-2, the SS calculates the Block error ratio. The SS resets both counters.

m)	The SS repeats steps j) to l) using USF/CS2 coding.

NOTE:	Since coding for USF-bits is identical for CS2, CS3, and CS4, it’s not required to perform the step for CS3 or CS4.

n)	The SS repeats steps i) to m) under extreme test conditions.

o)	The SS establishes normal test condition and a static channel. The SS sets the value of the USF/CS-1 to all values randomly, with the exception of the one allocated to the MS, transmitting at 3 dB below the level at which reference performance shall be met, and counts the number of times the MS transmits on the uplink. This is done for 2 000 blocks.

14.16.1a.5	Test requirements

In step a) the Packet Downlink Ack/Nack as sent by the MS shall indicate every block transmitted by the SS with incorrect BCS as not acknowledged.

The block error ratio, as calculated by the SS for different channels and under the different propagation conditions, under any combination of normal and extreme test voltages and ambient temperatures, shall not exceed the conformance requirement.

In step o) the MS shall transmit no more than 25 times.

In the case when downlink power control is not used and the output power used on the transmitted blocks is not equal to (BCCH level – Pb) then the MS is not required to fulfil 3GPP TS 05.05 requirements for the first 25 blocks addressed to this MS (3GPP TS 05.08, subclause 10.2.2).

NOTE:	This is stated in the Rel 99 version of 3GPP TS 05.08.

< Unchanged sections omitted >

## 14.18	EGPRS receiver tests

Statistical testing of receiver BLER performance

Error Definition

Block Error Ratio (BLER):

The Block Error Ratio is the ratio of blocks received in error to the total number of received blocks, where a block is defined as received in error if the error detection functions in the receiver, operating in accordance with 3GPP TS 05.03, indicate an error as the result of the Block Check Sequence (BCS).

For USF the Block Error Ratio is the ratio of incorrectly interpreted USF to the total number of received USF.

Test criteria

In the receiver tests for circuit switched channels, test error rates have been defined in order not to pass MS with a performance worse than the specification by 1 dB, with tests to be performed at the sensitivity and interference levels defined in 3GPP TS 05.05. For circuit switched channels 3GPP TS 05.05 defines the error rates at a fixed sensitivity or interference level.

For packet switched channels 3GPP TS 05.05 defines the receive or interference level at which a fixed Block Error Ratio is met. Therefore, for EGPRS the receiver is tested with a 1 dB offset in the receive level and the interference level.

If the error events can be assumed to be random independent variables, outputs of stationary random processes with identical Gaussian distributions, the previous figures suggest a number of events not lower than 200 in AWGN channel and not lower than 600 in a multipath environment.

For multipath propagation conditions the hypothesis of stationary random processes does not generally hold. In case of non frequency hopping operation mode, the radio channel may be assumed to change 10 times per wavelength of travelled distance and to be short term stationary in between. So, in this case, the required observation time for having good statistical properties should not be lower (with some rounding) than that reported in table 14.18-1.

Table 14.18-1: Minimum test time according to propagation profile

|  | GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  | DCS 1 800 and PCS 1 900 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation Conditions | TUlow | TUhigh | HT | RA | TUlow | TUhigh | HT | RA |
| Min. test time (s) | 500 | 30 | 15 | 6 | 500 | 15 | 7,5 | 6 |

Table 14.18-2 details, for the different test conditions, the minimum number of blocks required in order to meet points 1) to 3): the corresponding test time (point 4) can be consequently computed.

Table 14.18-2: Test conditions

| Type of test | Type of sub-test | Propagation/ frequency conditions | Specified BLER % | Minimum No of RLC blocks |
| --- | --- | --- | --- | --- |
| Sensitivity | PDTCH/MCS-1 to 4 | static | 10 | 2 000 |
| " | PDTCH/MCS-1 to 4 | TUhigh/no FH | 10 | 6 000 |
| " | PDTCH/MCS-1 to 4 | TUhigh/FH | 10 | 6 000 |
| " | PDTCH/MCS-1 to 4 | RA/no FH | 10 | 6 000 |
| " | PDTCH/MCS-1 to 4 | HT/no FH | 10 | 6 000 |
| " | PDTCH/MCS-5 to 9 | static | 10 | 2 000 |
| " | PDTCH/MCS-5 to 9 | TUhigh/no FH | 10 or 30 | 6 000 or 2 000 |
| " | PDTCH/MCS-5 to 9 | TUhigh/FH | 10 or 30 | 6 000 or 2 000 |
| " | PDTCH/MCS-5 to 9 | RA/noFH | 10 or 30 | 6 000 or 2 000 |
|  | PDTCH/MCS-5 to 9 | HT/noFH | 10 or 30 | 6 000 or 2 000 |
|  | PDTCH /DAS-5 to 7 | static | 10 | 2 000 |
|  | PDTCH /DAS-5 to 7 | Tuhigh/noFH | 10 | 6 000 |
|  | PDTCH /DAS-5 to 7 | Tuhigh/FH | 10 | 6 000 |
|  | PDTCH /DAS-5 to 7 | RA/no FH | 10 | 6 000 |
|  | PDTCH /DAS-5 to 7 | HT/no FH | 10 | 6 000 |
|  | PDTCH /DAS-8 to 9 | static | 10 | 2 000 |
|  | PDTCH /DAS-8 to 9 | Tuhigh/noFH | 10 | 6 000 |
|  | PDTCH /DAS-8 to 9 | Tuhigh/FH | 10 | 6 000 |
|  | PDTCH /DAS-8 to 9 | RA/no FH | 10 | 6 000 |
|  | PDTCH /DAS-8 to 9 | HT/no FH | 10 or 30 | 6 000 or 2 000 |
|  | PDTCH /DAS-10 to 12 | static | 10 | 2 000 |
|  | PDTCH /DAS-10 to 12 | Tuhigh/noFH | 10 or 30 | 6 000 or 2 000 |
|  | PDTCH /DAS-10 to 12 | Tuhigh/FH | 10 or 30 | 6 000 or 2 000 |
|  | PDTCH /DAS-10 to 12 | RA/no FH | 10 or 30 | 6 000 or 2 000 |
|  | PDTCH /DAS-10 to 12 | HT/no FH | n/a | n/a |
| " | USF/MCS-1to 4 | static | 1 | 20 000 |
| " | USF/MCS-1 to 4 | TUhigh/no FH | 1 | 60 000 |
| " | USF/MCS-1 to 4 | TUhigh/FH | 1 | 60 000 |
| " | USF/MCS-1 to 4 | RA/no FH | 1 | 60 000 |
| " | USF/MCS-1 to 4 | HT/no FH | 1 | 60 000 |
|  | USF/MCS-5 to 9 | static | 1 | 20 000 |
|  | USF/MCS-5 to 9 | Tuhigh/noFH | 1 | 60 000 |
|  | USF/MCS-5 to 9 | Tuhigh/FH | 1 | 60 000 |
|  | USF/MCS-5 to 9 | RA/no FH | 1 | 60 000 |
|  | USF/MCS-5 to 9 | HT/no FH | 1 | 60 000 |
|  | USF/DAS-5 to 7 | static | 1 | 20 000 |
|  | USF/DAS-5 to 7 | Tuhigh/noFH | 1 | 60 000 |
|  | USF/DAS-5 to 7 | Tuhigh/FH | 1 | 60 000 |
|  | USF/DAS-5 to 7 | RA/no FH | 1 | 60 000 |
|  | USF/DAS-5 to 7 | HT/no FH | 1 | 60 000 |
|  | USF/DAS-8 to 9 | static | 1 | 20 000 |
|  | USF/DAS-8 to 9 | Tuhigh/noFH | 1 | 60 000 |
|  | USF/DAS-8 to 9 | Tuhigh/FH | 1 | 60 000 |
|  | USF/DAS-8 to 9 | RA/no FH | 1 | 60 000 |
|  | USF/DAS-8 to 9 | HT/no FH | 1 | 60 000 |
|  | USF/DAS-10 to 12 | static | 1 | 20 000 |
|  | USF/DAS-10 to 12 | Tuhigh/noFH | 1 | 60 000 |
|  | USF/DAS-10 to 12 | Tuhigh/FH | 1 | 60 000 |
|  | USF/DAS-10 to 12 | RA/no FH | 1 | 60 000 |
|  | USF/DAS-10 to 12 | HT/no FH | 1 | 60 000 |
| Co-channel | PDTCH/MCS-1 to 4 | TUlow/no FH | 10 | 6 000, but minimum of 500 s |
| ,, | PDTCH/MCS-1 to 4 | TUhigh/no FH | 10 | 6 000 |
| ,, | PDTCHMCS-1 to 4 | TUhigh/FH | 10 | 6 000 |
| ,, | PDTCH/MCS-1 to 4 | RA/no FH | 10 | 6 000 |
| ,, | PDTCH/MCS-5 to 9 | TUlow/no FH | 10 or 30 | 6 000 or 2 000, but minimum of 500 s |
| ,, | PDTCH/MCS-5 to 9 | TUhigh/no FH | 10 or 30 | 6 000 or 2 000 |
| ,, | PDTCH/MCS-5 to 9 | TUhigh/FH | 10 or 30 | 6 000 or 2 000 |
| ,, | PDTCH/MCS-5 to 9 | RA/no FH | 10 or 30 | 6 000 or 2 000 |
| " | USF/MCS-1 to 4 | TUlow/no FH | 1 | 60 000 |
| " | USF/MCS-1 to 4 | TUhigh/no FH | 1 | 60 000 |
| " | USF/MCS-1 to 4 | TUhigh/FH | 1 | 60 000 |
| " | USF/MCS-1 to 4 | RA/no FH | 1 | 60 000 |
| " | USF/MCS-5 to 9 | TUlow/no FH | 1 | 60 000 |
| " | USF/MCS-5 to 9 | TUhigh/no FH | 1 | 60 000 |
| " | USF/MCS-5 to 9 | TUhigh/FH | 1 | 60 000 |
|  | USF/MCS-5 to 9 | RA/no FH | 1 | 60 000 |
| Adjacent Channel 200kHz | PDTCH/MCS-1 to 4 | TUlow/No FH | 10 | 6 000 |
| " | PDTCH/MCS-1 to 4 | TUhigh/NoFH | 10 | 6 000 |
| " | PDTCH/MSC-5 to 9 | TUlow/No FH | 10 or 30 | 6 000 or 2 000 |
| " | PDTCH/MSC-5 to 9 | TUhigh/No FH | 10 or 30 | 6 000 or 2 000 |
| " | USF/MCS-1 to 4 | TUlow/No FH | 1 | 60 000 |
| " | USF/MCS-1 to 4 | TUhigh/No FH | 1 | 60 000 |
| " | USF/MCS-5 to 9 | TUlow/No FH | 1 | 60 000 |
|  | USF/MCS-5 to 9 | TUhigh/No FH | 1 | 60 000 |
| Adjacent Channel 400kHz | PDTCH/MCS-1 to 4 | TUhigh/No FH | 10 | 6 000 |
| " | PDTCH/MCS-5 to 9 | TUhigh/No FH | 10 or 30 | 6 000 or 2 000 |
| " | USF/MCS-1 to 4 | TUhigh/No FH | 1 | 60 000 |
| " | USF/MCS-5 to 9 | TUhigh/No FH | 1 | 60 000 |
| Intermodula-tion Rejection | PDTCH/MCS-1 to 4 | static | 10 | 2 000 |
| " | PDTCH/MCS-5 to 9 | static | 10 | 2 000 |
| " | USF/MCS-1 to 4 | static | 1 | 20 000 |
| " | USF/MCS-1 to 9 | static | 1 | 20 000 |
| Blocking and Spurious | PDTCH/MCS-1 to 4 | static | 10 | 6 000 |
| " | PDTCH/MCS-5 to 9 | static | 10 or 30 | 6 000 or 2 000 |
| " | USF/MCS-1 to 4 | static | 1 | 60 000 |
| " | USF/MCS-5 to 9 | static | 1 | 60 000 |

NOTE 1:	For MCS-7, 8 and 9 the BLER of 10 % or 30 % is specified in the conformance requirements. For MCS-5 to 6 a BLER of 10 % is always applied.

NOTE 2:	For PDTCH sub-tests under fading conditions the number of RLC blocks indicated above shall be transmitted on each downlink timeslot of the multislot configuration.

NOTE 3:	For USF sub-tests under fading conditions, the number of RLC blocks indicated above shall be per uplink timeslot of the multislot configuration.

### 14.18.1	Minimum Input level for Reference Performance

14.18.1.1	Definition

The minimum input level is the signal level at the MS receiver input at which a certain BLER is met.

14.18.1.2	Conformance requirement

1.	The block error rate (BLER) performance for PDTCH/MCS1 to 4 shall not exceed 10 % at input levels according to the table 14.18-3a; and for PDTCH/MCS5 to 9 shall not exceed 10 % or 30 % depending on Coding Schemes at input levels according to the table 14.18-3b.

Table 14.18-3a: PDTCH Sensitivity Input Level for GMSK modulation

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| PDTCH/MCS-1 | dBm | -104 | -102,5 | -103 | -103 | -102 |
| PDTCH/MCS-2 | dBm | -104 | -100,5 | -101 | -100.5 | -100 |
| PDTCH/MCS-3 | dBm | -104 | -96,5 | -96,5 | -92,5 | -95,5 |
| PDTCH/MCS-4 | dBm | -101,5 | -91 | -91 | (note) | (note) |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| PDTCH/MCS-1 | dBm | -104 | -102,5 | -103 | -103 | -101,5 |
| PDTCH/MCS-2 | dBm | -104 | -100,5 | -101 | -100,5 | -99,5 |
| PDTCH/MCS-3 | dBm | -104 | -96,5 | -96,5 | -92,5 | -94,5 |
| PDTCH/MCS-4 | dBm | -101,5 | -90,5 | -90,5 | (note) | (note) |
| NOTE: PDTCH/MCS-4 can not meet the reference performance for some propagation conditions. |  |  |  |  |  |  |

The input levels given in the above Table are applicable to  GSM 400, GSM 700, GSM 850, GSM 900 and PCS 1 900 MS, and have to be corrected by the following values for the following classes of MS:

GSM 400 small MS	+2 dB;

GSM 700, GSM 850, GSM 900 small MS	+2 dB;

DCS 1800 class 1 or 2 MS	+2/+4 dB**;

DCS 1800 class 3 MS	+2 dB;

PCS 1 900 class 1 or 2 MS	+2 dB.

**	For all DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

3GPP TS 05.05, table 1a; 3GPP TS 05.05, subclause 6.2.

Table 14.18-3b: PDTCH Sensitivity Input Level for MS for 8-PSK modulation

| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |  |
| PDTCH/MCS-5 | dBm | -98 | -93 | -94 | -93 | -92 |
| PDTCH/MCS-6 | dBm | -96 | -91 | -91,5 | -88 | -89 |
| PDTCH/MCS-7 | dBm | -93 | -84 | -84 | (note 2) | -83 (note 3) |
| PDTCH/MCS-8 | dBm | -90,5 | -83 (note 3) | -83 (note 3) | (note 2) | (note 2) |
| PDTCH/MCS-9 | dBm | -86 | -78,5 (note 3) | -78,5 (note 3) | (note 2) | (note 2) |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| PDTCH/MCS-5 | dBm | -98 | -93,5 | -93,5 | -93 | -89,5 |
| PDTCH/MCS-6 | dBm | -96 | -91 | -91 | -88 | -83,5 |
| PDTCH/MCS-7 | dBm | -93 | -81,5 | -80,5 | (note 2) | (note 2) |
| PDTCH/MCS-8 | dBm | -90,5 | -80 (note 3) | -80 (note 3) | (note 2) | (note 2) |
| PDTCH/MCS-9 | dBm | -86 | (note 2) | (note 2) | (note 2) | (note 2) |
| NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TUhigh (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. NOTE 2: PDTCH for MCS-x can not meet the reference performance for some propagation conditions. NOTE 3: Performance is specified at 30% BLER for some cases. |  |  |  |  |  |  |

The input levels given in the above Table are applicable to Class 4 or Class 5 MS for GSM 400, GSM 700, GSM 850 and GSM 900 and to Class 1 or Class 2 MS for DCS 1 800 and PCS 1 900. For all other MS the input levels have to be corrected by the value of -2 dB.

3GPP TS 05.05, tables 1c; 3GPP TS 05.05, subclause 6.2

2.	The block error rate (BLER) performance for USF/MCS1 to 9 shall not exceed 1 % at input levels according to the tables 14.18-4a and 14.18-4b.

Table 14.18-4a: USF Sensitivity Input Level for GMSK modulation

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| USF/MCS-1 to 4 | dBm | -104 | -102,5 | -104 | -104 | -102,5 |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| USF/MCS-1 to 4 | dBm | -104 | -104 | -104 | -104 | -102,5 |

The input levels given in the above Table are applicable to  GSM 400, GSM 700, GSM 850, GSM 900and PCS 1 900 MS, and have to be corrected by the following values for the following classes of MS:

GSM 400 small MS	+2 dB;

GSM 700, GSM 850 and GSM 900 small MS		+2 dB;

DCS 1800 class 1 or 2 MS	+2/+4 dB**;

DCS 1800 class 3 MS	+2 dB;

PCS 1 900 class 1 or 2 MS	+2 dB.

**	For all DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

3GPP TS 05.05, table 1a; 3GPP TS 05.05, subclause 6.2.

Table 14.18-4b: USF Sensitivity Input Level for 8-PSK modulation

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| USF/MCS-5 to 9 | dBm | -102 | -97,5 | -99 | -100 | -99 |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| USF/MCS-5 to 9 | dBm | -102 | -99 | -99 | -100 | -99 |

The input levels given in the above Table are applicable to Class 4 or Class 5 MS for GSM 400, GSM 700, GSM 850 and GSM 900 and to Class 1 or Class 2 MS for DCS 1 800 and PCS 1 900. For all other MS the input levels have to be corrected by the value of -2 dB.

3GPP TS 05.05, table 1c; 3GPP TS 05.05, subclause 6.2

3.	The BLER shall not exceed the conformance requirements given in 1. and 2. under extreme conditions; 3GPP TS 05.05, subclause 6.2 and annex D subclauses D.2.1 and D.2.2.

4.	The reference sensitivity performance specified above need not be met in the following cases:

For MS at the static channel, if the received level on either of the two adjacent timeslots to the wanted exceed the wanted timeslot by more than 20 dB.

For MS on a multislot configuration, if the received level on any of the timeslots belonging to the same multislot configuration as the wanted time slot, exceed the wanted time slot by more than 6 dB.

The interfering adjacent time slots shall be static with valid GSM signals in all cases.

3GPP TS 05.05, subclause 6.2.

5.	For an MS allocated a USF on a PDCH with a random RF input or a valid PDCH signal with a random USF not equal to the allocated USF, the overall reception shall be such that the MS shall detect the allocated USF in less than 1 % of the radio blocks for GMSK modulated signals and 1 % for 8-PSK modulated signals. This requirement shall be met for all input levels up to -40 dBm for GMSK modulated signals and up to -40 dBm for 8-PSK modulated signals.

3GPP TS 05.05, subclause 6.4

3GPP TS 45.05 subclause 2:

For T-GSM 810 the requirements for GSM 900 shall apply, apart for those parameters for which a separate requirement exists.

14.18.1.3	Test purpose

1.	To verify that the MS sends a Packet Not Acknowledge in the Packet Downlink Ack/Nack in case of the Block Check Sequence indicating a Block Error.

2.	To verify that the MS does not exceed conformance requirement 1 for PDTCH with different coding schemes and under different propagation conditions with an allowance for the statistical significance of the test.

3.	To verify that the MS does not exceed conformance requirement 2 under HT propagation conditions with an allowance for the statistical significance of the test.

4.	To verify that the MS does not exceed conformance requirement 3 under STATIC, TUhigh, HT and RA propagation conditions for the PDTCH, and under HT propagation conditions for the USF, with an allowance for the statistical significance of the test.

5.	To verify that the MS meets the conformance requirements also 1 and 2 for the conditions allowed by conformance requirement 4, with an allowance for the statistical significance of the test.

6.	To verify that the MS meets conformance requirement 5, with an allowance for the statistical significance of the test.

14.18.1.4	Method of test

Initial conditions

NOTE 1:	The BA list sent on the BCCH will indicate at least six surrounding cells with at least one near to each band edge. It is not necessary to generate any of these BCCHs but, if provided the signal strengths of BCCHs shall be in the range 15 dBVemf(  ) to 35 dBVemf(  ). Surrounding cell signal levels and cell reselection parameters are set so that the MS will not try a cell reselection.

NOTE 2:	The ARFCN of any BCCH shall not be co-channel or on adjacent channels to the wanted traffic channel.

NOTE 3:	When frequency hopping is used under static conditions, the traffic channel may fall on any of the ARFCNs defined in clause 6.  When frequency hopping is used under non-static conditions any ARFCNs shall be chosen.

NOTE 4:	The PSI1 message is included in the PACCH  when the MS is in packet transfer mode. The PBCCH_CHANGE_MARK value in PSI1 is not changed. This, together with preventing cell reselection as per Note 1, ensures that the MS is highly unlikely to suspend the TBF (3GPP TS 04.60 subclause 5.5.1.4.2 Suspension of operation to receive system operation), and thus making the effect of TBF suspension statistically insignificant for the test result.

For both GMSK and 8-PSK modulations, a downlink TBF is set up according to the generic procedure specified in clause 40 for packet switched on an ARFCN in the Mid range. The power control parameter ALPHA (α) is set to 0. The SS shall transmit on the maximum number of receive timeslots. The SS commands the MS to transmit at maximum power.

Test procedure

For GMSK Modulation:

a)	The SS transmits packets under static conditions, using MCS-4 coding at a level of 1 dB above the level given in conformance reference 1. Out of the 400 blocks transmitted by the SS, 20 blocks are sent with incorrect BCS, at (pseudo) random positions. The SS checks, for the blocks it transmitted with incorrect BCS, whether or not the MS Packet Downlink Ack/Nack as sent by the MS indicates these blocks as not acknowledged.

b)	The SS transmits packets under static conditions, with the MS commanded to hopping mode using the hopping sequence used in clause 6, and using MCS-4 coding to the MS on all allocated timeslots, at a level of 1 dB above the level given in the table in conformance requirement 1. On the time slots not allocated to the MS, the SS transmits at a level of 20 dB above the level given in the table in conformance requirement 1. This implicitly tests adjacent time slot rejection.

c)	The SS counts the number of blocks transmitted with MCS-4 and the number of these blocks not acknowledged based on the content of the Ack/Nack Description information element (see 3GPP TS 04.60, subclause 12.3) in the Packet Downlink Ack/Nack as sent from the MS to the SS on the PACCH.

NOTE 5:	Due to the error rates related to the USF, the MS is likely to occasionally miss its USF for transmitting the Packet Downlink Ack/Nack. As this requirement is not verified in this part of the test, the SS then again assigns uplink resources so the MS can sent this message.

d)	Once the number of blocks transmitted with MCS-4 as counted in step c) reaches or exceeds the minimum number of blocks as given in table 14-18-2, the SS calculates the Block error ratio. The SS resets both counters.

e)	The SS repeats step b) to d) with the following two fading conditions and hopping modes: TUhigh/noFH and TUhigh/FH. For these tests with fading channels , the SS does not transmit on the timeslots not allocated to the MS.

f)	The SS repeats steps b) to d) using MCS-3 coding with RA/No FH, MCS-2 coding with HT/No FH and MCS-1 coding with TUhigh/No FH. For these tests, the SS does not transmit on the timeslots not allocated to the MS.

g)	The SS repeats steps b) to e) under extreme test conditions for MCS-4 coding only.

h)	This step is only performed for a multislot MS. The SS establishes the normal test conditions with the exceptions in the parameter settings of Packet Downlink Assignment message:

- P0 = 14 dB;

- BTS_PWR_CTRL_MODE = Mode A;

- PR_MODE = B.

Furthermore, the SS has to set the PR fields in the MAC headers of each downlink RLC data block to correspond the applied downlink power level, as defined below. The SS repeats steps b) to d) with only one of the active timeslots at 1 dB above the level at which the reference sensitivity performance shall be met, and all other timeslots belonging to the same multislot configuration at a level of 6 dB above this timeslot.

i)	The SS establishes the normal test conditions, and sets the fading function to HT/noFH. An uplink TBF shall be established.

j)	The SS sets the value of the USF/MCS-1 such as to allocate the uplink to the MS, transmitting at a level of 1 dB above the level given in the table in conformance requirement 2.

k)	The SS counts the number of times the USF is allocated to the MS, and the number of times the MS does not transmit while being allocated the uplink.

l)	Once the number of USF/MCS-1 allocating the uplink for the MS as counted in step k) reaches or exceeds the minimum number of blocks as given in table 14-18-2, the SS calculates the Block error ratio. The SS resets both counters

m)	The SS repeats steps i) to l) under extreme test conditions using  USF/MCS-1 coding.

n)	The SS establishes normal test condition and a static channel. The SS sets the value of the USF/MCS-1 to all values randomly, with the exception of the one allocated to the MS, transmitting at 3 dB below the level at which reference performance shall be met, and counts the number of times the MS transmits on the uplink. This is done for 2 000 blocks.

For 8-PSK Modulation:

a)	The SS transmits packets under static conditions, using MCS-8 coding at a level of 1 dB above the level given in conformance reference 1. Out of the 400 blocks transmitted by the SS, 20 blocks are sent with incorrect BCS, at (pseudo) random positions. The SS checks, for the blocks it transmitted with incorrect BCS, whether or not the MS Packet Downlink Ack/Nack as sent by the MS indicates these blocks as not acknowledged.

b)	The SS transmits packets under static conditions, with the MS commanded to hopping mode using the hopping sequence used in clause 6, and using MCS-8 coding to the MS on all allocated timeslots, at a level of 1 dB above the level given in the table in conformance requirement 1. On the time slots not allocated to the MS, the SS transmits at a level of 20 dB above the level given in the table in conformance requirement 1. This implicitly tests adjacent time slot rejection.

c)	The SS counts the number of blocks transmitted with MCS-8 and the number of these blocks not acknowledged based on the content of the Ack/Nack Description information element (see 3GPP TS 04.60, subclause 12.3) in the Packet Downlink Ack/Nack as sent from the MS to the SS on the PACCH.

NOTE 6:	Due to the error rates related to the USF, the MS is likely to occasionally miss its USF for transmitting the Packet Downlink Ack/Nack. As this requirement is not verified in this part of the test, the SS then again assigns uplink resources so the MS can sent this message.

d)	Once the number of blocks transmitted with MCS-8 as counted in step c) reaches or exceeds the minimum number of blocks as given in table 14-18-2, the SS calculates the Block error ratio. The SS resets both counters.

e)	The SS repeats step b) to d) with the following two fading conditions and hopping modes: TUhigh/noFH and TUhigh/FH. For these tests with fading channels, the SS does not transmit on the timeslots not allocated to the MS.

f)	The SS repeats steps b) to d) using MCS-9 with static condition, MCS-7 with TUhigh/FH, MSC-6 with HT/No FH and MSC-5 with RA/No FH. For these tests, the SS does not transmit on the timeslots not allocated to the MS.

g)	The SS repeats steps b) to e) under extreme test conditions for MCS-8 coding only.

h)	This step is only performed for a multislot MS. The SS establishes the normal test conditions with the exceptions in the parameter settings of Packet Downlink Assignment message:

- P0 = 14 dB;

- BTS_PWR_CTRL_MODE = Mode A;

- PR_MODE = B.

Furthermore, the SS has to set the PR fields in the MAC headers of each downlink RLC data block to correspond the applied downlink power level, as defined below. The SS repeats steps b) to d) with only one of the active timeslots at 1 dB above the level at which the reference sensitivity performance shall be met, and all other timeslots belonging to the same multislot configuration at a level of 6 dB above this timeslot.

i)	The SS establishes the normal test conditions, and sets the fading function to HT/noFH. An uplink TBF shall be established.

j)	The SS sets the value of the USF/MCS-5 such as to allocate the uplink to the MS, transmitting at a level of 1 dB above the level given in the table in conformance requirement 2.

k)	The SS counts the number of times the USF is allocated to the MS, and the number of times the MS does not transmit while being allocated the uplink.

l)	Once the number of USF/MCS-5 allocating the uplink for the MS as counted in step k) reaches or exceeds the minimum number of blocks as given in table 14-18-2, the SS calculates the Block error ratio. The SS resets both counters.

m)	The SS repeats steps j) to l) under extreme test conditions using USF/MCS-5 coding.

n)	The SS establishes normal test condition and a static channel. The SS sets the value of the USF/MCS-5 to all values randomly, with the exception of the one allocated to the MS, transmitting at 3 dB below the level at which reference performance shall be met, and counts the number of times the MS transmits on the uplink. This is done for 2 000 blocks.

14.18.1.5	Test requirements

In step a) the Packet Downlink Ack/Nack as sent by the MS shall indicate every block transmitted by the SS with incorrect BCS as not acknowledged.

The block error ratio, as calculated by the SS for different channels and under the different propagation conditions, under any combination of normal and extreme test voltages and ambient temperatures, shall not exceed the conformance requirement.

In step n) for both GMSK modulation and 8-PSK modulation the MS shall transmit no more than 25 times.

In the case when downlink power control is not used and the output power used on the transmitted blocks is not equal to (BCCH level – Pb) then the MS is not required to fulfil 3GPP TS 05.05 requirements for the first 25 blocks addressed to this MS (3GPP TS 05.08, subclause 10.2.2).

< Unchanged sections omitted >

### 14.18.1b	Minimum Input level for Reference Performance in TIGHTER configuration

14.18.1b.1	Definition

The minimum input level is the signal level at the MS receiver input at which a certain BLER is met.

14.18.1b.2	Conformance requirement

For a MS indicating support for TIGHTER Capability (see 3GPP TS 24.008), the minimum input signal levels for which the reference performance shall be met are specified in table 1w, according to the propagation condition. The performance requirements for GSM 400 and GSM 700 systems are as for GSM 900 in table 1w, except that the GSM 400 MS speed is doubled from that of GSM 900, e.g. TU50 becomes TU100, and the GSM 700 MS speed is increased by a factor of 1.2, e.g. TU50 becomes TU60.

1.	The block error rate (BLER) performance for PDTCH/MCS1 to 4 shall not exceed 10 % at input levels according to the table 14.18.1b-3a; and for PDTCH/MCS5 to 9 shall not exceed 10 % or 30 % depending on Coding Schemes at input levels according to the table 14.18.1b-3b.

Table 14.18.1b-3a: PDTCH Sensitivity Input Level for GMSK modulation

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| PDTCH/MCS-1 | dBm | -105,5 | -105 | -105,5 | -105,5 | -103 |
| PDTCH/MCS-2 | dBm | -105,5 | -103 | -103,5 | -103 | -101 |
| PDTCH/MCS-3 | dBm | -105,5 | -99 | -99 | -95 | -96,5 |
| PDTCH/MCS-4 | dBm | -103 | -93,5 | -93,5 | (note 2) | (note 2) |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| PDTCH/MCS-1 | dBm | (note 3) | -104,5 | -105,5 | -105,5 | -102,5 |
| PDTCH/MCS-2 | dBm | (note 3) | -102,5 | -103,5 | -103 | -100,5 |
| PDTCH/MCS-3 | dBm | (note 3) | -98,5 | -99 | -95 | -95,5 |
| PDTCH/MCS-4 | dBm | (note 3) | -92,5 | -93 | (note 2) | (note 2) |
| NOTE 2: PDTCH for MCS-x can not meet the reference performance for some propagation conditions. NOTE 3: The requirements for the DCS 1800 & PCS 1900 Static propagation condition are the same as for the GSM 850 & GSM 900 Static propagation condition, the requirements for the GSM 850 & GSM 900 TU50 (ideal FH) and DCS 1800 & PCS 1900 TU50 (ideal FH) propagation conditions are the same as for the DCS 1800 & PCS 1900 TU50 (no FH) propagation condition, and the requirements for the DCS 1800 & PCS 1900 RA130 (no FH) propagation condition are the same as for the GSM 850 & GSM 900 RA250 (no FH) propagation condition. |  |  |  |  |  |  |

The input levels given in the above Table are applicable to  GSM 400, GSM 700, GSM 850, GSM 900 and PCS 1 900 MS, and have to be corrected by the following values for the following classes of MS:

GSM 400 small MS	+2 dB;

GSM 700, GSM 850, GSM 900 small MS	+2 dB;

DCS 1800 class 1 or 2 MS	+2/+4 dB**;

DCS 1800 class 3 MS	+2 dB;

PCS 1 900 class 1 or 2 MS	+2 dB.

**	For all DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

3GPP TS 45.005, table 1w; 3GPP TS 45.005, subclause 6.2.

Table 14.18.1b-3b: PDTCH Sensitivity Input Level for MS for 8-PSK modulation

| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |  |
| PDTCH/MCS-5 | dBm | -101,5 | -97,5 | -98,5 | -99 | -97,5 |
| PDTCH/MCS-6 | dBm | -99,5 | -95,5 | -96 | -94 | -94,5 |
| PDTCH/MCS-7 | dBm | -96,5 | -88,5 | -88,5 | (note 2) | -88,5** |
| PDTCH/MCS-8 | dBm | -94 | -87,5** | -87,5** | (note 2) | (note 2) |
| PDTCH/MCS-9 | dBm | -89,5 | -83** | -83** | (note 2) | (note 2) |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| Type of |  | Propagation conditions |  |  |  |  |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| PDTCH/MCS-5 | dBm | (note 3) | -98,5 | -98,5 | -99 | -95 |
| PDTCH/MCS-6 | dBm | (note 3) | -96 | -96 | -94 | -89 |
| PDTCH/MCS-7 | dBm | (note 3) | -86,5 | -85,5 | (note 2) | (note 2) |
| PDTCH/MCS-8 | dBm | (note 3) | -85 ** | -85** | (note 2) | (note 2) |
| PDTCH/MCS-9 | dBm | (note 3) | (note 2) | (note 2) | (note 2) | (note 2) |
| Performance is specified at 30% BLER for those cases identified with mark **. NOTE 1: Ideal FH case assumes perfect decorrelation between bursts. This case may only be tested if such a decorrelation is ensured in the test. For TUhigh (ideal FH), sufficient decorrelation may be achieved with 4 frequencies spaced over 5 MHz. NOTE 2: PDTCH for MCS-x cannot meet the reference performance for some propagation conditions. NOTE 3: The requirements for the DCS 1800 & PCS 1900 Static propagation condition are the same as for the GSM 850 & GSM 900 Static propagation condition, the requirements for the GSM 850 & GSM 900 TU50 (ideal FH) and DCS 1800 & PCS 1900 TU50 (ideal FH) propagation conditions are the same as for the DCS 1800 & PCS 1900 TU50 (no FH) propagation condition, and the requirements for the DCS 1800 & PCS 1900 RA130 (no FH) propagation condition are the same as for the GSM 850 & GSM 900 RA250 (no FH) propagation condition. |  |  |  |  |  |  |

The input levels given in the above Table are applicable to Class 4 or Class 5 MS for GSM 400, GSM 700, GSM 850 and GSM 900 and to Class 1 or Class 2 MS for DCS 1 800 and PCS 1 900. For all other MS the input levels have to be corrected by the value of -2 dB.

3GPP TS 45.005, tables 1w; 3GPP TS 45.005, subclause 6.2

2.	The block error rate (BLER) performance for USF/MCS1 to 9 shall not exceed 1 % at input levels according to the tables 14.18-4a and 14.18-4b.

Table 14.18-4a: USF Sensitivity Input Level for GMSK modulation

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| USF/MCS-1 to 4 | dBm | -104 | -102,5 | -104 | -104 | -102,5 |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| USF/MCS-1 to 4 | dBm | -104 | -104 | -104 | -104 | -102,5 |

The input levels given in the above Table are applicable to  GSM 400, GSM 700, GSM 850, GSM 900and PCS 1 900 MS, and have to be corrected by the following values for the following classes of MS:

GSM 400 small MS	+2 dB;

GSM 700, GSM 850 and GSM 900 small MS		+2 dB;

DCS 1800 class 1 or 2 MS	+2/+4 dB**;

DCS 1800 class 3 MS	+2 dB;

PCS 1 900 class 1 or 2 MS	+2 dB.

**	For all DCS 1 800 class 1 and class 2 MS, a correction offset of +2dB shall apply for the reference sensitivity performance as specified in table 1a for the normal conditions defined in Annex D and an offset of +4 dB shall be used to determine all other MS performances.

3GPP TS 45.005, table 1a; 3GPP TS 45.005, subclause 6.2.

Table 14.18-4b: USF Sensitivity Input Level for 8-PSK modulation

| Type of |  | Propagation conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel |  | static | TUhigh (no FH) | TUhigh (ideal FH) | RA (no FH) | HT (no FH) |
| GSM 400, GSM 700, GSM 850 and GSM 900 |  |  |  |  |  |  |
| USF/MCS-5 to 9 | dBm | -102 | -97,5 | -99 | -100 | -99 |
| DCS 1 800 and PCS 1 900 |  |  |  |  |  |  |
| USF/MCS-5 to 9 | dBm | -102 | -99 | -99 | -100 | -99 |

The input levels given in the above Table are applicable to Class 4 or Class 5 MS for GSM 400, GSM 700, GSM 850 and GSM 900 and to Class 1 or Class 2 MS for DCS 1 800 and PCS 1 900. For all other MS the input levels have to be corrected by the value of -2 dB.

3GPP TS 45.005, table 1c; 3GPP TS 45.005, subclause 6.2

3.	The BLER shall not exceed the conformance requirements given in 1. and 2. under extreme conditions; 3GPP TS 45.005, subclause 6.2 and annex D subclauses D.2.1 and D.2.2.

4.	The reference sensitivity performance specified above need not be met in the following cases:

For MS at the static channel, if the received level on either of the two adjacent timeslots to the wanted exceed the wanted timeslot by more than 20 dB.

For MS on a multislot configuration, if the received level on any of the timeslots belonging to the same multislot configuration as the wanted time slot, exceed the wanted time slot by more than 6 dB.

The interfering adjacent time slots shall be static with valid GSM signals in all cases.

3GPP TS 45.005, subclause 6.2.

5.	For an MS allocated a USF on a PDCH with a random RF input or a valid PDCH signal with a random USF not equal to the allocated USF, the overall reception shall be such that the MS shall detect the allocated USF in less than 1 % of the radio blocks for GMSK modulated signals and 1 % for 8-PSK modulated signals. This requirement shall be met for all input levels up to -40 dBm for GMSK modulated signals and up to -40 dBm for 8-PSK modulated signals.

3GPP TS 45.005, subclause 6.4

3GPP TS 45.05 subclause 2:

For T-GSM 810 the requirements for GSM 900 shall apply, apart for those parameters for which a separate requirement exists.

14.18.1b.3	Test purpose

1.	To verify that the MS sends a Packet Not Acknowledge in the Packet Downlink Ack/Nack in case of the Block Check Sequence indicating a Block Error.

2.	To verify that the MS does not exceed conformance requirement 1 for PDTCH with different coding schemes and under different propagation conditions with an allowance for the statistical significance of the test.

3.	To verify that the MS does not exceed conformance requirement 2 under HT propagation conditions with an allowance for the statistical significance of the test.

4.	To verify that the MS does not exceed conformance requirement 3 under STATIC, TUhigh, HT and RA propagation conditions for the PDTCH, and under HT propagation conditions for the USF, with an allowance for the statistical significance of the test.

5.	To verify that the MS meets the conformance requirements also 1 and 2 for the conditions allowed by conformance requirement 4, with an allowance for the statistical significance of the test.

6.	To verify that the MS meets conformance requirement 5, with an allowance for the statistical significance of the test.

14.18.1b.4	Method of test

Initial conditions

NOTE 1:	The BA list sent on the BCCH will indicate at least six surrounding cells with at least one near to each band edge. It is not necessary to generate any of these BCCHs but, if provided the signal strengths of BCCHs shall be in the range 15 dBVemf(  ) to 35 dBVemf(  ). Surrounding cell signal levels and cell reselection parameters are set so that the MS will not try a cell reselection.

NOTE 2:	The ARFCN of any BCCH shall not be co-channel or on adjacent channels to the wanted traffic channel.

NOTE 3:	When frequency hopping is used under static conditions, the traffic channel may fall on any of the ARFCNs defined in clause 6.  When frequency hopping is used under non-static conditions any ARFCNs shall be chosen.

NOTE 4:	The PSI1 message is included in the PACCH  when the MS is in packet transfer mode. The PBCCH_CHANGE_MARK value in PSI1 is not changed. This, together with preventing cell reselection as per Note 1, ensures that the MS is highly unlikely to suspend the TBF (3GPP TS 04.60 subclause 5.5.1.4.2 Suspension of operation to receive system operation), and thus making the effect of TBF suspension statistically insignificant for the test result.

For both GMSK and 8-PSK modulations, a downlink TBF is set up according to the generic procedure specified in clause 40 for packet switched on an ARFCN in the Mid range. The power control parameter ALPHA (α) is set to 0. The SS shall transmit on the maximum number of receive timeslots. The SS commands the MS to transmit at maximum power.

Test procedure

For GMSK Modulation:

a)	The SS transmits packets under static conditions, using MCS-4 coding at a level of 1 dB above the level given in conformance reference 1. Out of the 400 blocks transmitted by the SS, 20 blocks are sent with incorrect BCS, at (pseudo) random positions. The SS checks, for the blocks it transmitted with incorrect BCS, whether or not the MS Packet Downlink Ack/Nack as sent by the MS indicates these blocks as not acknowledged.

b)	The SS transmits packets under static conditions, with the MS commanded to hopping mode using the hopping sequence used in clause 6, and using MCS-4 coding to the MS on all allocated timeslots, at a level of 1 dB above the level given in the table in conformance requirement 1. On the time slots not allocated to the MS, the SS transmits at a level of 20 dB above the level given in the table in conformance requirement 1. This implicitly tests adjacent time slot rejection.

c)	The SS counts the number of blocks transmitted with MCS-4 and the number of these blocks not acknowledged based on the content of the Ack/Nack Description information element (see 3GPP TS 04.60, subclause 12.3) in the Packet Downlink Ack/Nack as sent from the MS to the SS on the PACCH.

NOTE 5:	Due to the error rates related to the USF, the MS is likely to occasionally miss its USF for transmitting the Packet Downlink Ack/Nack. As this requirement is not verified in this part of the test, the SS then again assigns uplink resources so the MS can sent this message.

d)	Once the number of blocks transmitted with MCS-4 as counted in step c) reaches or exceeds the minimum number of blocks as given in table 14-18-2, the SS calculates the Block error ratio. The SS resets both counters.

e)	The SS repeats step b) to d) with the following two fading conditions and hopping modes: TUhigh/noFH and TUhigh/FH. For these tests with fading channels , the SS does not transmit on the timeslots not allocated to the MS.

f)	The SS repeats steps b) to d) using MCS-3 coding with RA/No FH, MCS-2 coding with HT/No FH and MCS-1 coding with TUhigh/No FH. For these tests, the SS does not transmit on the timeslots not allocated to the MS.

g)	The SS repeats steps b) to e) under extreme test conditions for MCS-4 coding only.

h)	This step is only performed for a multislot MS. The SS establishes the normal test conditions with the exceptions in the parameter settings of Packet Downlink Assignment message:

- P0 = 14 dB;

- BTS_PWR_CTRL_MODE = Mode A;

- PR_MODE = B.

Furthermore, the SS has to set the PR fields in the MAC headers of each downlink RLC data block to correspond the applied downlink power level, as defined below. The SS repeats steps b) to d) with only one of the active timeslots at 1 dB above the level at which the reference sensitivity performance shall be met, and all other timeslots belonging to the same multislot configuration at a level of 6 dB above this timeslot.

i)	The SS establishes the normal test conditions, and sets the fading function to HT/noFH. An uplink TBF shall be established.

j)	The SS sets the value of the USF/MCS-1 such as to allocate the uplink to the MS, transmitting at a level of 1 dB above the level given in the table in conformance requirement 2.

k)	The SS counts the number of times the USF is allocated to the MS, and the number of times the MS does not transmit while being allocated the uplink.

l)	Once the number of USF/MCS-1 allocating the uplink for the MS as counted in step k) reaches or exceeds the minimum number of blocks as given in table 14-18-2, the SS calculates the Block error ratio. The SS resets both counters

m)	The SS repeats steps i) to l) under extreme test conditions using  MCS-1 coding.

n)	The SS establishes normal test condition and a static channel. The SS sets the value of the USF/MCS-1 to all values randomly, with the exception of the one allocated to the MS, transmitting at 3 dB below the level at which reference performance shall be met, and counts the number of times the MS transmits on the uplink. This is done for 2 000 blocks.

For 8-PSK Modulation:

a)	The SS transmits packets under static conditions, using MCS-8 coding at a level of 1 dB above the level given in conformance reference 1. Out of the 400 blocks transmitted by the SS, 20 blocks are sent with incorrect BCS, at (pseudo) random positions. The SS checks, for the blocks it transmitted with incorrect BCS, whether or not the MS Packet Downlink Ack/Nack as sent by the MS indicates these blocks as not acknowledged.

b)	The SS transmits packets under static conditions, with the MS commanded to hopping mode using the hopping sequence used in clause 6, and using MCS-8 coding to the MS on all allocated timeslots, at a level of 1 dB above the level given in the table in conformance requirement 1. On the time slots not allocated to the MS, the SS transmits at a level of 20 dB above the level given in the table in conformance requirement 1. This implicitly tests adjacent time slot rejection.

c)	The SS counts the number of blocks transmitted with MCS-8 and the number of these blocks not acknowledged based on the content of the Ack/Nack Description information element (see 3GPP TS 04.60, subclause 12.3) in the Packet Downlink Ack/Nack as sent from the MS to the SS on the PACCH.

NOTE 6:	Due to the error rates related to the USF, the MS is likely to occasionally miss its USF for transmitting the Packet Downlink Ack/Nack. As this requirement is not verified in this part of the test, the SS then again assigns uplink resources so the MS can sent this message.

d)	Once the number of blocks transmitted with MCS-8 as counted in step c) reaches or exceeds the minimum number of blocks as given in table 14-18-2, the SS calculates the Block error ratio. The SS resets both counters.

e)	The SS repeats step b) to d) with the following two fading conditions and hopping modes: TUhigh/noFH and TUhigh/FH. For these tests with fading channels, the SS does not transmit on the timeslots not allocated to the MS.

f)	The SS repeats steps b) to d) using MCS-9 with static condition, MCS-7 with TUhigh/FH, MSC-6 with HT/No FH and MSC-5 with RA/No FH. For these tests, the SS does not transmit on the timeslots not allocated to the MS.

g)	The SS repeats steps b) to e) under extreme test conditions for MCS-8 coding only.

h)	This step is only performed for a multislot MS. The SS establishes the normal test conditions with the exceptions in the parameter settings of Packet Downlink Assignment message:

- P0 = 14 dB;

- BTS_PWR_CTRL_MODE = Mode A;

- PR_MODE = B.

Furthermore, the SS has to set the PR fields in the MAC headers of each downlink RLC data block to correspond the applied downlink power level, as defined below. The SS repeats steps b) to d) with only one of the active timeslots at 1 dB above the level at which the reference sensitivity performance shall be met, and all other timeslots belonging to the same multislot configuration at a level of 6 dB above this timeslot.

i)	The SS establishes the normal test conditions, and sets the fading function to HT/noFH. An uplink TBF shall be established.

j)	The SS sets the value of the USF/MCS-5 such as to allocate the uplink to the MS, transmitting at a level of 1 dB above the level given in the table in conformance requirement 2.

k)	The SS counts the number of times the USF is allocated to the MS, and the number of times the MS does not transmit while being allocated the uplink.

l)	Once the number of USF/MCS-5 allocating the uplink for the MS as counted in step k) reaches or exceeds the minimum number of blocks as given in table 14-18-2, the SS calculates the Block error ratio. The SS resets both counters.

m)	The SS repeats steps j) to l) under extreme test conditions using MCS-5 coding.

n)	The SS establishes normal test condition and a static channel. The SS sets the value of the USF/MCS-5 to all values randomly, with the exception of the one allocated to the MS, transmitting at 3 dB below the level at which reference performance shall be met, and counts the number of times the MS transmits on the uplink. This is done for 2 000 blocks.

14.18.1b.5	Test requirements

In step a) the Packet Downlink Ack/Nack as sent by the MS shall indicate every block transmitted by the SS with incorrect BCS as not acknowledged.

The block error ratio, as calculated by the SS for different channels and under the different propagation conditions, under any combination of normal and extreme test voltages and ambient temperatures, shall not exceed the conformance requirement.

In step n) for both GMSK modulation and 8-PSK modulation the MS shall transmit no more than 25 times.

In the case when downlink power control is not used and the output power used on the transmitted blocks is not equal to (BCCH level – Pb) then the MS is not required to fulfil 3GPP TS 45.005 requirements for the first 25 blocks addressed to this MS (3GPP TS 45.008, subclause 10.2.2).

< Unchanged sections omitted >

### 14.18.5	Blocking and spurious response

14.18.5.1	Definition

Blocking is a measure of the ability of the receiver to receive a modulated wanted input signal in the presence of an unwanted input signal, on frequencies other than those of the spurious responses or the adjacent channels, without exceeding a given degradation. "Wanted signal" in this test is the signal generated by the transmitted RLC data blocks.

14.18.5.2	Conformance requirement

1.	The blocking characteristics of the receiver are specified separately for in-band and out-of-band performance as identified in 3GPP TS 05.05 subclause 5.1.

2.	The block error rate (BLER) performance for PDTCH/MCS1 to 4 shall not exceed 10 % and for PDTCH/MCS5 to 9 shall not exceed 10 % or 30 % depending on Coding Schemes and for USF/MCS1 to 9 shall not exceed 1 % when the following signals are simultaneously input to the receiver; 3GPP TS 05.05, subclause 6.2:

-	a useful signal at frequency f0, 3 dB above the reference sensitivity level specified in table 14.18-3a for GMSK modulation and table 14.18-3b for 8-PSK modulation for PDTCH channels; and in tables 14.18-4a for GMSK modulation and 14.18-4b for 8-PSK modulation for USF channel with correction values as specified in 3GPP TS 05.05 subclause 6.2;

-	a continuous, static sine wave unwanted signal at a level as in the table 14.18-9 below and at a frequency (f) which is an integer multiple of 200 kHz.

with the following exceptions, called spurious response frequencies:

a)	GSM 400: in band, for a maximum of three occurrences. 3GPP TS 05.05, subclause 5.1.

GSM 700, GSM 850 or GSM 900: in band, for a maximum of six occurrences (which if grouped shall not exceed three contiguous occurrences per group). 3GPP TS 05.05, subclause 5.1.

DCS 1 800 and PCS 1 900: in band, for a maximum of twelve occurrences (which if grouped shall not exceed three contiguous occurrences per group). 3GPP TS 05.05, subclause 5.1.

b)	out of band, for a maximum of 24 occurrences (which if below f0 and grouped shall not exceed three contiguous occurrences per group). 3GPP TS 05.05, subclause 5.1.

where the above performance shall be met when the continuous sine wave signal (f) is set to a level of 70 dBµV (emf) (i.e. -43 dBm). 3GPP TS 05.05, subclause 5.1.

14.18.5.3	Test purpose

1.	To verify that the in band blocking performance is met without exceeding the total number of allowed in band spurious responses. An allowance is made for the statistical significance of the test.

2.	To verify that at selected out of band frequencies, the out of band blocking performance is met without exceeding the total number of allowed out of band spurious responses. An allowance is made for the statistical significance of the test.

NOTE:	Not all of the possible out of band frequencies are tested as this results in excessive test time. However, the total number of out of band spurious responses, specified in 3GPP TS 05.05, is allowed to ensure a fair test of the MS.

14.18.5.4	Method of test

Initial conditions

A call is set up according to the generic call set up procedure, except the BCCH frequency list shall be empty, on a TCH with an arbitrary ARFCN in the range supported by the MS. The power control level is set to maximum power. The ARFCN of the BCCH shall be the same - or at an offset of +/- 2 channels, than that of the ARFCN for the TCH.

The SS transmits EGPRS RLC data blocks containing random data.

In addition to the wanted Test Signal, the SS transmit a static unmodulated continuous interfering signal (Standard Test Signal I0).

Test procedure

For the ACK/NACK test steps the maximum number of supported time slots shall be used, and for the USF test steps the maximum supported symmetrical UL slot configuration shall be used.

For GMSK Modulation:

a)	The SS is set to produce a static GMSK wanted signal and a static interfering signal at the same time. The SS sets the amplitude of the wanted signal to 4 dB above the reference sensitivity level specified in table 14.18-3a for PDTCH channel and in table 14.18-4a for USF channel with correction values as specified in 3GPP TS 05.05 subclause 6.2.

b)	The SS transmits packets on PDTCH using MCS-4 coding to MS on all allocated timeslots.

c)	The unwanted signal is of frequency FB. It is applied in turn on the subset of frequencies calculated at step d) in the overall range 100 kHz to 12,75 GHz, where FB is an integer multiple of 200 kHz.

However, frequencies in the range FR ± 600 kHz are excluded.

NOTE:	Allowance must be made for possible spurious signals arising from the SS. These are particularly likely at sub harmonic frequencies nFB where n = 2, 3, 4, 5, etc.

d)	The frequencies at which the test is performed (adjusted to an integer multiple of 200 kHz channels most closely approximating the absolute frequency of the calculated blocking signal frequency) are the combined frequencies from i), ii) and iii) which follow:

i)	The total frequency range formed by:

GSM 400		the frequencies between Flo + (IF1 + IF2 + ... + IFn + 3,6 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 3,6 MHz).

GSM 700		the frequencies between Flo + (IF1 + IF2 + ... + IFn + 7,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 7,5 MHz).

GSM 850		the frequencies between Flo + (IF1 + IF2 + ... + IFn + 12,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 12,5 MHz).

P-GSM 900: the frequencies between Flo + (IF1 + IF2 + ... + IFn + 12,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 12,5 MHz).

E-GSM 900: the frequencies between Flo + (IF1 + IF2 + ... + IFn + 17,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 17,5 MHz).

DCS 1 800: the frequencies between Flo + (IF1 + IF2 + ... + IFn + 37,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 37,5 MHz).

PCS 1 900: the frequencies between Flo + (IF1 + IF2 + ... + IFn + 30 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 30 MHz).

and

the frequencies +100 MHz and -100 MHz from the edge of the relevant receive band.

Measurements are made at 200 kHz intervals.

ii)	The three frequencies IF1, IF1 + 200 kHz, IF1 - 200 kHz.

iii)	The frequencies:

mFlo + IF1;

mFlo - IF1;

mFR;

where m is all positive integers greater than or equal to 2 such that either sum lies in the range 100 kHz to 12,75 GHz.

The frequencies in step ii) and iii) lying in the range of frequencies defined by step i) above need not be repeated.

Where:

Flo 	- local oscillator applied to first receiver mixer

IF1 ... IFn 	- are the n intermediate frequencies

Flo, IF1, IF2 ... IFn	- shall be declared by the manufacturer in the PIXIT statement
	3GPP TS 51.010-1 annex 3.

e)	The level of the unwanted signal is set according to table 14.18-9.

Table 14.18-9a: Level of unwanted signals

|  | GSM450 |  | GSM480 |  | GSM 900 |  | DCS 1 800 | PCS 1 900 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Small MS | Other MS | Small MS | Other MS | Small MS | Other MS |  |  |
| FREQUENCY | LEVEL IN dBVemf( ) |  |  |  |  |  |  |  |
| FR ±600 kHz to FR ±800 kHz | 70 | 75 | 70 | 75 | 70 | 75 | 70 | 70 |
| FR ±800 kHz to FR ±1,6 MHz | 70 | 80 | 70 | 80 | 70 | 80 | 70 | 70 |
| FR ±1,6 MHz to FR ±3 MHz | 80 | 90 | 80 | 90 | 80 | 90 | 80 | 80 |
| 457,6 MHz to FR - 3 MHz | 90 | 90 | - | - | - | - | - | - |
| FR + 3 MHz to 473,6 MHz | 90 | 90 | - | - | - | - | - | - |
| 486 MHz to FR - 3MHz | - | - | 90 | 90 | - | - | - | - |
| FR + 3MHz to 502 MHz | - | - | 90 | 90 | - | - | - | - |
| 915 MHz to FR - 3 MHz | - | - | - | - | 90 | 90 | - | - |
| FR + 3 MHz to 980 MHz | - | - | - | - | 90 | 90 | - | - |
| 1 785 MHz to FR - 3 MHz | - | - | - | - | - | - | 87 | - |
| FR + 3 MHz to 1 920 MHz | - | - | - | - | - | - | 87 | - |
| 1 910 MHz to FR - 3 MHz | - | - | - | - | - | - | - | 87 |
| FR + 3 MHz to 2 010 MHz | - | - | - | - | - | - | - | 87 |
| 100 kHz to < 457,6 MHz | 113 | 113 | - | - | - | - | - | - |
| > 473,6MHz to 12,750 MHz | 113 | 113 | - | - | - | - | - | - |
| 100 kHz to < 486 MHz | - | - | 113 | 113 | - | - | - | - |
| > 502 MHz to 12,750 MHz | - | - | 113 | 113 | - | - | - | - |
| 835 MHz to < 915 MHz | - | - | - | - | 113 | 113 | - | - |
| > 980 MHz to 1 000 MHz | - | - | - | - | 113 | 113 | - | - |
| 100 kHz to < 835 MHz | - | - | - | - | 113 | 113 | - | - |
| > 1 000 MHz to 12,750 MHz | - | - | - | - | 113 | 113 | - | - |
| 100 kHz to 1 705 MHz | - | - | - | - | - | - | 113 | - |
| > 1 705 MHz to < 1 785 MHz | - | - | - | - | - | - | 101 | - |
| > 1 920 MHz to 1 980 MHz | - | - | - | - | - | - | 101 | - |
| > 1 980 MHz to 12,750 MHz | - | - | - | - | - | - | 113 | - |
| 100 kHz to < 1 830 MHz | - | - | - | - | - | - | - | 113 |
| 1 830 MHz to < 1 910 MHz | - | - | - | - | - | - | - | 101 |
| > 2 010 MHz to 2 070 MHz | - | - | - | - | - | - | - | 101 |
| > 2 070 MHz to 12,750 MHz | - | - | - | - | - | - | - | 113 |

Table 14-18-9b: Level of unwanted signals

|  | GSM 710 | GSM 750 | T-GSM 810 | GSM 850 |  |
| --- | --- | --- | --- | --- | --- |
| FREQUENCY | LEVEL IN dBVemf( ) |  |  |  |  |  |
| FR ±600 kHz to FR ±800 kHz | 70 | 70 | 70 |  | 70 |  |
| FR ±800 kHz to FR ±1,6 MHz | 70 | 70 | 70 |  | 70 |  |
| FR ±1,6 MHz to FR ±3 MHz | 80 | 80 | 80 |  | 80 |  |
| 678 MHz to FR - 3 MHz | 90 | - | - |  | - |  |
| FR + 3 MHz to 728 MHz | 90 | - | - |  | - |  |
| 727 MHz to FR – 3 MHz | - | 90 | - |  | - |  |
| FR + 3 MHz to 777 MHz | - | 90 | - |  | - |  |
| 831 MHz to FR - 3 MHz | - | - | 90 |  | - |  |
| FR + 3 MHz to 886 MHz | - | - | 90 |  | - |  |
| 849 MHz to FR – 3 MHz | - | - | - |  | 90 |  |
| FR + 3 MHz to 914 MHz | - | - | - |  | 90 |  |
| 678 MHz to FR - 3 MHz | 113 | - | - |  | - |  |
| FR + 3 MHz to 728 MHz | 113 | - | - |  | - |  |
| 100 kHz to < 727 MHz | - | 113 | - |  | - |  |
| > 777 MHz to 12,75 GHz | - | 113 | - |  | - |  |
| 100 kHz to 831 MHz | - | - | 113 |  | - |  |
| > 886 MHz to 12,75 MHz | - | - | 113 |  | - |  |
| 100 kHz to < 849 MHz | - | - | - |  | 113 |  |
| > 914 MHz to 12,75 GHz | - | - | - |  | 113 |  |

NOTE 1:	For E-GSM 900 MS the level of the unwanted signal in the band 905 MHz to 915 MHz is relaxed to 108 dBuVemf( ). 3GPP TS 05.05, subclause 5.1.

NOTE 2:	a) For R-GSM 900 MS the level of the unwanted signal in the band 880 MHz to 915 MHz is relaxed to 108 dBuVemf( ). 3GPP TS 05.05, subclause 5.1.
For ER-GSM MS the level of the unwanted signal in the band 880 MHz to 912 MHz is relaxed to 108 dBuVemf( ). 3GPP TS 45.005, subclause 5.1.
For ER-GSM MS the level of the unwanted signal in the band 912 MHz to 915 MHz is relaxed to 101 dBuVemf( ). 3GPP TS 45.005, subclause 5.1.

b) For R-GSM 900 small MS the level of the unwanted signal in the band 876 MHz to 915 MHz is relaxed to 106 dBuVemf( ). 3GPP TS 05.05, subclause 5.1.
For ER-GSM small MS the level of the unwanted signal in the band 873 MHz to 912 MHz is relaxed to 106 dBuVemf( ). 3GPP TS 45.005, subclause 5.1.
For ER-GSM small MS the level of the unwanted signal in the band 912 MHz to 915 MHz is relaxed to 99 dBuVemf( ). 3GPP TS 45.005, subclause 5.1.

NOTE 3:	a) For GSM 450 small MS the level of the unwanted signal in the band 450,4 MHz to 457,6 MHz is relaxed to 108 dBuVemf( ). 3GPP TS 05.05, subclause 5.1.

b) For GSM 480 small MS the level of the unwanted signal in the band 478,8 MHz to 486 MHz is relaxed to 108 dBuVemf( ). 3GPP TS 05.05, subclause 5.1.

f)	The SS counts the number of blocks transmitted with current coding scheme and the number of these blocks not acknowledged based on the content of the Ack/Nack Description information element (see 3GPP TS 04.60, subclause 12.3) in the Packet Downlink Ack/Nack as sent from the MS to the SS on the PACCH.

NOTE 1:	Due to the error rates related to the USF, the MS is likely to occasionally miss its USF for transmitting the Packet Downlink Ack/Nack. As this requirement is not verified in this part of the test, the SS then again assigns uplink resources so the MS can send this message.

g)	Once the number of blocks transmitted with the current coding scheme as counted in step f) reaches or exceeds the minimum number of blocks as given in table 14.18-2, the SS calculates the Block error ratio. The SS resets both counters.

If a failure is indicated, it is noted and counted towards the allowed exemption total. In the case of failures discovered at the predicted frequencies at steps d i), ii) or iii) the test is repeated on the adjacent channels ±200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond is also be tested. This process is repeated until all channels constituting the group of failures is known.

h)	The SS sets the value of the USF/MCS-1 such as to allocate the uplink to the MS.

i)	The unwanted signal is of frequency FB. It is applied in turn on the subset of frequencies calculated at step d) in the overall range 100 kHz to 12,75 GHz, where FB is an integer multiple of 200 kHz.

However, frequencies in the range FR ±600 kHz are excluded.

NOTE 2:	Allowance must be made for possible spurious signals arising from the SS. These are particularly likely at sub harmonic frequencies nFB where n = 2, 3, 4, 5, etc.

j)	The level of the unwanted signal is set according to table 14.18-9.

k)	The SS counts the number of times the USF is allocated to the MS, and the number of times the MS does not transmit while being allocated the uplink.

l)	Once the number of USF/MCS-1 allocating the uplink for the MS as counted in step k) reaches or exceeds the minimum number of blocks as given in table 14.18-2, the SS calculates the Block error ratio. The SS resets both counters. If a failure is indicated, it is noted and counted towards the allowed exemption total.

In the case of failures discovered at the predicted frequencies at steps d i), ii) or iii) the test is repeated on the adjacent channels ±200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond is also be tested. This process is repeated until all channels constituting the group of failures is known.

For 8-PSK Modulation:

a)	The SS is set to produce a static 8-PSK wanted signal and a static interfering signal at the same time. The SS sets the amplitude of the wanted signal to 4 dB above the reference sensitivity level specified in table 14.18-3b for PDTCH channel and in table 14.18-4b for USF channel with correction values as specified in 3GPP TS 05.05 subclause 6.2;

b)	The SS transmits packets on PDTCH using MCS-9 coding to MS on all allocated timeslots.

c)	The unwanted signal is of frequency FB. It is applied in turn on the subset of frequencies calculated at step d) in the overall range 100 kHz to 12,75 GHz, where FB is an integer multiple of 200 kHz.

However, frequencies in the range FR ±600 kHz are excluded.

NOTE 3:	Allowance must be made for possible spurious signals arising from the SS. These are particularly likely at sub harmonic frequencies nFB where n = 2, 3, 4, 5, etc.

d)	The frequencies at which the test is performed (adjusted to an integer multiple of 200 kHz channels most closely approximating the absolute frequency of the calculated blocking signal frequency) are the combined frequencies from i), ii) and iii) which follow:

i)	The total frequency range formed by:

GSM 400		the frequencies between Flo + (IF1 + IF2 + ... + IFn + 3,6 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 3,6 MHz).

GSM 700		the frequencies between Flo + (IF1 + IF2 + ... + IFn + 7,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 7,5 MHz).

GSM 850		the frequencies between Flo + (IF1 + IF2 + ... + IFn + 12,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 12,5 MHz).

P-GSM 900: the frequencies between Flo + (IF1 + IF2 + ... + IFn + 12,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 12,5 MHz).

E-GSM 900: the frequencies between Flo + (IF1 + IF2 + ... + IFn + 17,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 17,5 MHz).

DCS 1 800: the frequencies between Flo + (IF1 + IF2 + ... + IFn + 37,5 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 37,5 MHz).

PCS 1 900: the frequencies between Flo + (IF1 + IF2 + ... + IFn + 30 MHz)

and Flo - (IF1 + IF2 + ... + IFn + 30 MHz).

and

the frequencies +100 MHz and -100 MHz from the edge of the relevant receive band.

Measurements are made at 200 kHz intervals.

ii)	The three frequencies IF1, IF1 + 200 kHz, IF1 - 200 kHz.

iii)	The frequencies:

mFlo + IF1;

mFlo - IF1;

mFR;

where m is all positive integers greater than or equal to 2 such that either sum lies in the range 100 kHz to 12,75 GHz.

The frequencies in step ii) and iii) lying in the range of frequencies defined by step i) above need not be repeated.

Where:

Flo 	- local oscillator applied to first receiver mixer

IF1 ... IFn 	- are the n intermediate frequencies

Flo, IF1, IF2 ... IFn	- shall be declared by the manufacturer in the PIXIT statement
	3GPP TS 51.010-1 annex 3.

e)	The level of the unwanted signal is set according to table 14.18-9.

f)	The SS counts the number of blocks transmitted with current coding scheme and the number of these blocks not acknowledged based on the content of the Ack/Nack Description information element (see 04.60, 12.3) in the Packet Downlink Ack/Nack as sent from the MS to the SS on the PACCH.

NOTE 4:	Due to the error rates related to the USF, the MS is likely to occasionally miss its USF for transmitting the Packet Downlink Ack/Nack. As this requirement is not verified in this part of the test, the SS then again assigns uplink resources so the MS can send this message.

g)	Once the number of blocks transmitted with the current coding scheme as counted in step f) reaches or exceeds the minimum number of blocks as given in table 14.18-2, the SS calculates the Block error ratio. The SS resets both counters. If a failure is indicated, it is noted and counted towards the allowed exemption total.

In the case of failures discovered at the predicted frequencies at steps d i), ii) or iii) the test is repeated on the adjacent channels ±200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond is also be tested. This process is repeated until all channels constituting the group of failures is known.

h)	The SS sets the value of the USF/MCS-5 such as to allocate the uplink to the MS.

j)	The unwanted signal is of frequency FB. It is applied in turn on the subset of frequencies calculated at step d) in the overall range 100 kHz to 12,75 GHz, where FB is an integer multiple of 200 kHz.

However, frequencies in the range FR ±600 kHz are excluded.

NOTE 5:	Allowance must be made for possible spurious signals arising from the SS. These are particularly likely at sub harmonic frequencies nFB where n = 2, 3, 4, 5, etc.

k)	The level of the unwanted signal is set according to table 14.18-9.

l)	The SS counts the number of times the USF is allocated to the MS, and the number of times the MS does not transmit while being allocated the uplink.

m)	Once the number of USF/MCS-5 allocating the uplink for the MS as counted in step l) reaches or exceeds the minimum number of blocks as given in table 14.18-2, the SS calculates the Block error ratio. The SS resets both counters. If a failure is indicated, it is noted and counted towards the allowed exemption total.

In the case of failures discovered at the predicted frequencies at steps d i), ii) or iii) the test is repeated on the adjacent channels ±200 kHz away. If either of these two frequencies fail then the next channel 200 kHz beyond is also be tested. This process is repeated until all channels constituting the group of failures is known.

14.18.5.5	Test requirements

The block error ratio as calculated by the SS for different channels and coding schemes shall not exceed the conformance requirement. Testing the conformance requirement can be done either in the classical way with a fixed minimum number of samples (refer to section 14.18.5.5.2) or using statistical methods that lead to an early pass/fail decision with test time significantly reduced for MS with performance not on the limit (refer to section 14.18.5.5.1). Both methods are based on a bad DUT factor M = 1.5.

This shall apply under normal test voltage and ambient temperature, and with the interfering signal at any frequency in the range specified.

The following exceptions are allowed:

GSM 400:	A maximum of three failures in the band 457,6 MHz to 473,6 MHz for GSM450 and in the band 486,0 MHz to 502,0 MHz for GSM480

A maximum of 24 in the combined bands 100 kHz to 457,6 MHz and 473,6 MHz to 12,75 GHz for GSM 450 and in the combined bands 100 kHz to 486,0 MHz and 502,0 MHz to 12,75 GHz for GSM 480 (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

GSM 710:	A maximum of six failures  in the frequency band 678 MHz to 728 MHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 failures  in the combined bands 100 kHz to 678 MHz and 728 MHz to 12,75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

GSM 750:	A maximum of six failures in the frequency band 727 MHz to 782 MHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 failures in the combined bands 100 kHz to 727 MHz and 782 MHz to 12,75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

GSM 810:	A maximum of six failures  in the frequency band 831 MHz to 886 MHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 failures  in the combined bands 100 kHz to 831 MHz and 886 MHz to 12,75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

GSM 850:	A maximum of six failures in the frequency band 849 MHz to 914 MHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 failures in the combined bands 100 kHz to 849 MHz and 914 MHz to 12,75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

GSM 900:	A maximum of six failures in the band 915 MHz to 980 MHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 in the combined bands 100 kHz to 915 MHz and 980 MHz to 12,75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

DCS 1 800:	A maximum of twelve failures in the band 1 785 MHz to 1 920 MHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 in the combined bands 100 kHz to 1 785 MHz and 1 920 MHz to 12,75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

PCS 1 900:	A maximum of twelve failures in the band 1 910 MHz to 2 010 MHz (which, if grouped, shall not exceed three 200 kHz channels per group).

A maximum of 24 in the combined bands 100 kHz to 1 910 MHz and 2 010 MHz to 12,75 GHz (which, if below FR and grouped, shall not exceed three 200 kHz channels per group).

If the number of failures does not exceed the maximum allowed figures stated above, the test of 14.18.5.4 is repeated at the frequencies at which the failures occurred. The level of the unwanted signal is set to 70 dBuVemf(  ) and the performance requirement is once again that stated above.

The number of Error Events recorded in this test shall not exceed the test limit error rate values given above, when using either the accelerated BLER method or the maximum number of samples.

No failures are allowed at this lower unwanted signal level.

14.18.5.5.1	Statistical testing of blocking and spurious response performance with early decision

For more information on statistical testing of blocking and spurious response performance, especially the definition of limit lines, refer to Annex 7.

Wrong decision risk F for one single error rate test:

Fpass   ≠   Ffail	As the blocking test case comprises of many BLER tests the wrong decision risk for a fail decision of one single error rate test must be smaller than the wrong decision risk for a pass decision to avoid an increased probability of an erroneous fail decision.

Fpass   =   0.2%

Ffail    =   0.02%

Wrong decision probability D per test step:

Dpass    ≠   Dfail

Dpass    =   0.008%

Dfail     =      0.0008%

Parameters for limit lines:

1.	Dpass    =	0.008% 	wrong decision probability per test step for early pass decision.

	Dfail     = 	0.0008% 	wrong decision probability per test step for early fail decision.

2.	M	=	1.5	bad DUT factor

3.	ne	number of (error) events.

4.	ns	number of samples. The error rate is calculated from ne and ns.

Limit checking

For an early decision a minimum number of measured (error) events are necessary.

For an early pass decision	ne   ≥   1

For an early fail decision	ne   ≥   8

When the target test time has been reached the test is finished and a pass/fail decision can be made.

The statistical testing of the conformance requirement is done using table 14.18.5-1.

Table 14.18.5-1: Statistical test limits for blocking performance of EGPRS mobiles

| Blocking and spurious response for EGPRS mobiles |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Orig. BLER | Derived | Target number | Target test | Target test time |
|  | blocks per s | requirement | test limit | of samples | time (s) | (hh:mm:ss) |
| One time slot: |  |  |  |  |  |  |
| PDTCH/MCS-4 | 50 | 0,100000 | 0,125100 | 3221 | 64 | 00:01:04 |
| USF/MCS-1 | 50 | 0,010000 | 0,012510 | 32214 | 644 | 00:10:44 |
| PDTCH/MCS-9 | 50 | 0,100000 | 0,125100 | 3221 | 64 | 00:01:04 |
| USF/MCS-5 | 50 | 0,010000 | 0,012510 | 32214 | 644 | 00:10:44 |
| Two time slots: |  |  |  |  |  |  |
| PDTCH/MCS-4 | 100 | 0,100000 | 0,125100 | 3221 | 32 | 00:00:32 |
| USF/MCS-1 | 100 | 0,010000 | 0,012510 | 32214 | 322 | 00:05:22 |
| PDTCH/MCS-9 | 100 | 0,100000 | 0,125100 | 3221 | 32 | 00:00:32 |
| USF/MCS-5 | 100 | 0,010000 | 0,012510 | 32214 | 322 | 00:05:22 |
| Three time slots |  |  |  |  |  |  |
| PDTCH/MCS-4 | 150 | 0,100000 | 0,125100 | 3221 | 21 | 00:00:21 |
| USF/MCS-1 | 150 | 0,010000 | 0,012510 | 32214 | 215 | 00:03:35 |
| PDTCH/MCS-9 | 150 | 0,100000 | 0,125100 | 3221 | 21 | 00:00:21 |
| USF/MCS-5 | 150 | 0,010000 | 0,012510 | 32214 | 215 | 00:03:35 |
| Four time slots |  |  |  |  |  |  |
| PDTCH/MCS-4 | 200 | 0,100000 | 0,125100 | 3221 | 16 | 00:00:16 |
| USF/MCS-1 | 200 | 0,010000 | 0,012510 | 32214 | 161 | 00:02:41 |
| PDTCH/MCS-9 | 200 | 0,100000 | 0,125100 | 3221 | 16 | 00:00:16 |
| USF/MCS-5 | 200 | 0,010000 | 0,012510 | 32214 | 161 | 00:02:41 |

14.18.5.5.2	Fixed testing of blocking and spurious response performance with minimum number of samples

The fixed testing of the conformance requirement is done using  the minimum number of samples and the limit BLER given in table 14.18-2.

< End of changes >