3GPP TSG-RAN5 Meeting #102	R5-241376

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0605 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for support of ULCA for different type of CA Band combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per TS 38.101-1, clause 5.3A.5, the requirements for carrier aggregation are defined for carrier aggregation configurations. This implies that an UE may support UL CA for intraband contiguous CA configuration and may not do it for inter band configuration. This is also inline with the Note 2 from Table A.4.3.2A.2.1-3a in TS 38.508-2 which implies that the UL CA capabilities can be supported on a single band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Defined 3 PICS for each of the 3 CA configurations for UL CA with 2 carriers. There is a case for defining separate PICS for UL and DL CA for 2 and more carriers. However the current requirement for Signaling test cases is for UL CA with 2 carriers |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail test cases configuring UL CA with 2 carriers in a non-supported CA configuration |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### A.4.3.2A	NR CA Physical Layer Baseline Implementation Capabilities

NOTE:	See Annex B for status of completed NR CA configurations and power classes in this version of 3GPP UE conformance test specifications.

#### A.4.3.2A.1	General NR CA capabilities

Table A.4.3.2A.1-1: Downlink NR CA capabilities (for one or more of the supported NR CA configurations)

| Item | DL NR CA capability | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR CA with 2 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_DL_NR_CA_2CC |  |
| 2 | DL NR CA with 3 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_DL_NR_CA_3CC |  |
| 3 | DL NR CA with 4 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_DL_NR_CA_4CC |  |
| 4 | DL NR CA with 5 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_DL_NR_CA_5CC |  |
| 5 | DL NR CA with 6 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_DL_NR_CA_6CC |  |
| 6 | DL NR CA with 7 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_DL_NR_CA_7CC |  |
| 7 | DL NR CA with 8 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_DL_NR_CA_8CC |  |

Table A.4.3.2A.1-2: Uplink NR CA capabilities (for one or more of the supported NR CA configurations)

| Item | UL NR CA capability | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR Intra-band contiguous CA with 2 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_UL_NR_ intra_contiguous_CA_2CC |  |
| 1A | UL NR Intra-band non-contiguous CA with 2 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_UL_NR_ intra_non_contiguous_CA_2CC |  |
| 1B | UL NR Inter-band CA with 2 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_UL_NR_ inter_band_CA_2CC |  |
| 2 | UL NR CA with 3 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_UL_NR_CA_3CC |  |
| 3 | UL NR CA with 4 carriers | 38.101-1, 5.3A 38.101-2, 5.3A 38.101-3, 5.3A | pc_UL_NR_CA_4CC |  |
| 4 | UL NR CA with 5 carriers | 38.101-2, 5.3A 38.101-3, 5.3A | pc_UL_NR_CA_5CC |  |
| 5 | UL NR CA with 6 carriers | 38.101-2, 5.3A 38.101-3, 5.3A | pc_UL_NR_CA_6CC |  |
| 6 | UL NR CA with 7 carriers | 38.101-2, 5.3A 38.101-3, 5.3A | pc_UL_NR_CA_7CC |  |
| 7 | UL NR CA with 8 carriers | 38.101-2, 5.3A 38.101-3, 5.3A | pc_UL_NR_CA_8CC |  |