3GPP TSG-RAN5 Meeting # 102	R5-241405

# Athens, Greece, 26th February – 3rd March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1033 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Annex F for UE Maximum Output Power – EIRP with UL Gaps test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Apple Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Measurement uncertainty and test tolerances for UE Maximum Output Power – EIRP with UL Gaps test case are incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define Measurement uncertainty for UE Maximum Output Power – EIRP with UL Gaps test case and test tolerances as described in R5-241403 and R5-241404. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE Maximum Output Power – EIRP with UL Gaps test case will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependent on RAN5 discussion paper R5-241403 and CR R5-241404. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty (MTSU) for transmitter tests

| Sub clause | Maximum Test System Uncertainty | Derivation of MTSU |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.08 dB (FR2a, NTC testing) ±5.28 dB (FR2b, NTC testing) [±5.91] dB (FR2c, NTC testing) ±5.35 dB (FR2a, ETC testing) ±5.55 dB (FR2b, ETC testing) TBD (FR2c, ETC testing) PC1 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.40 dB (FR2b, NTC testing) ±5.60 dB (FR2a, ETC testing) ±5.67 dB (FR2b, ETC testing) PC5 Minimum peak EIRP, Max EIRP Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.60 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.3-1 in TR 38.903) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP Max Device size ≤ 30 cm ±4.61 dB (FR2a, NTC testing) ±4.81 dB (FR2b, NTC testing) [±5.44] dB (FR2c, NTC testing) ±4.85 dB (FR2a, ETC testing) ±5.07 dB (FR2b, ETC testing) TBD (FR2c, ETC testing) PC1 Max TRP Max Device size ≤ 30 cm ±4.64 dB (FR2a, NTC testing) ± 4.78 dB (FR2b, NTC testing) ± 4.90 dB (FR2a, ETC testing) ± 5.04 dB (FR2b, ETC testing) PC5 Max TRP Max Device size ≤ 30 cm ±4.64 dB (FR2a, NTC testing) ± 4.90 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.3-2 in TR 38.903) |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC3 Max Device size ≤ 30 cm ±4.78 dB (FR2a) ±5.38 dB (FR2b) TBD (FR2c) PC1 Max Device size ≤ 30 cm ±4.69 dB (FR2a) ±4.84 dB (FR2b) | MTSU = 1.00 x MU (from Table B.3-3 in TR 38.903) |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Max Device size ≤ 30 cm ±5.11 dB (FR2a, NTC testing) ±5.29 dB (FR2b, NTC testing) ±5.38 dB (FR2a, ETC testing) ±5.56 dB (FR2b, ETC testing) PC1 Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.50 dB (FR2b, NTC testing) ±5.60 dB (FR2a, ETC testing) ±5.77 dB (FR2b, ETC testing) PC5 Max Device size ≤ 30 cm ±5.33 dB (FR2a, NTC testing) ±5.60 dB (FR2a, ETC testing) | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 for FR2a, FR2b PC3 Max Device size ≤ 30 cm TBD (FR2c, NTC testing) TBD (FR2c, ETC testing) | MTSU = 1.00 x MU (from Table B.4-1 in TR 38.903) |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2.4_1 Configured transmitted power with Power Boost | Same as 6.2.1.1 |  |
| 6.2.5 UE Maximum Output Power – EIRP with UL Gaps | PC3 Max Device size ≤ 30 cm ±0.4 dB (FR2a & FR2b, NTC & ETC testing) |  |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous, Inter-band CA TBD |  |

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

## F.3.2	Measurement of transmitter

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Influence of noise is subtracted from MTSU before calculating the TT for lower limit Tx test cases.

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Sub clause | Test Tolerance (TT) | Formula for test requirement |
| --- | --- | --- |
| 6.2.1.1 UE maximum output power (EIRP) | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 2.99 dB (FR2a, NTC) 2.99 dB (FR2b, NTC) [3.37]dB (FR2c, NTC) 3.15 dB (FR2a, ETC) 3.15 dB (FR2b, ETC) TBD (FR2c, ETC) PC1 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.12 dB (FR2a, NTC) 3.12 dB (FR2b, NTC) 3.28 dB (FR2a, ETC) 3.28 dB (FR2b, ETC) PC5 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.12 dB (FR2a, NTC) Max EIRP 0 dB | PC3 Minimum peak EIRP TT = 0.60 x (MTSUIFF - 0.1) (FR2a) TT = 0.60 x (MTSUIFF - 0.3) (FR2b) TT = 0.60 x (MTSUIFF - 0.3) (FR2c) PC1 Minimum peak EIRP TT = 0.60 x (MTSUIFF – 0.13) (FR2a) TT = 0.60 x (MTSUIFF – 0.20) (FR2b) PC5 Minimum peak EIRP TT = 0.60 x (MTSUIFF – 0.13) (FR2a) |
| 6.2.1.1 UE maximum output power (TRP) | PC3 Max TRP IFF (Max Device size ≤ 30 cm) 2.77 dB (FR2a, NTC) 2.89 dB (FR2b, NTC) [3.26] dB (FR2c, NTC) 2.91 dB (FR2a, ETC) 3.04 dB (FR2b, ETC) TBD (FR2c, ETC) PC1 Max TRP IFF (Max Device size ≤ 30 cm) 2.78 dB (FR2a, NTC) 2.87 dB (FR2b, NTC) 2.94 dB (FR2a, ETC) 3.03 dB (FR2b, ETC) PC5 Max TRP IFF (Max Device size ≤ 30 cm) 2.78 dB (FR2a, NTC) 2.94 dB (FR2a, ETC) | Max TRP TT = 0.60 x MTSUIFF |
| 6.2.1.1_1 UE maximum output power – EIRP (Rel-16 and forward) | Same as 6.2.1.1 |  |
| 6.2.1.2 UE maximum output power (Spherical coverage) | PC1 IFF (Max Device size ≤ 30 cm) 2.69 dB (FR2a) 2.69 dB (FR2b) PC2 TBD PC3 IFF (Max Device size ≤ 30 cm) 2.69 dB (FR2a) 2.69 dB (FR2b) TBD (FR2c) PC4 TBD PC5 IFF (Max Device size ≤ 30 cm) 2.69 dB (FR2a) | PC3 TT = 0.60 x (MTSUIFF - 0.3) (FR2a) TT = 0.60 x (MTSUIFF - 0.9) (FR2b) PC1 TT = 0.60 x (MTSUIFF - 0.20) (FR2a) TT = 0.60 x (MTSUIFF - 0.35) (FR2b) PC5 TT = 0.60 x (MTSUIFF - 0.20) (FR2a) |
| 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) | Same as 6.2.1.2 |  |
| 6.2.2 UE maximum output power reduction | PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.24 dB (FR2a, NTC) 3.24 dB (FR2b, NTC) 3.41 dB (FR2a, ETC) 3.41 dB (FR2b, ETC) PC1 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.38 dB (FR2a, NTC) 3.38 dB (FR2b, NTC) 3.56 dB (FR2a, ETC) 3.56 dB (FR2b, ETC) PC5 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) 3.38 dB (FR2a, NTC) 3.56 dB (FR2a, ETC) | Minimum peak EIRP PC3 TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.31) (FR2b) PC1 TT = 0.65 x (MTSUIFF - 0.13) (FR2a) TT = 0.65 x (MTSUIFF - 0.3) (FR2b) PC5 TT = 0.65 x (MTSUIFF - 0.13) (FR2a) |
| 6.2.2_1 UE maximum output power reduction enhancements | Same as 6.2.2 for FR2a, FR2b PC3 Minimum peak EIRP IFF (Max Device size ≤ 30 cm) TBD (FR2c, NTC) TBD (FR2c, ETC) |  |
| 6.2.3 UE maximum output power with additional requirements | Same as 6.2.2 |  |
| 6.2.4 Configured transmitted power | TBD |  |
| 6.2.4_1 Configured transmitted power with Power Boost | Same as 6.2.1.1 |  |
| 6.2.5 UE Maximum Output Power – EIRP with UL Gaps | PC3 Relative EIRP IFF (Max Device size ≤ 30 cm) 0.26 dB (FR2a & FR2b, NTC & ETC) | PC3 Relative EIRP IFF (Max Device size ≤ 30 cm) TT = 0.65 x (MTSUIFF) (FR2a, FR2b) |
| 6.2A.1.1.1 UE maximum output power - EIRP and TRP for CA (2UL CA) | Intra-band contiguous CA Maximum aggregated BW ≤ 400MHz Same as 6.2.1 Maximum aggregated BW > 400MHz TBD Intra-band non-contiguous TBD |  |

## <<< END OF CHANGES 2>>>