3GPP TSG-RAN WG5 Meeting #102                                                       R5-241409

# Athens, Greece, 26th Feb – 1st Mar 2024

Source:	MediaTek Inc.

Title:		Discussion paper for handling Rel-16 features and test cases for Redcap Only UEs

Agenda item:	6.3.6.7

# Document for: Discussion and Decision

Agenda Item:

## Introduction

## At RAN5#100 [1] “Way forward for Rel-16 and Rel-17 legacy test cases applicability to RedCap UE” was agreed that introduces the way forward on how to handle Rel-16 and Rel-17 protocol (TS 38.523-1) legacy test cases applicability to Redcap UEs.

At RAN5#102 [2] “Discussion paper for selecting optional Rel-16 features and test case for Redcap Only UE” has been agreed that all Rel-17 test cases should be upgraded and only a date needs to be decided.

At RAN2#124 [3] “Discussion on SON/MDT reports for eRedCap” has been discussed.

## Discussion

Below is a list of Rel-16 legacy features with protocol test cases. Features excluded from the below list:

- features with no test cases

- features not applicable to RedCap:DC,CA and ENDC  (for full list of restrictions see clause 4.2.21.1 in TS 38.306)

- Conformance Test Aspects - Protocol enhancements for Mission Critical Services for Rel-16 (MCPTT, MCVideo, MCData)

| Name | Acronym | Rapporteur | TC count so far | Completion | Ind. interest so far |
| --- | --- | --- | --- | --- | --- |
| UE Conformance Test Aspects -UE Power Saving in NR | NR_UE_pow_sav-UEConTest | CATT | 5 | 100% | Yes |
| UE Conformance Test Aspects - Physical Layer Enhancements for NR URLLC | NR_L1enh_URLLC-UEConTest | Huawei | 5 | 100% | Yes |
| UE Conformance Test Aspects – Support of NR Industrial Internet of Things (IoT) | NR_IioT-UEConTest | CMCC | 6 | 100% | Yes |
| UE Conformance Test Aspects - NR-based access to unlicensed spectrum | NR_unlic-UEConTest | Qualcomm | 13 | 65% |  |
| UE Conformance Test Aspects - 5G V2X with NR sidelink | 5G_V2X_NRSL_eV2XARC-UEConTest | Huawei | 0 | 63% |  |
| UE Conformance Test Aspects - NR positioning support | NR_pos-UEConTest | CATT | 0 | 100% | Yes |
| UE Conformance Test Aspects - Optimisations on UE radio capability signalling - NR/E-UTRA Aspects | RACS-UEConTest | Qualcomm | 8 | 100% |  |
| UE Conformance Test Aspects - Enhancement of Network Slicing | eNS-UEConTest | CMCC | 5 | 67% | Yes |
| UE Conformance Test Aspects - Access Traffic Steering, Switch and Splitting support in 5G system | ATSSS-UEConTest | China Telecom | 0 | 100% | Yes |
| UE Conformance Test Aspects - Private Network Support for NG-RAN | NG_RAN_PRN_Vertical_LAN-UEConTest | Qualcomm | 12 | 100% | Yes |
| UE Conformance Test Aspects - Enhancements on MIMO for NR | NR_eMIMO-UEConTest | Huawei | 1 | 100% |  |
| UE Conformance Test Aspects - NR Mobility Enhancements | NR_Mob_enh-UEConTest | Huawei | 13 | 100% | Yes |
| UE Conformance Test Aspects - 2-step RACH for NR | NR_2step_RACH-UEConTest | ZTE, China Telecom | 6 | 100% | Yes |
| UE Conformance Test Aspects - SON (Self-Organising Networks) and MDT (Minimization of Drive Tests) support for NR | NR_SON_MDT-UEConTest | CMCC, Ericsson | 47 | 100% | Yes |
| UE Conformance Test Aspects - B1C Signal in BDS Positioning System Support for LTE and NR | B1C_BDS_pos_UEConTest | CATT | 0 | 100% |  |
| (Small) Test Technical Enhancements and Improvements for Rel-16 | TEI16_Test |  | 8 | 100% |  |
| UE Conformance Test Aspects - Support of eCall over IMS for NR | NR_EIEI-UEConTest | Qualcomm | 15 | 100% | Yes |

There is a large number of affected test cases and it could be challenging to update them at one meeting.

RAN2 discussion in [3] may impact how SON/MDT test cases need to be updated.

## 3.	Proposal

Proposal 1:	Track list of updated Rel-16 test cases in the attached spreadsheet.

Proposal 2:	Postpone updating SON/MDT test cases until discussion in [3] concludes.

## 4.	References

[1]	R5-235348: Way forward for Rel-16 and Rel-17 legacy test cases applicability to RedCap UE

[2]	R5-237399: Discussion paper for selecting optional Rel-16 features and test case for Redcap Only UE

[3]	R2-2312918: Discussion on SON/MDT reports for eRedCap