3GPP TSG-RAN WG5 Meeting #102	R5-241413

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	5.3.10.9

Source:	THALES

Title: 	Neighbor GSO satellite ephemeris file for NTN NR UE testing

Document for:	Discussion

# Introduction

The main objective of this technical discussion document is to present an updated methodology to generate satellite Ephemeris for NTN NR UE testing that incorporates both SIB19 (SIB31/SIB32 for NB-IoT) state vector and agreement reached at RAN4#106-bis with respect to NTN UE testing.

The data required by the Test Equipment to implement SIB19 (NTN dedicated SIB from 5G NR Rel-17) or SIB31/SIB32 (NTN dedicated SIB for NB-IoT starting from Rel-17) is based on two potential information: ECEF information and/or Orbital information. Effectively, it has been agreed in RAN1/RAN2 that UEs are provided with satellite ephemeris in ECI of the date (namely ECEF) in either state vector (position velocity) or orbital information (Kepler elements).

The testing methodology from this discussion document can apply for test preparation of NTN NR UE either for:

- Testing at the time of the current date using GNSS real time resources, or

- Testing at a previous date earlier than the current date (i.e. 1 day) using stored GNSS information or GNSS information to be retrieved from AT commands.

The recovery, the interpretation and processing of location information by the UE (pre-configured location, GNSS information or AT commands) is out scope of this document.

This approach is in line with maximum errors described in R1-2111122 "Considerations on UL timing and frequency synchronization in NTN" or the testing process described in R4-2302517 "RRM test cases methodology and configuration for SAN NTN assistance information parameters".

In addition to Range and Doppler effect evaluation, the delay, delay shift considered. Furthermore, the state vector (position, velocity) are given in SIB19/SIB31/SIB32 format using the dedicated transfer functions.

This work represents an incremental update of previous RAN5 contribution R5-233941 “Satellite Ephemeris file generation methodology for NTN NR UE testing” (THALES) and R5-237213 “GSO satellite Ephemeris file generation methodology for NTN NR UE testing” (THALES). This contribution further provides information with respect to neighbor GSO satellite ephemeris data and related channel Doppler/Delay values. As a matter of fact, the serving GSO satellite ephemeris data has been already provided in R5-237213.

Is important to mention that the process and provided data are applicable/are valid for both 5G NR NTN (SIB19) and NB-IoT NTN (SIB31/SIB32).

# Recall of RRC protocol (TS 38.331) in Rel-17

The specification TS 38.331 "Technical Specification Group Radio Access Network - NR - Radio Resource Control (RRC) protocol specification" indicates the required satellite ephemeris information to be included in SIB and transmitted/broadcasted by the satellite. The UE uses this information (namely SIB19 for NTN 5GNR) to perform UL Doppler and timing advance pre-compensation.

-- ASN1START

-- TAG-EPHEMERISINFO-START

EphemerisInfo-r17 ::=          CHOICE {

positionVelocity-r17           PositionVelocity-r17,

orbital-r17                    Orbital-r17

}

PositionVelocity-r17 ::=       SEQUENCE {

positionX-r17                  PositionStateVector-r17,

positionY-r17                  PositionStateVector-r17,

positionZ-r17                  PositionStateVector-r17,

velocityVX-r17                 VelocityStateVector-r17,

velocityVY-r17                 VelocityStateVector-r17,

velocityVZ-r17                 VelocityStateVector-r17

}

Orbital-r17 ::=                SEQUENCE {

semiMajorAxis-r17              INTEGER (0..8589934591),

eccentricity-r17               INTEGER (0..1048575),

periapsis-r17                  INTEGER (0..268435455),

longitude-r17                  INTEGER (0..268435455),

inclination-r17                INTEGER (-67108864..67108863),

meanAnomaly-r17                INTEGER (0..268435455)

}

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

-- TAG-EPHEMERISINFO-STOP

-- ASN1STOP

With the following description:

EphemerisInfo field descriptions

Eccentricity Satellite orbital parameter: eccentricity e, see NIMA TR 8350.2 [71]. Unit is radian. Step of 1.431 * 10-8. Actual value = field value * (1.431 * 10-8).

Inclination Satellite orbital parameter: inclination i, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

Longitude Satellite orbital parameter: longitude of ascending node , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

meanAnomaly Satellite orbital parameter: Mean anomaly M at epoch time, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

Periapsis Satellite orbital parameter: argument of periapsis , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

semiMajorAxis Satellite orbital parameter: semi major axis , see NIMA TR 8350.2 [71]. Unit is meter. Step of 4.249 * 10-3 m. Actual value = 6500000 + field value * (4.249 * 10-3).

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

Those transfer functions previously presented are directly incorporated in the GMAT script used for emulating satellite movement, relative speed and velocity with respect to ground UE.

# Agreement on NTN UE testing

The test of NTN UE for NGSO satellite has been discussed at several occasions during RAN4#108 Toulouse. Meanwhile, after the RAN4#108 meeting, RAN#101 decided the following Way Forward in RP-232682 “Summary of offline discussions for NTN testing”:

Way Forward:

- RAN tasks RAN4 to discuss and conclude in Oct meeting on

  - The condition for testing the frequency error requirements to verify NGSO UE pre-compensation functionality for Doppler shift, which will be implemented in the RAN5 NTN specifications

    - For example, setting an artificial fixed Doppler shift, which is randomly selected out of a range of Doppler shift values decided by RAN4, as the condition in a test

  - The condition for RRM uplink timing test cases to verify NGSO UE delay pre-compensation functionality, which can be implemented in RAN5 NTN specifications

    - For example, setting an artificial fixed delay, which is randomly selected out of a range of delay values decided by RAN4, as the condition in a test

    - Note: For all the other NTN RRM test cases (except NGSO scenario in UE timing TCs), zero doppler shift and zero timing shift are assumed to be configured for RAN5 conformance testing

- Further discuss TE-emulated channel model with delay and Doppler shifts matching the satellite propagator model in future release i.e. Rel-19

- No changes to the RAN4 core requirements

More discussion took place before the RAN4#108-bis meeting and during the meeting, which generated the following documents:

- R4-2317963, Topic summary for [108bis][332] LS_BSRF_RAN_task;

- R4-2316953, Ad-hoc meeting minutes on LS_BSRF_RAN_task.

At the end, the final agreements for NGSO/GSO testing reached at RAN4#108-bis were:

(priority) Issue 1-1: [NGSO][RF] Range of Doppler shift for RF frequency error requirements

agreement

- For one of the cases in the freq error test, the range of absolute value of Doppler is within [0.1 or 7.5ppm] greater than zero and less than or equal to 24ppm.

  - The range can be further limited, considering other factors like elevation angles and satellite height.

  - The exact value(s) and the number of values (>1) to be selected are left to RAN5.

    - At least the worst-case value needs to be selected

  - UE should derive the amount of Doppler to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

    - The same ephemeris info will be maintained during each single frequency error measurement of the TE

Issue 1-2: [NGSO][RF] Assumption of delay shift for RF frequency error requirements

Agreement:

- Constant value from the same ephemeris (i.e., orbit emulation) as the Doppler shift is derived

- UE is assumed to derive the amount of delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of delay

(priority) Issue 1-3: [NGSO][RRM] Range of Delay shift for RRM UL timing accuracy test cases

Agreement:

- The range of the one-way delay from UE to satellite is within 2ms (lowest value for LEO orbit 600km) to 6.67ms (highest for LEO orbit 1200 km)

  - Assuming that the absolute elevation angle is larger than 30 degree

  - The range can be further limited, considering other factors like elevation angles and satellite height.

  - The set of values and the number of values is to be introduced by RAN4.  RAN5 to select from this set according to RAN4’s test case revision.

    - At least the worst-case value needs to be selected

  - UE should derive the amount of time delay to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

    - The same ephemeris info will be maintained during each single timing accuracy measurement of the TE

Issue 1-4: [NGSO][RRM] Assumption of Doppler shift for RRM UL timing accuracy test cases

Agreement:

- Constant value from the same ephemeris (i.e., orbit emulation) as the delay is derived

- UE is assumed to derive the amount of Doppler to be pre-compensated based on the ephemeris info (SIB-19 or SIB-31) and UE location.

- The requirement is defined assuming the UE pre-compensates the amount of Doppler

Issue 1-5: [NGSO][RRM] Assumption of Doppler and timing shifts for RRM test cases other than UL timing accuracy

Agreement: Constant non-zero time delay, with constant non-zero Doppler value per satellite

Sub-topic 1-2: GSO testing

Agreement:  Apply the same principles for GSO as agreed for NGSO

Finally, the following Way Forward has been approved as follows at RAN4#108-bis:

- R4-2316967	WF on NTN RAN task

- Chair:  Issue 2-5 indicates non-zero Doppler for GSO.  In addition, zero Doppler shall be considered for GEO.

# Serving GSO satellite data

The UE selected location is given by TS 36.508 (and respectively by TS 38.508-1):

The generated report file for SIB19/SIB31 then contain satellite ephemeris for each time of the propagation, for 2.56 sec simulation step and 7° inclination angle (worst case for GSO):

The following [units] have been considered: satellite elevation in UE frame in degree, one-way Delay in [ms], Doppler in [ppm], so the results remain relevant and representative also for other frequency ranges to be studied in future releases.

## Further considerations for serving GSO satellite

For any given UE location, the daily elevation variation remains rather low compared to NGSO cases. However, the UE location is limited by the Earth visibility domain with the minimum angle of 10° or 30°.

As an indication, the Earth visibility domain as seen by a satellite in geostationary position and 0° inclination for a minimum elevation of 0° is given the following figure:

Figure 1. Visibility domain from a geostationary satellite, Minimum UE elevation=0

This table gives at satellite longitude the maximum UE latitude with its minimum elevation angle:

Table 1. Maximum UE latitude with satellite longitude at UE longitude

| Minimum UE Elevation (degree) | Maximum UE latitude with satellite longitude at UE longitude (spherical Earth) |
| --- | --- |
| 0 | 81.30 |
| 5 | 76.33 |
| 10 | 71.43 |
| 15 | 66.60 |
| 20 | 61.83 |
| 25 | 57.12 |
| 30 | 52.47 |

As also provided in previous THALES contributions, GSO data for other UE latitude (55 degrees) have been provided in order to consider an elevation angle up to 30 degrees:

Therefore, for this above generated file only, UE coordinates were:

- Latitude 55 degrees;

- Longitude 121.56076999 degrees;

- Altitude 0 m.

Otherwise, for the other generated files, the normal UE coordinates are considered such as:

- Latitude 25.08439333 degrees;

- Longitude 121.56076999 degrees;

- Altitude 0 m.

## Further considerations for the Maximum Doppler of the serving GSO satellite

The maximum Doppler during simulation is about ±0.1 ppm. As a matter of fact, this is a common value for geostationary satellite with a fixed (or static) UE location.

Although TR 38.811 shows higher values, is important to mention that these higher results were obtained with UE in motion embarked on an aircraft:

Table 2. Extract of TR 38.811 Rel-15

Finally, the Figure 2 below shows satellite movement and Doppler values seen by the UE during the simulation for a fixed UE position (24 hours have been simulated):

Figure 2. Satellite orbit evolution during 24hrs with UE Doppler

In Figure 2, because of the (non-zero) inclination angle and in practice also because of Earth homogeneity/irregular shape for instance, the satellite orbit evolution shows a satellite latitude variation and a longitude variation during a 24h period with non-zero Doppler as seen by the UE. Since the satellite longitude was selected the same as the UE longitude, there is a variation of satellite longitude with respect to UE position (selected as reference).

# Serving GSO satellite data

With respect to previous observation, the neighbor satellite position was therefore selected such as the longitude of the neighbor satellites is +/-30 degrees longitude with respect to (same initial) reference UE longitude located at:

- Latitude 25.08439333 degrees;

- Longitude 121.56076999 degrees;

- Altitude 0 m.

For instance, the position of one of the neighbor satellite (with longitude at 91.56076999) is described below:

Figure 3. Neighbor GSO at longitude 91.56076999 degrees

And the following generated files for the 2 neighbor satellites (with longitudes at 91.56076999 and 151.56076999 respectively) are given below:

Given said that, is important to test with different serving and neighbor GSO in more dynamic conditions since the relative Doppler between serving and neighbor GSO (delta value experienced by the UE from serving to neighbor) may reach up to ± 0.2 ppm with a fixed (or static) UE location condition. For a moving UE case with GSO, Doppler conditions may be even higher as previously mentioned.

# Overall Conclusion

Proposal 1: For GSO NTN UE testing, RAN5 can use the following example file for the serving GSO satellite:

Proposal 2: For GSO NTN UE testing, RAN5 can use the following example file for the neighbor GSO satellites (-30 and 30 degrees longitude from the central point of longitude of serving GSO satellites):

Proposal 3: Intermediate GSO ephemeris values can be also selected, depending on different company willingness to test with different (non-zero) Doppler/Delay values.

Proposal 4: The Ephemeris/Elevation/Delay/Doppler provided data are applicable/are valid for both 5G NR NTN (SIB19) and NB-IoT NTN (SIB31/SIB32).

Proposal 5: The Ephemeris/Elevation/Delay/Doppler values can be used to test for different frequencies since Doppler values are given in .ppm.