3GPP TSG-RAN5 Meeting #102	R5-241525

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3105 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of signaling test frequencies for DC_2A_n2A and DC_66A_n66A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | WE Certification, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R16-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | EN-DC combos DC_2A_n2A and DC_66A_n66A are missing test frequencies for signalling testing |  |  |  |  |  |  |  |  |
| Summary of change: |  | EN-DC combos DC_2A_n2A and DC_66A_n66A signalling test frequencies are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN-DC combos DC_2A_n2A and DC_66A_n66A signalling test frequencies are not defined leaving testing ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.3.2	Test frequencies for EN-DC band combinations for signalling testing

##### 6.2.3.2.1	General

The default channel bandwidths for EN-DC signalling test are specified per NR and E-UTRA band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

##### 6.2.3.2.2	E-UTRA 1CC and NR 1CC

For EN-DC Inter-band case with E-UTRA 1CC and NR 1CC (one E-UTRA band and one NR band) the EN-DC configurations are specified in clause 4.3.1.4.1.2 for EN-DC with NR FR1 and 4.3.1.5.1.2 for EN-DC with NR FR2.

The E-UTRA and NR test frequencies are specified in TS 36.508 [2], clause 6.2.3.1 for the E-UTRA band (E-UTRA f1, f2, f3 and f4); and in clause 6.2.3.1 for the NR band (NRf1, NRf2, NRf3, NRf4) and for the secondary NR band (NRf5, NRf6, NRf7) of the secondary EN-DC inter-band configuration.

For EN-DC Intra-band Contiguous case with E-UTRA 1CC and NR 1CC the EN-DC configurations and the test frequencies are specified in Table 6.2.3.2-1.

For EN-DC Intra-band Non-Contiguous with E-UTRA 1CC and NR 1CC case the EN-DC configurations and test frequencies are specified in Table 6.2.3.2-2.

For EN-DC Intra-Band Contiguous and EN-DC Intra-Band Non-Contiguous cases with E-UTRA 1CC and NR 1CC the mapping of frequency ranges to NR test frequencies NRf1, NRf2, NRf3, and NRf4 to PSCell; and to E-UTRA test frequencies f1, f2, f3, and f4 for PCell are as follows:

-	for band combinations with only one test frequency: Low Range (NRf1, f1);

-	for band combinations with up to two frequencies: Low Range (NRf1, f1), High Range (NRf2, f2);

-	for band combinations with up to three frequencies: Mid Range (NRf3, f3), Low Range (NRf1, f1) and High Range (NRf2, f2);

-	for band combinations with up to four frequencies: Mid-Low Range (NRf3, f3), High Range (NRf2, f2), Low Range (NRf1, f1) and Mid-High Range (NRf4, f4).

Table 6.2.3.2.2-1: Test frequencies for EN-DC Intra-band Contiguous configurations with E-UTRA 1CC and NR 1CC

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)41AA | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | Low, High | Same values as for Low and High range values in Table 4.3.1.4.2.41.1-2 (SCS 30 kHz, 30 kHz NR raster and NR CC at the band edges) and EN-DC channel bandwidth combination “E-UTRA: 20MHz + NR: 60MHz”. |  |  |  |  |  |  |  |  |  |  |  |
|  | NR CC1 | 60 | 162 | Downlink & Uplink | Low, High |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_(n)71AA | E-UTRA | 5 | 25 | Downlink | Low, Mid, High | Same values as for Low, Mid and High range values in Table 4.3.1.4.2.71.1-1 (SCS 15 kHz, 100 kHz NR raster and NR CC at the band edges) and EN-DC channel bandwidth combination “E-UTRA: 5MHz + NR: 5MHz”. |  |  |  |  |  |  |  |  |  |  |  |
|  | CC1 |  |  | Uplink | Low, Mid, High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR | 5 | 25 | Downlink | Low, Mid, High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC1 |  |  | Uplink | Low, Mid, High |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.2.2-2: Test frequencies for EN-DC Intra-Band Non-Contiguous configurations with E-UTRA 1CC and NR 1CC

| EN-DC channel bandwidth combination | CC | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) | offsetToPointA (SIB1) [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n2A | E-UTRA CC1 | 20 | 100 | Downlink | High | Same values as for Test Frequency ID = ”Low with maxWgap (NR – E-UTRA)” in Table 4.3.1.4.2.1-1 with NR SCS=15 kHz and CBW=10MHz; and E-UTRA CBW=20 MHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR CC1 | 10 | 52 | Downlink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_41A_n41A | E-UTRA CC1 | 20 | 100 | Downlink & Uplink | Low | Same values as for Test Frequency ID = ”Low with maxWgap (NR – E-UTRA)” in Table 4.3.1.4.41.1-1 with NR SCS=30 kHz and CBW=60MHz; and E-UTRA CBW=20 MHz. |  |  |  |  |  |  |  |  |  |  |  |
|  | NR CC1 | 60 | 162 | Downlink & Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_66A_n66A | E-UTRA CC1 | 20 | 100 | Downlink | High | Same values as for Test Frequency ID = ”Low with maxWgap (NR – E-UTRA)” in Table 4.3.1.4.66.1-1 with NR SCS=15 kHz and CBW=10MHz; and E-UTRA CBW=20 MHz. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | NR CC1 | 10 | 52 | Downlink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Uplink | Low |  |  |  |  |  |  |  |  |  |  |  |  |