3GPP TSG-RAN5 Meeting #102	R5-241532

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4185 | rev | 1 | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MBS TC 14.1.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The referred steps and tables are incorrect in the Specific message contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Referred table numbers were corrected. Editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will exist in this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-240190 with one revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-240190 -> R5-241532 |  |  |  |  |  |  |  |  |

[Start of change]

14.1.1.3	MBS Broadcast/ MCCH Information Acquisition/ MCCH Information change notification

14.1.1.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC IDLE state and camped on a cell providing SIB20 and interested to receive MBS broadcast service }

ensure that {

when { UE is receiving a notification that the MCCH information has changed due to the start of new MBS service }

then { UE starts acquiring the MBSBroadcastConfiguration message on MCCH from the slot in which the change notification was received }

}

(2)

with { UE in NR RRC CONNECTED state and is receiving data via broadcast MRB }

ensure that {

when { UE is receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS session }

then { UE starts acquiring the MBSBroadcastConfiguration message on MCCH from the slot in which the change notification was received }

}

14.1.1.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300, clause 16.10.6.2; TS 38.331, clauses 5.9.1.3, 5.9.2.2 and 5.9.2.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 16.10.6.2]

The following principles govern the MCCH structure:

…

-	MCCH uses a modification period and MCCH contents are only allowed to be modified at each modification period boundary; A notification mechanism is used to announce the change of MCCH contents due to broadcast session start, modification or stop and due to neighbouring cell information modification;

NOTE:	It is up to UE implementation to use the start and stop times in the USD to determine when to start monitoring the MCCH for the session the UE is interested in.

-	When the UE receives a MCCH change notification, it acquires the updated MCCH in the same MCCH modification period where the change notification is sent.

[TS 38.331, clause 5.9.1.3]

Change of MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period).

When the network changes (some of) the MCCH information, it notifies the UEs about the change starting from the beginning of the MCCH modification period via PDCCH which schedules the MCCH in every repetition in that modification period.

Upon receiving a change notification, a UE receiving or interested to receive MBS services transmitted using MBS broadcast acquires the new MCCH information starting from the same slot. The UE applies the previously acquired MCCH information until the UE acquires the new MCCH information. The notification is transmitted with a 2-bit bitmap, see TS 38.212 [17] clause 7.3.1.5.1. The MSB in the 2-bit bitmap, when set to '1', indicates the start of new MBS service(s). The LSB in the 2-bit bitmap, when set to '1', indicates modification of MCCH information other than the change caused by start of new MBS service(s), e.g. modification of a configuration of an on-going MBS session(s), MBS session(s) stop or neighbouring cell information modification.

[TS 38.331, clause 5.9.2.2]

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s). A UE that is receiving data via broadcast MRB shall apply the MCCH information acquisition procedure upon receiving a notification that the MCCH information has changed due to MCCH information modification other than the change caused by the start of new MBS service(s).

[TS 38.331, clause 5.9.2.3]

An MBS capable UE interested to receive or receiving an MBS broadcast service shall:

1>	if the procedure is triggered by an MCCH information change notification:

2>	start acquiring the MBSBroadcastConfiguration message on MCCH in the concerned cell from the slot in which the change notification was received;

…

14.1.1.3.3	Test description

14.1.1.3.3.1	Pre-test conditions

System Simulator:

-	The SS configures the NR Cell 1 as the "Serving cell".

-	System information combination NR-20 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	UE is made interested in receiving MBS Broadcast service with MBS Service ID ‘000001’H.

Preamble:

-	The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.1.1.3.3.2	Test procedure sequence

Table 14.1.1.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS updates MBSBroadcastConfiguration message to include the configuration for MBS Service ID ‘000001’H. | - | - | - | - |
| 2 | The SS transmits MCCH information change notification due to the start of new MBS service. | - | - | - | - |
| 3-11a2 | Steps 1 to 9a2 of the generic procedures described in TS 38.508-1 [4] subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C. | - | - | - | - |
| - | Exception: Step 12 is repeated 5 times. | - | - | - | - |
| 12 | The SS transmits a MBS Packet on the MTCH with LCID=1 and g-RNTI = ’0001’H. | <-- | MBS Packet. | - | - |
| 13 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 14 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 15 | Check: Is the number of reported MBS Packets received on the MTCH in step 14 greater than zero? | - | - | 1 | P |
| 16 | The SS updates MBSBroadcastConfiguration message to modify the configuration for MBS Service ID ‘000001’H. | - | - | - | - |
| 17 | The SS transmits MCCH information change notification due to MBS service modification. | - | - | - | - |
| - | Exception: Step 18 is repeated 5 times. | - | - | - | - |
| 18 | The SS transmits a MBS Packet on the MTCH with LCID=1 and g-RNTI = ’0002’H. | <-- | MBS Packet. | - | - |
| 19 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 20 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 21 | Check: Is the number of reported MBS Packets received on the MTCH in step 20 greater than the number of reported in step 15? | - | - | 2 | P |
| 22 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 23 | The SS updates MBSBroadcastConfiguration message to exclude the configuration for MBS Service ID ‘000001’H. | - | - | - | - |
| 24 | The SS transmits MCCH information change notification due to MBS service stop. | - | - | - | - |
| - | Exception: Step 25 is repeated 5 times. | - | - | - | - |
| 25 | The SS transmits a MBS Packet on the MTCH with LCID=1 and g-RNTI = ’0002’H. | <-- | MBS Packet. | - | - |
| 26-33 | Steps 1 to 8 of the generic procedures described in TS 38.508-1 [4] subclause 4.5.4.2-3 are performed. | - | - | - | - |
| 34 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 35 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 36 | Check: Is the number of reported MBS Packets received on the MTCH in step 35 equal to the number of reported in step 20? | - | - | 2 | P |

14.1.1.3.3.3	Specific message contents

Table 14.1.1.3.3.3-1: SIB1 of NR Cell 1 (preamble and all steps, Table 14.1.1.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 14.1.1.3.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| si-SchedulingInfo-v1700 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { |  | 1 entry |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  |  | entry 1 |  |
| si-BroadcastStatus-r17 |  | broadcasting |  |  |
| si-WindowPosition-r17 |  | 2 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 |  | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { |  | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |  |
| type1-r17 |  | sibType20 |  |  |
| } |  |  |  |  |
| valueTag-r17 |  | 0 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.1.3.3.3-2: ServingCellConfigCommonSIB (Table 14.1.1.3.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 14.1.1.3.3.3-3 |  |
| } |  |  |  |

Table 14.1.1.3.3.3-3: DownlinkConfigCommonSIB (Table 14.1.1.3.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 14.1.1.3.3.3-4 |  |
| } |  |  |  |

Table 14.1.1.3.3.3-4: BWP-DownlinkCommon (Table 14.1.1.3.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with condition MBS_Broadcast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.3.3.3-5: SIB20 of NR Cell 1 (preamble and all steps, Table 14.1.1.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB20-r17 ::= SEQUENCE { |  |  |  |
| mcch-Config-r17 SEQUENCE { |  |  |  |
| mcch-WindowStartSlot-r17 | 6 |  | SCS15 OR SCS 120 |
|  | 3 |  | SCS30 |
| mcch-WindowDuration-r17 | Not present |  |  |
| } |  |  |  |
| cfr-ConfigMCCH-MTCH-r17 | CFR-ConfigMCCH-MTCH-r17 with condition SIB1_BWP | TS 38.508-1 [4], Table 4.6.7-2 |  |
| } |  |  |  |

Table 14.1.1.3.3.3-6: ACTIVATE TEST MODE (preamble, Table 14.1.1.3.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.1.1.3.3.3-7: CLOSE UE TEST LOOP (step 11a1, Table 14.1.1.3.3.2-1)

Derivation Path: TS 38.508-1 [4], clause 4.7A.2, condition UE TEST LOOP MODE C and Broadcast MRB

Table 14.1.1.3.3.3-8: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (steps 13, 19 and 34, Table 14.1.1.3.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-9

Table 14.1.1.3.3.3-9: MBSBroadcastConfiguration (preamble, steps 1, 16 and 23, Table 14.1.1.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5ABA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSBroadcastConfiguration-r17 := SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mbsBroadcastConfiguration-r17 SEQUENCE { |  |  |  |
| mbs-SessionInfoList-r17 | MBS-SessionInfoList-Service2 | Table 14.1.1.3.3.3-10 | Preamble, Step 23 |
|  | MBS-SessionInfoList-Service1and2 | Table 14.1.1.3.3.3-11 | Step 1, Step 16 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.3.3.3-10: MBS-SessionInfoList-Service2 (Table 14.1.1.3.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBS-SessionInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17 { | 1 entry |  |  |
| MBS-SessionInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI-r17 with condition Service2 | Table 14.1.1.3.3.3-12 |  |
| g-RNTI-r17 | ’0003’H |  |  |
| mrb-ListBroadcast-r17 SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17 { | 1 entry |  |  |
| MRB-InfoBroadcast-r17[1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config-r17 SEQUENCE { |  |  |  |
| pdcp-SN-SizeDL-r17 | Not present |  |  |
| headerCompression-r17 CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| t-Reordering-r17 | Not present |  |  |
| } |  |  |  |
| rlc-Config-r17 SEQUENCE { |  |  |  |
| logicalChannelIdentity-r17 | 2 |  |  |
| sn-FieldLength-r17 | Not present |  |  |
| t-Reassembly-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.3.3.3-11: MBS-SessionInfoList-Service1and2 (Table 14.1.1.3.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBS-SessionInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17 { | 2 entries |  |  |
| MBS-SessionInfo-r17[1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI-r17 with condition Service2 | Table 14.1.1.3.3.3-12 |  |
| g-RNTI-r17 | ’0003’H |  |  |
| mrb-ListBroadcast-r17 SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17 { | 1 entry |  |  |
| MRB-InfoBroadcast-r17[1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config-r17 SEQUENCE { |  |  |  |
| pdcp-SN-SizeDL-r17 | Not present |  |  |
| headerCompression-r17 CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| t-Reordering-r17 | Not present |  |  |
| } |  |  |  |
| rlc-Config-r17 SEQUENCE { |  |  |  |
| logicalChannelIdentity-r17 | 2 |  |  |
| sn-FieldLength-r17 | Not present |  |  |
| t-Reassembly-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mtch-SchedulingInfo-r17 | Not present |  |  |
| mtch-NeighbourCell-r17 | Not present |  |  |
| pdsch-ConfigIndex-r17 | Not present |  |  |
| mtch-SSB-MappingWindowIndex-r17 | Not present |  |  |
| } |  |  |  |
| MBS-SessionInfo-r17[2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI-r17 with condition Service1 | Table 14.1.1.3.3.3-12 |  |
| g-RNTI-r17 | ’0001’H |  | Step 1 |
|  | ’0002’H |  | Step 16 |
| mrb-ListBroadcast-r17 SEQUENCE (SIZE (1..maxNrofMRB-Broadcast-r17)) OF MRB-InfoBroadcast-r17 { | 1 entry |  |  |
| MRB-InfoBroadcast-r17[1] SEQUENCE { |  | entry 1 |  |
| pdcp-Config-r17 SEQUENCE { |  |  |  |
| pdcp-SN-SizeDL-r17 | Not present |  |  |
| headerCompression-r17 CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| t-Reordering-r17 | Not present |  |  |
| } |  |  |  |
| rlc-Config-r17 SEQUENCE { |  |  |  |
| logicalChannelIdentity-r17 | 1 |  |  |
| sn-FieldLength-r17 | Not present |  |  |
| t-Reassembly-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mtch-SchedulingInfo-r17 | Not present |  |  |
| mtch-NeighbourCell-r17 | Not present |  |  |
| pdsch-ConfigIndex-r17 | Not present |  |  |
| mtch-SSB-MappingWindowIndex-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.3.3.3-12: TMGI-r17 (Table 14.1.1.3.3.3-10 and Table 14.1.1.3.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| TMGI-r17 ::= SEQUENCE { |  |  |  |  |
| plmn-Id-r17 CHOICE { |  |  |  |  |
| plmn-Index-r17 |  | 1 |  |  |
| } |  |  |  |  |
| serviceId-r17 |  | ‘000002’H | OCTET STRING (SIZE (3)) | Service2 |
|  |  | ‘000001’H | OCTET STRING (SIZE (3)) | Service1 |
| } |  |  |  |  |
| Condition | Explanation |  |  |  |
| Service1 | Broadcast MBS Service with MBS service id ‘000001’H |  |  |  |
| Service2 | Broadcast MBS Service with MBS service id ‘000002’H |  |  |  |

Table 14.1.1.3.3.3-13: Physical layer parameters for DCI format 4_0 (all steps, Table14.1.1.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.5.1-1, condition MCCH-RNTI |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| MCCH change notification | MSB indicates no new MBS service(s) start. LSB indicates no modification of MCCH information other than the change caused by start of new MBS service(s). | “00” | NOT (Step 1, Step 17, Step 24) |
|  | MSB indicates a new MBS service start. LSB indicates no modification of MCCH information other than the change caused by start of new MBS service(s). | “10” | Step 1 |
|  | MSB indicates no new MBS service(s) start. LSB indicates the modification of MCCH information other than the change caused by start of new MBS service(s). | “01” | Step 17, Step 24 |

[End of change]