3GPP TSG-RAN5 Meeting #102	R5-241534

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4227 | rev | 1 | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MBS broadcast TC 14.1.1.1-Test Loop Mode C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1.According to 38.321, LCID+G-RNTI identified a logical channel for broadcast MRB.Therefore, same LCID with different G-RNTI can not be treated the same broadcast MRB. [38.331 6.2.1] NOTE 1: For MBS broadcast, a logical channel is identified based on G-RNTI and LCID if the same LCID is allocated for logical channels corresponding to different G-RNTIs. 2.36.509/38.509 request SS start test loop mode C after MRB establishment. [36.509 5.4.2] A prerequisite for UE test loop mode C is that at least one MTCH data radio bearer has been established between SS and UE. [38.509 5.3.2] Same as TS 36.509 [6], subclause 5.4.2 with the following exceptions: - where MTCH data radio bearer is replaced by MBS radio bearer; |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update test procedure to Close loop mode C after LCID+G-RNTI updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-240435r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.1.1.1	MBS Broadcast/ MCCH Information Acquisition/ entering the cell providing SIB20

14.1.1.1.1	Test Purpose (TP)

(1)

with { UE in switched off state and interested to receive MBS broadcast services }

ensure that {

when { UE is switched on and camped on a cell providing SIB20 }

then { UE acquires the MBSBroadcastConfiguration message at the next repetition period and starts MBS reception }

}

(2)

with { UE in NR RRC_IDLE state and receiving MBS broadcast services }

ensure that {

when { UE reselects to a cell providing SIB20 }

then { UE acquires the MBSBroadcastConfiguration message at the next repetition period and starts MBS reception }

}

(3)

with { UE in NR RRC_INACTIVE state and receiving MBS broadcast services }

ensure that {

when { UE reselects to a cell providing SIB20 }

then { UE acquires the MBSBroadcastConfiguration message at the next repetition period and starts MBS reception }

}

(4)

with { UE in NR RRC_CONNECTED state and receiving MBS broadcast services }

ensure that {

when { UE handovers to a cell providing SIB20 }

then { UE acquires the MBSBroadcastConfiguration message at the next repetition period and starts MBS reception }

}

(5)

with { UE received SIB20 in a cell}

ensure that {

when { MCCH is only scheduled in the slot indicated by mcch-WindowStartSlot }

then { UE acquires the MBSBroadcastConfiguration message in the slot indicated by mcch-WindowStartSlot }

(6)

with { UE received SIB20 in a cell}

ensure that {

when { MCCH is scheduled starting from the slot indicated by mcch-WindowStartSlot and during mcch-WindowDuration (larger than 1 slot) }

then { UE acquires the MBSBroadcastConfiguration message starting from the slot indicated by mcch-WindowStartSlot and during mcch-WindowDuration }

14.1.1.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.300, clause 16.10.6.2; TS 38.331, clauses 5.9.1.1, 5.9.1.2, 5.9.2.2 and 5.9.2.3. Unless otherwise stated these are Rel-17 requirements.

[TS 38.300, clause 16.10.6.2]

MBS broadcast can be received by UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. A UE can receive the MBS configuration for broadcast session (e.g., parameters needed for MTCH reception) via MCCH in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state. The parameters needed for the reception of MCCH are provided via System Information.

The following principles govern the MCCH structure:

-	MCCH provides the list of all broadcast services with ongoing sessions transmitted on MTCH(s) and the associated information for broadcast session includes MBS session ID, associated G-RNTI scheduling information and information about neighbouring cells providing certain service on MTCH(s). MCCH content is transmitted within periodically occurring time domain windows, referred to as MCCH transmission window defined by MCCH repetition period, MCCH window duration and radio frame/slot offset;

…

[TS 38.331, clause 5.9.1.1]

UE receiving or interested to receive MBS broadcast service(s) applies MBS broadcast procedures described in this clause as well as the MBS Interest Indication procedure as specified in clause 5.9.4.

MBS broadcast configuration information is provided on MCCH logical channel. MCCH carries the MBSBroadcastConfiguration message which indicates the MBS broadcast sessions that are provided in the cell as well as the corresponding scheduling related information for these sessions. Optionally, the MBSBroadcastConfiguration message may also contain a list of neighbour cells providing the same broadcast MBS service(s) as provided in the current cell. The configuration information required by the UE to receive MCCH is provided in SIB20. Additionally, System Information provides also an information related to service continuity of MBS broadcast in SIB21.

[TS 38.331, clause 5.9.1.2]

The MCCH information (i.e. information transmitted in messages sent over MCCH) is transmitted periodically, using a configurable repetition period and within a configured transmission window. MCCH transmissions (and the associated radio resources and MCS) are indicated via the PDCCH addressed to MCCH-RNTI. PDCCH monitoring occasion(s) for MCCH transmission are determined according to the common search space indicated by searchspaceMCCH. If searchspaceMCCH is set to zero, PDCCH monitoring occasions for MCCH message reception in the MCCH transmission window are the same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. If searchspaceMCCH is not set to zero, PDCCH monitoring occasions for MCCH message are determined based on search space indicated by searchspaceMCCH. PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from one in the MCCH transmission window.

[TS 38.331, clause 5.9.2.2]

A UE shall apply the MCCH information acquisition procedure upon becoming interested to receive MBS broadcast services. A UE interested to receive MBS broadcast services shall apply the MCCH information acquisition procedure upon entering the cell providing SIB20 (e.g. upon power on, following UE mobility), upon receiving SIB20 of an SCell via dedicated signalling and upon receiving a notification that the MCCH information has changed due to the start of new MBS service(s).

[TS 38.331, clause 5.9.2.3]

An MBS capable UE interested to receive or receiving an MBS broadcast service shall:

…

1>	if the UE enters a cell broadcasting SIB20; or

1>	if the UE receives sCellSIB20:

2>	acquire the MBSBroadcastConfiguration message on MCCH in the concerned cell at the next repetition period.

14.1.1.1.3	Test description

14.1.1.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 (TAI-1) and NR Cell 2 (TAI-1).

-	The SS configures the NR Cell 1 as the "Serving cell" and NR Cell 2 as "Non-suitable "Off" cell".

-	System information combination NR-20 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 0N-B as defined in TS 38.508-1 [4], subclause 4.4A.

-	Before being switched off the UE is made interested in receiving MBS Broadcast service with MBS Service ID  ‘000001’H

14.1.1.1.3.2	Test procedure sequence

Table 14.1.1.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-21 | Steps 1 to 20a1 of the registration procedure described in TS 38.508-1 subclause 4.5.2.2-2 are performed on NR Cell 1 with condition UE TEST LOOP MODE C. NOTE: The UE performs registration and activate test mode C and the RRC connection is released. | - |  | - | - |
| 22 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. (Note 1) | - | - | - | - |
| 23-31 | Steps 1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 1 with condition UE TEST LOOP MODE C. | - | - | - | - |
| - | Exception: Step 32 is repeated 5 times | - | - | - | - |
| 32 | The SS transmits a MBS Packet on the MTCH with LCID=1 and g-RNTI = ’0001’H. | <-- | MBS Packet. | - | - |
| 33 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 34 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC:UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 35 | Check: Is the number of reported MBS Packets received on the MTCH in step 34 greater than zero? | - | - | 1,6 | P |
| 36 | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| 37 | The SS configures: -NR Cell 1 as the "Non-suitable cell". -NR Cell 2 as the "Serving cell". | - | - | - | - |
| 38 | Wait for 34[FR1]/130[FR2] seconds.(Note 2) | - | - | - | - |
| 39 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 2. (Note 1) | - | - | - | - |
| 39A-39H | Steps 1 to 8 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 2. | - | - | - | - |
| 39I | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 39J | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 39K-39L | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C. | - | - | - | - |
| 39M | The SS transmits an RRCRelease message. | <-- | NR RRC: RRCRelease | - | - |
| - | Exception: Step 40 is repeated 5 times | - | - | - | - |
| 40 | The SS transmits a MBS Packet on the MTCH with LCID= 1 and g-RNTI = ’FFF2’H. | <-- | MBS Packet | - | - |
| 41-48 | Steps 1 to 8 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed on NR Cell 2. | - | - | - | - |
| 49 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 50 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 51 | Check: Is the number of reported MBS Packets received on the MTCH in step 50 greater than zero? | - | - | 2,6 | P |
| 52 | SS change NR Cell 1 system information | - | - | - | - |
| 53 | The SS configures: -NR Cell 1 as the "Serving cell ". -NR Cell 2 as the "Suitable neighbour intra-frequency cell ". |  |  |  |  |
| 54 | The SS transmits an RRCReconfiguration message including reconfigurationWithSync to order the UE to perform intra-frequency handover to NR Cell 1 | <-- | NR RRC: RRCReconfiguration | - | - |
| 55 | Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 1? | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 56 | Wait for a scheduling period for SIB20. | - | - | - | - |
| 57 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 1. (Note 3) | - | - | - | - |
| 57A | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 57B | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 57C-57D | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C. | - | - | - | - |
| - | Exception: Step 58 is repeated 5 times | - | - | - | - |
| 58 | The SS transmits a MBS Packet on the MTCH with LCID=1 and g-RNTI = ’0002’H | <-- | MBS Packet | - | - |
| 59 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 60 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 61 | Check: Is the number of reported MBS Packets received on the MTCH in step 60 greater than zero? | - | - | 4,5 | P |
| - | EXCEPTION: Steps 62a1-62a13 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if inactiveState is configured | - | - | - | - |
| 62a1 | IF pc_inactiveState THEN the SS transmits an RRCRelease message with suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| 62a2 | SS change NR Cell 2 system information | - | - | - | - |
| 62a3 | The SS configures: -NR Cell 1 as the "Non-suitable cell". -NR Cell 2 as the "Serving cell". | - | - | - | - |
| 62a4 | Wait for 8[FR1]/27[FR2] seconds.(Note 2) | - | - | - | - |
| 62a5 | Wait for a period equal to the MCCH repetition period for the UE to receive MBSBroadcastConfiguration message on NR Cell 2. (Note 3) | - | - | - | - |
| 62a5A | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 62a5B | The UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest | - | - |
| 62a5C | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 62a5D | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 62a5E | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 62a5F | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 62a5G-62a5H | Steps 9a1 to 9a2 of the generic procedures described in TS 38.508-1 subclause 4.5.4.2-3 are performed with condition UE TEST LOOP MODE C. | - | - | - | - |
| 62a5I | The SS transmits an RRCRelease message with suspendConfig. | <-- | NR RRC: RRCRelease | - | - |
| - | Exception: Step 62a6 is repeated 5 times | - | - | - | - |
| 62a6 | The SS transmits a MBS Packet on the MTCH with LCID=1 and g-RNTI = ’FFF1’H. | <-- | MBS Packet | - | - |
| 62a7 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 62a8 | The UE transmits an RRCResumeRequest message. | --> | NR RRC: RRCResumeRequest | - | - |
| 62a9 | The SS transmits an RRCResume message. | <-- | NR RRC: RRCResume | - | - |
| 62a10 | The UE transmits an RRCResumeComplete message. | --> | NR RRC: RRCResumeComplete | - | - |
| 62a11 | The SS transmits an UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST message. | <-- | NR RRC: DLInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST | - | - |
| 62a12 | UE responds with UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE. | --> | NR RRC: ULInformationTransfer TC: UE TEST LOOP MODE C MBMS PACKET COUNTER RESPONSE | - | - |
| 62a13 | Check: Is the number of reported MBS Packets received on the MTCH in step 62a12 greater than zero? | - | - | 3,5 | P |
| Note 1: MAC PDU for MBSBroadcastConfiguration is 15 bytes (12 bytes RLC SDU + 1 byte UMD PDU header + 2 bytes MAC sub Header) and DL assignments are set to 96 bits (LRBs & IMCS as per 38.523-3[3] annex B) so that MBSBroadcastConfiguration need to be sent in 2 slots. Note 2: The wait time at steps 38 is the cell re-selection delay to a newly detectable cell, it can be expressed as: Tdetect,NR_Intra (as per TS 38.133 [30], clause 4.2.2.3) plus the time to read the system information TSI-NR (1280ms). Note 3: MAC PDU for MBSBroadcastConfiguration is 15 bytes (12 bytes RLC SDU + 1 byte UMD PDU header + 2 bytes MAC sub Header) and DL assignment is set to larger than 120 bits (LRBs & IMCS as per 38.523-3[3] annex B) so that MBSBroadcastConfiguration could to be sent in 1 slot. Note 4: The wait time at steps 62a4 is the cell re-selection delay to an already detected cell, it can be expressed as: Tevaluate,NR_Intra (as per TS 38.133 [30], clause 4.2.2.3) plus the time to read the system information TSI-NR (1280ms). |  |  |  |  |  |

14.1.1.1.3.3	Specific message contents

Table 14.1.1.1.3.3-1: SIB1 of NR Cell 1 and NR Cell 2 (preamble and all steps, Table 14.1.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| servingCellConfigCommon | ServingCellConfigCommonSIB | Table 14.1.1.1.3.3-2 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| si-SchedulingInfo-v1700 SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17 { | 1 entry |  |  |
| SchedulingInfo2-r17 [1] SEQUENCE { |  | entry 1 |  |
| si-BroadcastStatus-r17 | broadcasting |  |  |
| si-WindowPosition-r17 | 2 | entry number for si-SchedulingInfo in SIB1 +1 |  |
| si-Periodicity-r17 | 64 |  |  |
| sib-MappingInfo-r17 SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700 { | 1 entry |  |  |
| SIB-TypeInfo-v1700 [1] SEQUENCE { |  | entry 1 |  |
| sibType-r17 CHOICE { |  |  |  |
| type1-r17 | sibType20 |  |  |
| } |  |  |  |
| valueTag-r17 | 0 |  |  |
|  | 1 |  | step 52 and step 62a2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.1.3.3-2: ServingCellConfigCommonSIB (Table 14.1.1.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB | Table 14.1.1.1.3.3-3 |  |
| } |  |  |  |

Table 14.1.1.1.3.3-3: DownlinkConfigCommonSIB (Table 14.1.1.1.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-53 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DownlinkConfigCommonSIB ::= SEQUENCE { |  |  |  |
| initialDownlinkBWP | BWP-DownlinkCommon | Table 14.1.1.1.3.3-4 |  |
| } |  |  |  |

Table 14.1.1.1.3.3-4: BWP-DownlinkCommon (Table 14.1.1.1.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-10 with condition InitialBWP_SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-DownlinkCommon ::= SEQUENCE { |  |  |  |
| pdcch-ConfigCommon CHOICE { |  |  |  |
| setup | PDCCH-ConfigCommon with conditionMBS_Broadcast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.1.3.3-5: SIB20 of NR Cell 1 and NR Cell 2 (preamble and all steps, Table 14.1.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB20-r17 ::= SEQUENCE { |  |  |  |
| mcch-Config-r17 SEQUENCE { |  |  |  |
| mcch-WindowStartSlot-r17 | 6 |  | SCS15 OR SCS 120 |
|  | 3 |  | SCS30 |
|  | 2 |  | step52 and step 62a2 |
| mcch-WindowDuration-r17 | sl2 |  |  |
|  | Not present |  | step52 and step 62a2 |
| } |  |  |  |
| cfr-ConfigMCCH-MTCH-r17 | Not present |  | NR Cell 1 |
|  | CFR-ConfigMCCH-MTCH-r17 with condition SIB1_BWP | TS 38.508-1 [4], Table 4.6.7-2 | NR Cell 2 |
| } |  |  |  |

Table 14.1.1.1.3.3-6: ACTIVATE TEST MODE (step 10a1, Table 14.1.1.1.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.1.1.1.3.3-7: CLOSE UE TEST LOOP (step 31a1, step 39K, step 57C, step 62a5G, Table 14.1.1.1.3.2-1)

Derivation Path: 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Broadcast MRB

Table 14.1.1.1.3.3-8: UE TEST LOOP MODE C MBMS PACKET COUNTER REQUEST (step 33, step 49, step 59, step 62a11, Table 14.1.1.1.3.2-1)

Derivation Path: 36.508 [6], Table 4.7A-9

Table 14.1.1.1.3.3-9: MBSBroadcastConfiguration (step 22, step39, step57, step 62a5, Table 14.1.1.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-5ABA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBSBroadcastConfiguration-r17 := SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| mbsBroadcastConfiguration-r17 SEQUENCE { |  |  |  |
| mbs-SessionInfoList-r17 | MBS-SessionInfoList | Table 14.1.1.1.3.3-10 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note : The size for MBSBroadcastConfiguration-r17 is 12 bytes |  |  |  |

Table 14.1.1.1.3.3-10: MBS-SessionInfoList (Table 14.1.1.1.3.3-9)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBS-SessionInfoList-r17 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17 { | 1 entry |  |  |
| MBS-SessionInfo-r17[1] SEQUENCE { |  |  |  |
| g-RNTI-r17 | ’0001’H |  | step 22 for NR Cell 1 |
|  | ’0002’H |  | step 57 for NR Cell 1 |
|  | ’FFF2’H |  | step 39 for NR Cell 2 |
|  | ’FFF1’H |  | step 62a5 for NR Cell 2 |
| } |  |  |  |
| } |  |  |  |

Table 14.1.1.1.3.3-11: RRCRelease (step 62a1, step62a5I, Table 14.1.1.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-16 with condition NR_RRC_INACTIVE

Table 14.1.1.1.3.3-12: Paging (step 62a7, 62a5A, Table 14.1.1.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

<End of modified section 1>