3GPP TSG-RAN5 Meeting #102	R5-241565

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3063 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to SIB19 Common Config for NR-NTN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Timing advance value is missing for common NTN config |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added default values for ta-info-r17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NTN test cases will not be implementatble |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.6.3-84C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-240649r1: Table 4.6.3-84D Removed. TA-Info for GSO updated in the NTN-Config Table R5-240649r2: Updated Ephemeris reference R5-241565 is the final version of R5-240649: Unaltered sections are removed. TA value formatted to TAC |  |  |  |  |  |  |  |  |

<Beginning of changes>

### 4.6.3	Radio resource control information elements

<Unchanged sections omitted >

#### –	NTN-Config

Table 4.6.3-84C: NTN-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NTN-Config-r17 ::= SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  | GSO |
| epochTime-r17 SEQUENCE { |  |  | NGSO |
| sfn-r17 | FFS |  |  |
| subFrameNR-r17 | FFS |  |  |
| } |  |  |  |
| ntn-UlSyncValidityDuration-r17 | s240 |  |  |
| cellSpecificKoffset-r17 | 550 |  |  |
| kmac-r17 | Not present |  |  |
| ta-Info-r17 SEQUENCE { |  |  | GSO |
| ta-Common-r17 | 8243100 |  |  |
| ta-CommonDrift-r17 | 0 |  |  |
| ta-CommonDriftVariant-r17 | 0 |  |  |
| } |  |  |  |
| ta-Info-r17 | FFS |  | NGSO |
| ntn-PolarizationDL-r17 | linear |  |  |
| ntn-PolarizationUL-r17 | linear |  |  |
| ephemerisInfo-r17 | EphemerisInfo with condition GSO (See Cl 6.3.4.1) |  | GSO |
|  | EphemerisInfo with condition NGSO (See Cl 6.3.4.2) |  | NGSO |
| ta-Report-r17 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

#### –	NZP-CSI-RS-Resource

Table 4.6.3-85: NZP-CSI-RS-Resource

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | NZP-CSI-RS-ResourceId |  |  |
| resourceMapping | CSI-RS-ResourceMapping |  |  |
| powerControlOffset | -3 |  |  |
| powerControlOffsetSS | Not present |  |  |
| scramblingID | ScramblingId |  |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset |  |  |
| qcl-InfoPeriodicCSI-RS | TCI-StateId |  |  |
| } |  |  |  |

<End of changes >