3GPP TSG-RAN5 Meeting #102	R5-241579

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-2 | CR | 0339 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating applicability for video |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - As per 26.114 support of video codec is only required if the device supports video but at the moment the status is “m”. Additionally requirements for “constrained terminals” (many RedCap devices will be such terminals) is different: “ Constrained terminal: UE that is (i) operating in radio access capability category series "M" capable of supporting conversational services, and/or (ii) a wearable device which is constrained in size, weight or power consumption (e.g. connected watches), excluding smartphones and feature phones. ... MTSI clients in terminals offering speech communication shall support narrowband, wideband and super-wideband communication. The only exception to this requirement is for the MTSI client in constrained terminal offering speech communication, in which case the MTSI client in constrained terminal shall support narrowband and wideband, and should support super-wideband communication. ... MTSI clients in terminals offering video communication shall support: - H.264 (AVC) [24] Constrained Baseline Profile (CBP) Level 1.2; - H.265 (HEVC) [119] Main Profile, Main Tier, Level 3.1. The only exception to this requirement is for the MTSI client in constrained terminal offering video communication, in which case the MTSI client in constrained terminal should support H.265 (HEVC) Main Profile, Main Tier, Level 3.1. In addition they should support: - H.264 (AVC) [24] Constrained High Profile (CHP) Level 3.1. For backwards compatibility to previous releases, if H.264 (AVC) [24] Constrained High Profile Level 3.1 is supported, then H.264 (AVC) [24] Constrained Baseline Profile (CBP) Level 3.1 should also be offered.” - There was a note added to NG.114/video as a feature but no similar note was added to other Video fearue tag. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected status for video codec to align it with TS 26.114. - Adding note to the video feature tag |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Status for video codecs will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.7 and A.4.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

A.4.7	MTSI media

Table A.15: MTSI media

| Item | Media | Reference | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Speech | 26.114 [56], 5.2.1 | o | Rel-7 | pc_MTSI_Speech |  |
|  |  | IR.92 [83], 3.2 | m |  |  |  |
|  |  | IR 51 [84], 3 | m |  |  |  |
|  |  | NG.108 [87] | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 2 | Speech, AMR wideband | 26.114 [56], 5.2.1 | o | Rel-7 | pc_MTSI_Speech_AMRWB |  |
|  |  | IR.51 [84], 3 | m |  |  |  |
|  |  | IR.92 [83], 3.2 | m |  |  |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 3 | Video | 26.114 [56], 5.2.2 | o | Rel-7 | pc_MTSI_Video |  |
|  |  | IR.94 [75], 3.3 | m |  |  |  |
|  |  | IR.51 [84], 3 | m |  |  |  |
|  |  | NG.114 [96] | m NOTE 1 | Rel-15 |  |  |
| 4 | Video, H.263 Profile 3 | 26.114 [56], 5.2.2 | o | Rel-7 |  |  |
| 5 | Video, MPEG-4 | 26.114 [56], 5.2.2 | o | Rel-7 |  |  |
| 6 | Video, H.264 | 26.114 [56], 5.2.2 | o | Rel-7 |  |  |
| 7 | Void |  |  |  |  |  |
| 8 | Void |  |  |  |  |  |
| 9 | Video codec H.264 CBP Level 1.2 | 26.114 [56], 5.2.2 | o | Rel-8 | pc_VideoCodecH264CBP |  |
|  |  | IR.94 [75], 3.3 | m |  |  |  |
|  |  | IR.51 [84], 3 | m |  |  |  |
| 10 | Speech, EVS | 26.114 [56] | o | Rel-12 | pc_MTSI_Speech_EVS |  |
|  |  | NG.114 [96] | m | Rel-15 |  |  |
| 11 | Video codec H.265 (HEVC), Main Profile, Main Tier, Level 3.1 | 26.114 [56], 5.2.2 | c1 | Rel-15 | pc_VideoCodecH265Lv31 |  |
|  |  | NG.114 [96] | m NOTE 1 | Rel-15 |  |  |
| 12 | Video codec H.264 (AVC) CHP Level 3.1 | 26.114 [56], 5.2.2 | o | Rel-15 | pc_VideoCodecH264CHPLv31 |  |
|  |  | NG.114 [96] | m NOTE 1 | Rel-15 |  |  |
| 13 | Video codec H.264 (AVC) CBP Level 3.1 | 26.114 [56], 5.2.2 | o | Rel-15 | pc_VideoCodecH264CBPLv31 |  |
|  |  | NG.114 [96] | m NOTE 1 | Rel-15 |  |  |
| NOTE 1: As an exception to NG.114 support of MTSI Video Call is optional. c1: IF NOT A.15/3 OR [93] A.4.3.12-1/2 THEN o ELSE m |  |  |  |  |  |  |

[Skip unchanged text]

A.4.14	NG.114 capabilities

Table A.22: NG.114 capabilities

| Item |  | Ref. | Status | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | mmtel ICSI value | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_mmtel |  |
| 2 | audio feature tag | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_audio |  |
| 3 | video feature tag | NG.114 v1.0 [96] | m NOTE 1 | n/a | pc_NG114_v1_0_video |  |
| 4 | Void |  |  |  |  |  |
| 5 | Void |  |  |  |  |  |
| 6 | Void |  |  |  |  |  |
| 7 | Void |  |  |  |  |  |
| 8 | Annex C.3 default configuration for voice exempt | NG.114 v2.0 [98] | m | n/a | pc_NG114_v1_0_default_config_voice_exempt |  |
| 9 | Annex C.3 default configuration for video exempt | NG.114 v2.0 [98] | m | n/a | pc_NG114_v2_0_default_config_video_exempt |  |
| 10 | Annex C.3 default configuration for EVS/Br | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_default_config_evs_br |  |
| 11 | Annex C.3 default configuration for EVS/Bw | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_default_config_evs_bw |  |
| 12 | Void |  |  |  |  |  |
| 13 | Void |  |  |  |  |  |
| 14 | Forking behavior | NG.114 v1.0 [96] | m | n/a | pc_NG114_v1_0_forking |  |
| 15 | EVS Configuration B0 | NG.114 v1.0 [96] | o |  | pc_NG114_v1_0_EVS_B0 |  |
| 16 | EVS Configuration B1 | NG.114 v1.0 [96] | o |  | pc_NG114_v1_0_EVS_B1 |  |
| 17 | EVS Configuration B2 | NG.114 v1.0 [96] | o |  | pc_NG114_v1_0_EVS_B2 |  |
| 18 | EVS Configuration A1 | NG.114 v1.0 [96] | o |  | pc_NG114_v1_0_EVS_A1 |  |
| NOTE 1: As an exception to NG.114 support of MTSI Video Call is optional. |  |  |  |  |  |  |

[End of change]