3GPP TSG-RAN5 Meeting #102	R5-241588

# Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4215 | rev | 1 | Current version: | 17.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 9.1.11.6 for Rel-17 eNPN for Mobility management in Rel-17 eNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of Rel-17 eNPN TC 9.1.11.6 for Mobility management in Rel-17 eNPN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Rel-17 eNPN TC 9.1.11.6 for Mobility management in Rel-17 eNPN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNPN WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.11.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0433 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-240357. |  |  |  |  |  |  |  |  |

9.1.11.6	SNPN / EAP-AKA' related procedures / UE handling EAP-failure message / Register for onboarding services and access to an SNPN using credentials from a credentials holder

9.1.11.6.1	Test Purpose (TP)

(1)

with { The UE in 5GMM-REGISTERED-INITIATED state for onboarding service and SS initiates an EAP based primary authentication and key agreement procedure }

ensure that {

when { The SS sends an EAP-failure message within AUTHENTICATION REJECT }

then { The UE shall store the SNPN identity in the "permanently forbidden SNPNs" list for onboarding services, enter state 5GMM-DEREGISTERED.PLMN-SEARCH, and perform SNPN selection }

}

(2)

with { The UE in 5GMM-REGISTERED-INITIATED state using credentials from a credentials holder and SS initiates a EAP based primary authentication and key agreement procedure }

ensure that {

when { The SS sends an EAP-failure message within AUTHENTICATION REJECT }

then { The UE shall consider the entry of the selected entry of the "list of subscriber data" as invalid for 3GPP access until the UE is switched off or the entry is updated, and the USIM is considered invalid until switching off the UE }

}

9.1.11.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501 clauses 5.4.1.2.2.11 and 5.5.1.2.2, TS 23.122 clause 4.9.3.0. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, clause 5.4.1.2.2.11]

Upon receiving an EAP-failure message, the UE shall delete the partial native 5G NAS security context and shall delete the new KAUSF and the new KSEAF, if any were created as described in subclause 5.4.1.2.2.3.

The UE shall consider the procedure complete.

If the EAP-failure message is received in an AUTHENTICATION REJECT message:

1)	if the AUTHENTICATION REJECT message has been successfully integrity checked by the NAS:

-	the UE shall set the update status to 5U3 ROAMING NOT ALLOWED, delete the stored 5G-GUTI, TAI list, last visited registered TAI and ngKSI;

In case of PLMN, the USIM shall be considered invalid until switching off the UE or the UICC containing the USIM is removed.

In case of SNPN, if the UE is neither registered for onboarding services in SNPN nor performing initial registration for onboarding services in SNPN and the UE does not support access to an SNPN using credentials from a credentials holder, the entry of the "list of subscriber data" with the SNPN identity of the current SNPN shall be considered invalid until the UE is switched off or the entry is updated. Additionally, the UE shall consider the USIM as invalid for the current SNPN until switching off or the UICC containing the USIM is removed.

In case of SNPN, if the UE is neither registered for onboarding services in SNPN nor performing initial registration for onboarding services in SNPN and the UE supports access to an SNPN using credentials from a credentials holder, the UE shall consider the selected entry of the "list of subscriber data" as invalid until the UE is switched off or the entry is updated. Additionally, the UE shall consider the USIM as invalid for the entry until switching off or the UICC containing the USIM is removed.

If the UE is registered for onboarding services in SNPN or is performing initial registration for onboarding services in SNPN, the UE shall store the SNPN identity in the "permanently forbidden SNPNs" list for onboarding services, enter state 5GMM-DEREGISTERED.PLMN-SEARCH, and perform an SNPN selection or an SNPN selection for onboarding services according to 3GPP TS 23.122 [5];

-	if the UE is neither registered for onboarding services in SNPN nor performing initial registration for onboarding services in SNPN, the UE shall set:

i)	the counter for "SIM/USIM considered invalid for GPRS services" events, the counter for "USIM considered invalid for 5GS services over non-3GPP access" events, and the counter for "SIM/USIM considered invalid for non-GPRS services" events if maintained by the UE, in case of PLMN; or

ii)	the counter for "the entry for the current SNPN considered invalid for 3GPP access" events and the counter for "the entry for the current SNPN considered invalid for non-3GPP access" events in case of SNPN;

to UE implementation-specific maximum value.

If the UE is registered for onboarding services in SNPN or performing initial registration for onboarding services in SNPN, the UE shall set the SNPN-specific attempt counter for the current SNPN to the UE implementation-specific maximum value; and

-	if the UE is operating in single-registration mode, the UE shall handle EMM parameters, 4G-GUTI, last visited registered TAI, TAI list and eKSI as specified in 3GPP TS 24.301 [15] for the case when the authentication procedure is not accepted by the network. The USIM shall be considered as invalid also for non-EPS services until switching off or the UICC containing the USIM is removed; and

[TS 24.501, clause 5.5.1.2.2]

The UE in state 5GMM-DEREGISTERED shall initiate the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF,

…

If the UE supports access to an SNPN using credentials from a credentials holder and the UE is in its HPLMN or EHPLMN or a subscribed SNPN, the UE shall set the SSNPNSI bit to "SOR-SNPN-SI supported" in the 5GMM capability IE of the REGISTRATION REQUEST message.

[TS 23.122 clause 4.9.3.0]

The ME is configured with a "list of subscriber data" containing zero or more entries. Each entry of the "list of subscriber data" consists of:

…

The MS shall add an SNPN to the list of "permanently forbidden SNPNs" (for onboarding services, if the MS is registered for onboarding services in SNPN or performing initial registration for onboarding services in SNPN) which is, if the MS supports access to an SNPN using credentials from a credentials holder, associated with the selected entry of the "list of subscriber data" or the selected PLMN subscription, if a message with cause value #75 "Permanently not authorized for this SNPN", #3 "Illegal UE" (applicable in an onboarding SNPN only), #6 "Illegal ME" (applicable in an onboarding SNPN only), or #7 "5GS services not allowed" (applicable in an onboarding SNPN only) (see 3GPP TS 24.501 [64]) is received by the MS in response to an LR request from the SNPN.

The MS shall remove an SNPN from the list of "permanently forbidden SNPNs" (for onboarding services, if the MS is registered for onboarding services in SNPN or performing initial registration for onboarding services in SNPN) which is, if the MS supports access to an SNPN using credentials from a credentials holder, associated with the selected entry of the "list of subscriber data" or the selected PLMN subscription, if:

a)	there is a successful LR after a subsequent manual selection of the SNPN;

b)	the MS is configured to use timer T3245 and timer T3245 expires;

c)	the MS is not configured to use timer T3245, the timer T3247 expires and the value of the SNPN-specific attempt counter for that SNPN is less than the MS implementation specific maximum value as defined in 3GPP TS 24.501 [64];

d)	an entry of the "list of subscriber data" with the subscribed SNPN identity identifying the SNPN is updated or the USIM is removed if:

-	EAP based primary authentication and key agreement procedure using EAP-AKA'; or

-	5G AKA based primary authentication and key agreement procedure;

was performed in the selected SNPN; or

e)	the selected entry of the "list of subscriber data" is updated or USIM is removed for the selected PLMN subscription.

9.1.11.6.3	Test description

9.1.11.6.3.1	Pre-test conditions

System Simulator:

-	2 SNPN cell: NGC Cell A and B are configured according to Table 6.3.2.2-1 broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-26 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NGC cells.

UE:

-	The UE is in Automatic SNPN selection mode.

-	The UE is provisioned default UE credentials and pre-configured with onboarding SNPN selection information to allow access to SNPN identified by NGC Cell A and is configured with a "list of subscriber data" to allow access to SNPN identified by NGC Cell B.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN, and the UE is in state Switched OFF (state 0-A).

9.1.11.6.3.2	Test procedure sequence

Table 9.1.11.6.3.2-1/2 shows the cell configurations used during the test. Subsequent configurations marked “T1”, “T2” and “T3” are applied at the points indicated in the Main behaviour description in Table 9.1.11.6.3.2-3. Cell power is chosen for a serving cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 9.1.11.6.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NGC Cell A | NGC Cell B |  |  | Remark |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | “Off” |  |  |  |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 |  |  |  |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -88 |  |

Table 9.1.11.6.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NGC Cell A | NGC Cell B |  | Remark |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -82 | “Off” |  |  |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 |  |  |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -82 |  |

Table 9.1.11.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS adjusts cell levels according to row T1 of table 9.1.11.6.3.2-1/2. | - | - | - | - |
| 2 | The UE is switched on. | - | - | - | - |
| 3-5 | The UE establishes RRC connection and initiates registration procedure on NGC Cell A by executing steps 2-4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 6 | SS transmits a correct AUTHENTICATION REQUEST message with an EAP-Request/AKA'-challenge message. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 7 | The UE respond with a correct AUTHENTICATION RESPONSE message with an EAP-Response/AKA'-challenge message. | --> | 5GMM: AUTHENTICATION RESPONSE | - | - |
| 8 | The SS transmits an “EAP-failure” message within AUTHENTICATION REJECT | <-- | 5GMM: AUTHENTICATION REJECT | - | - |
| 9 | The SS releases the RRC connection | - | - | - | - |
|  | The SS adjusts cell levels according to row T2 of table 9.1.11.6.3.2-1/2. |  |  |  |  |
| 10 | Check: Does the UE transmit an RRCSetupRequest message for initial registration procedure on NGC Cell B within the next 30 seconds? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 11-12 | The UE establishes RRC connection and initiates registration procedure on Cell B by executing steps 3-4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 13 | The SS transmits a correct AUTHENTICATION REQUEST message with an EAP-Request/AKA'-challenge message. | <-- | 5GMM: AUTHENTICATION REQUEST | - | - |
| 14 | The UE respond with a correct AUTHENTICATION RESPONSE message with an EAP-Response/AKA'-challenge message. | --> | 5GMM: AUTHENTICATION RESPONSE | - | - |
| 15 | The SS transmits an “EAP-failure” message within AUTHENTICATION REJECT | <-- | 5GMM: AUTHENTICATION REJECT | - | - |
| 16 | The SS releases the RRC connection | - | - | - | - |
| 17 | Check: Does the UE transmit an RRCSetupRequest message for initial registration procedure on NGC Cell B within the next 30 seconds? | --> | NR RRC: RRCSetupRequest | 2 | F |
| 18 | The SS adjusts cell levels according to row T3 of table 9.1.11.6.3.2-1/2. | - | - | - | - |
| 19 | The UE is switched off by executing generic procedure in Table 4.9.6.4-1 in TS 38.508-1 [4]. | - | - | - | - |
| 20 | The UE is switched on. | - | - | - | - |
| 21 | Check: Does the UE transmit an RRCSetupRequest message for initial registration procedure on NGC Cell B within the next 30 seconds? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 22-39 | Steps 3 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 are performed on NGC Cell B. | - | - | - | - |

9.1.11.6.3.3	Specific message contents

Table 9.1.11.6.3.3-1: REGISTRATION REQUEST (step 5, Table 9.1.11.6.3.2-3)

Derivation path: TS 38.508-1 [4], table 4.7.1-6 with condition SNPN_ONBOARDING

Table 9.1.11.6.3.3-2: Message AUTHENTICATION REQUEST (step 6 and 13, Table 9.1.11.6.3.2-3)

Derivation path: TS 38.508-1 [4], table 4.7.1-1 with condition EAP-AKA

Table 9.1.11.6.3.3-3: Message AUTHENTICATION RESPONSE (step 7 and 14, Table 9.1.11.6.3.2-3)

Derivation path: TS 38.508-1 [4], table 4.7.1-2 with condition EAP-AKA

Table 9.1.11.6.3.3-4: AUTHENTICATION REJECT (step 8 and 15, Table 9.1.11.6.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.7.1-5 with condition EAP-AKA |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol discriminator |  | 5GMM |  |  |
| Security header type |  | ’0000’B | Plain 5GS NAS message, not security protected |  |
| Spare half octet |  | '0000'B |  |  |
| Authentication reject message identity |  | ‘0101 1000’B |  |  |
| EAP message |  | EAP-failure | EAP-failure |  |
| NOTE: This message is sent within SECURITY PROTECTED 5GS NAS MESSAGE message with Integrity protected and ciphered. |  |  |  |  |

Table 9.1.11.6.3.3-5: REGISTRATION REQUEST (step 23, Table 9.1.11.6.3.2-3)

| Derivation path: TS 38.508-1 [4], table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM capability |  |  |  |
| All octets with the exception of octet 7, bit 4 | Any allowed value |  |  |
| SOR-SNPN-SI (SSNPNSI) (octet 7, bit 4) | ‘1’B | SOR-SNPN-SI supported | pc_SSNPNSI |