3GPP TSG-RAN5 Meeting #102	R5-241597

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3038 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test procedure for 5G ProSe U2N Relay Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test procedure for 5G ProSe U2N Relay Discovery hasn’t be clarified according to TS 23.304 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test procedure for 5G ProSe U2N Relay Discovery |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink U2N Relay will be not implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.43(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.9.43	Test procedure for 5G ProSe U2N Relay Discovery

4.9.43.1	Scope

The purpose of this procedure is for 5G ProSe U2N Relay Discovery.

4.9.43.2	Procedure description

The HTTP signalling between the UE and the 5G DDNMF is done over TLS i.e. the UE connects to TCP port 443 (HTTPS) and starts TLS handshake.

4.9.43.2.1	Initial conditions

UE is authorised to perform NR sidelink communication.

4.9.43.2.2	Procedure description

Table 4.9.43.2.2-1: Procedure for 5G ProSe U2N Relay Discovery

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a ClientHello message including PSK-based ciphersuites. | --> | TLS: ClientHello | - | - |
| 2 | The SS transmits the ServerHello, ServerKeyExchange and ServerHelloDone messages indicating GBA as required. | <-- | TLS: ServerHello TLS: ServerKeyExchange TLS: ServerHelloDone | - | - |
| 3-6 | The generic procedure for GBA authentication according TS 34.229-5 [47] subclause A.22 (steps 1-4) takes place. | - | - | - | - |
| 7 | The UE transmits the ClientKeyExchange including a PSK identity, ChangeCipherSuite and Finished messages. | --> | TLS: ClientKeyExchange TLS: ChangeCipherSuite TLS: Finished | - | - |
| 8 | The SS transmits the ChangeCipherSuite and Finished messages. | <-- | TLS: ChangeCipherSuite TLS: Finished | - | - |
| 9 | The UE transmits a DISCOVERY_REQUEST message over the PC3a interface. | --> | HTTP Request containing DISCOVERY_REQUEST | - | - |
| 10 | The SS transmits a DISCOVERY_RESPONSE message over the PC3a interface. | <-- | HTTP Response containing DISCOVERY_RESPONSE | - | - |
| - | EXCEPTION: Steps 11a1 to 11a2 describe behaviour which depends on the 5G ProSe Procedure being applied | - | - | - | - |
| 11a1 | The UE transmits a MATCH_REPORT message over the PC3a interface. | --> | HTTP Request containing MATCH_REPORT | - | - |
| 11a2 | The SS transmits a MATCH_REPORT_ACK message over the PC3a interface. | <-- | HTTP Response containing MATCH_REPORT_ACK | - | - |
| - | EXCEPTION: Steps 12a1 to 12d2 describe behaviour which depends on the 5G ProSe Procedure being applied | - | - | - | - |
| 12a1 | IF L2RemoteUE AND Model A THEN The SS transmits PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement with RRC container. | <-- | PC5-S: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Announcement) | - | - |
| 12b1 | IF L2RemoteUE AND Model B THEN The UE transmits a PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Solicitation. | --> | PC5-S: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Solicitation) | - | - |
| 12b2 | The SS transmits a PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Response with RRC container. | <-- | PC5-S: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Response) | - | - |
| 12c1 | IF L2RelayUE AND Model A THEN The UE transmits PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Announcement with RRC container. | --> | PC5-S: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Announcement) | - | - |
| 12d1 | IF L2RelayUE AND Model B THEN The SS transmits a PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Solicitation. | <-- | PC5-S: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Solicitation) | - | - |
| 12d2 | The UE transmits a PROSE PC5 DISCOVERY message for UE-to-Network Relay Discovery Response with RRC container. | --> | PC5-S: PROSE PC5 DISCOVERY (for UE-to-Network Relay Discovery Response) | - | - |

4.9.43.3	Specific message contents

FFS

<End of Changed Section>