3GPP TSG-RAN5 Meeting #102	R5-241608

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5291 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of UAS EPS test case 10.10.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 10.10.2 [UAS / UE requested PDN connection establishment / UUAA Re-authentication and Re-authorization with USS] shall be added as specified in UAS EPS WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 10.10.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UAS EPS WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10.2 (New) |  |  |  |  |  |  |  |  |
| sss |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 36.523-2 CR 1443 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-241608 is the final version of R5-241040 with below change: 1. Updated style of test case title. |  |  |  |  |  |  |  |  |

<Start of modified section>

### 10.10.2	UAS / UE requested PDN connection establishment / UUAA Re-authentication and Re-authorization with USS

10.10.2.1	Test Purpose (TP)

(1)

with { UE having peformed successful UUAA-SM procedure for USS communication }

ensure that {

when { UE receives MODIFY EPS BEARER CONTEXT REQUEST message with service-level-AA container for UUAA Re-authentication and Re-authorization}

then { the UE transmits MODIFY EPS BEARER CONTEXT ACCEPT message }

}

10.10.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 6.3.13.2, 6.4.3.2, 6.4.3.3. Unless otherwise stated, these are Rel-17 requirements.

[TS 24.301, clause 6.4.3.2]

The MME shall initiate the EPS bearer context modification procedure by sending a MODIFY EPS BEARER CONTEXT REQUEST message to the UE, starting the timer T3486, and entering the state BEARER CONTEXT MODIFY PENDING (see example in figure 6.4.3.2.1).

The MME shall include an EPS bearer identity that identifies the EPS bearer context to be modified in the MODIFY EPS BEARER CONTEXT REQUEST message

…

[TS 24.301, clause 6.4.3.3]

If the EPS bearer context being modified is associated with a PDN connection for UAS services and the MODIFY EPS BEARER CONTEXT REQUEST message includes the service-level-AA container with the length of two octets in the Extended protocol configuration options IE, the UE supporting UAS services shall forward the contents of the service-level-AA container with the length of two octets to the upper layers.

If the EPS bearer context being modified is associated with a PDN connection for UAS services, the MODIFY EPS BEARER CONTEXT REQUEST message includes the Extended protocol configuration options IE containing the service-level-AA container with the length of two octets containing the service-level-AA response parameter with the SLAR field set to "Service level authentication and authorization was successful", the UE supporting UAS services:

a)	shall consider the UUAA procedure as successfully completed and provide the service-level-AA response to the upper layers;

b)	if the service-level-AA container with the length of two octets contains the service-level device ID parameter carrying a CAA-level UAV ID, shall provide the CAA-level UAV ID to the upper layers; and

c)	if the service-level-AA container with the length of two octets contains the service-level-AA payload type parameter with the value "UUAA payload" and the service-level-AA payload parameter carrying the UUAA payload, shall provide the UUAA payload to the upper layers.

Upon reception of a service-level-AA payload from the upper layers, the UE supporting UAS services shall include the Extended protocol configuration options IE in the MODIFY EPS BEARER CONTEXT ACCEPT message. In the Extended protocol configuration options IE, the UE shall include the service-level-AA container with the length of two octets. In the service-level-AA container with the length of two octets, the UE shall:

a)	include the service-level-AA payload parameter set to the service-level-AA payload received from the upper layers; and

b)	set the service-level-AA payload type parameter to the type of the service-level-AA payload.

…

[TS 24.301, clause 6.3.13.2]

After successful UUAA-SM procedure, the network may initiate the re-authentication or re-authorization procedure for the UE supporting UAS services as a part of network-initiated EPS bearer context modification procedure. If UUAA-SM fails during the re-authentication or a re-authorization procedure, or if the revocation of UUAA is initiated by the network, then the associated PDN connection for USS communication is released.

…

10.10.2.3	Test description

10.10.2.3.1	Pre-test conditions

System Simulator:

-	Cell A is configured according to Table 6.3.2.2-1 of TS 36.508 [18].

UE:

-	The UE is capable of UAS operation and provisioned with CAA-level UAV ID.

Preamble:

-	The UE has successfully performed generic test procedure for USS UAV Authorization/Authentication- Session Management in EPS according to TS 36.508[18] subclause 4.5A.31 by executing steps 1 to 9.

10.10.2.3.2	Test procedure sequence

Table 10.10.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a MODIFY EPS BEARER CONTEXT REQUEST message including the EPS bearer identity of the EPS bearer associated with UAS services and service-level-AA container for re-authentication and re-authorization. | <-- | MODIFY EPS BEARER CONTEXT REQUEST | - | - |
| 2 | Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? | --> | MODIFY EPS BEARER CONTEXT ACCEPT | 1 | P |
| 3-4 | Steps 8-9 of generic test procedure for UUAA-SM in EPS defined in TS 36.508 [18] subclause 4.5A.31 take place. | - | - | - | - |
| 5 | The SS releases the RRC Connection. | - | - | - | - |

10.10.2.3.3	Specific message contents

Table 10.10.2.3.3-1: Message MODIFY EPS BEARER REQUEST (step 1, Table 10.10.2.3.2-1)

| Derivation Path: TS 36.508[18] Table 4.7.3-18 |  |  |
| --- | --- | --- |
| Information Element | Value/Remark | Comment |
| EPS bearer identity | Same EPS Bearer Identity value as specified in PDN establishment for UAS services |  |
| Extended protocol configuration options |  |  |
| Service-level-AA container | ‘0041’H |  |
| Length of Service-level-AA container contents | ‘00011000’ | 24 octets |
| Service-level-AA container contents |  |  |
| Service-level-AA parameter |  |  |
| Service-level-AA device ID IEI | ‘10’H |  |
| Service-level-AA device ID length | ‘08’H |  |
| Service-level-AA device ID | Same value as Service-level-AA device ID sent by UE in PDN Connectivity Request |  |
| Service-level-AA parameter |  |  |
| Service-level-AA payload type IEI | ‘40’H |  |
| Service-level-AA payload type length | ‘01’H |  |
| Service-level-AA payload type | ‘01’H | UUAA payload |
| Service-level-AA parameter |  |  |
| Service-level-AA payload IEI | ‘70’H |  |
| Service-level-AA payload length | ‘0008’H |  |
| Service-level-AA payload | ‘ABCDEFGHABCDEFGH’H |  |

Table 10.10.2.3.3-2: Message MODIFY EPS BEARER ACCEPT (step 2, Table 10.10.2.3.2-1)

| Derivation Path: TS 36.508[18] Table 4.7.3-16 |  |  |
| --- | --- | --- |
| Information Element | Value/Remark | Comment |
| EPS bearer identity | Same EPS Bearer Identity value as specified in PDN establishment for UAS services |  |
| Extended protocol configuration options |  |  |
| Service-level-AA container | ‘0041’H |  |
| Length of Service-level-AA container contents | ‘00000000 00011000’ | 14 octets |
| Service-level-AA container contents |  |  |
| Service-level-AA parameter |  |  |
| Service-level-AA payload type IEI | ‘40’H |  |
| Service-level-AA payload type length | ‘01’H |  |
| Service-level-AA payload type | ‘01’H | UUAA payload |
| Service-level-AA parameter |  |  |
| Service-level-AA payload IEI | ‘70’H |  |
| Service-level-AA payload length | ‘0008’H | 8 octets |
| Service-level-AA payload | ‘ABCDEFGHABCDEFGH’H |  |

<End of modified section>