3GPP TSG-RAN5 Meeting #102	R5-241632

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 51.010-2 | CR | 4413 | rev | 1 | Current version: | 13.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removal of GEA1/GEA2 and verification of non-support of GEA1/GEA2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI_Test |  |  |  |  |  | Date: |  |  | 2024-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of ciphering algorithms GEA1 and GEA2 have been prohibited from all releases of 3GPP core specifications published in December 2023. 3GPP SA requested RAN5 to enable verification of non-support of GEA1 and GEA2 in all releases (R5-240018). RAN5 test specifications need to be updated to align with the latest version of core specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the Table A.2 and Annex B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RAN5 test specification won’t be aligned with core specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR is final version of R5-250514 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.4.3	Mobile Station Features

The supplier of the implementation shall state the support of the implementation for each of the questions concerning the mobile station features given in the table below.

Table A.2: Mobile Station Features

| Item | Mobile Station Feature |  | Ref. | Release | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Display of Called Number. |  | 3GPP TS 02.07 B.1.1 | Phase 2 | C.202 |  | TSPC_Feat_DCN |
| 2 | Indication of Call Progress Signals. |  | 3GPP TS 02.07 B.1.2 | Phase 2 | C.204 |  | TSPC_Feat_CPSind |
| 3 | Country/PLMN Indication. |  | 3GPP TS 02.07 B.1.3 | Phase 2 | C.202 |  | TSPC_Feat_PLMNind |
| <<SKIPPED ROWS>> |  |  |  |  |  |  |  |
|  |  |  |  | R99 | O |  |  |
| 74 | Support of Fine Time Assistance |  | 3GPP TS 44.031 A.4.2.4 | Rel-4 | C.215 |  | TSPC_Fine_Time_Assist |
| 75 | Void |  |  |  |  |  |  |
| 76 | Support of Encryption GEA3 |  | 3GPP TS 43.020 | Rel-6 | M |  | TSPC_Feat_GEA3 |
| <<SKIPPED ROWS>> |  |  |  |  |  |  |  |
| 134 | Support of IPA capability |  | 3GPP TS 44.018 | Rel-11 | O |  | TSPC_IMMEDIATE_PACKET_ASSIGNMENT |
| 135 | Void |  |  |  |  |  |  |
| 136 | Support of Low Access Priority and Extended Access Barring |  | 3GPP TS 24.008 1.8 TS 22.011 4.3.4 | Rel-10 | O |  | TSPC_LAP_EAB |
| <<SKIPPED ROWS>> |  |  |  |  |  |  |  |
| 151 | Support of Power Efficiency Operation |  | 3GPP TS 44.018 | Rel-13 | O |  | TSPC_PEO |
| 152 | Support of handling NAS reject messages without Integrity protection |  | 3GPP TS 24.008 | Rel-13 | M Note 2 |  | TSPC_NAS_rej_integrity |
| O.201 |  | Only one of these items can be supported |  |  |  |  |  |
| C.201 |  | IF A.3/1 OR A.3/2 OR A.4/20 OR A.4/21 THEN M ELSE N/A |  |  | -- TSPC_Serv_TS11 OR TSPC_Serv_TS12 OR TSPC_Serv_BS61 OR TSPC_Serv_BS81 |  |  |
| C.202 |  | IF A.2/27 THEN M ELSE N/A |  |  | -- TSPC_Feat_Alphanum_Display |  |  |
| C.203 |  | IF A.2/27 OR A.2/28 THEN M ELSE N/A |  |  | -- TSPC_AlphaNum_Display OR TSPC_Other_Means_of_Display |  |  |
| C.204 |  | IF A.2/29 THEN M ELSE N/A |  |  | -- TSPC_Speech_Indicator |  |  |
| C.205 |  | IF A.2/26 OR A.2/40 THEN M ELSE N/A |  |  | -- TSPC_Feat_Autocall |  |  |
| C.206 |  | IF A.1/16 OR A.1/17 THEN M ELSE N/A |  |  | -- TSPC_Feat_Ext_TA |  |  |
| C.207 |  | IF A.2/41 OR A.2/42 THEN M ELSE N/A |  |  | -- TSPC_GPRS OR TSPC_EGPRS |  |  |
| C.208 |  | IF A.2/41 THEN O ELSE N/A |  |  | -- TSPC_GPRS |  |  |
| C.209 |  | IF A.2/41 or A.2/42 THEN at least one of these items shall be supported ELSE N/A |  |  | -- TSPC_GPRS OR TSPC_EGPRS |  |  |
| C.210 |  | IF A.2/42 THEN O ELSE N/A |  |  | -- TSPC_EGPRS |  |  |
| C.211 |  | IF A.3/2 THEN M ELSE N/A |  |  | -- TSPC_Serv_TS12 |  |  |
| C.212 |  | Void |  |  |  |  |  |
| C.213 |  | Void |  |  |  |  |  |
| C.214 |  | IF (A.2/41 AND A.1/51) OR (A.2/42 AND A.1/66) THEN O ELSE N/A |  |  | -- (TSPC_GPRS AND TSPC_Type_GPRS_Multislot_operation) OR (TSPC_EGPRS AND TSPC_Type_EGPRS_Multislot_operation) |  |  |
| C.215 |  | IF A.2/59 OR A.2/94 OR A.2/60 OR A.2/95 THEN O ELSE N/A |  |  | -- TSPC_A-GPS_Based OR TSPC_MSB_A-GANSS OR TSPC_A-GPS_Assist OR TSPC_MSA_A-GANSS) |  |  |
| C.216 |  | IF A.2/71 THEN O ELSE N/A |  |  | -- TSPC_GAN |  |  |
| C.217 |  | IF A.2/96 or A.2/97 or A.2/98 or A.2/144 THEN at least one of these items shall be supported ELSE N/A |  |  | -- TSPC_GLONASS OR TSPC_MGPS OR TSPC_GALILEO OR TSPC_BDS |  |  |
| C.218 |  | IF A.2/99 OR A.2/100 THEN M ELSE N/A |  |  | -- TSPC_CS_EGAN OR TSPC_PS_EGAN |  |  |
| C.219 |  | IF A.2/91 THEN A ELSE IF A.2/125 THEN O ELSE N/A |  |  | -- TSPC_eCallOnly_Equipment OR TSPC_eCallCapableMS |  |  |
| C.220 |  | IF A.2/151 THEN M ELSE O |  |  | -- TSPC_Feat_Alphanum_Display |  |  |
| Note 1: Void Note 2: The support of handling NAS reject messages without Integrity protection is a mandatory feature from Rel-13 onwards and can be optionaly implemented since Rel-10. Note 3: Void |  |  |  |  |  |  |  |

Annex B (normative):
Applicability of the individual test

The applicability of each individual test is identified in the table B.1.

The applicability of every test is formally expressed by the use of Boolean expression that are based on parameters (ICS) included in annex A of the present document.

The columns in Table B.1 have the following meaning:

Clause column

The Clause column indicates the clause number for each test case as described in the 3GPP TS 51.010-1 or 3GPP TS 11.10-4 (tests 27.22.x) for which the applicability is identified.

Title column

The Title column indicates the title of each test case as described in the 3GPP TS 51.010-1 or 3GPP TS 11.10-4 (tests 27.22.x) for which the applicability is identified.

Release column

The Release column indicates the earliest release from which each test case is applicable, except if otherwise stated of an individual test case.

Applicability column

The Applicability column describes the applicability of the test in a verbal way.

Applicability Limitations column

The Applicability Limitations column describes limitations, redundancies or extensions of the applicability of the test using the following notations:

R	redundant – the requirement in this test is verified in another test.

Ri	Reduced applicability – the test is applicable ("A") or redundant ("R") depending on the support of other optional or conditional items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is used to avoid ambiguities.

Li	Limited execution – the test is applicable ("A"). The execution may be limited depending on the support of other optional or conditional items, e.g. some tests may not be repeated for all execution counters. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table.

Ei	Excluded applicability – the test is excluded (“E”) depending on the support of other optional or conditional items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table.

Xi	eXtended execution – the test conditions are eXtended (“X”) to allow an alternate execution of the targeted test with different initial conditions but testing the same requirements

Status column

The following notations are used for the Status column:

A		applicable - the test is applicable.

N/A	not applicable – in the given context, the test case is not applicable.

Ci	conditional – the test is applicable ("A") or not ("N/A") depending on the support of other optional or conditional items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." is used to avoid ambiguities.

Specific PICS Statements column

The Specific PICS Statements column shows PICS statements in their mnemonics form that are used in 3GPP TS 51.010-1 to specify or influence the performance or behaviour of the test.

Supported column

The following common notations are used for the Supported column:

Y or y	test is supported by the implementation

N or n	test is not supported by the implementation

N/A, n/a or - 	no answer required (allowed only if the status is N/A, directly or after evaluation of a conditional status)

Table B.1: Applicability of tests

| Clause | Title | Release | Applicability | Applicability Limitations | Status | Specific PICS Statements | Supported |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 11.1.1 | Mobile Terminated (MT) calls | Phase 2 | Each MT Bearer Service and MT Teleservice supported by the MS |  | C31 |  |  |
| <<SKIPPED ROWS>> |  |  |  |  |  |  |  |
| 44.2.5.2.4 | Ciphering mode / Cipher key Kc128 and algorithmn changes | Rel-9 | All GPRS MS supporting GEA4 |  | C482 | TSPC_operation_mode_B TSPC_operation_mode_C TSPC_AddInfo_on_auto_GPRS_AP TSPC_Feat_OnOff TSPC_Feat_GEA3 |  |
| 44.2.5.2.5 | Ciphering mode / Non support of GEA1 / Non support of GEA2 | R97 | All GPRS MS |  | C215 | TSPC_operation_mode_B TSPC_operation_mode_C TSPC_AddInfo_on_auto_GPRS_AP TSPC_Feat_OnOff |  |
| 44.2.6.1 | General Identification | R97 | All GPRS MS | X1 | C215 | TSPC_operation_mode_B TSPC_operation_mode_C TSPC_AddInfo_on_auto_GPRS_AP TSPC_Feat_OnOff |  |
| <<SKIPPED ROWS>> |  |  |  |  |  |  |  |
| Note 1: Early implemented features can be tested regardless of the release, e.g. eCall is a Rel-8 feature but conformance testing of early implementations of eCall is also allowed in Rel-7 etc. Note 2: When selecting TC for Testing the TIGHTER, if TIGHTER TC is not available then the legacy TC shall be run |  |  |  |  |  |  |  |

Table B.1a: Applicability of tests – Conditions definitions

| C1 | IF NOT A.25/50 THEN A ELSE N/A | -- NOT TSPC_AddInfo_ApplAlwaysRun |
| --- | --- | --- |
| C2 | IF A.25/1 THEN A ELSE N/A | -- TSPC_AddInfo_HalfRate |
| C408 | IF A.25/114 AND A.25/19 THEN A ELSE N/A | -- TSPC_AddInfo_TTY AND TSPC_AddInfo_MTsvc |
| C409 | void |  |
| <<SKIPPED ROWS>> |  |  |
| C410 | IF A.1/188 THEN A ELSE N/A | -- TSPC_EGPRS_Multislot_Uplink |
| C411 | IF A.25/26 AND A.25/19 THEN A ELSE N/A | -- TSPC_AddInfo_CCprotocol_oneBC AND TSPC_AddInfo_MOsvc |
| C412 | IF A.25/19 OR A.25/20 THEN A ELSE N/A | -- TSPC_AddInfo_MOsvc OR TSPC_AddInfo_MTsvc |
| C413 | IF A.25/60 OR A.25/148 THEN A ELSE N/A | -- TSPC_AddInfo_PermAntenna OR TSPC_AddInfo_TempAntenna |
| C414 | IF A.25/139 THEN A ELSE N/A | -- TSPC_Repeated_SACCH |
| C415 | Void |  |
| C416 | IF A.2/41 AND A.2/76 THEN A ELSE N/A | -- TSPC_GPRS AND TSPC_Feat_GEA3 |
| C417 | IF A.2/41 AND NOT (A.1/84 OR A.1/95) THEN A ELSE N/A | -- TSPC_GPRS AND NOT (TSPC_Type_GPRS_Multislot_Class18 OR TSPC_Type_GPRS_Multislot_Class29) |
| C418 | IF A.2/41 AND NOT (A.1/84 OR A.1/90 OR A.1/91 OR A.1/92 OR A.1/93 OR A.1/94 OR A.1/95) THEN A ELSE N/A | -- TSPC_GPRS AND NOT (TSPC_Type_GPRS_Multislot_Class18 OR TSPC_Type_GPRS_Multislot_Class24 OR TSPC_Type_GPRS_Multislot_Class25 OR TSPC_Type_GPRS_Multislot_Class26 OR TSPC_Type_GPRS_Multislot_Class27 OR TSPC_Type_GPRS_Multislot_Class28 OR TSPC_Type_GPRS_Multislot_Class29) |
| <<SKIPPED ROWS>> |  |  |
| C593 | IF (A.2/89 AND A.2/128) THEN A ELSE N/A | -- TSPC_EGPRS2A AND TSPC_TIGHTER_EGPRS2 |
| C594 | IF A.2/134 THEN A ELSE N/A | -- TSPC_IMMEDIATE_PACKET_ASSIGNMENT |
| C595 | Void |  |
| C596 | Void |  |
| C597 | IF A.2/130 THEN A ELSE N/A | -- TSPC_FANR_Capability |
| C598 | IF A.2/41 AND A.2/138 THEN A ELSE N/A | -- TSPC_GPRS AND TSPC_NMO_I_Behaviour |
| C599 | IF A.2/41 AND A.2/139 THEN A ELSE N/A | -- TSPC_GPRS AND TSPC_AttachWithIMSI |
| <<SKIPPED ROWS>> |  |  |
| C600 | IF A.2/136 THEN A ELSE N/A | -- TSPC_LAP_EAB |
| C601 | IF A.2/137 THEN A ELSE N/A | -- TSPC_MinimumPeriodicSearchTimer |
| C623 | IF A.2/151 AND A.2/72 THEN A ELSE N/A | -- TSPC_PEO AND TSPC_GERAN_FEATURE_PACKAGE_1 |
| C624 | IF (A.25/2 OR A.25/3) AND A.25/169 THEN A ELSE N/A | -- TSPC_ No_SIM_Test Execution AND (TSPC_AddInfo_Full_rate_version_1 OR TSPC_AddInfo_Half_rate_version_1) |
| C625 | Void |  |
| Note1: This test case concerns a feature introduced in R97, but it is applicable only for R99 and later as it has been created late. |  |  |