3GPP TSG-RAN5 Meeting #102	R5-241633

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

Source:	Huawei, HiSilicon, China Telecom

Title:	Discussion to update MUSIM gap TC

Agenda item:	6.4.7

Document for:	Discussion and Endorsement

Discussion

Issue: How to make sure that RAN5 TC are testing the MUSIM gap pattern supported by UE?

[38.306]

musim-GapPreference-r17

Indicates whether the UE supports providing MUSIM assistance information with MUSIM gap preference and related MUSIM gap configuration, as defined in TS 38.331 [9]. UE supporting this feature supports 3 periodic gaps and 1 aperiodic gap.

[38.133]

UE supporting MUSIM capability shall support the MUSIM gap patterns listed in Table 9.1.10-1 based on UE’s capability specified in TS38.306[14] and the applicability specified in Table 9.1.10-2

Table 9.1.10-1: MUSIM Gap Pattern Configurations

| MUSIM Gap Pattern Id | MUSIM Gap Length (MGL, ms) | MUSIM Gap Repetition Period (MGRP, ms) |
| --- | --- | --- |
| 0 | 6 | 40 |
| 1 | 6 | 80 |
| 2 | 3 | 40 |
| 3 | 3 | 80 |
| 4 | 6 | 20 |
| 5 | 6 | 160 |
| 6 | 4 | 20 |
| 7 | 4 | 40 |
| 8 | 4 | 80 |
| 9 | 4 | 160 |
| 10 | 3 | 20 |
| 11 | 3 | 160 |
| 12 | 10 | 80 |
| 13 | 20 | 160 |
| 14 | 6 | 320 |
| 15 | 6 | 640 |
| 16 | 6 | 1280 |
| 17 | 6 | 2560 |
| 18 | 10 | 320 |
| 19 | 10 | 640 |
| 20 | 10 | 1280 |
| 21 | 10 | 2560 |
| 22 | 20 | 320 |
| 23 | 20 | 640 |
| 24 | 20 | 1280 |
| 25 | 20 | 2560 |
| 26 | 20 | 5120 |
| 27 | 10 | NA |
| 28 | 20 | NA |
| Note 1: Measurement gap pattern #27, #28 are the aperiodic gap pattern without MGRP. |  |  |

Observation 1: 38.133 list 27 periodic MUSIM gaps and 2 aperiodic MUSIM gaps, but 38.306 only require UE to support 3 periodic MUSIM gaps and 1 aperiodic MUSIM gap.

Therefore, core spec does not require UE to support all kinds of gaps and it does not list which gap pattern is mandatory and which gap pattern is optional, either.

[38.331]

MUSIM-GapConfig field descriptions

musim-AperiodicGap Indicates the MUSIM aperiodic gap as specified in TS 38.133 [14] clause 9.1.10. If UE indicates the musim-Starting-SFN-AndSubframe when requesting aperiodic gap the network can only configure the aperiodic gap with the same start point or no aperiodic gap. If the field musim-Starting-SFN-AndSubframe is absent for aperiodic gap, network can configure any timing as the starting point for aperiodic gap or configure no aperiodic gap.

musim-GapInfo Indicates the values for musim-GapLength and musim-GapRepetitionAndOffset. When network provides periodic gap, network always signals the musim-GapLength and musim-GapRepetitionAndOffset as indicated by the UE's preferred MUSIM gap configuration.

Observation 2: According to 38.331, for periodic gap, network always configured the musim-GapLength and musim-GapRepetitionAndOffset as indicated by the UE's preferred periodic MUSIM gap configuration. For aperiodic gap, network always configured the same musim-Starting-SFN-AndSubframe as indicated by the UE's preferred aperiodic MUSIM gap configuration. The UE's preferred MUSIM gap configuration is indicated in the MUSIM UAI.

Therefore, if UE only support part of MUSIM gap listed in 38.133, and UE always indicated that supported MUSIM gap pattern to network through MUSIM UAI, network have no chance to configure MUSIM gap pattern unsupported by the UE. In addition, If TC directly (e.g 38.523-1 8.1.2.1.6) configure MUSIM gap to UE, the UE behaviour is not specified by 38.331.

[38.331 5.7.4.2]

A UE capable of providing MUSIM assistance information for gap preference may initiate the procedure if it was configured to do so, upon determining it needs the gaps, or upon change of the gap preference information.

[38.509 5.13]

The SS uses the Set MUSIM UAI test procedure to trigger a change of preference in the MUSIM UE that will cause the indication of UE Assistance Information (UAI) by the UE, see figure 5.8.1-1. The different types of information that can be sent in a SET MUSIM UAI REQUEST is preferred RRC state and gap preference for MUSIM, see clause 6.13.

Observation 3: Accroding to 38.331, UE may not trigger MUSIM UAI automatically after network configuration. In order to trigger MUSIM UAI, RAN5 design Set MUSIM UAI test function to force UE to report any type of MUSIM gap in MUSIM UAI.

“Set MUSIM UAI test function” is designed based on an assumption that UE support all the MUSIM gap pattern listed in 38.133. However based on observation 1/2, there is not enough evidence to support this assumption.

[RP-230751 R18 NR_DualTxRx_MUSIM]

3. Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2]

- Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2]

  - The following MUSIM gap requirements are considered

    - Measurements in Network A

    - Measurements in Network B in RRC idle/inactive

    - Note: it is up to RAN4 decision whether to define requirements for Network B.

Observation 4: RAN4 has defined the RRM requirement for Rel-17 MUSIM gaps in R18 WI NR_DualTxRx_MUSIM. It is highly possible that RRM MUSIM gaps TC will face similar issue in the future.

Proposal

Based on the above discussion, the followings are proposed:

Proposed1: Add MUSIM UAI procedure in 38,523-1 8.1.2.1.6 before MUSIM gap configuration.

Proposed2: UE is required to report 3 periodic MUSIM gap and 1 aperiodic MUSIM gap in MUSIM UAI, the specific MUSIM UAI trigger method in based on UE implementation or based on test function, the gap pattern is chosen by UE.

Proposed3: SS only configure and test the MUSIM gap reported in MUSIM UAI.

Proposed4: When MUSIM gap is configured, SS only test UE behavior outside the gap.

Proposed5: When MUSIM gap is releaseed, SS can test UE behavior inside the gap to make sure UE have release the gap successfully.

SS configure gap based on UAI content