3GPP TSG-RAN5 Meeting #102	R5-241636

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0082 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Set MUSIM UAI test function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon,China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - "Set MUSIM UAI test function" is introduced to trigger UE send perference MUSIM periodic gap(s) and/or aperiodic gap.Aperiodic gap configuration method has already included in clause 6.13 but it is missing in clause 5.13.3. Therefore It is needed to update 5.13.3. - For periodic MUSIM gap, the starting SFN number and starting subframe number are absent; Gap length, gap repetition period and gap offset are mandatory. For aperiodic MUSIM gap, the starting SFN number and starting subframe number are optional; Gap length is mandatory; gap repetition period and gap offset are absent. The current SET MUSIM UAI REQUEST in 6.13 does not indicated UE how to distinguish periodic MUSIM gap and aperiodic MUSIM gap, and it does not indicated how to set gap repetition period and gap offset when the test function is setting aperiodic MUSIM gap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update MUSIM aperiodic gap in clause 5.13.3 so that this clause could align with clause 6.13. - C2..C0 and D3...D0 are moved to octet 1, and add F0 in octet 1 to indicate type of the gap(aperiodic gap or periodic gap). For aperiodic gap, E13..E0 is used to indicate the gap offset. For periodic gap, E13..E0 is used to indicate gap start SFN and subframe number. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is not clear that "Set MUSIM UAI test function" can trigger UE to send perference MUSIM aperiodic gap in 5.13.3 - It is not clear how to distinguish periodic MUSIM gap and aperiodic MUSIM gap |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.13.3, 6.13.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | revised from R5-240415r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

5.13	Set MUSIM UAI test function

5.13.1	General

The SS uses the Set MUSIM UAI test procedure to trigger a change of preference in the MUSIM UE that will cause the indication of UE Assistance Information (UAI) by the UE, see figure 5.8.1-1. The different types of information that can be sent in a SET MUSIM UAI REQUEST is preferred RRC state and gap preference for MUSIM, see clause 6.13.

Figure 5.13.1-1: Set MUSIM UAI test procedure

5.13.2	Initiation

The SS requests the UE to set the MUSIM related UE Assistance Information by transmitting a SET MUSIM UAI REQUEST message and the UE confirms by responding with a SET MUSIM UAI RESPONSE message.

### 5.13.3	Reception of SET MUSIM UAI REQUEST message by UE

When UE receives SET MUSIM UAI REQUEST message then the UE shall:

1>	if the MUSIM UE is operating in RRC_CONNECTED state:

2>	set its preferred RRC state within the release preference, to the equivalent value as received in the musim-PreferredRRC-State-r17 of the SET MUSIM UAI REQUEST message.

2>	transmit SET MUSIM UAI RESPONSE message.

1>	if the UE has a preference for MUSIM periodic gap(s) and/or aperiodic gap:

2>	set its preferred gap preference, to the equivalent value as received in the musim-GapPreferenceList of the SET MUSIM UAI REQUEST message.

2>	transmit SET MUSIM UAI RESPONSE message.

1>	else:

2>	the UE behaviour is unspecified.

<End of modified section 1>

<Start of modified section 2>

6.13	SET MUSIM UAI REQUEST

6.13.1	SET MUSIM UAI REQUEST

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| musim-PreferredRRC-State |  | M | V | 1 |
| musim-GapPreferenceList |  | CV-musimGap | TLV | 5*(NC+1)+2 |

| Condition | Explanation |
| --- | --- |
| CV-musimGap | 0 ≤ NC ≤ 3. |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | octet 1 |

where musim-PreferredRRC-State is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  | E1 | E0 | octet 3 |

E1=0 and E0=0; idle.

E1=0 and E0=1; inactive.

E1=1 and E0=0: outOfConnected.

E1=1 and E0=1: Reserved.

where musim-GapPreferenceList is:

| 8 7 6 5 4 3 2 1 |  |
| --- | --- |
| musim-GapPreferenceList Value type | octet 1 |
| Length of musim-GapPreferenceList Value contents in bytes | octet 2 |
| musim-GapPreferenceList IE #0 | octet 3 octet 4 octet 5 |
| … |  |
| musim-GapPreferenceList IE #NC | octet 3+5 * NC octet 4+5 * NC octet 5+5 * NC |

And where musim-GapPreferenceList Value type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | octet 1 |

And where musim-GapPreferenceList IE #n is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| C2 | C1 | C0 | D3 | D2 | D1 |  | D0 | F0 | octet 1 |
| E13 | E12 | E11 | E10 | E9 | E8 |  | E7 | E6 | octet 2 |
| E5 | E4 | E3 | E2 | E1 | E0 |  | Reserved |  | octet 3 |

F0 = 0; Indicates periodic MUSIM gap.

F0 = 1; Indicates aperiodic MUSIM gap.

C2..C0 = 0..4 Indicates the length of the UE's MUSIM gap as specified in TS 38.133 [14] clause 9.1.10. This field is mandatory present for both periodic gap and aperiodic gap preference indication (binary coded, C2 is most significant bit and C0 least significant bit).

C2..C0 = 0; ms3.

C2..C0 = 1; ms4.

C2..C0 = 2; ms6.

C2..C0 = 3; ms10.

C2..C0 = 4; ms20.

D3..D0 = 0..8 Indicates the gap repetition period in ms for the periodic MUSIM gap as specified in TS 38.133 [14] clause 9.1.10 (binary coded, D3 is most significant bit and D0 least significant bit). If D3-D0 is not 0..8, the gap repetition period is NA.

D3..D0 = 0; ms20.

D3..D0 = 1; ms40.

D3..D0 = 2; ms80.

D3..D0 = 3; ms160.

D3..D0 = 4; ms320.

D3..D0 = 5; ms640.

D3..D0 = 6; ms1280.

D3..D0 = 7; ms2560.

D3..D0 = 8; ms5120.

E13..E0= 0.. (gap repetition period-1) Indicates the gap offset in number of subframes for the periodic MUSIM gap as specified in TS 38.133 [14] clause 9.1.10 if F0 = 0. (binary coded, E13 is most significant bit and E0 least significant bit).

E13..E4= 0..1023 Indicates gap starting SFN number for the aperiodic MUSIM gap preference if F0=1 (binary coded, E13 is most significant bit and E4 least significant bit).

E3..E0= 0..9 Indicates gap starting subframe number for the aperiodic MUSIM gap preference if F0=1(binary coded, E3 is most significant bit and E0 least significant bit).

If E3..E0 is not 0..9 and F0=1, the gap starting subframe number indicated by E3..E0 is not valid, and gap starting SFN number indicated by E13..E4 is not valid, either. The musim-Starting-SFN-AndSubframe-r17 in the MUSIM UAI set to not present.

<End of modified section 2>