3GPP TSG-RAN5 Meeting # 102	R5-241676

# Athens, Greece, 26th February – 3rd March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 3021 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex A.3 update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some testing aspects related to satellite access are missing in current 3GPP TS 36.521-3 according to R4-2402910. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update clause A.10.4 based on 3GPP TS 36.133 clause A.3.28.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM tests will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8a (New), A.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is dependent on R4-2403452. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-241389 and the outcome of 1 revision. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

# A.8	PDSCH Reference Channels for Cat-M1 UEs

## A.8.1	FDD in CEModeA

Table A.8.1-1: PDSCH Reference Channel for Cat-M1 FDD in CEModeA

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.20 FDD | R.21 FDD | R.28 FDD | R.29 FDD | R.32 FDD | R.33 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 16 | 16 | 16 | 16 | 1 | 1 |
| Frequency hopping |  | ON | ON | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 | 2 | 2 |
| PDSCH Narrowband |  | 2nd | 2nd | 2nd | 2nd | 2nd | 2nd |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 | 4 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD | TBD | TBD | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |  |

## A.8.2	HD-FDD in CEModeA

Table A.8.2-1: PDSCH Reference Channel for Cat-M1 HD-FDD in CEModeA

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.10 HD-FDD | R.11 HD-FDD | R.18 HD-FDD | R.19 HD-FDD | R.24 HD-FDD | R.25 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 16 | 16 | 16 | 16 | 1 | 1 |
| Frequency hopping |  | ON | ON | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 | 2 | 2 |
| PDSCH Narrowband |  | 1st | 1st | 1st | 1st | 1st | 1st |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 4 | 4 | 4 | 4 | 4 | 4 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD | TBD | TBD | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |  |  |

## A.8.3	TDD in CEModeA

Table A.8.3-1: PDSCH Reference Channel for Cat-M1 TDD in CEModeA

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| PDSCH Reference channel |  | R.16 TDD | R.17 TDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 |
| Allocated subframes per Radio Frame |  | 4 | 4 |
| TDD Uplink-Downlink Configuration |  | 0 | 0 |
| TDD Special Subframe Configuration |  | 6 | 6 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| All Sub-Frames except 1, 6 (TDD) | Bits | 32 | 32 |
| All Sub-Frames except 1, 6 (TDD DwPTS) | - | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  | 1 | 1 |
| All Sub-Frames except 1, 6 (TDD) |  | 1 | 1 |
| All Sub-Frames except 1, 6 (TDD DwPTS) |  | 0 | 0 |
| Maximum number of repetitions |  | 16 | 16 |
| Frequency hopping |  | ON | ON |
| Number of narrowbands for frequency hopping |  | 2 | 2 |
| PDSCH Narrowband |  | 2nd | 2nd |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 |
| Frequency hopping interval | subframes | 10 | 10 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |

## A.8.4	FDD in CEModeB

Table A.8.4-1: PDSCH Reference Channel for Cat-M1 FDD in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.22 FDD | R.23 FDD | R.30 FDD | R.31 FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 |  | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | - | 10 | 10 | 10 | 10 |
| Modulation | - | QPSK | QPSK | QPSK | QPSK |
| Target coding rate | - | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | - | 1 | 1 | 1 | 1 |
| Maximum number of repetitions | - | 192 | 192 | 192 | 192 |
| Frequency hopping | - | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 |
| PDSCH Narrowband | - | 2nd | 2nd | 2nd | 2nd |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

## A.8.5	HD-FDD in CEModeB

Table A.8.5-1: PDSCH Reference Channel for Cat-M1 HD-FDD in CEModeB

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PDSCH Reference channel |  | R.12 HD-FDD | R.13 HD-FDD | R.20 HD-FDD | R.21 HD-FDD |
| Carrier bandwidth | MHz | 10 | 10 | 5 | 5 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 | 2 | 2 |
| Allocated subframes per Radio Frame | subframes | 10 | 10 | 10 | 10 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |  |  |
| Sub-Frames 0 ~ 9 | Bits | 32 | 32 | 32 | 32 |
| Number of Code Blocks per Sub-Frame |  |  |  |  |  |
| Sub-Frames 0 ~ 9 |  | 1 | 1 | 1 | 1 |
| Maximum number of repetitions |  | 192 | 192 | 192 | 192 |
| Frequency hopping |  | ON | ON | ON | ON |
| Number of narrowbands for frequency hopping | narrowbands | 2 | 2 | 2 | 2 |
| PDSCH Narrowband |  | 1st | 1st | 1st | 1st |
| Frequency HoppingOffset | narrowbands | 7 | 7 | 3 | 3 |
| PDSCH start symbol | symbols | 2 | 2 | 2 | 2 |
| Frequency hopping interval | subframes | 8 | 8 | 8 | 8 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |  |  |

## A.8.6	TDD in CEModeB

Table A.8.6-1: PDSCH Reference Channel for Cat-M1 TDD in CEModeB

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| PDSCH Reference channel |  | R.18 TDD | R.19 TDD |
| Carrier bandwidth | MHz | 10 | 10 |
| Number of transmitter antennas |  | 1 | 2 |
| Allocated resource blocksNote1 | PRBs | 2 | 2 |
| Allocated subframes per Radio Frame |  | 4 | 4 |
| TDD Uplink-Downlink Configuration |  | 0 | 0 |
| TDD Special Subframe Configuration |  | 6 | 6 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| All Sub-Frames except 1, 6 (TDD) | Bits | 32 | 32 |
| All Sub-Frames except 1, 6 (TDD DwPTS) | - | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  | 1 | 1 |
| All Sub-Frames except 1, 6 (TDD) |  | 1 | 1 |
| All Sub-Frames except 1, 6 (TDD DwPTS) |  | 0 | 0 |
| Maximum number of repetitions |  | 192 | 192 |
| Frequency hopping |  | ON | ON |
| Number of narrowbands for frequency hopping |  | 2 | 2 |
| PDSCH Narrowband |  | 2nd | 2nd |
| Frequency HoppingOffset | narrowbands | 7 | 7 |
| PDSCH start symbol | symbols | 2 | 2 |
| Frequency hopping interval | subframes | 20 | 20 |
| Max. Throughput averaged over 1 frame | kbps | TBD | TBD |
| Cell ID |  | Note 2 | Note 2 |
| NOTE 1: Allocation is located in the middle of narrowband. NOTE 2: Cell ID shall depend upon the test configuration. |  |  |  |

# A.8a	Reference Configurations for satellites for Cat-M1 UEs

## A.8a.1	General setup for SIB31

The general parameters for SIB31 setup is specified in Table A.8a.1-1.

Table A.8a.1-1: Test cases for NTN specific requirements

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO, GSO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO, GSO: 900s |
| cellSpecificKoffset | slot | LEO: 8 GEO, GSO: 256 |
| kmac | slot | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ntn-PolarizationDL |  | linear |
| ntn-PolarizationUL |  | linear |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |
| ta-Report |  | Not configured |

## A.8a.2	Satellite specific configuration for serving cell

Table A.8a.2-1: SIB31 parameters setup for Serving satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | SSC.1 | SSC.2 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | slot | 258 | 14 |
| k-Mac-r17 | slot | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |  |

## A.8a.3	Satellite specific configuration for neighbour cell

Table A.8a.3-1: SIB31 parameters setup for neighbour satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for neighbour satellite |  | NSC.1 | NSC.2 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | slot | 258 | 14 |
| k-Mac-r17 | slot | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |  |

# A.9	Reference PRACH Configurations

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

## A.10.4	Satellite specific parameters configuration

### A.10.4.1	General setup for SIB-31-NB

The general parameters for SIB31-NB setup is specified in Table A.10.4.1-1.

Table A.10.4.1-1: Test cases for NTN specific requirements

| Parameter | Unit | Test 1 |
| --- | --- | --- |
| Interval between adjacent epoch time | s | LEO: 2.56 GEO, GSO: 10.24 |
| ntn-UlSyncValidityDuration | s | LEO: 5s GEO, GSO: 900s |
| cellSpecificKoffset | slot | LEO: 8 GEO, GSO: 256 |
| kmac | slot | Not configured |
| ta-Common |  | 0 |
| ta-CommonDrift |  | 0 |
| ta-CommonDriftVariant |  | 0 |
| ntn-PolarizationDL |  | linear |
| ntn-PolarizationUL |  | linear |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |
| ta-Report |  | Not configured |

### A.10.4.2	Satellite specific configuration for serving cell

The general parameters for SIB31-NB setup for serving satellite is specified in Table A.10.4.2-1.

Table A.10.4.2-1: SIB31-NB parameters setup for Serving satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for serving satellite |  | SSC.1 | SSC.2 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | slot | 258 | 14 |
| k-Mac-r17 | slot | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |  |

### A.10.4.3	Satellite specific configuration for neighbour cell

Table A.10.4.3-1: SIB31-NB parameters setup for neighbour satellite

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference configuration for neighbour satellite |  | NSC.1 | NSC.2 |
| Scenario |  | GSO or GEO (as per test definition) | NGSO |
| Interval between adjacent epoch time | s | 10.24 | 2.56 |
| ul-SyncValidityDuration-r17 | s | 900 | 5 |
| k-Offset-r17 | slot | 258 | 14 |
| k-Mac-r17 | slot | Not configured | Not configured |
| nta-Common-r17 |  | 0 | 0 |
| nta-CommonDrift-r17 |  | 0 | 0 |
| nta-CommonDriftVariation-r17 |  | 0 | 0 |
| ephemerisInfo |  | According to TS 36.133 [5] Annex B.8 |  |

## <<< END OF CHANGES 2>>>