3GPP TSG-RAN5 Meeting #102	R5-241686

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0105 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new PIXITs for REL-16 TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2024-03-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | New test cases in Rel-16 have introduced a need for additional PIXITs |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new PIXITs to Table 9.2.1-1 Added a new PIXITs to Table 9.3.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testers would not be able to use the correct parameters in a test case and the DUT could then fail. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is a final version of R5-240902 |  |  |  |  |  |  |  |  |

<START OF CHANGES>

## 9.2	MCX

### 9.2.1	MCX Client PIXIT

Table 9.2.1-1: MCX Client Common PIXIT

| Parameter Name |  | Parameter Type |  |  | Default Value |  |  | Supported Values |  |  | Description |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Client relevant IXIT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| px_MCX_Client_B_ID |  | charstring |  |  | "urn:uuid:cdcd34fb-c2f1-4488-a03a-6b5a5d360c47" |  |  |  |  |  | MCX client ID for a remote user: As defined in TS 24.379 [9] clause 4.10 it is a UUID URN according to RFC 4122 [62], i.e. has the format "urn:uuid:XXXXXXXX-YYYY-ZZZZ-yyyy-zzzzzzzzzzzz" |  |  |
| Users relevant IXIT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| px_MCPTT_ID_User_A |  | charstring |  |  | "mcptt-user-A-id@ mcptt-op.gov" |  |  | NOTE 1 |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_ID_User_A |  | charstring |  |  |  |  |  | NOTE 1 |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |  |
| px_MCData_ID_User_A |  | charstring |  |  |  |  |  | NOTE 1 |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_User_A_Alias |  | charstring |  |  | "mcptt-user-A-alias" |  |  |  |  |  | Alphanumeric alias of MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_User_A_Alias |  | charstring |  |  | "mcvideo-user-A-alias" |  |  |  |  |  | Alphanumeric alias of MCVideo user. Ref. TS 24.483 [13]. |  |  |
| px_MCData_User_A_Alias |  | charstring |  |  | "mcdata-user-A-alias" |  |  |  |  |  | Alphanumeric alias of MCData user. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_ID_FA_A |  | charstring |  |  | "mcptt-FA-A-id@ mcptt-op.gov" |  |  |  |  |  | The value is a "uri" attribute. TS 24.483 [13] clause 5.2.48W6 |  |  |
| px_MCData_ID_FA_A |  | charstring |  |  | "mcdata-FA-A-id@ mcptt-op.gov" |  |  |  |  |  | The value is a "uri" attribute. TS 24.483 [13] clause 10.2.97B3 |  |  |
| px_MCX_User_A_ParticipantType |  | charstring |  |  | "first responder" |  |  |  |  |  | Participant type of the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCX_DomainName_Organization_A |  | charstring |  |  | "mcptt-op.gov" |  |  |  |  |  | Domain name of the organization the user belongs to. |  |  |
| px_MCX_User_A_username |  | charstring |  |  | "MCPTT#U01" |  |  |  |  |  | UE's User username used for user authentication |  |  |
| px_MCX_User_A_password |  | charstring |  |  | "psw@MCPTT&7" |  |  |  |  |  | UE's User password used for user authentication |  |  |
| px_MCX_TokenRsaPrivateKey |  | octetstring |  |  |  |  |  |  |  |  | Private key being used to create signature for ID and Access Token |  |  |
| px_MCX_TrK |  | bitstring |  |  |  |  |  |  |  |  | Transport key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |  |
| px_MCX_TrK_ID |  | B32_Type |  |  |  |  |  |  |  |  | Transport key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |  |
| px_MCX_InK |  | bitstring |  |  |  |  |  |  |  |  | Integrity protection key for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |  |
| px_MCX_InK_ID |  | B32_Type |  |  |  |  |  |  |  |  | Integrity protection key ID for XML confidentiality and between the KMS and the MC KM client according to TS 33.180 [43] clause 9.3.3. |  |  |
| px_MCPTT_ID_User_B |  | charstring |  |  | "mcptt-user-B-id@ mcptt-op.gov" |  |  |  |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_ID_User_B |  | charstring |  |  | "mcvideo-user-B-id@ mcptt-op.gov" |  |  |  |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |  |
| px_MCData_ID_User_B |  | charstring |  |  | "mcdata-user-B-id@ mcptt-op.gov" |  |  |  |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |  |
| px_MCX_User_B_ParticipantType |  | charstring |  |  | "first responder" |  |  |  |  |  | Participant type of the MCX user. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_ID_User_C |  | charstring |  |  | "mcptt-user-C-id@ mcptt-op.gov" |  |  |  |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_ID_User_C |  | charstring |  |  | "mcvideo-user-C-id@ mcptt-op.gov" |  |  |  |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |  |
| px_MCData_ID_User_C |  | charstring |  |  | "mcdata-user-C-id@ mcptt-op.gov" |  |  |  |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |  |
| px_MCX_User_C_ParticipantType |  | charstring |  |  | "first responder" |  |  |  |  |  | Participant type of the MCX user. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_ID_User_D |  | charstring |  |  | "mcptt-user-D-id@ mcptt-op.gov" |  |  |  |  |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_ID_User_D |  |  | charstring |  |  | "mcvideo-user-D-id@ mcptt-op.gov" |  |  |  |  |  | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user. Ref. TS 24.483 [13]. |  |
| px_MCData_ID_User_D |  |  | charstring |  |  | "mcdata-user-D-id@ mcptt-op.gov" |  |  |  |  |  | MCData user identity (MCData ID) which is a globally unique identifier within the MCData service that represents the MCData user. Ref. TS 24.483 [13]. |  |
| px_MCX_SIP_PublicUserId_A_1 |  | charstring |  |  |  |  |  |  |  |  | (First) public user id of the SIP subscriber (user A) for MCX; the parameter shall be a SIP URI |  |  |
| px_MCX_SIP_PrivateUserId_A |  | charstring |  |  |  |  |  |  |  |  | Private user id of the SIP subscriber (user A) for MCX |  |  |
| px_MCX_SIP_PrivateUserId_B |  |  | charstring |  |  |  |  |  |  |  |  | Private user id of the SIP subscriber (user B) for MCX |  |  |
| px_MCX_SIP_HomeDomain_A |  | charstring |  |  |  |  |  |  |  |  | Home domain name of the SIP subscriber (user A) for MCX |  |  |
| px_MCX_SIP_PublicUserId_B |  | charstring |  |  |  |  |  |  |  |  | Public user id of user B |  |  |
| px_MCX_SIP_PublicUserId_C |  | charstring |  |  |  |  |  |  |  |  | Public user id of user C |  |  |
| px_MCX_SIP_RegistrationWithTemporaryIdentities |  | boolean |  |  |  |  |  |  |  |  | If true the UE shall derive the public user id, private user id and home domain name from the IMSI for SIP registration |  |  |
| Groups relevant IXIT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| px_MCPTT_Group_A_ID |  | charstring |  |  | "mcptt-group-A@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_Group_A_ID |  | charstring |  |  | "mcvideo-group-A@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCData_Group_A_ID |  | charstring |  |  | "mcdata-group-A@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_Group_A_Name |  | charstring |  |  | "mcptt-group-A-name" |  |  |  |  |  | A human readable Group name for the group |  |  |
| px_MCVideo_Group_A_Name |  | charstring |  |  | "mcvideo-group-A-name" |  |  |  |  |  | A human readable Group name for the group |  |  |
| px_MCData_Group_A_Name |  | charstring |  |  | "mcdata-group-A-name" |  |  |  |  |  | A human readable Group name for the group |  |  |
| px_MCX_Group_A_Owner_Organization |  | charstring |  |  | "mcx-op.gov" |  |  |  |  |  | Indicates the group's owner organization the group belongs to. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_Group_A_preferred_VCodec |  | charstring |  |  | "AMR-WB" |  |  |  |  |  | Preferred voice codec for the group (a RTP payload). MCPTT clients shall support the AMR-WB codec. RFC 4566 [25] TS 26.171 [26] |  |  |
| px_MCPTT_Group_B_ID |  | charstring |  |  | "mcptt-group-B@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_Group_B_ID |  | charstring |  |  | "mcvideo-group-B@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCData_Group_B_ID |  | charstring |  |  | "mcdata-group-B@mcptt-op.gov" |  |  |  |  |  | Group ID for a group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCPTT_Group_T_ID |  | charstring |  |  | "mcptt-group-T@mcptt-op.gov" |  |  |  |  |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCVideo_Group_T_ID |  | charstring |  |  | "mcvideo-group-T@mcptt-op.gov" |  |  |  |  |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| px_MCData_Group_T_ID |  | charstring |  |  | "mcdata-group-T@mcptt-op.gov" |  |  |  |  |  | Group ID for a temporary group. Value is an "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. Ref. TS 24.483 [13]. |  |  |
| Miscellaneous IXIT |  |  |  |  |  |  |  |  |  |  |  |  |  |
| px_MCX_APN |  | charstring |  |  | "mcx-apn" |  |  |  |  |  | A single APN which the UE shall use to access each and all MCX relevant services. The APN is provided in the initial UE configuration as specified in TS 36.579-1 [2] Table 5.5.8.1-1. |  |  |
| px_MCX_InitialRegistration_TypeOfPDN1 |  | MCX_Registration_PDN_Type |  |  | mcx |  |  | ims, internet, mcx |  |  | First PDN registered during initial registration (either ‘ims’ or ‘internet’ or ‘mcx’; ‘none’ is not applicable as first PDN) |  |  |
| px_MCX_InitialRegistration_TypeOfPDN2 |  | MCX_Registration_PDN_Type |  |  | none |  |  | ims, internet, mcx, none |  |  | Second PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no second PDN connectivity requested by the UE during initial registration |  |  |
| px_MCX_InitialRegistration_TypeOfPDN3 |  | MCX_Registration_PDN_Type |  |  | none |  |  | ims, internet, mcx, none |  |  | Third PDN registered during initial registration; in addition to ‘ims’ or ‘internet’ or ‘mcx’ it may be ‘none’ to indicate that there is no third PDN connectivity requested by the UE during initial registration |  |  |
| px_MCX_CoordinateLatitude_Client_B |  | float |  |  | none |  |  |  |  |  | Simulated latitude value sent to the UE to inform about Client_B location |  |  |
| px_MCX_CoordinateLongitude_Client_B |  | float |  |  | none |  |  |  |  |  | Simulated longitude value sent to the UE to inform about Client_B location |  |  |
| NOTE 1: According to TS 23.280 [54] clause 8.1.2 a MC service ID shall be a URI; nevertheless in context of this specification only URIs consisting of userinfo, host and path elements (see RFC 3986 [53]) are supported. Editor's note: A similar note may need to be added to other PIXITs too |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 9.2.2	MCX Server PIXIT

Table 9.2.2-1: MCX Server Common PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_MCX_GroupCreationXUI | charstring | "mcx-gms@mcptt-op.gov" |  | Indicates the group creation XUI information for creation of groups. Ref. TS 23.003 [21]. |
| px_MCX_IdMS_auth_IPAddress | charstring |  |  | IPv4/IPv6 address of the IdMS authorization endpoint |
| px_MCX_IdMS_auth_Port | integer |  |  | Port number of the IdMS authorization endpoint |
| px_MCX_IdMS_auth_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the IdMS authorization endpoint |
| px_MCX_IdMS_token_IPAddress | charstring |  |  | IPv4/IPv6 address of the IdMS token endpoint |
| px_MCX_IdMS_token_Port | integer |  |  | Port number of the IdMS token endpoint |
| px_MCX_IdMS_token_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the IdMS token endpoint |
| px_MCX_HTTP_Proxy_IPAddress | charstring |  |  | IPv4/IPv6 address of the HTTP proxy |
| px_MCX_HTTP_Proxy_Port | integer |  |  | Port number of the HTTP proxy |
| px_MCX_HTTP_Proxy_Certificate | charstring |  |  | Identifier or file name of the certificate to be used during establishment of the TLS tunnel to the HTTP proxy |
| px_MCX_InitialConfigServer_IPAddress | charstring |  |  | IPv4/IPv6 address of the server optionally being used to download the Initial UE Configuration document |
| px_MCX_InitialConfigServer_Port | integer |  |  | Port number of the server optionally being used to download the Initial UE Configuration document |
| px_MCX_InitialConfigServer_UriPath | charstring | "/cms/initial-ue-config" |  | URI Path component: Absolute path used for HTTP requests addressing a server to download the UE initial configuration document |
| px_MCX_TLS_CipherSuite | TLS_CIPHER_Type | TLS_RSA_WITH_NULL_SHA |  | Cipher suite to be used for TLS connections |
| px_MCX_OAuth_ClientId_A | charstring |  |  | Client ID of the UE's MCX application as used in OAuth signalling with the IdMS |
| px_MCX_OAuth_RedirectURI_A | charstring |  |  | Redirect URI used by the UE implementation (user agent, MCX client and OS) to redirect the OAuth authentication response to the MCX client application |

### 9.2.3	MCX Constant PIXIT Definitions

Several parameters for MCX conformance testing can be defined as constants as they are neither preconfigured at the UE nor at the SS. Table 9.2.3-1 lists these constants.

Table 9.2.3-1: MCX Constant PIXIT Definitions

| Constant Name | Constant Type | Value | Description |
| --- | --- | --- | --- |
| tsc_MCX_KMS_Hostname | charstring | "kms." & px_MCX_DomainName_Organization_A | FQDN of the KMS; used in initial UE configuration as domain name for the 'kms' URI in the App-Server-Info. |
| tsc_MCX_CMS_Hostname | charstring | "cms." & px_MCX_DomainName_Organization_A | FQDN of the CMS; used in initial UE configuration as domain name for the 'cms' URI in the App-Server-Info. |
| tsc_MCX_GMS_Hostname | charstring | "gms." & px_MCX_DomainName_Organization_A | FQDN of the GMS; used in initial UE configuration as domain name for the 'gms' URI in the App-Server-Info. |
| tsc_MCX_KMS_CertUri | charstring | "certificate1." & tsc_MCX_KMS_Hostname | Name of the KMS certificate sent to the UE during MCX user authentication |
| tsc_MCX_IdMS_auth_UriPath | charstring | "/idms/auth" | URI path to address the IdMS authorization endpoint |
| tsc_MCX_IdMS_token_UriPath | charstring | "/idms/token" | URI path to address the IdMS token endpoint |
| tsc_MCX_IdMS_userauth_UriPath | charstring | "/idms/userauth" | relative URI at IdMS used for user authentication |
| tsc_MCX_KMS_init_UriPath | charstring | "/keymanagement/identity/v1/init" | Path of the Request-URI for "KMS Initialize" request according to TS 33.180 [43] D.2.3 |
| tsc_MCX_KMS_keyprov_UriPath | charstring | "/keymanagement/identity/v1/keyprov" | Path of the Request-URI for "KMS KeyProvision" request according to TS 33.180 [43] D.2.4 |
| tsc_MCX_KMS_ClientReqUrl_init | charstring | "https://" & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_init_UriPath | used as <ClientReqUrl> entry in the KMS Key Set and as Request-URI for "KMS Initialize" request according to TS 33.180 [43] D.2.3 (NOTE 1) |
| tsc_MCX_KMS_ClientReqUrl | charstring | "https://" & tsc_MCX_KMS_Hostname & tsc_MCX_KMS_keyprov_UriPath | used as <ClientReqUrl> entry in the KMS Key Set and as Request-URI for "KMS KeyProvision" request according to TS 33.180 [43] D.2.4 (NOTE 1) |
| tsc_MCX_GMSURI | charstring | "subscription-proxy." & tsc_MCX_GMS_Hostname | used for <GMS-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9; sub-domain of the GMS's hostname |
| tsc_MCX_CMSXCAPRootURI | charstring | "http://xcap." & tsc_MCX_CMS_Hostname | used for <CMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9C |
| tsc_MCX_GMSXCAPRootURI | charstring | "http://xcap." & tsc_MCX_GMS_Hostname | used for <GMS-XCAP-root-URI> element in the MCS UE initial configuration document according to TS 24.484 [14] clause 7.2.2.7 and TS 24.483 [13] clause 8.2.9B |
| tsc_MCX_MC_ID_User_A | charstring | px_MCX_User_A_username | According to TS 24.482 [12] clause 6.3.1 the MC ID set to the user name |
| tsc_MCPTT_PublicServiceId_A | charstring | "sip:" & "mcptt-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCPTT Server which is simulated by the SS |
| tsc_MCVideo_PublicServiceId_A | charstring | "sip:" & "mcvideo-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCVideo Server which is simulated by the SS |
| tsc_MCData_PublicServiceId_A | charstring | "sip:" & "mcdata-server-A@" & px_MCX_DomainName_Organization_A | The URI of the MCData Server which is simulated by the SS |
| tsc_MCPTT_PublicServiceId_B | charstring | "sip:" & "mcptt-server-B@" & px_MCX_DomainName_Organization_A | The URI of a second MCPTT Server which is implemented in the DUT used in MCPTT Server testing. |
| tsc_MCPTT_PublicServiceId_PF_A | charstring | "sip:" & "participating-mcptt-function-A@" & px_MCX_DomainName_Organization_A | The URI of the participating MCPTT function which configures the location reporting at the UE according to TS 24.379 [9] clause 13.2.2. |
| tsc_MCVideo_PublicServiceId_PF_A | charstring | "sip:" & "participating-mcvideo-function-A@" & px_MCX_DomainName_Organization_A | The URI of the participating MCVideo function which configures the location reporting at the UE according to TS 24.281 [55] clause 18.2.2. |
| tsc_MCData_PublicServiceId_PF_A | charstring | "sip:" & "participating-mcdata-function-A@" & px_MCX_DomainName_Organization_A | The URI of the participating MCData function which configures the location reporting at the UE according to TS 24.282 [57] clause 17.2.2. |
| tsc_MCX_SessionID_B | charstring | "sip:" & "sessionB@cf." & px_MCX_DomainName_Organization_A | The URI of the MCPTT session B identity. Ref. TS 24.483 [13]. SIP URI according to TS 24.379 [9] clause 4.5. |
| tsc_MCPTT_SessionId | charstring | "sip:" & "mcptt-session-A@cf." & px_MCX_DomainName_Organization_A | Service Id for MCPTT calls as used as Contact URI of the MCPTT server |
| tsc_MCVideo_SessionId | charstring | "sip:" & "mcvideo-session-A@cf." & px_MCX_DomainName_Organization_A | Service Id for MCVideo calls as used as Contact URI of the MCVideo server |
| tsc_MCData_SessionId | charstring | "sip:" & "mcdata-session-A@cf." & px_MCX_DomainName_Organization_A | Service Id for MCData calls as used as Contact URI of the MCData server |
| tsc_MCData_MSF_Hostname | charstring | "msf." & px_MCX_DomainName_Organization_A | Hostname of the message storage function (MSF) |
| tsc_MCData_MSF_URI | charstring | "http://" & tsc_MCData_MSF_Hostname & "/userA/files" | Absolute URI of the message storage function (MSF) |
| tsc_MCPTT_Group_A_ProSeLayer2GroupID | octetstring | '00000A'O | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |
| tsc_MCPTT_Group_D_ProSeLayer2GroupID | octetstring | '00000D'O | Indicates the Prose layer-2 group ID for the group. Ref. TS 23.303 [24]. |
| tsc_MCVideo_VSF_Hostname | charstring | "vsf." & px_MCX_DomainName_Organization_A | Hostname of the video storage function (VSF) where video files are stored |
| tsc_MCVideo_VSF_URI | charstring | "http://" & tsc_MCVideo_VSF_Hostname & "/userA/files" | Absolute URI of the message storage function (MSF) where video files are stored |
| NOTE 1: According to the NOTE at step 0 in clause 5.2.3 of TS 33.180 [43] the connection between the KMS and the HTTP Proxy is secured and therefore the URIs of the KMS are assumed to have “https” rather than “http” as scheme. |  |  |  |

<END OF CHANGES>