3GPP TSG-RAN5 Meeting #102	R5-241723

# Athens, Greece, 26th Feb – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3026 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Test procedure for establishing unicast mode ProSe Direct communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The format for conditional events description need refined. - Typos in related clause - The steps in tables are not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Refine the format for conditional events description. - Correction on typos - Specify the steps in tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.39, 4.9.40, 4.9.41 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Final version of R5-240347. |  |  |  |  |  |  |  |  |

<Change starts>

4.9.39	Test procedure for establishing unicast mode ProSe Direct communication / Initiating UE side

4.9.39.1	Scope

The purpose of this procedure is for establishing unicast mode ProSe Direct communication / Initiating UE side.

4.9.39.2	Procedure description

4.9.39.2.1	Initial conditions

The UE is in state 1N-B, 3N-B or 4-A.

4.9.39.2.2	Procedure sequence

Table 4.9.39.2.2-1: Procedure for establishing unicast mode ProSe Direct communication / Initiating UE side

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| - | EXCEPTION: Steps 1a1 to 1b2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - |
| 1a1 | IF the UE is in state 1N-B or 4-A THEN the UE is configured by upper layer to establish unicast mode link. NOTE: This can be done by sending AT COMMAND +CCUTLE to close test loop function. | - | - |
| 1b1 | ELSE IF the UE is in state 3N-B THEN the SS transmits a CLOSE UE TEST LOOP message. | <-- | TC: CLOSE UE TEST LOOP |
| 1b2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | TC: CLOSE UE TEST LOOP COMPLETE |
| 2 | The UE sends a PROSE DIRECT LINK ESTABLISHMENT REQUEST message. | --> | PC5-S: PROSE DIRECT LINK ESTABLISHMENT REQUEST |
| 3 | The NR-SS-UE sends a PROSE DIRECT LINK SECURITY MODE COMMAND message. | <-- | PC5-S: PROSE DIRECT LINK SECURITY MODE COMMAND |
| 4 | The UE sends a PROSE DIRECT LINK SECURITY MODE COMPLETE message. | --> | PC5-S: PROSE DIRECT LINK SECURITY MODE COMPLETE |
| 5 | The NR-SS-UE sends a DIRECT LINK ESTABLISHMENT ACCEPT message. | <-- | PC5-S: PROSE DIRECT LINK ESTABLISHMENT ACCEPT |
| 6 | The UE sends an RRCReconfigurationSidelink message to establish a unicast mode SL-DRB. NOTE: Unless specifically specified in the test case prose, the UE shall select the DRB parameters as specified in the pre-configured parameters. | --> | PC5-RRC: RRCReconfigurationSidelink |
| 7 | The NR-SS-UE sends an RRCReconfigurationCompleteSidelink message. | <-- | PC5-RRC: RRCReconfigurationCompleteSidelink |
| - | EXCEPTION: Steps 8a1 to 8b2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that takes place if a procedure parameter has a particular value. | - | - |
| 8a1 | IF the UE is in state 1N-B or 4-A THEN the SS sends AT COMMAND +CCUTLE to open test loop function | - | - |
| 8b1 | ELSE IF the UE is in state 3N-B THEN the SS transmits an OPEN UE TEST LOOP message. | <-- | TC: OPEN UE TEST LOOP |
| 8b2 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | TC: OPEN UE TEST LOOP COMPLETE |

4.9.39.3	Specific message contents

All specific message contents shall be according to subclause 4.6, 4.7 and 4.7B with the exceptions below.

Table 4.9.39.3-1: +CCUTLE (Step 1a1, Table 4.9.39.2.2-1)

Derivation Path: Table 4.7B-1 with condition Close and Transmit

Table 4.9.39.3-2: CLOSE UE TEST LOOP (Step 1b1, Table 4.9.39.2.2-1)

| Derivation Path: 36.508 [2] Table 4.7A-3 with condition UE TEST LOOP MODE E(V2X Transmission) |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE test loop mode E LB setup |  |  |  |  |
| Communication Transmit or Receive |  | 0 0 0 0 0 0 0 1 | ‘01’ indicates V2X UE triggered to transmit NR sidelink communication with single spatial layer. |  |

4.9.40	Test procedure for establishing unicast mode ProSe Direct communication / Peer UE side

4.9.40.1	Scope

The purpose of this procedure is for establishing unicast mode ProSe Direct communication / Peer UE side.

4.9.40.2	Procedure description

4.9.40.2.1	Initial conditions

The UE is in state 1N-B, 3N-B or 4-A.

4.9.40.2.2	Procedure sequence

Table 4.9.40.2.2-1: Procedure for establishing unicast mode ProSe Direct communication / Peer UE side

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U – S | Message |
| 1 | The NR-SS-UE sends a PROSE DIRECT LINK ESTABLISHMENT REQUEST message. | <-- | PC5-S: PROSE DIRECT LINK ESTABLISHMENT REQUEST |
| 2 | The UE sends a PROSE DIRECT LINK SECURITY MODE COMMAND message. | --> | PC5-S: PROSE DIRECT LINK SECURITY MODE COMMAND |
| 3 | The NR-SS-UE sends a PROSE DIRECT LINK SECURITY MODE COMPLETE message. | <-- | PC5-S: PROSE DIRECT LINK SECURITY MODE COMPLETE |
| 4 | The UE sends a PROSE DIRECT LINK ESTABLISHMENT ACCEPT message. | --> | PC5-S: PROSE DIRECT LINK ESTABLISHMENT ACCEPT |
| 5 | The NR-SS-UE sends an RRCReconfigurationSidelink message to establish a unicast mode SL-DRB. | <-- | PC5-RRC: RRCReconfigurationSidelink |
| 6 | The UE sends an RRCReconfigurationCompleteSidelink message. | --> | PC5-RRC: RRCReconfigurationCompleteSidelink |

4.9.40.3	Specific message contents

All specific message contents shall be according to subclause 4.6 and 4.7B.

Table 4.9.40.3-1: PROSE DIRECT LINK ESTABLISHMENT REQUEST (Step 1, Table 4.9.40.2.2-1)

Derivation Path: Table 4.7.7-1 with condition Rx

Table 4.9.40.3-2: PROSE DIRECT LINK SECURITY MODE COMMAND (Step 2, Table 4.9.40.2.2-1)

Derivation Path: Table 4.7.7-2 with condition Tx

Table 4.9.40.3-3: PROSE DIRECT LINK SECURITY MODE COMPLETE (Step 3, Table 4.9.40.2.2-1)

Derivation Path: Table 4.7.7-14 with condition Rx

Table 4.9.40.3-4: PROSE DIRECT LINK ESTABLISHMENT ACCEPT (Step 4, Table 4.9.40.2.2-1)

Derivation Path: Table 4.7.7-2 with condition Tx

Table 4.9.40.3-5: RRCReconfigurationSidelink (Step 5, Table 4.9.40.2.2-1)

Derivation Path: Table 4.6.1A-3 with condition RX and SL_DRB

Table 4.9.40.3-6: RRCReconfigurationCompleteSidelink (Step 6, Table 4.9.40.2.2-1)

Derivation Path: Table 4.6.1A-4 with condition Tx

4.9.41	Test procedure for registration of NR sidelink U2N Relay / Relay UE side

4.9.41.1	Scope

The purpose of this procedure is testing relay UE behaviour when remote UE of NR sidelink U2N Relay is performing registration procedure.

4.9.41.2	Procedure description

4.9.41.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

-	1 NR-SS-UE

-	1 GNSS simulator

User Equipment:

-	The UE is in state 3N-A.

4.9.41.2.2	Procedure sequence

Table 4.9.41.2.2-1: Procedure for registration of NR sidelink U2N Relay / Relay UE side

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Trigger UE to reset or clear the current UTC time that has been calculated from GNSS. NOTE: The UTC time can be reset or clear on the UE using AT command (+CUTCR). |  |  |  |  |
| - | EXCEPTION: Steps 2a1 and 2b1 describe the SS sequence depending on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 2a1 | IF Cast type = Unicast AND UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.39 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 2b1 | ELSE IF Cast type = Unicast AND NR-SS-UE initiated unicast mode ProSe Direct communication THEN generic procedure specified in subclause 4.9.40 is performed to establish unicast mode sidelink communication between the UE and the NR-SS-UE. | - | - | - | - |
| 3 | The NR-SS-UE1 transmits an RRCSetupRequest message. | <-- | PC5-RRC: RRCSetupRequest | - | - |
| 4 | The UE transmits a SidelinkUEInformationNR message. | --> | NR RRC: SidelinkUEInformationNR | - | - |
| 5 | The SS transmits an RRCReconfiguration message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message | --> | NR RRC: RRCReconfigurationComplete | - |  |
| 7 | The UE relays the RRCSetupRequest message received at step 3. | --> | NR RRC: RRCSetupRequest | - | - |
| 8 | The SS transmits an RRCSetup message. | <-- | NR RRC: RRCSetup | - | - |
| 9 | The UE relays the RRCSetup message received at step 8. | --> | PC5-RRC: RRCSetup | - | - |
| 10 | The NR-SS-UE1 transmits an RRCSetupComplete message. | <-- | PC5-RRC: RRCSetupComplete | - | - |
| 11 | The UE relays the RRCSetupComplete message received at step 10. | --> | NR RRC: RRCSetupComplete | - | - |
| 12 | The SS transmits a SecurityModeCommand message. | <-- | NR RRC: SecurityModeCommand | - | - |
| 13 | The UE relays the SecurityModeCommand message received at step 12. | --> | PC5-RRC: SecurityModeCommand | - | - |
| 14 | The NR-SS-UE1 transmits a SecurityModeComplete message. | <-- | PC5-RRC: SecurityModeComplete | - | - |
| 15 | The UE relays the SecurityModeComplete message received at step 14. | --> | NR RRC: SecurityModeComplete | - | - |
| 16 | The SS transmits an RRCReconfiguration message. | <-- | NR RRC: RRCReconfiguration | - | - |
| 17 | The UE relays the RRCReconfiguration message received at step 16. | --> | PC5-RRC: RRCReconfiguration | - | - |
| 18 | The NR-SS-UE1 transmits an RRCReconfigurationComplete message | <-- | PC5-RRC: RRCReconfigurationComplete | - | - |
| 19 | The UE relays the RRCReconfigurationComplete message received at step 18. | --> | NR RRC: RRCReconfigurationComplete | - | P |

4.9.41.3	Specific message contents

Table 4.9.41.3-1: RRCSetupRequest (Steps 3 and 7, Table 4.9.41.2.2-1)

Derivation Path: Table 4.6.1-23 with condition L2RemoteUE

Table 4.9.41.3-2: SidelinkUEInformationNR (Step 4, Table 4.9.41.2.2-1)

Derivation Path: Table 4.6.1-28A with condition L2RelayUE

Table 4.9.41.3-3: RRCReconfiguration (Step 5, Table 4.9.41.2.2-1)

Derivation path: Table 4.6.1-13 with condition L2RelayUE and RELAY

Table 4.9.41.3-4: RRCReconfiguration (Steps 16 and 17, Table 4.9.41.2.2-1)

Derivation path: Table 4.6.1-13 with condition L2RemoteUE and RELAY

<Change ends>