3GPP TSG-RAN5 Meeting #102	R5-241726

# Athens, Greece, 26th Feb 2024 – 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2653 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Message exceptions clarifications for 6.3G.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test requirements for ON power in test case 6.3G.3.3 SRS time mask for Tx Diversity, require to test limits applicable to PC2 and PC1.5 as per respective referred Table 6.2G.1.5-1 and Table 6.2G.1.5-2. The test case description is based on 6.3.3.6 where a message exception sets P-Max = 23dBm, what makes PC2 and PC1.5 higher power unreacheable. Given that during RAN5#92-e, as per discussion paper R5-214323 and associated CRs to test case 6.3.3.6, it was agreed to limit P-Max = 23dBm for any power class in order to not damage the test equipment when it is set to the high sensitivity when measuring the OFF power. Therefore, the same must apply in equivalent TxD test case 6.3G.3.3 and test requirements need to be updated to be limited to PC3 only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In 6.3G.3.3 test case requirement, replaced reference to Table 6.2G.1.5-1 and Table 6.2G.1.5-2 by reference to Table 6.2G.1.5-4, the one applicable for PC3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3G.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1 of R5-240622: -Corrected WI code from TEI15_Test to TEI17_Test. -As per offline disscussion, Reverted message exception clarification and corrected test requirement instead. This CR is eventually replaced by R5-241779 just to have a title more consistent with the eventual changes agreed in this revision. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.3G.3.3	SRS time mask for Tx Diversity

6.3G.3.3.1	Test purpose

Same test purpose as in 6.3.3.6.1.

6.3G.3.3.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE, Power Class 2 and Power Class 3 UE release 15 and forward that support Tx diversity.

6.3G.3.3.3	Minimum conformance requirements

For UE supporting Tx diversity, the SRS time mask requirements in clause 6.3.3.6.3 apply at each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.3G.3.

6.3G.3.3.4	Test description

Same test description as in clause 6.3.3.6.4 with following exceptions:

-	The ON power is measured as sum of both transmit antenna connectors.

-	The OFF power is measured at each transmit antenna connector.

6.3G.3.3.5	Test requirement

The measured ON power and OFF power shall not exceed the values specified in Table 6.3G.3.3.5-1.

Table 6.3G.3.3.5-1: SRS time mask

|  |  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power |  | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth |  | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power |  | Same as Table 6.2G.1.5-4 |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT of OFF power for each frequency and channel bandwidth is specified in Table 6.3G.3.3.5-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3G.3.3.5-2: Test Tolerance (Transmit OFF power and SRS time mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 dB | 1.8 dB |
| 40MHz < BW ≤ 100MHz | 1.7 dB | 1.8 dB |

## <<< END OF CHANGES >>>