3GPP TSG-RAN5 Meeting #102 	R5-241737

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2704 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Reference sensitivity for CA_n28A-n41A-n79A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In Table 7.3A.0.5-2, minimum conformance requirement for CA_n28-n41-n79 does not match TS 38.101-1. - In Table 7.3A.2_1.4.1-1, n28 UL frequency is defined for DL_n28A-n41A-n79A_UL_n41A-n79A. - In Table 7.3A.2_1.4.1-1, n79 DL frequency for DL_n28A-n41A-n79A_UL_n28A-n41A is incorrect. - In Table 7.3A.2_1.4.1-1, text style error exists, and the bottom of the table is broken. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In Table 7.3A.0.5-2, minimum conformance requirement for CA_n28-n41-n79 was corrected according to TS 38.101-1. - In Table 7.3A.2_1.4.1-1, n28 UL frequency for DL_n28A-n41A-n79A_UL_n41A-n79A was removed. - In Table 7.3A.2_1.4.1-1, n79 DL frequency for DL_n28A-n41A-n79A_UL_n28A-n41A was corrected from 4480 to 4600. - In Table 7.3A.2_1.4.1-1, text style error was corrected, and the bottom of the table was reconstructed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CA_n28A-n41A-n79A will not be tested correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.0.5, 7.3A.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Table 7.3A.0.5-2 has further errors for other CA combinations, but it was not corrected in this CR due to the huge amount of corrections. It should be corrected in the future meeting. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-241097 and the outcome of 2 revisions to this document. r1: Unnecessary color mark was removed. r2: Unnecessary line was removed. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3A.0.5	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA

For inter-band carrier aggregation with uplink assigned to two NR bands given in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a, the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a. For these test points the reference sensitivity requirement specified in Table 7.3.2.3-1 and Table 7.3.2.3-2 are relaxed by the amount of the corresponding parameter MSD given in Table 7.3A.0.5-1 and Table 7.3A.0.5-1a.

<<Unchanged sections skipped>>

Editor’s Note:

The n78 UL RB size of 10MHz BW in CA_n1-n3-n78 is different from the Table 7.3A.5-2 in TS 38.101-1 (V17.9.0), since the values in the latter cannot be applied to DFT-s-OFDM. The closest smaller valid value has been applied in Table 7.3A.0.5-2.

Table 7.3A.0.5-2: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCR | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |
| CA_n28-n41-n79 | n28 | N/A | 5 | N/A | 780 | 13.0 | FDD | IMD31 |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n28 | 720 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4600 | 10.1 | TDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | N/A | 10 | N/A | 2645 | 10.4 | TDD | IMD4 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 6: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,f,c) as defined in clause 6.2A.4 |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.3A.2_1	Reference sensitivity power level for 3DL CA exceptions

7.3A.2_1.1	Test purpose

To verify the ability of UE that support CA to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when CA exceptions are allowed.

A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3A.2_1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support inter-band NR 3DL CA with UL CA support

7.3A.2_1.3	Minimum requirements

The minimum conformance requirements are defined in clause 7.3A.0.

7.3A.2_1.4	Test description

7.3A.2_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.3A.2_1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.3A.2_1.4.1-1: Test Configuration Table for inter-band 3DL CA exceptions

| Initial conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |
| Test CC Combination setting (CBW) as specified in subclause Table 5.5A.3.2-1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  |  |  | Refer to “NRB_PCC”and “NRB_SCC” columns |  |  |  |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.2.3-4 for the band with active uplink carrier |  |  |  |  |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / channel BW |  |  |  |  |  |  |  |  | DL Allocation |  |  |  | UL allocation (Note 2) |  |  |
|  | CA configuration |  |  |  |  |  | PCC | SCC1 | SCC2 | CC Mod | PCC & SCC RB allocation |  |  | CC Mod | PCC, SCC1 or SCC2 RB allocation |  |
|  | PCC |  | SCC1 |  | SCC2 |  |  |  |  |  | PCC | SCC1 | SCC2 |  |  |  |
|  | Band | Range | Band | Range | Band | Range |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n3A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n3 | UL 1750/ DL 1845 | n78 | 3700 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n1 | UL 1950/ DL 2140 | n3 | UL 1770/ DL 1865 | n78 | 3360 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n1A-n3A-n78A_UL_n1A-n78A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n1 | UL 1950/ DL 2140 | n78 | 3780 | n3 | UL 1770/ DL 1865 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n5A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 839/ DL 884 | n48 | 3640 | n2 | DL 1962 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n48A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n5 | UL 844/ DL 1889 | n48 | DL 3593 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1907.5/ DL 1987.5 | n77 | 3305 | n5 | DL 887.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 846.5/ DL 891.5 | n77 | 3680 | n2 | DL 1987 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n5A-n77A_UL_n2A-n5A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n5 | UL 830/ DL 875 | n77 | DL 3540 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n66 | UL 1770/ DL 2190 | n48 | DL 3625 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n2A-n48A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1905/ DL 1985 | n48 | 3560 | n66 | DL 2155 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n48A-n66A_UL_n48A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1735/ DL 2135 | n48 | 3695 | n2 | DL 1960 | 5MHz | 10Mhz | 5Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3620 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n66 | UL 1740/ DL 2140 | n77 | DL 3900 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n2A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n2 | UL 1855/ DL 1935 | n77 | 3970 | n66 | DL 2115 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n2 | UL 1880/ DL 1960 | n77 | 3500 | n66 | DL 2140 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n2 | UL 1885/ DL 1965 | n77 | 3915 | n66 | DL 2175 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n2A-n66A-n77A_UL_n66A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n66 | UL 1760/ DL 2160 | n77 | 3720 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n66 | UL 1760/ DL 2160 | n77 | 3350 | n2 | DL 1960 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n66 | UL 1760/ DL 2160 | n77 | 3620 | n2 | DL 1960 | 5Mhz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2518 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 2 | n3 | UL 1715/ DL 1810 | n28 | UL 743/ DL 798 | n41 | 2687 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n3A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n41 | 2510 | n28 | UL 735/ DL 790 | 5MHz | 5MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n41A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 710.5/ DL 765.5 | n41 | 2543 | n3 | UL 1737.5/ DL 1832.5 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n28A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1720/ DL 1815 | n28 | UL 733/ DL 788 | n77 | 4173 | 5MHz | 5MHz | 10MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n28A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n77 | 3320 | n3 | UL 1755/ DL 1850 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n3A-n28A-n77A_UL_n3A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n3 | UL 1712.5/ DL 1807.5 | n77 | 4195 | n28 | UL 715/ DL 770 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n48A-n66A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 829/ DL 874 | n66 | UL 1760/ DL 2160 | n48 | DL 3622 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n66A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | n5 | UL 845/ DL 890 | n66 | UL 1775/ DL 2175 | n77 | DL 3465 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 24 | n5 | UL 826.5/ DL 871.5 | n66 | UL 1712.5/ DL 2112.5 | n77 | DL 4192 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| 34 | n5 | UL 835/ DL 880 | n66 | UL 1735/ DL 2135 | n77 | DL 3535 | 5MHz | 5Mhz | 10Mhz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n5A-n66A-n77A_UL_n5A-n77A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n5 | UL 826.5/ DL 871.5 | n77 | 3795 | n66 | DL 2142 | 5MHz | 10MHz | 5MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n41A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n41 | 2600 | n28 | UL 725/ DL 780 | n79 | 4600 | 10MHz | 5 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n41A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 720/ DL 780 | n41 | 2600 | n79 | 4600 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Default Test Settings for a DL_n28A-n41A-n79A_UL_n28A-n79A Configuration (Inter-band) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | n28 | UL 735/ DL 790 | n41 | 2645 | n79 | 4850 | 5MHz | 10 MHz | 40 MHz | CP-OFDM QPSK | Full RB |  |  | DFT-s-OFDM QPSK | REFSENS_CA_3 | REFSENS_CA_3 |
| Note 1: CA Configuration Test CC Combination test settings are checked separately for each CA Configuration. Note 2: REFSENS refers to the PCC bands and PCC NRB ‘s single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3. REFSENS_CA_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to intermodulation interference of 3DL/2UL according to Table 7.3A.0.5-2 for PC3. Note 3: In a band where UE supports 4Rx, the test needs to be performed only with 4Rx antennas connected. Note 4: This step can be skipped for PC2 since no corresponding exceptions are defined for PC2 cases in Table 7.3A.0.5-2a |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

4.	The UL and Reference Measurement Channel is set according to Tables 7.3A.2_1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3A.1_1.4.3.

<<End of change>>