3GPP TSG-RAN5 Meeting #102	R5-241768

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2677 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to AMPR, ASEM and ASE for intra-band CA for CA_NS_04 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_RF_FR1 |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR corrects some errors in AMPR, ASEM and ASE for CA_NS_04. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correction of minimum requirement in 6.2A.3.0.1.1.2. - For AMPR, correcting the test requirement for CA_NS_04, and adding test requirement for PC2 with TxD. - For ASEM and ASE, correcting the test IDs as currently it’s not aligned with TP analysis. Correcting the refered table number from AMPR. - Correcting the RB allocation in 6.1A for BWagg = 160MHz and DFT-s-OFDM, which doesn’t satisfy the 2x*3y*5z restriction. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test of CA_NS_04 is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1A, 6.2A.3, 6.5A.2.3.1, 6.5A.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-240998 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.1A	General

Editor’s note:

Uplink RB allocations for intra-band UL CA with mixed numerology is FFS

The minimum requirements for band combinations including Band n41 also apply for the corresponding band combinations with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said band combinations with Band n90 are not listed in the tables below but are covered by this specification.

Uplink RB allocations for intra-band CA given in Table 6.1A-1a are used throughout this section, unless otherwise stated by the test case.

Table 6.1A-1a: Common uplink configuration for intra-band contiguous 2UL CA (contiguous RB allocation)

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 5+15 | CP | 52 | 0@0 | 52@1 | 104 | 25@0 | 79@0 |
|  |  |  | DFT-s | 50 | 0@0 | 50@0 | 100 | 25@0 | 75@0 |
| 20 |  | 10+10 | CP | 52 | 26@26 | 26@0 | 104 | 52@0 | 52@0 |
|  |  |  | DFT-s | 50 | 25@27 | 25@0 | 100 | 50@2 | 50@0 |
|  |  | 15+5 | CP | 52 | 52@26 | 0@0 | 104 | 79@0 | 25@0 |
|  |  |  | DFT-s | 50 | 50@29 | 0@0 | 100 | 75@4 | 25@0 |
|  | 15 | 5+20 | CP | 654 | 0@0 | 65@7 | 131 | 25@0 | 106@0 |
|  |  |  | DFT-s | 64 | 0@0 | 64@7 | 125 | 25@0 | 100@0 |
|  |  | 10+15 | CP | 654 | 20@32 | 45@0 | 131 | 52@0 | 79@0 |
|  |  |  | DFT-s | 65 | 20@32 | 45@0 | 125 | 50@2 | 75@0 |
|  |  | 15+10 | CP | 654 | 47@32 | 18@0 | 131 | 79@0 | 52@0 |
|  |  |  | DFT-s | 65 | 45@34 | 20@0 | 125 | 75@4 | 50@0 |
|  |  | 20+5 | CP | 654 | 65@32 | 0@0 | 131 | 106@0 | 25@0 |
| 25 |  |  | DFT-s | 64 | 64@35 | 0@0 | 125 | 100@6 | 25@0 |
|  | 30 | 10+15 | CP | 604 | 9@15 | 21@0 | 124 | 24@0 | 38@0 |
|  |  |  | DFT-s | 58 | 9@15 | 20@0 | 120 | 24@0 | 36@0 |
|  |  | 15+10 | CP | 604 | 23@15 | 7@0 | 124 | 38@0 | 24@0 |
|  |  |  | DFT-s | 58 | 20@18 | 9@0 | 120 | 36@2 | 24@0 |
|  | 15 | 10+20 | CP | 79 | 13@39 | 66@0 | 158 | 52@0 | 106@0 |
|  |  |  | DFT-s | 76 | 12@40 | 64@0 | 150 | 50@2 | 100@0 |
|  |  | 15+15 | CP | 79 | 40@39 | 39@0 | 158 | 79@0 | 79@0 |
|  |  |  | DFT-s | 76 | 40@39 | 36@0 | 150 | 75@4 | 75@0 |
|  |  | 20+10 | CP | 79 | 67@39 | 12@0 | 158 | 106@0 | 52@0 |
|  |  |  | DFT-s | 76 | 64@42 | 12@0 | 150 | 100@6 | 50@0 |
|  | 30 | 10+20 | CP | 74 | 5@19 | 32@0 | 150 | 24@0 | 51@0 |
|  |  |  | DFT-s | 74 | 5@19 | 32@0 | 148 | 24@0 | 50@0 |
| 30 |  | 15+15 | CP | 76 | 19@19 | 19@0 | 152 | 38@0 | 38@0 |
|  |  |  | DFT-s | 72 | 20@18 | 16@0 | 144 | 36@2 | 36@0 |
|  |  | 20+10 | CP | 74 | 32@19 | 5@0 | 150 | 51@0 | 24@0 |
|  |  |  | DFT-s | 74 | 32@19 | 5@0 | 148 | 50@1 | 24@0 |
|  | 15 | 15+20 | CP | 93 | 33@46 | 60@0 | 185 | 79@0 | 106@0 |
|  |  |  | DFT-s | 92 | 32@47 | 60@0 | 175 | 75@4 | 100@0 |
|  |  | 20+15 | CP | 93 | 60@46 | 33@0 | 185 | 106@0 | 79@0 |
|  |  |  | DFT-s | 92 | 60@46 | 32@0 | 175 | 100@6 | 75@0 |
|  | 30 | 15+20 | CP | 88 | 16@22 | 28@0 | 178 | 38@0 | 51@0 |
|  |  |  | DFT-s | 86 | 16@22 | 27@0 | 172 | 36@2 | 50@0 |
| 35 |  | 20+15 | CP | 88 | 29@22 | 15@0 | 178 | 51@0 | 38@0 |
|  |  |  | DFT-s | 86 | 27@24 | 16@0 | 172 | 50@1 | 36@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 10+30 | CP | 106 | 0@0 | 106@1 | 212 | 52@0 | 160@0 |
|  |  |  | DFT-s | 100 | 0@52 | 100@0 | 210 | 50@2 | 160@0 |
| 40 |  | 20+20 | CP | 106 | 53@53 | 53@0 | 212 | 106@0 | 106@0 |
|  |  |  | DFT-s | 104 | 54@52 | 50@0 | 200 | 100@6 | 100@0 |
|  |  | 30+10 | CP | 106 | 106@53 | 0@0 | 212 | 160@0 | 52@0 |
|  |  |  | DFT-s | 100 | 100@60 | 0@0 | 210 | 160@0 | 50@0 |
|  | 30 | 10+30 | CP | 1004 | 0@0 | 50@1 | 204 | 24@0 | 78@0 |
|  |  |  | DFT-s | 100 | 0@0 | 50@1 | 198 | 24@0 | 75@0 |
|  |  | 20+20 | CP | 1004 | 26@25 | 24@0 | 204 | 51@0 | 51@0 |
|  |  |  | DFT-s | 100 | 25@26 | 25@0 | 200 | 50@1 | 50@0 |
|  |  | 30+10 | CP | 1004 | 50@25 | 0@0 | 204 | 78@0 | 24@0 |
|  |  |  | DFT-s | 100 | 50@27 | 0@0 | 198 | 75@3 | 24@0 |
|  | 15 | 15+30 | CP | 1064 | 26@53 | 80@0 | 239 | 79@0 | 160@0 |
|  |  |  | DFT-s | 118 | 18@61 | 100@0 | 235 | 75@4 | 160@0 |
| 45 | 30 | 15+30 | CP | 116 | 9@29 | 49@0 | 232 | 38@0 | 78@0 |
|  |  |  | DFT-s | 114 | 9@29 | 48@0 | 222 | 36@2 | 75@0 |
|  | 15 | 10+40 | CP | 134 | 0@0 | 134@15 | 268 | 52@0 | 216@0 |
|  |  |  | DFT-s | 128 | 0@0 | 128@12 | 266 | 50@2 | 216@0 |
|  |  | 20+30 | CP | 133 | 40@66 | 93@0 | 266 | 106@0 | 160@0 |
|  |  |  | DFT-s | 130 | 40@66 | 90@0 | 260 | 100@6 | 160@0 |
|  |  | 30+20 | CP | 133 | 94@66 | 39@0 | 266 | 160@0 | 106@0 |
|  |  |  | DFT-s | 130 | 90@70 | 40@0 | 260 | 160@0 | 100@0 |
| 50 |  | 40+10 | CP | 134 | 134@67 | 0@0 | 268 | 216@0 | 52@0 |
|  |  |  | DFT-s | 128 | 128@76 | 0@0 | 266 | 216@0 | 50@0 |
|  | 30 | 10+40 | CP | 1284 | 0@0 | 64@8 | 260 | 24@0 | 106@0 |
|  |  |  | DFT-s | 128 | 0@0 | 64@8 | 248 | 24@0 | 100@0 |
|  |  | 20+30 | CP | 128 | 19@32 | 45@0 | 258 | 51@0 | 78@0 |
|  |  |  | DFT-s | 126 | 18@33 | 45@0 | 250 | 50@1 | 75@0 |
|  |  | 30+20 | CP | 128 | 46@32 | 18@0 | 258 | 78@0 | 51@0 |
|  |  |  | DFT-s | 126 | 45@33 | 18@0 | 250 | 75@3 | 50@0 |
|  |  | 40+10 | CP | 1284 | 64@32 | 0@0 | 260 | 106@0 | 24@0 |
|  |  |  | DFT-s | 128 | 64@34 | 0@0 | 248 | 100@6 | 24@0 |
|  | 15 | 15+40 | CP | 1474 | 6@73 | 141@0 | 295 | 79@0 | 216@0 |
|  |  |  | DFT-s | 143 | 8@71 | 135@0 | 291 | 75@4 | 216@0 |
| 55 | 30 | 15+40 | CP | 144 | 2@36 | 70@0 | 288 | 38@0 | 106@0 |
|  |  |  | DFT-s | 136 | 4@34 | 64@0 | 272 | 36@2 | 100@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 10+50 | CP | 161 | 0@0 | 161@28 | 322 | 52@0 | 270@0 |
|  |  |  | DFT-s | 160 | 0@0 | 160@28 | 320 | 50@2 | 270@0 |
|  |  | 20+40 | CP | 161 | 26@80 | 135@0 | 322 | 106@0 | 216@0 |
|  |  |  | DFT-s | 160 | 25@81 | 135@0 | 316 | 100@6 | 216@0 |
|  |  | 30+30 | CP | 160 | 80@80 | 80@0 | 320 | 160@0 | 160@0 |
|  |  |  | DFT-s | 160 | 80@80 | 80@0 | 320 | 160@0 | 160@0 |
|  |  | 40+20 | CP | 161 | 136@80 | 25@0 | 322 | 216@0 | 106@0 |
|  |  |  | DFT-s | 160 | 135@81 | 25@0 | 316 | 216@0 | 100@0 |
|  |  | 50+10 | CP | 161 | 161@80 | 0@0 | 322 | 270@0 | 52@0 |
|  |  |  | DFT-s | 160 | 160@82 | 0@0 | 320 | 270@0 | 50@0 |
|  | 30 | 10+50 | CP | 156 | 0@0 | 78@15 | 314 | 24@0 | 133@0 |
|  |  |  | DFT-s | 150 | 0@0 | 75@14 | 304 | 24@0 | 128@0 |
| 60 |  | 20+40 | CP | 156 | 12@39 | 66@0 | 314 | 51@0 | 106@0 |
|  |  |  | DFT-s | 152 | 12@39 | 64@0 | 300 | 50@1 | 100@0 |
|  |  | 30+30 | CP | 156 | 39@39 | 39@0 | 312 | 78@0 | 78@0 |
|  |  |  | DFT-s | 152 | 40@38 | 36@0 | 300 | 75@3 | 75@0 |
|  |  | 40+20 | CP | 156 | 67@39 | 11@0 | 314 | 106@0 | 51@0 |
|  |  |  | DFT-s | 152 | 64@42 | 12@0 | 300 | 100@6 | 50@0 |
|  |  | 50+10 | CP | 156 | 78@39 | 0@0 | 314 | 133@0 | 24@0 |
|  |  |  | DFT-s | 150 | 75@44 | 0@0 | 304 | 128@5 | 24@0 |
|  | 15 | 20+50 | CP | 188 | 12@94 | 176@0 | 376 | 106@0 | 270@0 |
|  |  |  | DFT-s | 181 | 1@105 | 180@0 | 370 | 100@6 | 270@0 |
|  |  | 30+40 | CP | 188 | 66@94 | 122@0 | 376 | 160@0 | 216@0 |
|  |  |  | DFT-s | 184 | 64@96 | 120@0 | 376 | 160@0 | 216@0 |
|  |  | 40+30 | CP | 188 | 122@94 | 66@0 | 376 | 216@0 | 160@0 |
|  |  |  | DFT-s | 184 | 120@96 | 64@0 | 376 | 216@0 | 160@0 |
|  |  | 50+20 | CP | 188 | 176@94 | 12@0 | 376 | 270@0 | 106@0 |
|  |  |  | DFT-s | 181 | 180@90 | 1@0 | 370 | 270@0 | 100@0 |
|  | 30 | 20+50 | CP | 184 | 5@46 | 87@0 | 368 | 51@0 | 133@0 |
|  |  |  | DFT-s | 174 | 6@45 | 81@0 | 356 | 50@1 | 128@0 |
|  |  | 30+40 | CP | 184 | 32@46 | 60@0 | 368 | 78@0 | 106@0 |
| 70 |  |  | DFT-s | 184 | 32@46 | 60@0 | 350 | 75@3 | 100@0 |
|  |  | 40+30 | CP | 184 | 60@46 | 32@0 | 368 | 106@0 | 78@0 |
|  |  |  | DFT-s | 184 | 60@46 | 32@0 | 350 | 100@6 | 75@0 |
|  |  | 50+20 | CP | 184 | 87@46 | 5@0 | 368 | 133@0 | 51@0 |
|  |  |  | DFT-s | 174 | 81@52 | 6@0 | 356 | 128@5 | 50@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | 30+50 | CP | 215 | 53@107 | 162@0 | 430 | 160@0 | 270@0 |
|  |  |  | DFT-s | 210 | 50@110 | 160@0 | 430 | 160@0 | 270@0 |
|  |  | 40+40 | CP | 216 | 108@108 | 108@0 | 432 | 216@0 | 216@0 |
|  |  |  | DFT-s | 216 | 108@108 | 108@0 | 432 | 216@0 | 216@0 |
|  |  | 50+30 | CP | 215 | 163@107 | 52@0 | 430 | 270@0 | 160@0 |
|  |  |  | DFT-s | 210 | 162@108 | 48@0 | 430 | 270@0 | 160@0 |
|  | 30 | 30+50 | CP | 210 | 25@53 | 80@0 | 422 | 78@0 | 133@0 |
| 80 |  |  | DFT-s | 210 | 25@53 | 80@0 | 406 | 75@3 | 128@0 |
|  |  | 40+40 | CP | 212 | 53@53 | 53@0 | 424 | 106@0 | 106@0 |
|  |  |  | DFT-s | 208 | 54@52 | 50@0 | 400 | 100@6 | 100@0 |
|  |  | 50+30 | CP | 210 | 80@53 | 25@0 | 422 | 133@0 | 78@0 |
|  |  |  | DFT-s | 210 | 80@53 | 25@0 | 406 | 128@5 | 75@0 |
|  | 15 | 40+50 | CP | 243 | 95@121 | 148@0 | 486 | 216@0 | 270@0 |
| 90 |  |  | DFT-s | 240 | 96@120 | 144@0 | 486 | 216@0 | 270@0 |
|  |  | 50+40 | CP | 243 | 149@121 | 94@0 | 486 | 270@0 | 216@0 |
|  |  |  | DFT-s | 240 | 150@120 | 90@0 | 486 | 270@0 | 216@0 |
|  | 30 | 40+50 | CP | 238 | 46@60 | 73@0 | 478 | 106@0 | 133@0 |
|  |  |  | DFT-s | 234 | 45@61 | 72@0 | 456 | 100@6 | 128@0 |
|  |  | 50+40 | CP | 238 | 73@60 | 46@0 | 478 | 133@0 | 106@0 |
|  |  |  | DFT-s | 234 | 72@61 | 45@0 | 456 | 128@5 | 100@0 |
|  | 15 | 50+50 | CP | 270 | 135@135 | 135@0 | 540 | 270@0 | 270@0 |
|  |  |  | DFT-s | 270 | 135@135 | 135@0 | 540 | 270@0 | 270@0 |
| 100 | 30 | 50+50 | CP | 2644 | 67@66 | 65@0 | 532 | 133@0 | 133@0 |
|  |  |  | DFT-s | 256 | 64@69 | 64@0 | 512 | 128@5 | 128@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 15+90 | CP | 282 | 0@0 | 141@33 | 566 | 38@0 | 245@0 |
|  |  |  | DFT-s | 270 | 0@0 | 135@30 | 558 | 36@2 | 243@0 |
| 105 |  | 25+80 | CP | 2804 | 0@0 | 140@5 | 564 | 65@0 | 217@0 |
|  |  |  | DFT-s | 270 | 0@0 | 135@3 | 560 | 64@1 | 216@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 10+100 | CP | 296 | 0@0 | 148@50 | 594 | 24@0 | 273@0 |
|  |  |  | DFT-s | 288 | 0@0 | 144@48 | 588 | 24@0 | 270@0 |
|  |  | 20+90 | CP | 296 | 0@0 | 148@23 | 592 | 51@0 | 245@0 |
|  |  |  | DFT-s | 288 | 0@0 | 144@21 | 586 | 50@1 | 243@0 |
|  |  | 30+80 | CP | 294 | 4@74 | 143@0 | 590 | 78@0 | 217@0 |
|  |  |  | DFT-s | 294 | 3@74 | 144@0 | 582 | 75@3 | 216@0 |
|  |  | 40+70 | CP | 294 | 32@74 | 115@0 | 590 | 106@0 | 189@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 50+60 | CP | 294 | 59@74 | 88@0 | 590 | 133@0 | 162@0 |
|  |  |  | DFT-s | 282 | 60@73 | 81@0 | 580 | 128@5 | 162@0 |
|  |  | 60+50 | CP | 294 | 88@74 | 59@0 | 590 | 162@0 | 133@0 |
|  |  |  | DFT-s | 282 | 81@81 | 60@0 | 580 | 162@0 | 128@0 |
|  |  | 70+40 | CP | 294 | 115@74 | 32@0 | 590 | 189@0 | 106@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 80+30 | CP | 294 | 143@74 | 4@0 | 590 | 217@0 | 78@0 |
|  |  |  | DFT-s | 292 | 144@73 | 2@0 | 582 | 216@1 | 75@0 |
| 110 |  | 90+20 | CP | 296 | 148@74 | 0@0 | 592 | 245@0 | 51@0 |
|  |  |  | DFT-s | 288 | 144@80 | 0@0 | 586 | 243@2 | 50@0 |
|  |  | 100+10 | CP | 296 | 148@74 | 0@0 | 594 | 273@0 | 24@0 |
|  |  |  | DFT-s | 288 | 144@81 | 0@0 | 588 | 270@3 | 24@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 15+100 | CP | 310 | 0@0 | 155@40 | 622 | 38@0 | 273@0 |
|  |  |  | DFT-s | 300 | 0@0 | 150@37 | 612 | 36@2 | 270@0 |
|  |  | 25+90 | CP | 3084 | 0@0 | 154@12 | 620 | 65@0 | 245@0 |
|  |  |  | DFT-s | 300 | 0@0 | 150@10 | 614 | 64@1 | 243@0 |
|  |  | 90+25 | CP | 3084 | 154@77 | 0@0 | 620 | 245@0 | 65@0 |
| 115 |  |  | DFT-s | 300 | 150@85 | 0@0 | 614 | 243@2 | 64@0 |
|  |  | 100+15 | CP | 310 | 155@78 | 0@0 | 622 | 273@0 | 38@0 |
|  |  |  | DFT-s | 300 | 150@86 | 0@0 | 612 | 270@3 | 36@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 20+100 | CP | 324 | 0@0 | 162@30 | 648 | 51@0 | 273@0 |
|  |  |  | DFT-s | 324 | 0@0 | 162@30 | 640 | 50@1 | 270@0 |
|  |  | 30+90 | CP | 322 | 0@0 | 161@3 | 646 | 78@0 | 245@0 |
|  |  |  | DFT-s | 320 | 0@0 | 160@2 | 636 | 75@3 | 243@0 |
|  |  | 40+80 | CP | 322 | 25@81 | 136@0 | 646 | 106@0 | 217@0 |
|  |  |  | DFT-s | 320 | 25@81 | 135@0 | 632 | 100@6 | 216@0 |
|  |  | 50+70 | CP | 3204 | 53@80 | 107@0 | 644 | 133@0 | 189@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 60+60 | CP | 324 | 81@81 | 81@0 | 648 | 162@0 | 162@0 |
|  |  |  | DFT-s | 324 | 81@81 | 81@0 | 648 | 162@0 | 162@0 |
|  |  | 70+50 | CP | 3204 | 109@80 | 51@0 | 644 | 189@0 | 133@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 80+40 | CP | 322 | 136@81 | 25@0 | 646 | 217@0 | 106@0 |
|  |  |  | DFT-s | 318 | 135@82 | 24@0 | 632 | 216@1 | 100@0 |
|  |  | 90+30 | CP | 322 | 161@81 | 0@0 | 646 | 245@0 | 78@0 |
|  |  |  | DFT-s | 320 | 160@83 | 0@0 | 636 | 243@2 | 75@0 |
| 120 |  | 100+20 | CP | 324 | 162@81 | 0@0 | 648 | 273@0 | 51@0 |
|  |  |  | DFT-s | 324 | 162@81 | 0@0 | 640 | 270@3 | 50@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 25+100 | CP | 3364 | 0@0 | 168@19 | 676 | 65@0 | 273@0 |
|  |  |  | DFT-s | 324 | 0@0 | 162@16 | 668 | 64@1 | 270@0 |
|  |  | 100+25 | CP | 3364 | 168@84 | 0@0 | 676 | 273@0 | 65@0 |
| 125 |  |  | DFT-s | 324 | 162@95 | 0@0 | 668 | 270@3 | 64@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 30+100 | CP | 350 | 0@0 | 175@10 | 702 | 78@0 | 273@0 |
|  |  |  | DFT-s | 324 | 0@0 | 162@3 | 690 | 75@3 | 270@0 |
|  |  | 40+90 | CP | 350 | 18@88 | 157@0 | 702 | 106@0 | 245@0 |
|  |  |  | DFT-s | 340 | 20@86 | 150@0 | 686 | 100@6 | 243@0 |
|  |  | 50+80 | CP | 3484 | 46@87 | 128@0 | 700 | 133@0 | 217@0 |
|  |  |  | DFT-s | 346 | 45@88 | 128@0 | 688 | 128@5 | 216@0 |
|  |  | 60+70 | CP | 350 | 74@88 | 101@0 | 702 | 162@0 | 189@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 70+60 | CP | 350 | 101@88 | 74@0 | 702 | 189@0 | 162@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 80+50 | CP | 3484 | 130@87 | 44@0 | 700 | 217@0 | 133@0 |
|  |  |  | DFT-s | 346 | 128@89 | 45@0 | 688 | 216@1 | 128@0 |
| 130 |  | 90+40 | CP | 350 | 157@88 | 18@0 | 702 | 245@0 | 106@0 |
|  |  |  | DFT-s | 340 | 160@85 | 10@0 | 686 | 243@2 | 100@0 |
|  |  | 100+30 | CP | 350 | 175@88 | 0@0 | 702 | 273@0 | 78@0 |
|  |  |  | DFT-s | 324 | 162@108 | 0@0 | 690 | 270@3 | 75@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 40+100 | CP | 378 | 11@95 | 178@0 | 758 | 106@0 | 273@0 |
|  |  |  | DFT-s | 372 | 6@100 | 180@0 | 740 | 100@6 | 270@0 |
|  |  | 50+90 | CP | 3764 | 39@94 | 149@0 | 756 | 133@0 | 245@0 |
|  |  |  | DFT-s | 372 | 36@97 | 150@0 | 742 | 128@5 | 243@0 |
|  |  | 60+80 | CP | 378 | 67@95 | 122@0 | 758 | 162@0 | 217@0 |
|  |  |  | DFT-s | 368 | 64@98 | 120@0 | 756 | 162@0 | 216@0 |
|  |  | 70+70 | CP | 3764 | 95@94 | 93@0 | 756 | 189@0 | 189@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 80+60 | CP | 378 | 122@95 | 67@0 | 758 | 217@0 | 162@0 |
|  |  |  | DFT-s | 368 | 120@97 | 64@0 | 756 | 216@1 | 162@0 |
|  |  | 90+50 | CP | 3764 | 151@94 | 37@0 | 756 | 245@0 | 133@0 |
| 140 |  |  | DFT-s | 372 | 150@95 | 36@0 | 742 | 243@2 | 128@0 |
|  |  | 100+40 | CP | 378 | 178@95 | 11@0 | 758 | 273@0 | 106@0 |
|  |  |  | DFT-s | 372 | 180@93 | 6@0 | 740 | 270@3 | 100@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 50+100 | CP | 4044 | 32@101 | 170@0 | 812 | 133@0 | 273@0 |
|  |  |  | DFT-s | 388 | 32@101 | 162@0 | 796 | 128@5 | 270@0 |
|  |  | 60+90 | CP | 406 | 60@102 | 143@0 | 814 | 162@0 | 245@0 |
|  |  |  | DFT-s | 396 | 54@108 | 144@0 | 810 | 162@0 | 243@0 |
|  |  | 70+80 | CP | 4044 | 88@101 | 114@0 | 812 | 189@0 | 217@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 80+70 | CP | 4044 | 116@101 | 86@0 | 812 | 217@0 | 189@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 90+60 | CP | 406 | 143@102 | 60@0 | 814 | 245@0 | 162@0 |
|  |  |  | DFT-s | 396 | 144@101 | 54@0 | 810 | 243@2 | 162@0 |
|  |  | 100+50 | CP | 4044 | 172@101 | 30@0 | 812 | 273@0 | 133@0 |
| 150 |  |  | DFT-s | 388 | 162@111 | 32@0 | 796 | 270@3 | 128@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 60+100 | CP | 434 | 53@109 | 164@0 | 0 | @162 | @0 |
|  |  |  | DFT-s | 432 | 54@108 | 162@0 | 864 | 162@0 | 270@0 |
|  |  | 70+90 | CP | 4324 | 81@108 | 135@0 | 0 | @189 | @0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 80+80 | CP | 4324 | 109@108 | 107@0 | 864 | 216@1 | 216@0 |
|  |  |  | DFT-s | 432 | 108@109 | 108@0 | 864 | 216@1 | 216@0 |
|  |  | 90+70 | CP | 4324 | 137@108 | 79@0 | 868 | 245@0 | 189@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
| 160 |  | 100+60 | CP | 434 | 164@109 | 53@0 | 870 | 273@0 | 162@0 |
|  |  |  | DFT-s | 432 | 162@111 | 54@0 | 864 | 270@3 | 162@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 70+100 | CP | 4604 | 74@115 | 156@0 | 924 | 189@0 | 273@0 |
|  |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  |  | 80+90 | CP | 4604 | 102@115 | 128@0 | 924 | 217@0 | 245@0 |
|  |  |  | DFT-s | 456 | 100@117 | 128@0 | 918 | 216@1 | 243@0 |
|  |  | 90+80 | CP | 4604 | 130@115 | 100@0 | 924 | 245@0 | 217@0 |
|  |  |  | DFT-s | 456 | 128@117 | 100@0 | 918 | 243@2 | 216@0 |
|  |  | 100+70 | CP | 4604 | 158@115 | 72@0 | 924 | 273@0 | 189@0 |
| 170 |  |  | DFT-s | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 80+100 | CP | 4884 | 95@122 | 149@0 | 980 | 217@0 | 273@0 |
|  |  |  | DFT-s | 480 | 96@121 | 144@0 | 972 | 216@1 | 270@0 |
|  |  | 90+90 | CP | 4884 | 123@122 | 121@0 | 980 | 245@0 | 245@0 |
|  |  |  | DFT-s | 480 | 120@125 | 120@0 | 972 | 243@2 | 243@0 |
|  |  | 100+80 | CP | 4884 | 151@122 | 93@0 | 980 | 273@0 | 217@0 |
| 180 |  |  | DFT-s | 480 | 150@123 | 90@0 | 972 | 270@3 | 216@0 |

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | OFDM | RB allocation (Inner Full) |  |  | RB allocation (Outer Full) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 90+100 | CP | 516 | 116@129 | 142@0 | 1036 | 245@0 | 273@0 |
|  |  |  | DFT-s | 504 | 108@137 | 144@0 | 1026 | 243@2 | 270@0 |
|  |  | 100+90 | CP | 5164 | 144@129 | 114@0 | 1036 | 273@0 | 245@0 |
| 190 |  |  | DFT-s | 504 | 144@129 | 108@0 | 1026 | 270@3 | 243@0 |
|  | 15 | N/A |  |  |  |  |  |  |  |
|  | 30 | 100+100 | CP | 544 | 137@136 | 135@0 | 1092 | 273@0 | 273@0 |
| 200 |  |  | DFT-s | 540 | 135@138 | 135@0 | 1080 | 270@3 | 270@0 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3 NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0 NOTE 3: RB allocation for configurations where at least one CC is DFT-s-OFDM with CBW=70MHz is FFS NOTE 4: The allocated RB number NRB_alloc is ceil(NRB_agg/2) -1∙2^μ in order to meet Inner RB allocation definition (RBStart,Low ≤ RBStart ≤ RBStart,High) described in subclause 6.2A.2.0. |  |  |  |  |  |  |  |  |  |

Table 6.1A-1b: Common uplink configuration for intra-band contiguous 2UL CA (non-contiguous RB allocation)

| BWchannel_CA (MHz) | SCS for all CC (kHz) | Channel bandwidth combination (MHz) | RB allocation (Inner) |  |  | RB allocation (Outer1) |  |  | RB allocation (Outer2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 | NRB_alloc | CC1 LCRB1@RBStart1 | CC2 LCRB2@RBStart2 |
| 20 | 15 | 5+15 | 30 | 1@25 | 1@29 | 66 | 1@17 | 1@57 | 104 | 1@0 | 1@78 |
|  |  | 10+10 | 34 | 1@35 | 1@16 | 63 | 1@20 | 1@30 | 104 | 1@0 | 1@51 |
|  |  | 15+5 | 37 | 1@52 | 1@9 | 66 | 1@37 | 1@23 | 104 | 1@0 | 1@24 |
| 25 | 15 | 5+20 | 34 | 1@25 | 1@33 | 85 | 1@21 | 1@80 | 131 | 1@0 | 1@106 |
|  |  | 10+15 | 43 | 1@44 | 1@34 | 80 | 1@26 | 1@53 | 131 | 1@0 | 1@78 |
|  |  | 15+10 | 43 | 1@43 | 1@6 | 80 | 1@25 | 1@25 | 131 | 1@0 | 1@51 |
|  |  | 20+5 | 48 | 1@67 | 1@8 | 85 | 1@49 | 1@27 | 131 | 1@0 | 1@24 |
|  | 30 | 10+15 | 40 | 1@21 | 1@16 | 76 | 1@12 | 1@25 | 124 | 1@0 | 1@37 |
|  |  | 15+10 | 40 | 1@21 | 1@2 | 76 | 1@12 | 1@11 | 124 | 1@0 | 1@23 |
| 30 | 15 | 10+20 | 54 | 1@51 | 1@52 | 98 | 1@28 | 1@73 | 158 | 1@0 | 1@105 |
|  |  | 15+15 | 51 | 1@53 | 1@24 | 95 | 1@30 | 1@45 | 158 | 1@0 | 1@78 |
|  |  | 20+10 | 54 | 1@53 | 1@0 | 99 | 1@31 | 1@23 | 158 | 1@0 | 1@51 |
|  | 30 | 10+20 | 48 | 1@24 | 1@23 | 94 | 1@13 | 1@35 | 150 | 1@0 | 1@50 |
|  |  | 15+15 | 46 | 1@25 | 1@9 | 92 | 1@15 | 1@22 | 152 | 1@0 | 1@37 |
|  |  | 20+10 | 46 | 1@29 | 1@0 | 94 | 1@14 | 1@9 | 150 | 1@0 | 1@23 |
| 35 | 15 | 15+20 | 62 | 1@60 | 1@42 | 114 | 1@35 | 1@69 | 185 | 1@0 | 1@105 |
|  |  | 20+15 | 62 | 1@61 | 1@16 | 113 | 1@35 | 1@41 | 185 | 1@0 | 1@78 |
|  | 30 | 15+20 | 58 | 1@29 | 1@19 | 110 | 1@17 | 1@33 | 178 | 1@0 | 1@50 |
|  |  | 20+15 | 58 | 1@30 | 1@7 | 110 | 1@17 | 1@20 | 178 | 1@0 | 1@37 |
| 40 | 15 | 10+30 | 53 | 1@52 | 1@52 | 130 | 1@40 | 1@117 | 212 | 1@0 | 1@159 |
|  |  | 20+20 | 70 | 1@71 | 1@34 | 129 | 1@42 | 1@64 | 212 | 1@0 | 1@105 |
|  |  | 30+10 | 52 | 1@110 | 1@1 | 130 | 1@40 | 1@9 | 212 | 1@0 | 1@51 |
|  | 30 | 10+30 | 66 | 1@34 | 1@42 | 126 | 1@19 | 1@57 | 204 | 1@0 | 1@77 |
|  |  | 20+20 | 64 | 1@34 | 1@14 | 124 | 1@19 | 1@29 | 204 | 1@0 | 1@50 |
|  |  | 30+10 | 46 | 1@56 | 1@0 | 126 | 1@19 | 1@3 | 204 | 1@0 | 1@23 |
| 45 | 15 | 15+30 | 79 | 1@79 | 1@78 | 146 | 1@46 | 1@112 | 239 | 1@0 | 1@159 |
|  | 30 | 15+30 | 74 | 1@38 | 1@36 | 142 | 1@22 | 1@54 | 232 | 1@0 | 1@77 |
| 50 | 15 | 10+40 | 57 | 1@52 | 1@56 | 165 | 1@50 | 1@162 | 268 | 1@0 | 1@215 |
|  |  | 20+30 | 87 | 1@89 | 1@69 | 161 | 1@53 | 1@107 | 266 | 1@0 | 1@159 |
|  |  | 30+20 | 87 | 1@88 | 1@14 | 161 | 1@52 | 1@52 | 266 | 1@0 | 1@105 |
|  |  | 40+10 | 55 | 1@162 | 1@0 | 165 | 1@52 | 1@0 | 268 | 1@0 | 1@51 |
|  | 30 | 10+40 | 50 | 1@24 | 1@24 | 160 | 1@23 | 1@78 | 260 | 1@0 | 1@105 |
|  |  | 20+30 | 82 | 1@43 | 1@32 | 156 | 1@25 | 1@51 | 258 | 1@0 | 1@77 |
|  |  | 30+20 | 82 | 1@43 | 1@5 | 156 | 1@24 | 1@23 | 258 | 1@0 | 1@50 |
|  |  | 40+10 | 48 | 1@83 | 1@0 | 160 | 1@27 | 1@0 | 260 | 1@0 | 1@23 |
| 55 | 15 | 15+40 | 84 | 1@79 | 1@83 | 182 | 1@55 | 1@157 | 295 | 1@0 | 1@215 |
|  | 30 | 15+40 | 78 | 1@38 | 1@38 | 178 | 1@27 | 1@77 | 288 | 1@0 | 1@105 |
| 60 | 15 | 10+50 | 108 | 1@52 | 1@54 | 197 | 1@61 | 1@205 | 322 | 1@0 | 1@269 |
|  |  | 20+40 | 107 | 1@106 | 1@106 | 195 | 1@61 | 1@149 | 322 | 1@0 | 1@215 |
|  |  | 30+30 | 104 | 1@107 | 1@50 | 193 | 1@63 | 1@95 | 320 | 1@0 | 1@159 |
|  |  | 40+20 | 105 | 1@112 | 1@0 | 195 | 1@62 | 1@40 | 322 | 1@0 | 1@105 |
|  |  | 50+10 | 52 | 1@220 | 1@0 | 192 | 1@79 | 1@0 | 322 | 1@0 | 1@51 |
|  | 30 | 10+50 | 48 | 1@24 | 1@23 | 192 | 1@29 | 1@100 | 314 | 1@0 | 1@132 |
|  |  | 20+40 | 100 | 1@51 | 1@49 | 190 | 1@29 | 1@72 | 314 | 1@0 | 1@105 |
|  |  | 30+30 | 98 | 1@52 | 1@22 | 188 | 1@30 | 1@45 | 312 | 1@0 | 1@77 |
|  |  | 40+20 | 96 | 1@59 | 1@0 | 190 | 1@29 | 1@17 | 314 | 1@0 | 1@50 |
|  |  | 50+10 | 44 | 1@112 | 1@0 | 186 | 1@41 | 1@0 | 314 | 1@0 | 1@23 |
| 70 | 15 | 20+50 | 107 | 1@106 | 1@106 | 229 | 1@73 | 1@195 | 376 | 1@0 | 1@269 |
|  |  | 30+40 | 126 | 1@121 | 1@89 | 229 | 1@72 | 1@140 | 376 | 1@0 | 1@215 |
|  |  | 40+30 | 126 | 1@126 | 1@35 | 229 | 1@74 | 1@86 | 376 | 1@0 | 1@159 |
|  |  | 50+20 | 106 | 1@165 | 1@0 | 229 | 1@73 | 1@31 | 376 | 1@0 | 1@105 |
|  | 30 | 20+50 | 100 | 1@51 | 1@49 | 224 | 1@35 | 1@95 | 368 | 1@0 | 1@132 |
|  |  | 30+40 | 122 | 1@61 | 1@43 | 224 | 1@34 | 1@67 | 368 | 1@0 | 1@105 |
|  |  | 40+30 | 122 | 1@62 | 1@16 | 224 | 1@35 | 1@40 | 368 | 1@0 | 1@77 |
|  |  | 50+20 | 98 | 1@85 | 1@0 | 224 | 1@35 | 1@13 | 368 | 1@0 | 1@50 |
| 80 | 15 | 30+50 | 143 | 1@143 | 1@125 | 261 | 1@83 | 1@183 | 430 | 1@0 | 1@269 |
|  |  | 40+40 | 142 | 1@144 | 1@69 | 260 | 1@84 | 1@127 | 432 | 1@0 | 1@215 |
|  |  | 50+30 | 143 | 1@144 | 1@16 | 261 | 1@84 | 1@74 | 430 | 1@0 | 1@159 |
|  | 30 | 30+50 | 138 | 1@70 | 1@60 | 256 | 1@40 | 1@89 | 422 | 1@0 | 1@132 |
|  |  | 40+40 | 136 | 1@70 | 1@31 | 256 | 1@41 | 1@62 | 424 | 1@0 | 1@105 |
|  |  | 50+30 | 138 | 1@71 | 1@6 | 256 | 1@40 | 1@34 | 422 | 1@0 | 1@77 |
| 90 | 15 | 40+50 | 161 | 1@163 | 1@107 | 294 | 1@96 | 1@173 | 486 | 1@0 | 1@269 |
|  |  | 50+40 | 161 | 1@162 | 1@52 | 294 | 1@95 | 1@118 | 486 | 1@0 | 1@215 |
|  | 30 | 40+50 | 156 | 1@80 | 1@51 | 290 | 1@47 | 1@85 | 478 | 1@0 | 1@132 |
|  |  | 50+40 | 156 | 1@80 | 1@24 | 290 | 1@47 | 1@58 | 478 | 1@0 | 1@105 |
| 100 | 15 | 50+50 | 178 | 1@180 | 1@87 | 326 | 1@106 | 1@161 | 540 | 1@0 | 1@269 |
|  | 30 | 20+80 | 98 | 1@51 | 1@48 | 324 | 1@53 | 1@163 | 536 | 1@0 | 1@216 |
|  |  | 50+50 | 172 | 1@88 | 1@40 | 322 | 1@52 | 1@79 | 532 | 1@0 | 1@132 |
|  |  | 80+20 | 94 | 1@171 | 1@0 | 320 | 1@58 | 1@0 | 536 | 1@0 | 1@50 |
| 105 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+90 | 76 | 1@38 | 1@37 | 344 | 1@55 | 1@188 | 566 | 1@0 | 1@244 |
|  |  | 25+80 | 126 | 1@64 | 1@61 | 340 | 1@54 | 1@158 | 564 | 1@0 | 1@216 |
| 110 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 10+100 | 48 | 1@24 | 1@23 | 360 | 1@58 | 1@213 | 594 | 1@0 | 1@272 |
|  |  | 20+90 | 96 | 1@50 | 1@46 | 356 | 1@57 | 1@183 | 592 | 1@0 | 1@244 |
|  |  | 30+80 | 154 | 1@78 | 1@76 | 358 | 1@58 | 1@158 | 590 | 1@0 | 1@216 |
|  |  | 40+70 | 194 | 1@99 | 1@89 | 358 | 1@59 | 1@131 | 590 | 1@0 | 1@188 |
|  |  | 50+60 | 192 | 1@98 | 1@60 | 356 | 1@58 | 1@102 | 590 | 1@0 | 1@161 |
|  |  | 60+50 | 192 | 1@99 | 1@32 | 356 | 1@59 | 1@74 | 590 | 1@0 | 1@132 |
|  |  | 70+40 | 194 | 1@98 | 1@5 | 358 | 1@58 | 1@47 | 590 | 1@0 | 1@105 |
|  |  | 80+30 | 152 | 1@142 | 1@0 | 358 | 1@59 | 1@20 | 590 | 1@0 | 1@77 |
|  |  | 90+20 | 95 | 1@199 | 1@0 | 348 | 1@72 | 1@0 | 592 | 1@0 | 1@50 |
|  |  | 100+10 | 52 | 1@248 | 1@0 | 324 | 1@112 | 1@0 | 594 | 1@0 | 1@23 |
| 115 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 15+100 | 76 | 1@38 | 1@37 | 376 | 1@60 | 1@209 | 622 | 1@0 | 1@272 |
|  |  | 25+90 | 127 | 1@65 | 1@62 | 374 | 1@61 | 1@182 | 620 | 1@0 | 1@244 |
|  |  | 90+25 | 122 | 1@185 | 1@0 | 374 | 1@61 | 1@2 | 620 | 1@0 | 1@64 |
|  |  | 100+15 | 74 | 1@237 | 1@0 | 354 | 1@97 | 1@0 | 622 | 1@0 | 1@37 |
| 120 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 20+100 | 98 | 1@50 | 1@47 | 392 | 1@64 | 1@208 | 648 | 1@0 | 1@272 |
|  |  | 30+90 | 154 | 1@78 | 1@76 | 390 | 1@63 | 1@179 | 646 | 1@0 | 1@244 |
|  |  | 40+80 | 208 | 1@106 | 1@103 | 390 | 1@63 | 1@151 | 646 | 1@0 | 1@216 |
|  |  | 50+70 | 210 | 1@107 | 1@78 | 390 | 1@64 | 1@125 | 644 | 1@0 | 1@188 |
|  |  | 60+60 | 210 | 1@107 | 1@49 | 390 | 1@64 | 1@96 | 648 | 1@0 | 1@161 |
|  |  | 70+50 | 210 | 1@107 | 1@22 | 390 | 1@64 | 1@69 | 644 | 1@0 | 1@132 |
|  |  | 80+40 | 206 | 1@115 | 1@0 | 390 | 1@63 | 1@40 | 646 | 1@0 | 1@105 |
|  |  | 90+30 | 152 | 1@170 | 1@0 | 390 | 1@63 | 1@12 | 646 | 1@0 | 1@77 |
|  |  | 100+20 | 94 | 1@227 | 1@0 | 378 | 1@85 | 1@0 | 648 | 1@0 | 1@50 |
|  | 15 | N/A |  |  |  |  |  |  |  |  |  |
| 125 | 30 | 25+100 | 126 | 1@65 | 1@62 | 406 | 1@66 | 1@203 | 676 | 1@0 | 1@272 |
|  |  | 100+25 | 122 | 1@213 | 1@0 | 404 | 1@72 | 1@0 | 676 | 1@0 | 1@64 |
| 130 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 30+100 | 152 | 1@78 | 1@75 | 424 | 1@70 | 1@203 | 702 | 1@0 | 1@272 |
|  |  | 40+90 | 208 | 1@106 | 1@103 | 424 | 1@70 | 1@175 | 702 | 1@0 | 1@244 |
|  |  | 50+80 | 228 | 1@116 | 1@96 | 422 | 1@68 | 1@145 | 700 | 1@0 | 1@216 |
|  |  | 60+70 | 230 | 1@117 | 1@69 | 424 | 1@70 | 1@119 | 702 | 1@0 | 1@188 |
|  |  | 70+60 | 230 | 1@117 | 1@42 | 424 | 1@70 | 1@92 | 702 | 1@0 | 1@161 |
|  |  | 80+50 | 228 | 1@116 | 1@12 | 422 | 1@68 | 1@61 | 700 | 1@0 | 1@132 |
|  |  | 90+40 | 206 | 1@143 | 1@0 | 424 | 1@69 | 1@35 | 702 | 1@0 | 1@105 |
|  |  | 100+30 | 150 | 1@199 | 1@0 | 424 | 1@70 | 1@8 | 702 | 1@0 | 1@77 |
| 140 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 40+100 | 210 | 1@106 | 1@104 | 458 | 1@75 | 1@197 | 758 | 1@0 | 1@272 |
|  |  | 50+90 | 248 | 1@125 | 1@115 | 456 | 1@73 | 1@167 | 756 | 1@0 | 1@244 |
|  |  | 60+80 | 250 | 1@127 | 1@89 | 456 | 1@75 | 1@140 | 758 | 1@0 | 1@216 |
|  |  | 70+70 | 246 | 1@125 | 1@58 | 456 | 1@75 | 1@113 | 756 | 1@0 | 1@188 |
|  |  | 80+60 | 248 | 1@126 | 1@32 | 456 | 1@74 | 1@84 | 758 | 1@0 | 1@161 |
|  |  | 90+50 | 248 | 1@125 | 1@3 | 456 | 1@73 | 1@55 | 756 | 1@0 | 1@132 |
|  |  | 100+40 | 208 | 1@170 | 1@0 | 458 | 1@74 | 1@29 | 758 | 1@0 | 1@105 |
| 150 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 50+100 | 262 | 1@133 | 1@130 | 490 | 1@80 | 1@191 | 812 | 1@0 | 1@272 |
|  |  | 60+90 | 268 | 1@136 | 1@107 | 490 | 1@80 | 1@162 | 814 | 1@0 | 1@244 |
|  |  | 70+80 | 266 | 1@135 | 1@78 | 490 | 1@80 | 1@135 | 812 | 1@0 | 1@216 |
|  |  | 80+70 | 266 | 1@135 | 1@50 | 490 | 1@80 | 1@107 | 812 | 1@0 | 1@188 |
|  |  | 90+60 | 268 | 1@135 | 1@23 | 490 | 1@79 | 1@78 | 814 | 1@0 | 1@161 |
|  |  | 100+50 | 262 | 1@143 | 1@0 | 490 | 1@80 | 1@51 | 812 | 1@0 | 1@132 |
| 160 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 60+100 | 288 | 1@145 | 1@125 | 524 | 1@86 | 1@185 | 870 | 1@0 | 1@272 |
|  |  | 70+90 | 286 | 1@145 | 1@98 | 522 | 1@85 | 1@156 | 868 | 1@0 | 1@244 |
|  |  | 80+80 | 284 | 1@144 | 1@68 | 522 | 1@85 | 1@128 | 868 | 1@0 | 1@216 |
|  |  | 90+70 | 286 | 1@145 | 1@42 | 522 | 1@85 | 1@100 | 868 | 1@0 | 1@188 |
|  |  | 100+60 | 288 | 1@145 | 1@15 | 524 | 1@86 | 1@74 | 870 | 1@0 | 1@161 |
| 170 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 70+100 | 304 | 1@153 | 1@115 | 558 | 1@92 | 1@181 | 924 | 1@0 | 1@272 |
|  |  | 80+90 | 304 | 1@154 | 1@88 | 556 | 1@91 | 1@151 | 924 | 1@0 | 1@244 |
|  |  | 90+80 | 304 | 1@154 | 1@60 | 556 | 1@91 | 1@123 | 924 | 1@0 | 1@216 |
|  |  | 100+70 | 304 | 1@153 | 1@31 | 558 | 1@92 | 1@97 | 924 | 1@0 | 1@188 |
| 180 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 80+100 | 322 | 1@163 | 1@106 | 590 | 1@96 | 1@173 | 980 | 1@0 | 1@272 |
|  |  | 90+90 | 322 | 1@164 | 1@79 | 588 | 1@96 | 1@144 | 980 | 1@0 | 1@244 |
|  |  | 100+80 | 324 | 1@164 | 1@52 | 590 | 1@96 | 1@117 | 980 | 1@0 | 1@216 |
| 190 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 90+100 | 342 | 1@173 | 1@98 | 624 | 1@103 | 1@169 | 1036 | 1@0 | 1@272 |
|  |  | 100+90 | 342 | 1@173 | 1@70 | 624 | 1@103 | 1@141 | 1036 | 1@0 | 1@244 |
| 200 | 15 | N/A |  |  |  |  |  |  |  |  |  |
|  | 30 | 100+100 | 360 | 1@182 | 1@88 | 656 | 1@108 | 1@162 | 1092 | 1@0 | 1@272 |
| NOTE 1: BWchannel_CA is defined in clause 5.3A.3 NOTE 2: NRB_alloc for intra-band contiguous UL CA is defined in 6.2A.2.0 |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2A.3	UE additional maximum output power reduction for CA

6.2A.3.0	Minimum conformance requirements

6.2A.3.0.1	UE additional maximum output power reduction for Intra-band contiguous CA

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6]. Relation between NR CA band and NR frequency band is specified in Table 5.2A.1-1.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2A.1.0.4-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2.0.4. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

Table 6.2A.3.0.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable CA band(s) for each CA_NS value. The CA_NS_xy value indicates the additional unwanted emissions requirements that apply for intra-band contiguous CA bands with NS_xy indicated or configured in multiple uplink serving cells, except CA_NS_01 that indicates the general emission requirements for intra-band contiguous CA bands. The mapping of NR CA band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2A.3.0.1-2. For any NR CA band not listed in Table 6.2A.3.0.1-2 the network signalling label CA_NS_01 applies.

Table 6.2A.3.0.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR CA Band | Aggregated channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_01 | 6.5A.2.2.1 6.5A.3.2.1 | Table 5.2A.1-1 | All applicable NR CA bands | All applicable NR CA configurations | N/A |
| CA_NS_04 | 6.5A.2.3.1.1 6.5A.3.3.1.1 | CA_n41 | Table 5.5A.1-1 | 6.2A.3.1.1.1 | 6.2A.3.1.1.1 |
| CA_NS_27 | 6.5A.2.3.1.2 6.5A.3.3.1.2 | CA_n48 | Table 5.5A.1-1 | 6.2A.3.1.1.2 | 6.2A.3.1.1.2 |
| CA_NS_46 | 6.5A.3.3.1.3 | CA_n7 | Table 5.5A.1-1 | 6.2A.3.1.1.3 | 6.2A.3.1.1.3 |
| CA_NS_55 | See CA_NS_01 | CA_n77 | Table 5.5A.1-1 | 6.2A.2.1 | See CA_NS_01 |

For UEs configured with intra-band contiguous CA in n77 and if NS_01 is indicated for an uplink component carrier in the range 3450-3650 MHz and NS_01 or NS_57 for another uplink component carrier in the range 3650-3980 MHz, the allowed additional spurious emission and maximum output power reduction requirements are according to CA_NS_01.

Table 6.2A.3.0.1-2: Mapping of network signalling label

| NR CA band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| CA_n41 | CA_NS_01 | CA_NS_04 |  |  |  |  |  |  |
| CA_n48 | CA_NS_01 | CA_NS_27 |  |  |  |  |  |  |
| CA_n7 | CA_NS_01 | CA_NS_46 |  |  |  |  |  |  |
| CA_n77 | CA_NS_01 | CA_NS_55 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

6.2A.3.0.1.1	A-MPR for CA_NS_04

6.2A.3.0.1.1.1	Contiguous allocations

For all waveform type, modulations and scs when Fedge, low - BWChannel_CA ≥ 2490.5 MHz, A-MPR = MPR

For all modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

if the RB allocation is an inner allocation as defined in clause 6.2A.2.0.4, then A-MPR = MPR

Except for RBstart ≤ 0.33*BWchannel_CA/0.18MHz, AMPR= max (MPR, AMPRcc).

if the RB allocation is an outer allocation as defined in clause 6.2A.2.0.4,

then A-MPR = MPR+1.5dB for BW Class B A-MPR = MPR for BW class C.

Where

-	MPR is the MPR as defined in Table 6.2A.2.0.4-1, Table 6.2A.2.0.4-1a and Table 6.2A.2.0.4-1b for PC3 and PC2 respectively and the respective CA bandwidth class

-	AMPRcc is defined as the PC3_A2 or PC2_A4 AMPR in table 6.2.3.3.2-2 for PC3 and PC2 respectively.

6.2A.3.0.1.1.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.0.1.1-1 or Table 6.5A.3.3.0.1.1-1. And uses MPR for all other cases.

The UE determines the AMPR type as follows:

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA ≥ 2490.5 MHz,

if allocation is an inner or outer 1 allocation as defined in Table 6.2A.2.0.4-2 then A-MPR = MPR

if allocation is an outer 2 allocation as defined in Table 6.2A.2.0.4-2 then A-MPR = MPR+1dB

For all waveform types, modulations and SCS when Fedge, low - BWChannel_CA <  2490.5 MHz

If AND( MIN(FIM3,low_block,high, SEM-13,low) < Ffilter,low ,  MAX( SEM-13,high, FIM3,high_block,low ) > Ffilter,high )

if RB allocation is an inner or outer 1 allocation as defined in Table 6.2A.2.0.4-2 then A-MPR = MPR

if RB allocation is an outer 2 allocation as defined in Table6.2A.2.0.4-2 then A-MPR = MPR+1dB

Else

A-MPR = A-MPRIM3 defined in Clause 6.2A.3.0.1.1.3.

where

-	MPR is the MPR as defined in Table 6.2A.2.0.4-2, Table 6.2A.2.0.4-3 and Table 6.2A.2.0.4-4 for PC3 and PC2 respectively and the respective CA bandwidth class

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-	FIM3,high_block,low = (2 * Fhigh_alloc,low_edge) – Flow_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-	Ffilter,high = 2745 MHz

-	SEM-13,high = Threshold frequency where upper spectral emission mask for upper channel drops from -13 dBm / 1MHz to -25 dBm / 1MHz, as specified in Clause 6.5A.2.3.0.1.1.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.3.0.1.1.

6.2A.3.0.1.1.3	AMPRIM3 to meet -25dBm/MHz

AMPR in this clause is for intra-band contiguous CA_n41B and CA_n41C. The allowed maximum output power reduction is defined as:

AMPRIM3=MA, Where MA is defined as follows

MA =	13;	0 ≤ B < 2.16

11.5;	2.16 ≤ B < 3.24

10.5;	3.24 ≤ B < 5.04

9.5;	5.04 ≤ B < 10.08

8;	10.08 ≤ B < 16.56

7;	16.56 ≤ B < 21.96

6;	21.96 ≤ B

Where:

B=(LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000

and LCRB1, SCS1 are for CC1, LCRB2, SCS2 are for CC2, CC1 is the component carrier with lower frequency.

The normative reference for this requirement is TS 38.101-1 [2] subclause 6.2A.3.1

6.2A.3.0.2	FFS

6.2A.3.0.3	UE additional maximum output power reduction for inter-band CA

Unless otherwise stated, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in subclause 6.2.3 apply for each uplink component carrier.

Unless specified in Table 6.2A.3.0.3-1, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply only to the indicated carrier. The requirements in Table 6.2A.3.0.3-1 are specified in terms of an additional spectrum emission requirement with their associated network signalling values and the allowed A-MPR. Unless otherwise stated, the combined requirements and allowed A-MPR are applicable on both bands when both component carriers are active. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet the additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 tables 6.2.2-1.

The emission requirements specified in Table 6.2A.3.0.3-1 also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.2A.3.0.3-1: Additional Requirements for uplink inter-band carrier aggregation (two-bands)

| 4 | Band | Applied NS | Requirements (clause) | A-MPR (table/clause) | Note |
| --- | --- | --- | --- | --- | --- |
|  | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 |  |
| CA_n1-n3 |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 | 1 |
|  | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 |  |
|  | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 |  |
| CA_n1-n8 |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 | 1 |
|  | n8 | 43 | 6.5.3.3.5 | Clause 6.2.3.6 |  |
|  |  | 43U | 6.5.3.3.5, 6.5.2.4.2 | Clause 6.2.3.6 |  |
| CA_n1-n78 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 | 1 |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 |  |
| CA_n1-n79 | n1 | 05 | 6.5.3.3.4 | Clause 6.2.3.4 | 1 |
|  |  | 05U | 6.5.3.3.4, 6.5.2.4.2 | Clause 6.2.3.4 |  |
| CA_n3-n41 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
|  | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n3-n78 | n3 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n5-n77 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n5-n78 | n5 | 100 | 6.5.2.4.2 | Table 6.2.3.1-2 | 1 |
| CA_n8-n78 | n8 | 43 | 6.5.3.3.5 | Clause 6.2.3.6 | 1 |
|  |  | 43U | 6.5.3.3.5, 6.5.2.4.2 | Clause 6.2.3.6 |  |
| CA_n24-n41 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n41 | 04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | Clause 6.2.3.3.2 |  |
| CA_n24-n48 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n48 | 27 | 6.5.2.3.3.8, 6.5.3.3.3.14 | Clause 6.2.3.3.16 |  |
| CA_n24-n77 | n24 | 56 | 6.5.3.3.3.27 | Clause 6.2.3.3.30 |  |
|  | n77 | 55 | NOTE 6 | N/A |  |
| CA_n28-n41 | n28 | 17 | 6.5.3.3.2 | N/A | 2 |
|  | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| CA_n28-n78 | n28 | 17 | 6.5.3.3.2 | N/A | 2 |
| CA_n28-n79 | n28 | 17 | 6.5.3.3.2 | N/A | 2 |
| CA_n41-n79 | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| NOTE 1: NS_05U, NS_43U and NS_100 can be signalled for NR bands that have UTRA services deployed and the requirements in clause 6.5.2.4.2 are only applicable to the signalling carrier. NOTE 2: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 3: TBD NOTE 4: TBD NOTE 5: TBD NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. |  |  |  |  |  |

6.2A.3.1	UE additional maximum output power reduction for CA (2UL CA)

Editor’s note: This clause is complete for AMPR testing. But the following aspects are either missing for not yet determined:

- For intra-band non-contiguous CA, tests for network signalling value other than NS_04 are not complete.

6.2A.3.1.1	Test purpose

Additional emission requirements for CA can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the CA maximum output power as specified in Table 6.2A.1.0.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2A.2. Outer and inner allocation notation used in clause 6.2A.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2A.3.1.2	Test applicability

The requirements of this test apply in test case 6.5A.2.3 Additional Spectrum Emission mask for CA for network signalling values CA_NS_04 to all types of NR UE release 15 and forward.

The requirements of this test apply in test case 6.5A.3.3 Additional Spurious Emissions for CA for network signalling values CA_NS_04 to all types of NR UE release 15 and forward.

6.2A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in 6.2A.3.0.

6.2A.3.1.4	Test description

6.2A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state. For the UE maximum output power modified by A-MPR specified in table 6.2A.3.0.3-1, the power limits specified in subclause 6.2A.4.1.3 apply.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing and are shown in table 6.2A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.2A.3.1.4.1-1: Test Configuration Table for inter-band CA (network signalling value NS_47/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] clause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] clause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range for PCC and SCC PCC specified in Table 6.2.3.4.1-18a and 6.2.3.4.1-18b |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] clause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest,Highest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL | UL configuration |  |  |  |  |  |  |
|  | configuration | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 | N/A for A-MPR | Default | 30 | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | testing | Default | 30 | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | Default | 30 | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.4.1-2: Test Configuration Table for inter-band CA (network signalling value NS_100/NS_01)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest NRB_agg for PCC and SCC Highest NRB_agg for PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest,Highest |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Inner Full |
| 2 |  | CP-OFDM 256QAM | CP-OFDM 256QAM |  | Outer Full | Outer Full |
| 3 |  | DFT-s-OFDM QPSK | CP-OFDM 256QAM |  | Inner Full | Outer Full |

Table 6.2A.3.1.4.1-3: Test Configuration Table for inter-band CA (network signalling value NS_43/NS_01, NS_43U/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range, High range for PCC and SCC unless otherwise stated in Fc column for PCC |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg, Highest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 |  | 910 | 10MHz | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | N/A | 907.5 | 15MHz | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | 907.5 | 15MHz | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |

Table 6.2A.3.1.4.1-4: Test Configuration Table for inter-band CA (network signalling value NS_56/NS_04)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  | SCS (15 kHz) | SCS (60 kHz) |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 10 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 10 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 10 | DFT-s-OFDM QPSK | 20@20 | 4@4 |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 100 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 100 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |

Table 6.2A.3.1.4.1-5: Test Configuration Table for inter-band CA (network signalling value NS_56/NS_27)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  | Low SCS | High SCS |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 15 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 15 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 15 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 40 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 40 | CP-OFDM 256QAM | Edge 1RB Left | Edge 1RB Left |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 40 | DFT-s-OFDM QPSK | 15@10 | 5@3 |

Table 6.2A.3.1.4.1-6: Test Configuration Table for inter-band CA (network signalling value NS_56/NS_55)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  |  | Low range for PCC and Low and High range for SCC as specified in test parameters |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest and Highest for PCC as specified in test parameters Lowest and Highest for SCC as specified in test parameters |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest as specified in test parameters |  |  |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |  |  |  |
|  |  | FC (MHz) | SCS | Ch BW (MHz) | Modulation | RB allocation | FC (MHz) | Ch BW (MHz) | Modulation | RB allocation |  |
|  |  |  |  |  |  |  |  |  |  | SCS (15 kHz) | SCS (60 kHz) |
| 1 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 2 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 10 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 3 |  | Low | 15 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 4 | N/A | Low | 30 | 5 | DFT-s-OFDM QPSK | Inner 1RB Right | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 5 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 10 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 6 |  | Low | 30 | 5 | DFT-s-OFDM QPSK | Edge 1RB Left | Low | 10 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 7 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | 18@20 | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 8 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 100 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 9 |  | Low | 15 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 10 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | 4@5 | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| 11 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 100 | CP-OFDM 256QAM | Outer Full | Outer Full |
| 12 |  | Low | 60 | 10 | DFT-s-OFDM QPSK | Edge 1RB Left | High | 100 | DFT-s-OFDM QPSK | Inner Full | Inner Full |

Table 6.2A.3.1.4.1-7: Test Configuration Table for intra-band contiguous CA for CA_NS_04 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  | Refering to Test Frequency Column |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest NRB_agg Highest NRB_agg |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |
| Test ID | Test Frequency | Downlink Configuration | Uplink Configuration |  |
|  |  |  | Modulation | RB allocation for PCC&SCC (NOTE 1) |
| 1 | High range |  | CP-OFDM QPSK | Outer_Full |
| 2 | High range |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Low range |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low range |  | CP-OFDM QPSK | Edge_1RB_Left |
| 5 | Low range |  | DFT-s-OFDM Pi/2 BPSK | Inner_Full |
| 6 | Low range |  | DFT-s-OFDM Pi/2 QPSK | Inner_Full |
| 7 | Low range |  | DFT-s-OFDM Pi/2 16QAM | Inner_Full |
| 8 | Low range |  | DFT-s-OFDM Pi/2 64QAM | Inner_Full |
| 9 | Low range |  | DFT-s-OFDM Pi/2 256QAM | Inner_Full |
| 10 | Low range |  | CP-OFDM QPSK | Inner_Full |
| 11 | Low range |  | CP-OFDM 16QAM | Inner_Full |
| 12 | Low range |  | CP-OFDM 64QAM | Inner_Full |
| 13 | Low range |  | CP-OFDM 256QAM | Inner_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1A-1a. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A3-1. |  |  |  |  |

Table 6.2A.3.1.4.1-8: Test Configuration Table for inter-band CA (network signalling value NS_17/NS_01)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  |  | Low range, High range for PCC and SCC unless otherwise stated in Fc column for PCC |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  | See Ch BW column for PCC Lowest NRB_agg, Highest NRB_agg for SCC |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 15kHz |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |  |
| Test ID | DL configuration | UL configuration |  |  |  |  |  |  |
|  |  | PCC |  |  |  |  | SCC |  |
|  |  | FC (MHz) | Ch BW (MHz) | Modulation |  | RB allocation | Modulation | RB allocation |
| 1 |  | 745.5 | 5 | DFT-s-OFDM QPSK |  | Inner Full | DFT-s-OFDM QPSK | Inner Full |
| 2 | N/A | 743 | 10 | CP-OFDM 256QAM |  | Outer Full | CP-OFDM 256QAM | Outer Full |
| 3 |  | 743 | 10 | CP-OFDM 256QAM |  | Outer Full | DFT-s-OFDM QPSK | Inner Full |

Table 6.2A.3.1.4.1-9: Test Configuration Table for PC3 inter-band CA or PC2 inter-band CA with both bands supporting PC3 (network signalling value NS_17/NS_47)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Mid range for PCC Test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz and 10MHz for PCC 30 MHz for SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest as specified in test parameters |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Edge_1RB_Right |
| 2 |  | CP-OFDM 256QAM | DFT-s-OFDM QPSK |  | Outer Full | Edge_1RB_Left |
| 3 |  | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Edge_1RB_Left |

Table 6.2A.3.1.4.1-9a: Test Configuration Table for PC2 inter-band CA with PCC supporting PC3 and SCC supporting PC2 (network signalling value NS_17/NS_47)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Mid range for PCC Test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz and 10MHz for PCC 30 MHz for SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest as specified in test parameters |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM PI/2 BPSK |  | Inner Full | Edge_1RB_Right |
| 2 |  | CP-OFDM 256QAM | DFT-s-OFDM PI/2 BPSK |  | Outer Full | Edge_1RB_Left |
| 3 |  | DFT-s-OFDM QPSK | DFT-s-OFDM PI/2 BPSK |  | Inner Full | Edge_1RB_Left |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2A.3.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2A.3.1.4.3.

6.2A.3.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.2A.3.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause 9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to tables in 6.2A.3.1.4.1 as appropriate on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level corresponding to Power Class 3.

6.	Measure the sum of mean transmitted power over all component carriers in the CA configuration of the radio access mode. The period of measurement shall be at least the continuous duration of 1ms uplink. For TDD only slots consisting of only UL symbols are under test. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test. For power class 2 intra-band contiguous carrier aggregation, the additional spectrum emission mask is measured as the sum from both UE transmit antenna connectors when UE indicates support for dualPA-Architecture IE.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration tables in 6.2A.3.1.4.1 send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2A.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

6.2A.3.1.4.3.1	Message contents exceptions (network signalling value " NS_100" on PCC)

Table 6.2A.3.1.4.3.1-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_100" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_100) |  | not for band n65 |
|  | 2 (NS_100) |  | for band n65 |

6.2A.3.1.4.3.2	Message contents exceptions (network signalling value " NS_43" on PCC)

Table 6.2A.3.1.4.3.2-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_43) |  |  |

6.2A.3.1.4.3.3	Message contents exceptions (network signalling value " NS_43U" on PCC)

Table 6.2A.3.1.4.3.3-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_43U" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 3 (NS_43U) |  |  |

6.2A.3.1.4.3.4	Message contents exceptions (network signalling value "NS_47" on PCC)

Table 6.2A.3.1.4.3.4-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_47) |  |  |

6.2A.3.1.4.3.5	Message contents exceptions (network signalling value "NS_56" on PCC)

Table 6.2A.3.1.4.3.5-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_56) |  |  |

6.2A.3.1.4.3.6	Message contents exceptions (network signalling value "CA_NS_04" on PCC and SCC)

Table 6.2A.3.1.4.3.6-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "CA_NS_04" on PCC and SCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (CA_NS_04) |  |  |

Table 6.2A.3.1.4.3.6-2: FrequencyInfoUL-SIB for intra-band contiguous CA (contiguous RB allocation) for CA_NS_04

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 16 |  | Power class 3 and Test IDs 5 |
|  | 15 |  | Power class 3 and Test IDs 6, 7 |
|  | 14 |  | Power class 3 and Test IDs 2, 4, 8 |
|  | 12 |  | Power class 3 and Test IDs 10, 11, 12 |
|  | 11 |  | Power class 3 and Test IDs 9 |
|  | 10 |  | Power class 3 and Test IDs 1, 3, 13 |
|  | 19 |  | Power class 2 and Test IDs 5 |
|  | 19 |  | Power class 2 and Test IDs 2, 4 |
|  | 17 |  | Power class 2 and Test IDs 6, 7 |
|  | 16 |  | Power class 2 and Test IDs 8 |
|  | 14 |  | Power class 2 and Test IDs 10, 11, 12 |
|  | 14 |  | Power class 2 and Test IDs 1, 3, 9 |
|  | 11 |  | Power class 2 and Test IDs 13 |

6.2A.3.1.4.3.7	Message contents exceptions (network signalling value "NS_17" on PCC)

Table 6.2A.3.1.4.3.7-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_17" on PCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 1 (NS_17) |  |  |

6.2A.3.1.4.3.8	Message contents exceptions (network signalling value "NS_47" on SCC)

Table 6.2A.3.1.4.3.8-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_47" on SCC

| Derivation Path: TS 38.508-1 [5] clause 4.6.3, Table 4.6.3-1 AdditionalSpectrumEmission |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 2 (NS_47) |  |  |

6.2A.3.1.5	Test requirement

The maximum output power, derived in step 6 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.3.1.5-1, Table 6.2A.3.1.5-2, Table 6.2A.3.1.5-3 as appropriate. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in 6.2A.4.0 apply. The test requirement of configurations for CA operating band including Band n41 also apply for the corresponding CA operating bands with Band n90 replacing Band n41.

Table 6.2A.3.1.5-0: Test Tolerance (Additional Maximum Power Reduction (A-MPR))

| TT for overall output power (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PCell |  |  |  |  |  |
|  |  |  | BW ≤ 40MHz |  |  | 40MHz < BW ≤ 100MHz |  |  |
|  |  |  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| SCell | BW ≤ 40MHz | f ≤ 3.0GHz | 0.7 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  | 40MHz < BW ≤ 100MHz | f ≤ 3.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 3.0GHz < f ≤ 4.2GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |
|  |  | 4.2GHz < f ≤ 6.0GHz | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 | 1.0 |

Table 6.2A.3.1.5-1: UE Power Class 3 test requirement (network signalling value NS_100/NS_01) for CA_n3-n78 and CA_n8-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 25+TT | 14.5-TT |
| 3 | 23 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23 (22.72) | 3 (52) | 25+TT | 20-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1a: UE Power Class 2 with both bands supporting PC3 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26 (24.52) | 3 | 28+TT | 23-TT (21.5-TT2) |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 19.5 | 5 | 28+TT | 14.5-TT |
| 3 | 26 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 23.9 (22.72) | 3 (52) | 28+TT | 20.9-TT (17.7-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1b: UE Power Class 2 with PCC supporting PC3 and SCC supporting PC2 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 26 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 28+TT | 16.3-TT |
| 3 | 26 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 24.6 (23.62) | 3 (52) | 28+TT | 21.6TT (20.6-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1c: UE Power Class 2 with supporting higherPowerLimit-r17 test requirement (network signalling value NS_100) for CA_n3-n78

| Test ID | PPowerclass(dBm) | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCC | SCC | PCC | SCC | PCC | SCC |  |  |  |  |  |
| 1 | 27.8 | 0 | 0 | 0 | 0 | 0 (1.52) | 0 (1.52) | 3 | 27.8 (26.32) | 3 | 29.8+TT | 24.8TT (23.3-TT2) |
| 2 | 27.8 | 6.5 | 6.5 | 6.5 | 0 | 0 | 0 | 3 | 21.3 | 5 | 29.8+TT | 16.3-TT |
| 3 | 27.8 | 0 | 6.5 | 0 | 0 | 0 (1.52) | 0 | 3 | 24.6 (23.62) | 3 | 29.8+TT | 21.6-TT (20.6+-TT2) |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: For transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: UE Power Class 3 test requirement (network signalling value NS_43/NS_01, NS_43U/NS_01) for CA_n8-n78

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 9.0 | 0 | 0 | 0 | 3 | 18.4 | 5 | 25+TT | 13.4-TT |
| 3 | 23 | 6.5 | 0 | 9.0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| NOTE 1: PPowerClass is the maximum UE power specified in 6.2A.1 without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: UE Power Class 3 test requirement (network signalling value NS_47/NS_01) for CA_n41-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 7 | 0 | 0 | 0 | 3 | 9.8 | 7 | 25+TT | 2.8-TT |
| 3 | 23 | 6.5 | 0 | 7 | 0 | 0 | 0 | 3 | 20 | 6 | 25+TT | 14-TT |

Table 6.2A.3.1.5-4: UE Power Class 3 test requirement (network signalling value NS_56/NS_04) for CA_n24-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 1 | 6.5 | 14 | 8 | 0 | 0 | 3 | 16.0 | 5 | 25+TT | 11-TT |
| 3 | 23 | 1 | 0 | 14 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 4 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 1 | 6.5 | 8 | 8 | 0 | 0 | 3 | 18.0 | 4 | 25+TT | 14-TT |
| 6 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 1 | 6.5 | 12 | 8 | 0 | 0 | 3 | 16.5 | 5 | 25+TT | 11.5-TT |
| 9 | 23 | 1 | 0 | 12 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 6.5 | 8 | 8 | 0 | 0 | 3 | 18.0 | 4 | 25+TT | 14-TT |
| 12 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |

Table 6.2A.3.1.5-5: UE Power Class 3 test requirement (network signalling value NS_56/NS_27) for CA_n24-n48

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 1 | 6.5 | 14 | 6 | 0 | 0 | 3 | 17.2 | 5 | 25+TT | 12.2-TT |
| 3 | 23 | 1 | 0 | 14 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 4 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 1 | 6.5 | 8 | 6 | 0 | 0 | 3 | 18.8 | 4 | 25+TT | 14.8-TT |
| 6 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 1 | 6.5 | 12 | 11.5 | 0 | 0 | 3 | 14.3 | 5 | 25+TT | 9.3-TT |
| 9 | 23 | 1 | 0 | 12 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 6.5 | 8 | 11.5 | 0 | 0 | 3 | 16.6 | 5 | 25+TT | 11.6-TT |
| 12 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |

Table 6.2A.3.1.5-6: UE Power Class 3 test requirement (network signalling value NS_56/NS_55) for CA_n24-n77

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 1 | 6.5 | 14 | 0 | 0 | 0 | 3 | 17.2 | 5 | 25+TT | 12.2-TT |
| 3 | 23 | 1 | 0 | 14 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 4 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 5 | 23 | 1 | 6.5 | 8 | 0 | 0 | 0 | 3 | 18.8 | 4 | 25+TT | 14.8-TT |
| 6 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 7 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 8 | 23 | 1 | 6.5 | 12 | 0 | 0 | 0 | 3 | 17.6 | 5 | 25+TT | 12.6-TT |
| 9 | 23 | 1 | 0 | 12 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 10 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 11 | 23 | 1 | 6.5 | 8 | 0 | 0 | 0 | 3 | 18.8 | 4 | 25+TT | 14.8-TT |
| 12 | 23 | 1 | 0 | 8 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |

Table 6.2A.3.1.5-7: UE Power Class 3 test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 2 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 3 | 23 | 8 | - | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |
| 4 | 23 | 8 | - | 8 | 1.5 | 2 | 15 | 5 | 25+TT | 10-TT |
| 5 | 23 | 2.5 | 3.5 | 3.5 | 0 | 2 | 19.5 | 3.5 | 25+TT | 16-TT |
| 6 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 7 | 23 | 2.5 | 4 | 4 | 0 | 2 | 19 | 3.5 | 25+TT | 15.5-TT |
| 8 | 23 | 5 | 4.5 | 5 | 0 | 2 | 18 | 4 | 25+TT | 14-TT |
| 9 | 23 | 7 | 6 | 7 | 0 | 2 | 16 | 5 | 25+TT | 11-TT |
| 10 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 11 | 23 | 3.5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 12 | 23 | 5 | 5.5 | 5.5 | 0 | 2 | 17.5 | 5 | 25+TT | 12.5-TT |
| 13 | 23 | 7 | 8 | 8 | 0 | 2 | 15 | 5 | 25+TT | 10-TT |

Table 6.2A.3.1.5-8: UE Power Class 2 not supporting TxD test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8 | - | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 2 | 26 | 8 | - | 8 | 1.5 | 3 | 18 | 4 | 28+TT | 14-TT |
| 3 | 26 | 8 | - | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 4 | 26 | 8 | - | 8 | 1.5 | 3 | 18 | 4 | 28+TT | 14-TT |
| 5 | 26 | 2.5 | 5.5 | 5.5 | 0 | 3 | 20.5 | 2.5 | 28+TT | 17.5-TT |
| 6 | 26 | 2.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17-TT |
| 7 | 26 | 2.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17-TT |
| 8 | 26 | 5 | 6.5 | 6.5 | 0 | 3 | 19.5 | 3.5 | 28+TT | 16-TT |
| 9 | 26 | 7 | 8 | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 10 | 26 | 3.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 11 | 26 | 3.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 12 | 26 | 5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 13 | 26 | 7 | 10 | 10 | 0 | 3 | 16 | 5 | 28+TT | 11-TT |

Table 6.2A.3.1.5-8a: UE Power Class 2 supporting TxD test requirement (network signalling value CA_NS_04) for CA_n41C

| Test ID | PPowerclass | MPR (dB) | AMPRcc (dB) | AMPR(dB) | ΔTC,c (dB) | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) |  |  |  |  |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 2 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 3 | 26 | 8.5 | - | 8.5 | 0 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 4 | 26 | 8.5 | - | 8.5 | 1.5 | 3 | 17.5 | 5 | 28+TT | 12.5-TT |
| 5 | 26 | 3.5 | 5.5 | 5.5 | 0 | 3 | 20.5 | 2.5 | 28+TT | 18-TT |
| 6 | 26 | 3.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17.5-TT |
| 7 | 26 | 3.5 | 6 | 6 | 0 | 3 | 20 | 2.5 | 28+TT | 17.5-TT |
| 8 | 26 | 6 | 6.5 | 6.5 | 0 | 3 | 19.5 | 3.5 | 28+TT | 16-TT |
| 9 | 26 | 8 | 8 | 8 | 0 | 3 | 18 | 4 | 28+TT | 14-TT |
| 10 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 11 | 26 | 4.0 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 12 | 26 | 5.5 | 7.5 | 7.5 | 0 | 3 | 18.5 | 4 | 28+TT | 14.5-TT |
| 13 | 26 | 7.5 | 10 | 10 | 0 | 3 | 16 | 5 | 28+TT | 11-TT |

Table 6.2A.3.1.5-9: UE Power Class 3 test requirement (network signalling value NS_17/NS_01) for CA_n28-n78 and CA_n28-n79

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 16.5 | 5 | 25+TT | 11.5-TT |
| 3 | 23 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 20 | 6 | 25+TT | 14-TT |

Table 6.2A.3.1.5-10: UE Power Class 2 test requirement (network signalling value NS_17/NS_01) for CA_n28-n79

| Test ID | PPowerclass | PPowerclass,c |  | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW (PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 23 | 26 | 0 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |
| 2 | 26 | 23 | 26 | 6.5 | 6.5 | 0 | 0 | 0 | 0 | 3 | 21.2 | 5 | 28+TT | 16.2-TT |
| 3 | 26 | 23 | 26 | 6.5 | 0 | 0 | 0 | 0 | 0 | 3 | 26 | 3 | 28+TT | 23-TT |

Table 6.2A.3.1.5-11: UE Power Class 3 test requirement (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 23 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |
| 2 | 23 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 25+TT | 14-TT |
| 3 | 23 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23 | 3 | 25+TT | 20-TT |

Table 6.2A.3.1.5-11a: UE Power Class 2 test requirement with both bands supporting PC3 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 1 | 0 | 2 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 1 | 0 | 7 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 1 | 0 | 7 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |

Table 6.2A.3.1.5-11b: UE Power Class 2 test requirement with with PCC supporting PC3 and SCC supporting PC2 (network signalling value NS_17/NS_47) for CA_n28-n41

| Test ID | PPowerclass | MPRc (dB) |  | AMPRc (dB) |  | ΔTC,c (dB) |  | TL (dB) | PCMAX_L (dBm) | TLOW(PCMAX_L) | Upper limit | Lower limit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (dBm) | PCC | SCC | PCC | SCC | PCC | SCC |  |  | (dB) | (dBm) | (dBm) |
| 1 | 26 | 0 | 3.5 | 0 | 5 | 0 | 0 | 3 | 25 | 3 | 28+TT | 22-TT |
| 2 | 26 | 6.5 | 3.5 | 0 | 10 | 0 | 0 | 3 | 19 | 5 | 28+TT | 14-TT |
| 3 | 26 | 0 | 3.5 | 0 | 10 | 0 | 0 | 3 | 23.8 | 3 | 28+TT | 20.8-TT |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<Unchanged Text Skipped>

6.5A.2.3.1	Additional spectrum emission mask for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling value other than CA_NS_04 are not complete.

6.5A.2.3.1.1	Test purpose

To verify that the power of any UE emission shall not exceed specified level for the specified channel bandwidth under the deployment scenarios where additional requirements are specified.

6.5A.2.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.2.3.0.

6.5A.2.3.1.4	Test description

6.5A.2.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown below. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Table 6.5A.2.3.1.4.1-1: Test Configuration Table (network signalling value "CA_NS_04")

Same test configuration as listed in Table 6.2A.3.1.4.1-7 shall be used with the following exceptions:

- Only Test IDs 1~4 are tested.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to the test configuration tables in clause 6.2A.2.1.4.1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.2.3.1.4.3

6.5A.2.3.1.4.2	Test procedure

<Unchanged Text Skipped>

6.5A.3.3.1	Additional spurious emissions for CA (2UL CA)

Editor’s note: The following aspects are either missing or not yet determined:

-	Tests for network signalling value other than NS_04 are not complete.

6.5A.3.3.1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5A.3.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.5A.3.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in subclause 6.5A.3.3.0.

6.5A.3.3.1.4	Test description

6.5A.3.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, are shown in Table 6.5A.3.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.3.1.4.1-1: Test Configuration Table (network signalling value "CA_NS_04")

Same test configuration as listed in Table 6.2A.3.1.4.1-7 shall be used with the following exceptions:

- Only Test IDs 3~13 are tested.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1 and C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.5A.3.3.1.4.3.

< End of Changes >