3GPP TSG-RAN5 Meeting #102	R5-241804

# Athens, Greece, February 26 – March 1 , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0003 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clear-up CR for Editor notes of applicability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC, MediaTek, CAICT, Sporton, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For the TCs which have updated applicability in TS 36.521-2, editor notes of applicability need to be removed in TS 36.521-4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor notes of applicability have been removed in TS 36.521-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TCs are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1, 6.2A.2, 6.2A.3, 6.2A.4, 6.2B.1, 6.2B.2, 6.2B.3, 6.2B.4, 6.3A.1, 6.3A.2, 6.3A.3.1, 6.3A.3.2.1, 6.3A.3.2.2, 6.3A.4.1, 6.3A.4.2, 6.3A.4.3, 6.3B.1, 6.3B.2, 6.3B.3.1, 6.3B.3.2, 6.3B.4.1, 6.3B.4.2, 6.4A.2.1, 6.4A.2.2, 6.4A.2.3, 6.4A.2.4, 6.4B.2.1, 6.4B.2.2, 6.4B.2.3, 6.5A.2, 6.5A.3.2, 6.5A.3.4, 6.5A.4.2, 6.5A.4.3, 6.5A.4.4, 6.5B.2, , 6.5B.3.2, 6.5B.3.4, 6.5B.4.2, 6.5B.4.3, 6.5B.4.4, 6.6B, 7.3A, 7.3B, 7.4A, 7.4B, 7.5A, 7.5B, 7.6A.2, 7.6A.3, 7.6A.4, 7.6B.2, 7.6B.3, 7.7A, 7.7B, 7.8A, 7.8B, 7.9A, 7.9B, 8.2.1.1.1, 8.3.1.1.1, 8.3.1.1.2, 8.3.1.1.3, 8.3.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is related to R5-240073. R5-240073 updates related TCs applicability in TS 36.521-2. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2A.1	UE maximum output power for category M1

6.2A.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

<< Unchanged Parts Skipped >>

6.2A.2	UE maximum output power reduction for category M1

6.2A.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

<< Unchanged Parts Skipped >>

6.2A.3	UE additional maximum output power reduction for category M1 UE

6.2A.3.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

<< Unchanged Parts Skipped >>

6.2A.4	Configured transmitted Power for category M1

6.2A.4.1	Test purpose

To verify the UE does not exceed the minimum between the PEMAX maximum allowed UL TX Power signalled by the E-UTRAN and the PUMAX maximum UE power the UE power class.

<< Unchanged Parts Skipped >>

6.2B.1	UE maximum output power for category NB1 and NB2

6.2B.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

<< Unchanged Parts Skipped >>

6.2B.2	UE maximum output power reduction for category NB1 and NB2

6.2B.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

<< Unchanged Parts Skipped >>

6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

6.2B.3.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2B.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

<< Unchanged Parts Skipped >>

6.2B.4	Configured transmitted Power for category NB1 and NB2

6.2B.4.1	Test purpose

To verify the UE does not exceed the minimum between the PEMAX maximum allowed UL TX Power signalled by the E-UTRAN and the PUMAX maximum UE power for the UE power class.

<< Unchanged Parts Skipped >>

6.3A.1	UE Minimum output power for category M1

6.3A.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

<< Unchanged Parts Skipped >>

6.3A.2	Transmit OFF power for category M1

6.3A.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

<< Unchanged Parts Skipped >>

6.3A.3.1	General ON/OFF time mask for category M1

6.3A.3.1.1	Test purpose

To verify that the general ON/OFF time mask meets the requirements given in TS 36.521-1[14] clause 6.3.4.1.5.

The time mask for transmit ON/OFF defines the ramping time allowed for the UE between transmit OFF power and transmit ON power.

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

<< Unchanged Parts Skipped >>

6.3A.3.2.1	PRACH time mask for UE category M1

6.3A.3.2.1.1	Test purpose

To verify that the PRACH time mask meets the requirements given in TS 36.521-1[14] clause 6.3.4.2.1.5.

The time mask for PRACH time mask defines the ramping time allowed for the UE between transmit OFF power and transmit ON power when transmitting the PRACH.

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

<< Unchanged Parts Skipped >>

6.3A.3.2.2	SRS time mask for UE category M1

6.3A.3.2.2.1	Test purpose

Same test purpose as in TS 36.521-1[14] clause 6.3.4.2.2.1.

<< Unchanged Parts Skipped >>

6.3A.4.1	Power Control Absolute power tolerance for UE category M1

6.3A.4.1.1	Test purpose

To verify the ability of the UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e. transmission gap is larger than 20 ms.

<< Unchanged Parts Skipped >>

6.3A.4.2	Power Control Relative power tolerance for UE category M1

6.3A.4.2.1	Test purpose

To verify the ability of the UE transmitter to set its output power relatively to the power in a target sub-frame relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is ≤ 20 ms.

<< Unchanged Parts Skipped >>

6.3A.4.3	Aggregate power control tolerance for UE category M1

6.3A.4.3.1	Test purpose

To verify the ability of category M1 UE to maintain its power level in non-contiguous transmission in response to 0 dB TPC commands with respect to the first UE transmission, when the power control parameters specified in TS 36.213 are constant.

<< Unchanged Parts Skipped >>

6.3B.1	UE Minimum output power for category NB1 and NB2

6.3B.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

<< Unchanged Parts Skipped >>

6.3B.2	Transmit OFF power for category NB1 and NB2

6.3B.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

<< Unchanged Parts Skipped >>

6.3B.3.1	General ON/OFF time mask for category NB1 and NB2

6.3B.3.1.1	Test purpose

To verify that the general ON/OFF time mask meets the requirements given in 6.3B.3.1.5.

The time mask for transmit ON/OFF defines the ramping time allowed for the UE between transmit OFF power and transmit ON power.

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

<< Unchanged Parts Skipped >>

6.3B.3.2	NPRACH time mask for category NB1 and NB2

6.3B.3.2.1	Test purpose

To verify that the NPRACH time mask meets the requirements given in 6.3B.3.2.5.

The time mask for NPRACH time mask defines the ramping time allowed for the UE between transmit OFF power and transmit ON power when transmitting the NPRACH.

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

<< Unchanged Parts Skipped >>

6.3B.4.1	Power Control Absolute power tolerance for category NB1 and NB2

6.3B.4.1.1	Test purpose

To verify the ability of the UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e., transmission gap is larger than 20 ms.

<< Unchanged Parts Skipped >>

6.3B.4.2 Power Control Relative power tolerance for category NB1 and NB2

6.3B.4.2.1	Test purpose

To verify the ability of the UE transmitter to set its output power relatively to the power in a target sub-frame relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is ≤ 20 ms.

<< Unchanged Parts Skipped >>

6.4A.2.1	Error Vector Magnitude (EVM) for category M1

6.4A.2.1.1	Test purpose

Same test purpose as in TS 36.521-1[14] clause 6.5.2.1EA.1.1

<< Unchanged Parts Skipped >>

6.4A.2.2	Carrier leakage for category M1

6.4A.2.2.1	Test purpose

Carrier leakage expresses itself as unmodulated sine wave with the carrier frequency or centre frequency of aggregated transmission bandwidth configuration. It is an interference of approximately constant amplitude and independent of the amplitude of the wanted signal. Carrier leakage interferes with the centre sub carriers of the UE under test (if allocated), especially, when their amplitude is small. The measurement interval is defined over one slot in the time domain.

The purpose of this test is to exercise the UE transmitter to verify its modulation quality in terms of carrier leakage.

<< Unchanged Parts Skipped >>

6.4A.2.3	In-band emissions for non allocated RB for category M1

6.4A.2.3.1	Test purpose

<< Unchanged Parts Skipped >>

6.4A.2.4	EVM equalizer spectrum flatness for category M1

6.4A.2.4.1	Test purpose

<< Unchanged Parts Skipped >>

6.4B.2.1	Error Vector Magnitude (EVM) for Category NB1 and NB2

6.4B.2.1.1	Test purpose

<< Unchanged Parts Skipped >>

6.4B.2.2	Carrier leakage for Category NB1 and NB2

6.4B.2.2.1	Test purpose

<< Unchanged Parts Skipped >>

6.4B.2.3	In-band emissions for Category NB1 and NB2

6.4B.2.3.1	Test purpose

The in-band emissions are a measure of the interference falling into the non-allocated tones.

<< Unchanged Parts Skipped >>

6.5A.2	Occupied bandwidth for category M1

6.5A.2.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE are less than their specific limits.

<< Unchanged Parts Skipped >>

6.5A.3.2	Spectrum emission mask for category M1

6.5A.3.2.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

<< Unchanged Parts Skipped >>

6.5A.3.4	Adjacent Channel Leakage Ratio for category M1

<< Unchanged Parts Skipped >>

6.5A.4.2	Transmitter Spurious emissions for category M1

6.5A.4.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

<< Unchanged Parts Skipped >>

6.5A.4.3	Spurious emission band UE co-existence for category M1

6.5A.4.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

<< Unchanged Parts Skipped >>

6.5A.4.4	Additional spurious emissions for category M1

6.5A.4.4.0	General

<< Unchanged Parts Skipped >>

6.5B.2	Occupied bandwidth for category NB1 and NB2

6.5B.2.1	Test purpose

To verify that the UE occupied bandwidth for all transmission bandwidth configurations supported by the UE are less than their specific limits.

<< Unchanged Parts Skipped >>

6.5B.3.2	Spectrum emission mask for category NB1 and NB2

6.5B.3.2.1	Test purpose

To verify that the power of the category NB1 and NB2 UE emission shall not exceed specified level for the specified channel bandwidth.

<< Unchanged Parts Skipped >>

6.5B.3.4	Adjacent Channel Leakage Ratio for category NB1 and NB2

6.5B.3.4.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to adjacent channels in terms of Adjacent Channel Leakage power Ratio (ACLR).

<< Unchanged Parts Skipped >>

6.5B.4.2	Transmitter Spurious emissions for category NB1 and NB2

6.5B.4.2.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions.

<< Unchanged Parts Skipped >>

6.5B.4.3	Spurious emission band UE co-existence for category NB1 and NB2

6.5B.4.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

<< Unchanged Parts Skipped >>

6.5B.4.4	Additional spurious emissions for category NB1 and NB2

6.5B.4.4.0	General

<< Unchanged Parts Skipped >>

6.6B	Transmit intermodulation for category NB1 and NB2

6.6B.1	Test purpose

To verify that the UE transmit intermodulation does not exceed the described value in the test requirement.

<< Unchanged Parts Skipped >>

7.3A	Reference sensitivity power level for UE category M1

7.3A.1	Test purpose

<< Unchanged Parts Skipped >>

7.3B	Reference sensitivity power level for UE category NB1 and NB2

7.3B.1	Test purpose

<< Unchanged Parts Skipped >>

7.4A	Maximum input level for category M1

7.4A.1	Test purpose

<< Unchanged Parts Skipped >>

7.4B	Maximum input level for category NB1 and NB2

7.4B.1	Test purpose

<< Unchanged Parts Skipped >>

7.5A	Adjacent Channel Selectivity for category M1

7.5A.1	Test purpose

<< Unchanged Parts Skipped >>

7.5B	Adjacent Channel Selectivity for category NB1 and NB2

7.5B.1	Test purpose

<< Unchanged Parts Skipped >>

7.6A.2	In-band blocking for category M1

7.6A.2.1	Test Purpose

<< Unchanged Parts Skipped >>

7.6A.3	Out-of-band blocking for category M1

7.6A.3.1	Test Purpose

<< Unchanged Parts Skipped >>

7.6A.4	Narrow band blocking for category M1

<< Unchanged Parts Skipped >>

7.6B.2	In-band blocking for category NB1 and NB2

7.6B.2.1	Test Purpose

<< Unchanged Parts Skipped >>

7.6B.3	Out-of-band blocking for category NB1 and NB2

<< Unchanged Parts Skipped >>

7.7A	Spurious response for category M1

7.7A.1	Test Purpose

<< Unchanged Parts Skipped >>

7.7B	Spurious response for category NB1 and NB2

<< Unchanged Parts Skipped >>

7.8A	Intermodulation characteristics for category M1

7.8A.1	Test purpose

<< Unchanged Parts Skipped >>

7.8B	Intermodulation characteristics for category NB1 and NB2

7.8B.1	Test purpose

<< Unchanged Parts Skipped >>

7.9A	Spurious emissions for category M1

7.9A.1	Test Purpose

<< Unchanged Parts Skipped >>

7.9B	Spurious emissions for category NB1 and NB2

7.9B.1	Test Purpose

<< Unchanged Parts Skipped >>

8.2.1.1.1	PDSCH in standalone mode for UE category M1 under NTN fading conditions

8.2.1.1.1.1	Test purpose

<< Unchanged Parts Skipped >>

8.3.1.1.1	Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2 under NTN fading conditions

Editor’s Note: This test is incomplete. The following aspects are not yet determined:

- Addition to applicability spec is pending.

- Annex G.3.5 Minimum Test time is pending.

8.3.1.1.1.1	Test purpose

<< Unchanged Parts Skipped >>

8.3.1.1.2	Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone mode for category NB1 and NB2

8.3.1.1.2.1	Test purpose

<< Unchanged Parts Skipped >>

8.3.1.1.3	Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2

8.3.1.1.3.1	Test purpose

<< Unchanged Parts Skipped >>

8.3.1.2.1	Demodulation of NPDCCH single-antenna performance for category NB1 and NB2

8.3.1.2.1.1	Test purpose

<< END OF CHANGES >>