3GPP TSG-RAN5 Meeting #102	R5-241814

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0795 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of MMSE-IRC CQI reporting test case with FDD 2Rx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2024-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MMSE-IRC test case for wideband CQI reporting with inter-cell interference under FDD 2Rx is uncompleted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of test description and test requirement parts for MMSE-IRC test case for wideband CQI reporting with inter-cell interference under FDD 2Rx in 6.2.2.1.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This WI can not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-240254r1: Correct meeting information in the coversheet. |  |  |  |  |  |  |  |  |

<Start of change>

6.2.2.1.2.3	2Rx FDD FR1 Wideband CQI reporting with inter-cell intereference

Editor's Note: This test case is incomplete in following aspects:

-	TE connection diagram is TBD.

-	MU/TT analysis pending.

6.2.2.1.2.3.1	Test purpose

Verify that the UE is tracking the channel variations and selecting the largest transport format possible based on inter-cell interference mitigation receiver.

6.2.2.1.2.3.2	Test applicability

This test applies to all types of NR UEs and E-UTRAN UEs supporting EN-DC for release 15 and release 16 supporting MMSE-IRC processing for scenarios with inter-cell and intra-cell inter-user interference.

This test applies to all types of release 17 and forward NR UEs and E-UTRAN UEs supporting EN-DC.

6.2.2.1.2.3.3	Minimum conformance requirements

For the parameters specified in Table 6.2.2.1.2.3.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following,

a)	the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥ where is specified in Table 6.2.2.1.2.3.3-2;

b)	when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to 0.02.

Table 6.2.2.1.2.3.3-1: Wideband CQI reporting test with inter-cell interference

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 10 | 10 |
| Duplex Mode |  |  |  | FDD | FDD |
| Subcarrier spacing |  |  | kHz | 15 | 15 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 | Row 5,4 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | - |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | As specified in Table A.4-2, TBS.2-1 | - |
| INR (Note 6) |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×2 | 1×2 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR is defined in clause B.6.1. |  |  |  |  |  |

Table 6.2.2.1.2.3.3-2: Minimum requirements

| Parameters | Test 1 |
| --- | --- |
|  | 1.9 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.1.2.3.

6.2.2.1.2.3.4	Test description

6.2.2.1.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure TBD for TE diagram and section A.3.2.3 for UE diagram.

2.	The parameter settings for the serving cell (Cell 1) and interfering cell (Cell 2) are set up according to Table 6.1.2-1 and Table 6.2.2.1.2.3.3-1 as appropriate.

3.	Downlink signals for NR serving cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On, for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.15_1.3.3.

6.2.2.1.2.3.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband CQI value reported from UE. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Continue to gather data, record the BLER (NACK / ACK + NACK) and measure the average throughput according to Annex G.3.3 and G.3.4. Declare the throughput as .

2.	Deactivate i.e., stop transmitting anything from Cell 2 and set the Cell 1 parameters of bandwidth, the propagation condition, antenna configuration and measurement channel according to Table 6.2.2.1.2.3.3-1 as appropriate.

3.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband CQI value reported from UE. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Continue to gather data, measure the average throughput according to Annex G.3.3 and G.3.4. Declare the throughput as .

4.	If the ratio ( /) ≥  specified in Table 6.2.2.1.2.3.3-2 and the BLER (NACK / ACK + NACK) calculated in step 2 is greater or equal to 0.02, then pass the UE for this test. Otherwise fail the UE.

6.2.2.1.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 5.4.2.

6.2.2.1.2.3.5	Test requirement

The pass fail decision is as specified in the test procedure in clause 6.2.2.1.2.3.4.2.

The test tolerance for this test is FFS.

<End of change>